3GPP TSG RAN WG1 #107-e		R1-2111868

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.3.2

Source:	Apple

Title:	URLLC uplink enhancements for unlicensed spectrum

Document for:	Discussion/Decision

# Introduction

A work item on enhanced IIoT and URLLC was approved in RAN#86, and the WID was further updated in RP-201310 (RAN#88, June 2020) [1]. One objective is about the uplink enhancements for URLLC in unlicensed spectrum:

- Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]:

  - Specify support for UE-initiated COT for FBE with minimum specification effort

  - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

In this contribution, we discuss the remaining issues and share our views based on the agreements made in previous meetings (see Appendix A).

# UE-initiated COT for FBE

## Configuration parameters

UE-initiated COT can be considered enabled once the FFP periodicity and offset are configured for a UE, and no separate parameter is needed. There has been some consideration on whether additional level of control may be beneficial. In particular, we think it may be desirable to provide the flexibility to disable UE-initiated COT for P-CSI and/or SRS, for UE/gNB power saving purpose.

Proposal 1: UE-initiated COT is considered enabled once the FFP periodicity and offset are configured for a UE. Introduce a RRC parameter to disable UE-initiated COT for P-CSI and/or SRS.

## Segmentation for PUSCH repetition type B

For PUSCH repetition Type B, it has been agreed that:

Agreement

In semi-static channel access mode, for PUSCH repetition Type B: If a nominal repetition overlaps with a set of symbols in an idle period associated to gNB’s FFP in case UE shares gNB-initiated COT for the nominal repetition or associated to UE’s FFP in case UE assumes UE-initiated COT for the nominal repetition, all the symbols in the idle period should be considered as invalid symbols which are not considered for an actual repetition as in Rel-16.

- Segmentation before and/or after the idle period is applied when applicable.

- FFS on impact of processing timeline for PUSCH on the UE behaviour

One open issue is how to handle the case when the PUSCH processing timeline cannot be satisfied. Especially for CG, whether the UE shares gNB’s COT or initiates its own COT is determined by whether the UE detects any DL transmission from the gNB. When the UE detects the DL transmission, it may not satisfy the timeline required for PUSCH processing to handle segmentation around the idle period properly. In fact, this can be extended to a more generic issue on whether the UE should assume gNB’s COT or UE’s COT when the UL processing timeline is not satisfied. We propose that this is left to UE implementation.

Proposal 2: When UE-initiated COT is enabled, if the time when the COT initiator is determined does not satisfy the processing timeline for a UL transmission, whether the UE assumes the UL transmission corresponds to gNB’s COT or UE’s COT is up to UE implementation.

## Wideband operation

For wideband operation, whether the UE should assume the same COT initiator across RB sets and/or multiple CCs was discussed in RAN1#106b-e, and the following was proposed:

Proposal 6-3 (updated):

If, In semi-static channel access mode, when operating on multiple carriers/LBT BWs, the assumptions regarding the COT initiator for a transmission are should be aligned across all carriers/LBT BWs at any transmission time. To align the assumptions,

- a UE could assume to operate as an initiating device for a LBT BW i) if the UE didn’t assess and didn’t receive inication from the gNB that it shall operate as a responding device for any of LBT BWs, and ii) if the UE assesses or has received indication from the gNB that it shall operate as an initiating device for the LBT BW.

- a UE could not assume to operate as an initiating device for any of LBT BWs i) if the UE assesses or has received inication from the gNB that it shall operate as a responding device for at least one LBT BW, and the UE could assume to operate as a responding device for a LBT BW i) if the UE assesses or has received indication from the gNB that it shall operate as a responding device for the LBT BW.

Generally speaking, we think it makes sense to align the assumption on COT initiator for a transmission within a carrier, even if it spans across multiple RB sets (LBT bandwidth). This is conceptually cleaner. Moreover, it is critical for PUSCH repetition Type B, where the segmentation depends on the assumption for COT initiator, and it is not possible to have different segmentation decision on different RB sets.

On the other hand, for transmissions on multiple carriers, we do not see the need to align the assumption on COT initiator. It has been mentioned that a UE transmitting in the gNB’s idle period on one carrier may block the gNB’s channel access on another carrier in case of intra-band CA. However, gNB has different tools to handle such cases, e.g. by proper resource allocation, or DCI format 2_0 to prevent UE from transmitting in gNB’s idle period. In addition, if such blocking could occur, the gNB should avoid scheduling DL and UL on different carriers in any case. So we do not see any special handling needed here. From UE implementation perspective, forcing the alignment across multiple carriers introduces unnecessary complication.

Proposal 3: If UE-initiated COT is enabled in semi-static channel access mode, when operating on multiple LBT BWs on a carrier, the assumptions regarding the COT initiator for a UL transmission should be aligned across all LBT BWs for the UL transmission:

- A UE assumes to operate as an initiating device for a UL transmission

  - if the UL transmission is dynamically scheduled and the UE has received indication from the gNB that it shall operate as an initiating device, or

  - if the UL transmission is configured and the UE determines that it shall operator as an initiating device for any of the LBT BWs.

- A UE assumes to operate as a responding device for a UL transmission

  - if the UL transmission is dynamically scheduled and the UE has received indication from the gNB that it shall operate as a responding device, or

  - if the UL transmission is configured and the UE determines that it shall operator as a responding device for all the LBT BWs.

# Other

We have agreed to support PUSCH repetition Type B for CG on unlicensed spectrum. But it is unclear whether we also support it for DG. Technically it makes sense to support both.

From UE feature perspective, we agreed to reuse FG11-5 to indicate the support on unlicensed band. However, FG11-5 does not differentiate CG and DG, which means that the UE either support both DG and CG or support neither.

Proposal 4: Support PUSCH repetition Type B for DG on unlicensed spectrum in Rel-17.

Enhancements to UL cancellation indication for NR-U

During the discussions, it has been mentioned that there are some existing features that can be used to cancel configured UL transmissions, such as dynamic SFI. Another such mechanism is UL cancellation indication (CI) introduced in Rel-16 URLLC. One issue of the current UL CI mechanism for NR-U is that interlaced frequency resource allocation is used in NR-U, and UL CI does not take this into account.

For example, as shown in Figure 1, the gNB schedules a high priority transmission PUSCH1 on interlace 1, which requires the cancellation of another UE having the same resource allocation (not shown in the figure) previously. The UL CI has 4 bits in frequency domain, and each bit corresponds to a set of contiguous RBs based on Rel-16 definition. Due to the interlaced structure of PUSCH1 resource allocation, all the 4 bits in UL CI needs to be set to 1. This causes all the PUSCHs using resources in the other interlaces also being cancelled unnecessarily, e.g. PUSCH2 that has resources allocated in interlace 2. This can cause a lot of unnecessary cancellation of other UL transmissions.

Figure 1 UL cancellation indication with interlaced frequency resource allocation

Therefore, we see the need to enhance the UL CI mechanism to efficiently handle interlaced frequency resource allocation. A straightforward way is to design the bitmap based on the interlaces instead of contiguous RBs.

Proposal 5: Enhance the UL cancellation indication mechanism to efficiently handle interlaced frequency resource allocation in NR-U UL.

# Conclusion

In this contribution, we discussed the UE-initiated COT for FBE and potential configured grant enhancements for unlicensed spectrum, and proposed the following:

Proposal 1: UE-initiated COT is considered enabled once the FFP periodicity and offset are configured for a UE. Introduce a RRC parameter to disable UE-initiated COT for P-CSI and/or SRS.

Proposal 2: When UE-initiated COT is enabled, if the time when the COT initiator is determined does not satisfy the processing timeline for a UL transmission, whether the UE assumes the UL transmission corresponds to gNB’s COT or UE’s COT is up to UE implementation.

Proposal 3: If UE-initiated COT is enabled in semi-static channel access mode, when operating on multiple LBT BWs on a carrier, the assumptions regarding the COT initiator for a UL transmission should be aligned across all LBT BWs for the UL transmission:

- A UE assumes to operate as an initiating device for a UL transmission

  - if the UL transmission is dynamically scheduled and the UE has received indication from the gNB that it shall operate as an initiating device, or

  - if the UL transmission is configured and the UE determines that it shall operator as an initiating device for any of the LBT BWs.

- A UE assumes to operate as a responding device for a UL transmission

  - if the UL transmission is dynamically scheduled and the UE has received indication from the gNB that it shall operate as a responding device, or

  - if the UL transmission is configured and the UE determines that it shall operator as a responding device for all the LBT BWs.

Proposal 4: Support PUSCH repetition Type B for DG on unlicensed spectrum in Rel-17.

Proposal 5: Enhance the UL cancellation indication mechanism to efficiently handle interlaced frequency resource allocation in NR-U UL.

# Reference

- RP-210854, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, Nokia, Nokia Shanghai Bell, RAN#91e, March 2021.

- ETSI EN 301 893 V2.1.1, 5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU.

# Appendix A: Previous Agreements

## RAN1#102-e

Agreements:

- For semi-static channel access mode,

- If sensing is needed, it is performed immediately before the configured/scheduled transmission opportunity.

- For operation with semi-static channel access, the Rel-16 random starting offsets for UL configured grants with Full BW allocation when UE initiates a COT, is not supported.

Agreements:

- For semi-static channel access mode,

  - When gNB operates as an initiating device

    - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initates a COT

  - When a UE operates as an initiating device

    - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initates a COT

  - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB

    - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB

  - When the gNB shares a COT initiated by a UE during an FFP associated with the UE

    - The gNB is not allowed to transmit during the idle period of that the FFP in which the gNB shares the COT initiated by the UE

  - FFS whether/how to support additional restrictions to the idle period

Agreements:

- For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode

  - FFS the case when the UE is IDLE/INACTIVE mode

Agreements:

- A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9us immediately before the UL transmission burst.

Agreements:

- Conditions on the channel access procedures with respect to sensing duration and transmission gap for UE-initiated COT with UE-to-gNB COT sharing is similar as those for gNB initiated COT and gNB-to-UE COT sharing in Rel-16 by exchanging UE and gNB roles.

Agreements:

- UE-to-gNB COT sharing in semi-static channel access mode is supported.

  - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP.

    - FFS details

  - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP.

    - FFS whether/how UE to gNB COT sharing when the gap is >16us

Agreements:

For semi-static channel access mode,

o    Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT.

o    FFS: FFP Periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT.

Agreements:

- For semi-static channel access mode,

- FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling.

  - FFS on to be provided by SIB-1

- FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

Agreements:

- At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

Conclusion:

Further study and decide how to harmonize the CG features for Rel-16 URLLC and Rel-16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision.

## RAN1#103-e

Agreements:

- In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200ms

Conclusion:

- For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB.

Agreements:

  - UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16us is supported

Conclusion:

If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X.

If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X  is the FFP associated with Y.

Note 1: One of the devices X and Y is a UE and the other is its serving gNB.

Note 2: Whether or not there is additional restriction on idle period is still FFS.

Update on 11/11

Agreements:

- The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s).

  - Note: UE initiated FBE configuration is configured per serving cell

Agreements:

In semi-static channel access mode,  FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT.

o    Note: Any value for the period, shall be at least 1ms and at most 10ms.

o    Note: Aim for low complexity operation to handle gNB and UE COT interactions

Agreements:

In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT.

- UE determines the initiator of a COT based on at least one of the following alternatives:

  - Alt 1: Introduce additional bit field in the scheduling DCI

  - Alt 2: Based on ChannelAccess-CPext field in DCI

  - Alt. 3: Based on a predetermined rule(s)

  - Alt. 4: Based on RRC signalling

  - Alt. 5: Based on MAC CE

  - FFS other alternatives

- FFS on overriding possibility and/or the assumption

- Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreements:

In semi-static channel access mode:

- When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

  - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration.

- When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE:

  - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

- FFS on other conditions for determining the corresponding UE or gNB initiated COT

- Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreements:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

- Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

- Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions.

## RAN1#104-e

Agreement:

- PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG

  - FFS whether/how to enhance

Agreement:

- In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5,10}.

  - FFS on other values

Agreement:

- In semi-static channel access mode:

- An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT

- FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability

- FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary.

  - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT

    - FFS on X (e.g. X=0, or X= even index number)

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Agreement:

- In semi-static channel access mode, sharing a UE initiated COT through the gNB to other intra-cell UEs for UL transmissions, is not supported.

## RAN1#104b-e

Agreements:

- Support explicit RRC configuration for the UE-FFP parameters including period and offset in RRC connected mode.

Agreements:

- For semi-static channel access mode, the offset value for configuration of a UE-FFP for a serving cell has a symbol level granularity.

Agreement:

- For semi-static channel access mode, in addition to the agreed set of period values for configuration of a UE-FFP for a serving cell:

  - Do not support any additional period value

Agreement:

- For semi-static channel access mode, the starting point of first UE FFP for a serving cell

  - is relative to the boundary of the radio frame of even index number (i.e. X=even indexed number in RAN1#104-e agreement).

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The UL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI  DL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The DL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

Agreement:

- Select one of the following options (aiming for RAN1#105-e):

  - Option 1: Do not support PUSCH repetition Type Bwhen using based on NR-U Rel-16 based CG for unlicensed band operation.

  - Option 2: Support enhancements of PUSCH repetition Type B when using based on NR-U Rel-16based CG for unlicensed band operation. FFS whether/how to enhance

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP.

  - FFS on details

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details

Conclusion:

- In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT

  - Note: the gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI.

Agreement:

- Option 2-b and option 3 are not considered further for the agreement in RAN1#103-e regarding CG harmonization

## RAN1#105-e

Agreement:

- Option 1 is taken in the following agreement:

Agreement:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

  - If cg-RetransmissionTimer-r16 is configured, “CG-UCI based procedures” should also be enabled by X.

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions

- Alt-a is taken in the following agreement:

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT..

- Alt-a is taken in the following agreement:

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

## RAN1#106-e

Agreement

In semi-static channel access mode, the content in a scheduling DCI that indicates the assumption on the COT-initiator for the scheduled transmission is determined based on the channel access field in the DCI.

Agreement

In semi-static channel access mode,

- The inclusion of the channel access field in Rel-16 DCI 0_1 and 1_1 in Rel-17 DCI 0_2 and 1_2, respectively, is supported.

Agreement

In semi-static channel access mode, the size of channel access field in a scheduling DCI with format 0_0/1_0, 0_1/1_1, 0_2/1_2 is 2 bits.

Conclusion

Any UL or DL transmission that is expected to occur, should be associated to a Channel Occupancy (CO) with a corresponding FFP. When a transmission is associated to a CO with a corresponding FFP:

- The association of the transmission to a CO with corresponding FFP is based on either of the following assumption:

  - “Initiating COT”: This assumption implies that the transmission would initiate a CO corresponding the FFP.

  - “Sharing COT”: This assumption implies that the transmission would share a CO corresponding to the FFP.

- The association assumption is validated as follows:

  - “Initiating COT” assumption is validated if the transmission would start at the FFP boundary and would end before idle period of the FFP.

  - “Sharing COT” assumption is validated if the transmission would start after the FFP boundary and would end before idle period of the FFP and the CO corresponding to the FFP is initiated.

- A transmission based on a CO association assumption can occur if the CO association assumption is validated and if the following sensing conditions are met:

  - For CO association assumption as “Initiating COT”:

    - If a CCA is successful before the transmission.

  - For CO association assumption as “Sharing COT”

    - If the gap between the beginning of the transmission and the end of previous one sharing the same CO in that FFP is more than 16us and if a CCA is successful before the transmission.

    - IF the gap between the beginning of the transmission and the end of previous one sharing the same CO in that FFP is at most 16us

Agreement

In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include at least scheduled DL transmission or a DCI intended for the UE that initiated that FFP.

- FFS whether/how the DL transmission burst can include transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission

Agreement

In semi-static channel access mode, the content of the channel access field in a DCI scheduling a UL transmission for a UE determines an index to a row in Table 1 with Alt-1 (Option 1)

TABLE 1

| Bit field mapped to index | Channel Access Type | The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211] | Initiator of a channel occupancy associated to UL transmission described in Clause x.x in TS 37.213 |
| --- | --- | --- | --- |
| 0 | No sensing as defined in Clause 4.3 in TS 37.213 | 0 | Alt-1:gNB Alt-2: UE-initiated COT if condition A, otherwise gNB’s COT |
| 1 | No sensing as defined in Clause 4.3 in TS 37.213 | 2 | Alt-1:gNB Alt-2: UE-initiated COT if condition A, otherwise gNB’s COT |
| 2 | 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 3 | 9us sensing [within a 25us interval] as defined in Clause 4.3 in TS 37.213 | 0 | UE |

Note: The last row in Table 1 is only applicable when the UE can operate as an initiating device as configured by gNB.

Agreement

In semi-static channel access mode, when the gNB schedules by a DCI a UL transmission in a later g-FFP that is different from the g-FFP that carries the scheduling DCI:

- The UE follows the indicated COT initiator as the following:

  - If the UE validates the indicated COT initiator assumption and satisfies the applicable sensing conditions, the transmission occurs. Otherwise, the transmission is dropped.

Conclusion

There is no consensus in RAN1 to support UE-initiated COT for semi-static channel occupancy in IDLE/INACTIVE mode.

Agreement

Replace “9us sensing [within a 25us interval] as defined in Clause 4.3 in TS 37.213” with “9us sensing as defined in Clause x.x in TS 37.213” in the last row of Table 1 in the previous agreement and add the following notes to Table 1:

- Note 1: The intention of Clause x.x above is to describe the LBT procedure from a UE perspective when this operates as initiating device.

- Note 2: A UE operating as initiating device may transmit an UL transmission burst(s) within its u-FFP immediately after sensing the channel to be idle for at least a sensing slot duration  if the gap between the UL transmission burst(s) and any previous transmission burst is more than

Agreement

When a UE operates as an initiating device, and the gNB shares a UE’s FFP for DL transmission, regardless of the gap between any UL and DL bursts, no restriction is imposed on the maximum duration of each of the DL bursts such that each can continue until the UE FFP idle period starts.

- Note: The applicability of the EDT calculation based on the UE’s transmit power to the UE COT initiation in accordance to the UL-DL gap duration and/or the content of the DL burst is separately discussed

Agreement

Do not support PUSCH repetition Type B based on NR-U Rel-16 CG for unlicensed band operation.