3GPP TSG RAN WG1#107-e	 	 	R1-2112108

e-Meeting, November 11th – 19th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on mitigating UE and gNB Rx/Tx timing delays

Agenda Item:	8.5.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#91-e meeting, updated WID on NR positioning was agreed [1]. The work item includes RAN1-led objectives as follows:

The objective of this work item is to specify solutions to enable RAT dependent (for both FR1 and FR2) and RAT independent NR positioning enhancements for improving positioning accuracy, latency, network and/or device efficiency. The objective of this work item also includes the support of GNSS enhancements.

The specific objectives of this work are:

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1, RAN2, RAN3, RAN4]

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1, RAN2, RAN3]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

Notes:

Solutions for RAT-dependent positioning enhancements are designed to operate in both frequency ranges (i.e. FR1 & FR2)

RAN working groups will take into account the work from other working group SA/CT which involves same objectives.

- The WID is subject to further update in RAN #91 for RAN1/2/3/4 scoping.

In this contribution, we present our view on mitigating UE and gNB Rx/Tx timing delays for Rel-17 NR positioning.

- Potential enhancements for mitigating UE and gNB Rx/Tx timing delays

  - Number of measurement occasions in a single measurement instance

Reporting number of measurement occasions in a single measurement instance was discussed and the following agreement and proposal were made at the previous meetings [2][5].

RAN1#104-e meeting

Agreement:

Support enabling

- A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and

- A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and

- Each measurement instance is reported with its own timestamp

  - FFS: The measurement instances are within a [configured] measurement time window

- FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set

  - FFS: N (including N=1)

- FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions

  - FFS: M (including M=1)

- FFS: details of signalling, procedures, and UE capability if any

- FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric

- Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s).

- Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

RAN1#106b-e meeting

(Round 4) Proposal 5-3

- Each measurement instance in a UE measurement report can be configured by LMF with at least N=1 instances of the DL-PRS Resource Set

- Each measurement instance in a TRP measurement report can be configured by LMF with at least M=1 SRS measurement time occasions.

- FFS: Maximum number of measurement instances in a single measurement report

- Send LS to RAN4 to inform RAN4 about RAN1’s decision.

From TEG error estimation perspective, it would be important to avoid averaging measurements with multiple time occasions i.e., to perform measurement with a single time occasion. A small number of PRS measurement time occasions per measurement instance would be beneficial for both a UE and a TRP. Thus, at least N=1 for a UE measurement and M=1 for a TRP measurement should be supported in Rel-17.

Proposal 1:

- Rel-17 should support the followings:

  - Each measurement instance in a UE measurement report can be configured by LMF with at least N=1 instances of the DL-PRS Resource Set

  - Each measurement instance in a TRP measurement report can be configured by LMF with at least M=1 SRS measurement time occasions.

  - Reporting association information of UL-PRS resources with UE Tx TEGs

Regarding reporting association information of UL-PRS resources with UE Tx TEGs, the following agreement and working assumption were made at the previous meetings [3][4].

RAN1#105-e meeting

Agreement:

- For mitigating UE Tx timing errors for UL TDOA, support  one of the following options:

  - Option 1:

    - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs.

    - FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs

  - Option 2:

    - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs.

    - Support the serving gNB to forward the association information provided by the UE to the LMF

    - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs

- FFS: UE should be able to report capability information related to Tx TEGs to LMF via LPP signalling

- Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF

RAN1#106b-e meeting

Working assumption:

- For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA.

  - The serving gNB should forward the association information provided by the UE to the LMF.

    - FFS: whether to support the serving gNB to forward the association information to the neighboring gNBs

  - UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB.

- For mitigating UE Tx timing errors for Multi-RTT, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT.

  - FFS: whether to support the LMF to forward the association information to the serving and neighboring gNBs

  - UE should report its capability of supporting multiple UE Tx TEGs for Multi-RTT directly to the LMF.

- FFS: Mitigation of UE Tx timing errors when Multi-RTT, UL-TDOA and/or DL-TDOA are used.

How to report the association information of UL-PRS resources with UE Tx TEGs was discussed, and then, two options were made as in the above agreement. The two options were discussed in the last meeting, and the above working assumption was made. Considering RAN1#107-e is the last RAN1 meeting in Rel-17 WI phase, the above working assumption should be confirmed.

In case of hybrid positioning (e.g. when Multi-RTT, UL-TDOA and/or DL-TDOA are used) where the UE reports measurement results via LPP signaling, we think the UE should provide the association information of UL-PRS resources with UE Tx TEGs directly to the LMF similar to M-RTT.

Proposal 2:

- The working assumption made at RAN1#106b-e meeting regarding the association information of SRS resources with UE Tx TEGs should be confirmed

Observation 1:

- In case of hybrid positioning where UE reports measurement results via LPP signaling, it may be reasonable that UE provides the association information of UL-PRS resources with UE Tx TEGs directly to the LMF

  - Definition of UE Rx-Tx time difference

Regarding definition of UE Rx-Tx time difference, the following proposal was made at the last meeting [5].

(Round 3)Proposal 3.3-2a

- Consider supporting one of the following alternatives related to the UE Rx-Tx time difference (decision to be made in RAN1#106b):

  - Option 1:

    - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

    - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

      - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

  - Option 2:

    - Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information

      - Option 2A: The TA change information is included in the UE Tx TEG report

      - Option 2B: The TA change information is included in the Rx-Tx measurement report

      - Note: TA change information corresponds to: Tx Timing change with a timestamp that this change occurred.

  - Option 3:

    - Send an LS to RAN4, requesting RAN4 to make the decision to select Option 1 or Option 2

- If RAN1 makes the decision to adopt either Option 1 or Option 2, send an LS to RAN4 to check if RAN4 has any issue to support the option.

Regarding Option 2 in the above proposal, there are two options (i.e. Option 2A and Option 2B) to report the Timing Adjustment (TA) change information. We think it needs to consider when the TA change occur. If the TA change value depends on UE Tx TEG, the TA change information should be reported in the UE Tx TEG report. On the other hand, if TA change is expected to occur at every measurement, the TA change information should be reported in Rx-Tx measurement report.

Observation 2:

- Regarding how to report TA change information, it may need to discuss when the TA change occur

- Conclusion

In this contribution, we discussed on mitigating UE and gNB Rx/Tx timing delays. Based on the discussion, we made following observations and proposals.

Observation 1:

- In case of hybrid positioning where UE reports measurement results via LPP signaling, it may be reasonable that UE provides the association information of UL-PRS resources with UE Tx TEGs directly to the LMF

Observation 2:

- Regarding how to report TA change information, it may need to discuss when the TA change occur

Proposal 1:

- Rel-17 should support the followings:

  - Each measurement instance in a UE measurement report can be configured by LMF with at least N=1 instances of the DL-PRS Resource Set

  - Each measurement instance in a TRP measurement report can be configured by LMF with at least M=1 SRS measurement time occasions.

Proposal 2:

- The working assumption made at RAN1#106b-e meeting regarding the association information of SRS resources with UE Tx TEGs should be confirmed

References

- RP-210903, 3GPP RAN#91-e, “Revised WID on NR Positioning Enhancements”, Electronic Meeting, Mar. 2021.

- 3GPP RAN1#104-e, Chairman’s Notes, Feb. 2021.

- 3GPP RAN1#105-e, Chairman’s Notes, May. 2021.

- 3GPP RAN1#106b-e, Chairman’s Notes, Oct. 2021.

- R1-2110579, 3GPP RAN1#106b-e, “FL Summary #4 for accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays”

Appendix

At the RAN1#104-e meeting, the following definitions of word in this agenda item were agreed [2].

Agreement:

The following definitions are used for the purpose of discussion of internal timing errors (these terms are not agreed to be included in the specifications):

- Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error.

- Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error.

- UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin.

- TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin.

- UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin.

- TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin.

- UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

- TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.