3GPP TSG RAN WG1 #107-e			R1-2112145

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.12

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on UE features for NR MBS

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.16.12 regarding UE features for NR MBS and captures the following email discussion.

[107-e-R17-UE-features-MBS-01] Email discussion UE features for NR MBS – Shinya (DOCOMO) - 1st check point: November 15 - Final check point: November 19

In the Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e [1], there are following feature groups for NR MBS.

- 33-1	Broadcast

- 33-2	Dynamic scheduling for multicast

- 33-2-x	Multiple G-RNTIs for group-common PDSCHs

- 33-3-1	Slot-level repetition for group-common PDSCH

- 33-3-2	FDM-ed unicast PDSCH and group-common PDSCH

- 33-3-3	Intra-slot TDM-ed unicast PDSCH and group-common PDSCH

- 33-4	NACK-only based HARQ-ACK feedback for multicast

- 33-5-1	SPS group-common PDSCH for multicast

- 33-5-2	Multiple SPS group-common PDSCH configuration

- 33-6-1	DL priority indication for multicast in DCI

- 33-6-2	Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE

- 33-6-3	More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot

- 33-7	Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback

The issues to be discussed are tagged and colour coded with High priority, Medium priority, or Low priority, considering RAN2 impact especially for capability signaling design.

- 33-1: Broadcast

In [1], FG 33-1 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. |  | Yes |  |  | Per UE | No | No |  |  | Optional without capability signalling |

Following feedbacks are provided in contributions for the RAN1#107-e meeting.

| [2] | Huawei, HiSilicon, CBN | FG 33-1 is the feature group for broadcast reception which targets UEs for all RRC states and is supposed to be received by UEs which has not entered the RRC_CONNECTED state yet. Hence, reporting such capability to network is not needed and the column of “Need for the gNB to know if the feature is supported” should be “No”. Proposal 1: “Need for the gNB to know if the feature is supported” for FG 33-1 should be “No”. FG 33-1 should be optional without capability signalling. |
| --- | --- | --- |
| [3] | ZTE | Rel-17 is the first release that supports broadcast and multicast for NR. To facilitate the MBS commercial process, it is beneficial to define some basic feature groups for MBS. For now, FG33-1 and FG33-2 can be set as the basic UE feature group for MBS. We can further merge or add more basic functionalities into FG33-1 and FG33-2 if needed. Proposal 1: Define FG33-1 and FG33-2 as basic feature group for MBS. |
| [4] | vivo | For broadcast, multiple G-RNTIs facilitate different services, and thus, support of G-RNTIs should be included in broadcast feature 33-1. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Mandatory/Optional. 33. NR_MBS; 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - Support slot-level repetition for MTCH - Support of G-RNTIs for broadcast. FFS details of N; Yes; Optional without capability signalling. |
| [5] | Spreadtrum Communications | 33. NR_MBS; 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI.; Yes; Per Band; No; No; Optional without capability signalling. |
| [6] | Nokia, Nokia Shanghai Bell | - 33-1: - Confirm it is per UE. Further discussion needed on whether there is a need for FR1/FR2 differentiation - Optional with capability signalling. The network needs to know if any UE supports the feature. |
| [8] | Intel Corporation | - FG 33-1: This group should be renamed to “Broadcast for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs” to capture the fact that low QoS broadcast transmission is supported by both connected and idle/inactive mode UEs Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC). 33. NR_MBS; 33-1; Broadcast for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI.; Yes; Per UE. |
| [9] | Samsung | Mandatory/optional and DCI format For FG 33-1, this feature should be optional. Instead, it is proposed to note that this is a basic FG for MBS. Also, DCI format 1_0 in component 5 is not agreed. The same comment for component 4 of 33-2 and component 1 of 33-6-1. Features; Index; Feature group; Components; Mandatory/Optional. 33. NR_MBS; 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format [1_0] with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI.; Optional without capability signalling This is a basic FG for MBS.. |
| [11] | Qualcomm Incorporated | 33. NR_MBS; 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of [Nbroadcast>=1 (FFS: value of Nbroadcast)] group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for MTCH. - Support of CFR configuration for broadcast. - Support of CORESET and common search space in a CFR for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between other PDSCH and group-common PDSCH for broadcast in different slots. - Support MCCH change notification indication via DCI. - Support of slot-level repetition for group-common PDSCH scheduled by using DCI format 1_0 - FFS: Support of TRS for GC-PDCCH/PDSCH configured in a CFR for broadcast; Yes; Per Band; N/A; N/A; Optional without capability signalling. |
| [12] | MediaTek Inc. | Regarding the CSS type for MCCH and MTCH broadcast reception, the following agreements was achieved in RAN1#105-e meeting: Agreement: For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs support the same CSS type for MCCH and MTCH.. Therefore, we suggest to add a new bullet, e.g., “Support the same CSS type for MCCH and MTCH reception”. Proposal 1: “Support the same CSS type for MCCH and MTCH reception” is added as the one component of FG 33-1. Regarding whether capability signalling is necessary for FG 33-1, we think that the broadcast mechanism is a basic function for UE supporting MBS and it is no need to define a capability signalling. Proposal 2: For FG 33-1, the capability signalling is no need for UE supporting MBS broadcast. Regarding the CFR number for broadcast reception, it is no clear motivation to support multiple CFR. We had agreed that the number of CFRs for multicast is no more than one, and the corresponding agreement is copied as following. We suggest the similar mechanism can be reused for broadcast reception. Agreement: The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17.. Proposal 3: For FG 33-1, adding a sub-bullet that “only one CFR is supported for broadcast” for the 3rd component. |
| [13] | Ericsson | In the broadcast features, DCI format should be in bracket to signify that a new DCI format value should be devised as “Fallback MBS DCI”. Moreover, the feature 33-1/1 mention MCCH-RNTI, which has not been agreed, and should be removed for now as there is an alternative being discussed for change notification. Finally, the support of interslot TDM between unicast and broadcast is limited to RRC CONNECTED state. The following shows the revise feature 33-1 (unchanged columns removed) 33. NR_MBS; 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format [1_0] with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots in RRC_CONNECTED. - Support MCCH change notification indication via DCI.. |

Discussion

Medium priority proposal 2-1:

- FG 33-1 is supported as a basic FG for MBS

  - Support: ZTE, Samsung

| Company | Comment |
| --- | --- |
| MediaTek | Regarding the concept “basic FG”, moderator clarified it that “The concept of basic FG was introduced in Rel-16 NR-U and SL. The FG itself is supported as optional one, but a UE supporting e.g. SL must indicate the FG is supported” in last meeting. Does it means the capability signaling is optional without UE capability as discussed in question 2-2? In addition, for Rel-16 NR SL, it has dedicated ITS band for SL reception, which means that the UE is mandatory to support the SL reception if the UE support the ITS band. However, for the UE supporting the Rel-17 MBS does not have a dedicated band to only to receive the MBS services. So, it is still not clear for us if FG 33-1 as a basic feature. We are fine to support the FG 33-1 is the optional without capability signaling. |
| Nokia, NSB | Support. |
| Huawei, HiSilicon | Support. |
| ZTE | Support |
| Samsung | Support |
| NTT DOCOMO | Support |

Medium priority question 2-2:

- Companies are encouraged to provide views on whether capability signaling is necessary for FG 33-1, i.e., whether to support as optional with capability signaling or optional without capability signaling

  - Optional with capability signaling: Nokia, NSB

  - Optional without capability signaling: Huawei, HiSilicon, CBN, MediaTek

  - Optional: Samsung

| Company | Comment |
| --- | --- |
| MediaTek | Optional without capability signaling. |
| Nokia, NSB | Optional with capability signaling. The network needs to know if there are UEs supporting the feature. |
| Huawei, HiSilicon | Without capability signaling. This is feature for broadcast maybe received by IDLE/INACTIVEs only. |
| ZTE | We slightly prefer “with capability signalling”. With reporting this capability, network can get the info on how many UEs in this cell support broadcast reception (instead of interested in broadcast reception, which is reported by interest report). |

Medium priority question 2-3:

- Companies are encouraged to provide views on whether the type of FG 33-1 should be per UE or per band

  - Per UE: Nokia, NSB

  - Per band: Spreadtrum, Qualcomm

| Company | Comment |
| --- | --- |
| Nokia, NSB | Per UE. |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. We prefer per band for broadcast in Rel17 NR MBS. |
| Huawei, HiSilicon | Could be per band. |

Medium priority question 2-4:

- Companies are encouraged to provide views on whether FR1/FR2 differentiation is necessary for FG 33-1

| Company | Comment |
| --- | --- |
| Nokia, NSB | Support FR1/FR2 differentiation, as it is unlikely both will be available at the same time. |
| Qualcomm | Yes. But if the type of FG33-1 is per band, it implicitly saying FR1/FR2 differentiation for FG33-1. |
| Huawei, HiSilicon | Agree with Qualcomm |

Low priority proposal 2-5:

- Name of FG 33-1 is revised as “Broadcast for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs”

- Components of FG 33-1 is revised as

  - Remove component 1: Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI

  - Component 2: Support of [Nbroadcast>=1 (FFS: value of Nbroadcast)] group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for MTCH.

  - Component 3: Support of CFR configuration for broadcast

    - Only one CFR is supported for broadcast

  - Component 4: Support of CORESET and common search space in a CFR for broadcast.

  - Component 5: Support of DCI format [1_0] with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast.

  - Component 6: Support of inter-slot TDM between unicast other PDSCH and group-common PDSCH for broadcast in different slots in RRC_CONNECTED.

  - Add a component for support of slot-level repetitions for MTCH

  - Add a component for support of the same CSS type for MCCH and MTCH reception

  - FFS: add a component for support of TRS for GC-PDCCH/PDSCH configured in a CFR for broadcast

| Company | Comment |
| --- | --- |
| Qualcomm | We support it except the following subbullet should be in [] to, which is still under discussion in RAN1. - [Only one CFR is supported for broadcast] |

Low priority question 2-6:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 33-1 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |

- 33-2 to 33-2-x: Dynamic scheduling for multicast

In [1], FGs 33-2 to 33-2-x are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. |  | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2-x | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107-e meeting.

| [2] | Huawei, HiSilicon, CBN | One component for FG 33-2 for now is supporting ACK/NACK based HARQ-ACK feedback as [2], which can be more specific as supporting Type-1/Type-2 HARQ-ACK codebook based on inter-slot TDM-ed unicast and group-common PDSCH. Proposal 2: Update bullet 6 for FG 33-2 to be “support of Type-1/Type-2 HARQ-ACK codebook based on inter-slot TDM-ed unicast and group-common PDSCH, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling.” |
| --- | --- | --- |
| [3] | ZTE | Rel-17 is the first release that supports broadcast and multicast for NR. To facilitate the MBS commercial process, it is beneficial to define some basic feature groups for MBS. For now, FG33-1 and FG33-2 can be set as the basic UE feature group for MBS. We can further merge or add more basic functionalities into FG33-1 and FG33-2 if needed. Proposal 1: Define FG33-1 and FG33-2 as basic feature group for MBS. |
| [4] | vivo | For multicast, the capability for group-common PDSCH reception doesn’t have to be coupled with HARQ-ACK feedback. In general, there exist three reactions when UE receives group-common PDSCH: does not send any HARQ-ACK feedback; sends NACK-only based HARQ-ACK feedback; sends ACK/NACK based HARQ-ACK feedback. Thus, support of ACK/NACK based HARQ-ACK feedback should be separated from 33-2. Support of FDM-ed HARQ-ACK codebook should be separated from 33-3-2, and support of TDM-ed HARQ-ACK codebook should be separated from 33-3-3. Furthermore, similar as that in NR Rel-15, support of type 1 and type 2 codebook should be separated as different UE capabilities. Regarding the retransmission for multicast, whether to differentiate PTP (re)transmissions for unicast and PTP (re)transmissions multicast is under discussion and thus, it’s better to wait for further progress to determine whether support of PTP retransmission for multicast is a separate UE capability. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Mandatory/Optional. 33. NR_MBS; 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support PTM retransmission for multicast. - FFS: Support PTP retransmission for multicast. FFS whether/how to separate the above capabilities from FG 33-2; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-1; ACK/NACK based HARQ-ACK feedback for multicast; - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling.; 33-2; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-2; TDM-ed Type-1 HARQ-ACK codebook for multicast; - Support TDM-ed Type-1 HARQ-ACK codebook for multicast.; 33-2, 33-3-3, 33-4-1; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-3; FDM-ed Type-1 HARQ-ACK codebook for multicast; - Support FDM-ed Type-1 HARQ-ACK codebook for multicast.; 33-2, 33-3-2, 33-4-1; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-4; Type 2 HARQ-ACK codebook for multicast; - Support Type-2 HARQ-ACK codebook for multicast.; 33-2, 33-4-1; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-5; NACK-only based HARQ-ACK feedback for multicast; - Support NACK-only based HARQ-ACK feedback; 33-2; Yes; Optional with capability signalling. |
| [5] | Spreadtrum Communications | FG33-2: In Rel-17 MBS, HARQ-ACK feedback is introduced for multicast, and ACK/NACK based HARQ feedback and NACK-only are included. The PUCCH resource for each UE in the group is orthogonal by gNB’s implementation, to improve the reliability. But in our understanding, HARQ-ACK feedback is not the basic operation of multicast, as we have done in Rel-16 sidelink PC5, and also like FG33-4. In general, multicast transmission is more reliability than unicast. In addition, slot-level repetition is also can be utilized for the reliability. From the perspective of UE, power cost can be saved if it doesn’t support to feedback. Thus, we think whether to support HARQ-ACK feedback respectively of feedback option is up to UE capability. Proposal 1: Component 6 in FG33-2 can be split as one separated FG, and as one optional UE capability. Proposal 2: Component 7 in FG33-2 can be split as one separated FG, and as one optional UE capability. Proposal 3: Component 8 in FG33-2 can be split as one separated FG, and as one optional UE capability. 33. NR_MBS; 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. FFS whether/how to separate the above capabilities from FG 33-2; Yes; Per Band; No; No; Optional with capability signalling. 33. NR_MBS; 33-2-1; Support ACK/NACK based HARQ-ACK feedback; - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling.; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2-2; PTM retransmission for multicast; - Support PTM retransmission for multicast.; 33-2-1; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2-3; PTP retransmission for multicast; - Support PTP retransmission for multicast; 33-2-1; Yes; Per UE; No; No; Optional with capability signalling. |
| [6] | Nokia, Nokia Shanghai Bell | - 33-2: - Add 33-1 as pre-requisite |
| [7] | OPPO | Discussion on FG 33-2 For FG of dynamic scheduling for multicast, at least component 1~5 are considered as basic functions. HARQ-ACK feedback is not always enabled, or it may be disabled by default. “Support ACK/NACK based HARQ-ACK feedback” can be separated from FG 33-2. Besides, NACK-only seems another option of HARQ-ACK feedback, it can also be captured together with this component. “Support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling” is considered as basic, since this enabling/disabling function should be supported by UEs. For components 7 and 8, both are related to HARQ-ACK feedback. If HARQ-ACK feedback is separated, at least PTP retransmission for multicast can also be separated together with it. - “Support ACK/NACK based HARQ-ACK feedback” can be separated from FG 33-2. - “Support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling” is not separated from FG 33-2. - “Support PTP retransmission for multicast” can be separated from FG 33-2, and can be merged together with “Support ACK/NACK based HARQ-ACK feedback”. |
| [8] | Intel Corporation | - FG 33-2: - This group should be renamed to “Multicast for RRC_CONNECTED UEs” to capture the fact that Multicast is supported only for connected mode UEs in Rel-17. - The support of ACK/NACK based HARQ feedback should be separated into a separate dependent feature group since the default mode is HARQ disabled and RRC is used to enable it. UEs may not support HARQ feedback. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC). 33. NR_MBS; 33-2; Dynamic scheduling for multicast Multicast for RRC_CONNECTED UEs; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether/how to separate the above capabilities from FG 33-2; Yes; Per UE. 33. NR_MBS; 33-x; ACK/NACK based HARQ-ACK feedback for multicast; - Support ACK/NACK based HARQ-ACK feedback, - Support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling; 33-2; Yes; Per UE. |
| [10] | Apple | Dynamic scheduling for multicast For component 6 Support ACK/NACK based HARQ-ACK feedback, it would be preferred this component is treated as an independent FG, the fundamental function for multicast service is receiving the multicast data without HARQ feedback, any feedback mode is the enhanced feature which is subjected to UE capability. In additional, supporting enable/disabling HARQ-ACK feedback is another UE feature. For component 8 Support PTP re-transmission for multicast, this component can be removed from FG32-2 as independent FG. PTM is the basic function for multicast, if re-transmission is via PTP, several issues are still on discussion, e.g., differentiation PTP for unicast and PTP retransmission of PTM, LBRM/TBS determination for PTP re-transmission. Additional capability would be required for UE to support PTP retransmission. Proposal 1: Supporting the following components as independent FG from FG33-2, - Support of PTP retransmission for multicast - Support of ACK/NACK based HARQ-ACK feedback - Support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling |
| [11] | Qualcomm Incorporated | 33. NR_MBS; 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for multicast. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration in a CFR for multicast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast in different slots. - Support of slot-level repetition for group-common PDSCH scheduled by using DCI format 1_0; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2a; Dynamic multicast with DCI format 1_1; - Support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2b; ACK/NACK-based feedback for Dynamic scheduling for multicast and PTM retransmission for Dynamic scheduling for multicast; - Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback per G-RNTI for multicast by RRC signaling - Support of PTM retransmission for dynamically scheduled multicast based on G-RNTI; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2c; DCI-based ACK/NACK-based feedback for Dynamic scheduling for multicast and PTM retransmission for Dynamic scheduling for multicast; - Support DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2d; PTP retransmission for Dynamic scheduling for multicast; - Support of PTP retransmission for dynamically scheduled multicast based on C-RNTI; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2e; HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast; - Support of selection of ACK/NACK-based or NACK-only-based HARQ-ACK feedback for multicast per G-RNTI.; 33-2b, 33-4; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2-x; Multiple G-RNTIs for group-common PDSCHs; Capability on max number of G-RNTIs for multicast FFS details.; 33-2; Yes; Per FSPC; N/A; N/A; FFS: Values = {2, …}; Optional with capability signalling. 33. NR_MBS; 33-2-y; Multiple G-CS-RNTIs for SPS group-common PDSCHs; Capability on max number of G-CS-RNTIs for multicast FFS details.; 33-5-1; Yes; Per FSPC; N/A; N/A; FFS: Values = {2, …}; Optional with capability signalling. |
| [12] | MediaTek Inc. | 33-2: Dynamic scheduling for multicast Regarding the CFR number for multicast reception, the following agreement was achieved in last RAN1 meeting: Agreement: The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17.. Thus, we prefer to update the 2nd component based on the latest agreement. Proposal 4: For FG 33-2, adding a sub-bullet that “only one CFR is supported for multicast reception” for the 2nd component. In last RAN1 meeting, the HARQ-ACK enabling/disabling was agreed for multicast reception, and the corresponding agreement is copied as following. Agreement: Confirm the WA made in RAN1#106-e meeting regarding enabling/disabling HARQ-ACK feedback. Agreement: Update the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback as follows: Working assumption: For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH: - ……………………………….. - UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details. - ……………………….. Agreement: The group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling. Agreement: If the group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is not configured, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.. According to the agreements, we can see that the HARQ-ACK can be disabled/enabled and the UE capability for enabling/disabling is introduced. In other words, the UE supporting multicast does not meant it is definitely to support HARQ-ACK. Thus, we suggest the HARQ-ACK feedback can be as a separate FG. Proposal 5: For FG 33-2, the HARQ-ACK feedback mechanism can be as a separate FG. 33-2-X: Multiple G-RNTIs for group-common PDSCHs In the ongoing RAN2#116-e meeting, one-to-many mapping between G-RNTI and MBS sessions/services was agreed as following. one-to-many mapping between G-RNTI and MBS sessions is supported and it is assumed that this does not introduce additional specification work.. Since the multiple MBS sessions associated with one G-RNTI is supported, the motivation for supporting multiple G-RNTIs for group-common PDSCH is not clear. In addition, supporting multiple G-RNTIs will increase the probability of false alarm for PDCCH detection, which will degrade the system performance. We think one G-NRTI is sufficient for Rel-17 UE supporting multicast services. Proposal 6: Only one G-RNTI is sufficient for Rel-17 UE supporting multicast services. Proposal 7: For FG 33-2-X, RAN1 needs to re-consider the motivation for UE supporting multiple G-RNTIs. |
| [13] | Ericsson | Feature 33-3 is based on the following agreement: Agreement: For Rel-17 MBS UE, the UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s).. Therefore, feature 5 in 33-2 should be per CC, not per UE. Additionally, the new fallback/non fallback DCIs for MBS are not yet agreed but will be different from formats 1_0 or 1_1. We propose to put the format in brackets for now. Finally, the support of CFR configuration feature can be very large, and it is not entirely clear yet whether CFR needs to be configured at all for some cases (for example, when MBS bandwidth is the same as the active BWP. Therefore we propose to put it in bracket for now and make progress on what is included under the support of CFR. 33. NR_MBS; 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - [Support of CFR configuration for multicast.] - Support of CORESET and common search space configuration for multicast. - Support of [DCI format 1_0 / 1_1] with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. - Support PTM retransmission for multicast. - Support PTP retransmission for multicast.; Yes; Per UE Feature 32-2-5 per CC; No; No; Optional with capability signalling. |

Discussion

[FL1] High priority question 3-1:

- Companies are encouraged to provide views on whether to separate following capabilities from FG 33-2

  - Support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast

  - Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling

    - Whether to separate as

      - FG for support of ACK/NACK based HARQ-ACK feedback for multicast

        - FG for support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling

        - FG for support of enabling/disabling ACK/NACK based HARQ-ACK feedback indicated by DCI

        - FG for Support of selection of ACK/NACK-based or NACK-only-based HARQ-ACK feedback for multicast per G-RNTI

    - Note: whether/how to separate FGs for support of TDM/FDM-ed Type-1/2 HARQ-ACK codebooks for multicast is discussed in question 5-1

  - Support of PTM retransmission for multicast

    - Whether to merge with the FG for support of ACK/NACK based HARQ-ACK feedback for multicast

  - Support of PTP retransmission for multicast

    - Whether to merge with the FG for support of ACK/NACK based HARQ-ACK feedback for multicast

| Company | Comment |
| --- | --- |
| vivo | For the 2nd sub-bullet, support to separate from FG 33-2 as - FG for support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling And support of enabling/disabling ACK/NACK based HARQ-ACK feedback indicated by DCI shall be another separate FG. For the 3rd sub-bullet, support to merge it with the FG for support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling For the 4nd sub-bullet, we can wait for more progress to identify whether UE capability is needed for PTP retransmission. |
| MediaTek | For the 1st sub-bullet, the motivation is not clear for us. For the 2nd sub-bullet, we support it as a separate FG, and it also align with the RAN1’ previous meeting. |
| ZTE | Regarding “Support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast”, we don’t think we need to have separate FGs for DCI 1_0 and DCI 1_1 scheduling multicast. Like in Rel-15, UE has to support both DCI 1_0 and DCI 1_1 scheduling unicast. If UE supports multicast, UE should support both DCI 1_0 and DCI 1_1 all together. Regarding “FG for support of ACK/NACK based HARQ-ACK feedback for multicast”, we think support of ACK/NACK based feedback should be included in the basic FG of multicast. Otherwise, if UE doesn’t support feedback at all, it would be difficult for network to guarantee the service performance. Dynamic indication of feedback can be a separate UE FG. Regarding “Support of PTM retransmission for multicast” and “Support of PTP retransmission for multicast”, we think at least one of them should be included in the basic FG group for multicast. |
| Nokia, NSB | We don’t see a need for splitting the capabilities. |
| Qualcomm | Support. For the 1st sub-bullet, DCI format 1_0 can be included in the basic format for multicast. Support DCI format 1_1 with configurable fields to be a separate FG. For the 2nd sub-bullet, need to separate each FG for support of ACK/NACK based HARQ-ACK feedback for multicast, RRC-configured enabling/disabling, GC-DCI-based enabling/disabling (on condition of DCI format 1_1), and RRC-configured feedback mode selection. For the 3rd sub-bullet, it can be merged with the support of ACK/NACK based HARQ-ACK feedback for multicast For the 4th subbullet, it should be separate FG on condition of FG for ACK/NACK feedback for multicast. |
| FL2 | Summary of companies view - Support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast: - Support: Qualcomm - DCI format 1_0 can be included in the basic format for multicast. Support DCI format 1_1 with configurable fields to be a separate FG - Not support: MediaTek, ZTE, Nokia, NSB - Same as unicast - Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling - Not support: Nokia, NSB - Whether to separate as - FG for support of ACK/NACK based HARQ-ACK feedback for multicast - FG for support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling: vivo, MediaTek, Qualcomm - FG for support of enabling/disabling ACK/NACK based HARQ-ACK feedback indicated by DCI: vivo, Qualcomm - FG for Support of selection of ACK/NACK-based or NACK-only-based HARQ-ACK feedback for multicast per G-RNTI: Qualcomm - Note: whether/how to separate FGs for support of TDM/FDM-ed Type-1/2 HARQ-ACK codebooks for multicast is discussed in question 5-1 - Support of PTM retransmission for multicast - Not support: Nokia, NSB - Whether to merge with the FG for support of ACK/NACK based HARQ-ACK feedback for multicast: vivo, Qualcomm - Support of PTP retransmission for multicast - Support: Qualcomm - on condition of FG for ACK/NACK feedback for multicast - Not support: Nokia, NSB - Wait for progress in AI 8.12: vivo - Whether to merge with the FG for support of ACK/NACK based HARQ-ACK feedback for multicast Based on the above, following proposal is made: [FL2] High priority proposal 3-1a: - Support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast is kept in component 4 in FG 33-2 [FL2] High priority proposal 3-1b: - FG 33-2 is separated as following FGs - FG 33-2: current FG 33-2 without components 6 and 7 - FG 32-2a: ACK/NACK-based feedback for dynamic scheduling for multicast and PTM retransmission for dynamic scheduling for multicast - FG 32-2b: DCI-based ACK/NACK-based feedback for dynamic scheduling for multicast and PTM retransmission for dynamic scheduling for multicast - FG 32-2c: HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast - FFS whether to separate the capability for support of PTP retransmission for multicast 33. NR_MBS; 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. Whether to separate the capability for support of PTP retransmission for multicast; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2a; ACK/NACK-based feedback for dynamic scheduling for multicast and PTM retransmission for dynamic scheduling for multicast; - Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback per G-RNTI for multicast by RRC signaling - Support of PTM retransmission for dynamically scheduled multicast based on G-RNTI; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2b; DCI-based ACK/NACK-based feedback for dynamic scheduling for multicast and PTM retransmission for dynamic scheduling for multicast; Support DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2c; HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast; Support of selection of ACK/NACK-based or NACK-only-based HARQ-ACK feedback for multicast per G-RNTI.; 33-2a, 33-4; Yes; Per UE; No; No; Optional with capability signalling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| Huawei, HiSilicon | - Support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast should be kept in component 4 in FG 33-2 - We assume component 6) and 7) should be included in FG 33-2. And Enabling/disabling HARQ-ACK can be separate |
| Spreadtrum | Proposal 3-1a: Ok Proposal 3-1b: - For the first two sub-bullet, we are fine. - For the third sub-bullet, regarding the naming of FG33-2b, we think ‘and PTM retransmission for dynamic scheduling for multicast’can de removed, for FG33-2a has included this part. DCI-based ACK/NACK-based feedback for dynamic scheduling for multicast and PTM retransmission for dynamic scheduling for multicast - For the fourth sub-bullet, we are not clear why it is UE’s capability. In our understanding, if UE supports both FG33-2a and FG33-4, it is up to gNB to configure the specific feedback option, no additional UE capability is needed. |
| NTT DOCOMO | For FG 33-2c, we have the similar view with Spreadtrum. |
| OPPO | Proposal 3-1a: OK. Proposal 3-1b: Generally OK. |
| FL3 | Above proposals tagged FL2 were discussed in the GTW session on Nov. 16 but could not be agreed. Regarding proposal 3-1a, companies still have different view whether to separate DCI format 1_1 as separate FG. Companies are encouraged to provide further comments whether DCI format 1_1 can be optional for scheduling group-common PDSCH. [FL3] High priority proposal 3-1a: - Support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast is kept in component 4 in FG 33-2 Regarding proposal 3-1b, companies still have different view whether/how to separate FG 33-2, while some convergence can be seen that: - Some basic feature for HARQ-ACK can be included in FG 33-2, while companies may have different understanding whether ACK/NACK based or NACK-only based HARQ-ACK is mandatory for group-common PDSCH - No strong concern on separate FG for enabling/disabling ACK/NACK-based HARQ-ACK feedback - No strong concern on separate FG for DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback - No sufficient support to have separate FG for HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast Based on the above, the proposal is updated as follows. Companies are encouraged to provide further comments. [FL3] High priority proposal 3-1b: - FG 33-2 is separated as following FGs - FG 33-2: current FG 33-2 without the capability for support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling - Support at least one of NACK-only and ACK/NACK based HARQ-ACK feedback as a component of FG 33-2, FFS which one - FG 33-2a: Enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - FG 33-2b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - FFS whether to separate the capability for HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast - FFS whether to separate the capability for support of PTP retransmission for multicast 33. NR_MBS; 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. [Support at least one of NACK-only and ACK/NACK based HARQ-ACK feedback, FFS which one] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of PTP retransmission for multicast; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2a; Enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback per G-RNTI for multicast by RRC signaling; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2b; DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| Spreadtrum | Proposal 3-1a: we prefer DCI format 1_1 is optional. In our understanding, DCI format 1_0 can be used for broadcast and multicast, and DCI format 1_1 is mainly for MIMO. Thus, it is enough to take DCI format 1_0 as basic feature. In addition, in AI8.12.1, we are discussing DCI size alignment for DCI 1_1 for MBS, where one possible alternative is to keep the size of DCI 1_1 for MBS to align with other DCI formats. In Rel-15, other DCI formats are optional UE capability signaling. So for UEs not supporting other DCI formats but want to support MBS, it can select not to support DCI 1_1 for MBS, in order to keep the procedure of DCI size alignment not changed in its implementation, and no additional impact on unicast transmission. Proposal 3-1b: not ok for FG33-2, and prefer the version from FL2. In our understanding, HARQ-ACK feedback operation is not the basic feature for multicast, like what we have done in R16 sidelink. Regarding the reliability issue, there are many other ways, e.g., repetition. No feedback is also beneficial for power saving for UEs. It is very valuable at least for MBS only UEs, and is beneficial for MBS quick commercialization. |
| ZTE | Proposal 3-1a: We think DCI format 1_1 should be merged together with DCI format 1_0. Most of the functionalities related to DCI fields in DCI 1_1 will be optional UE feature, UE has the flexibility to report support or not support for them already. We don’t think adding DCI 1_1 itself into the basic multicast basic FG will increase the test burden. Proposal 3-1b: RAN2 has defined two QoS modes for MBS, one is low QoS mode mainly for broadcast and another is higher QoS mode mainly for multicast. If HARQ feedback is not included in the basic FG for multicast, multicast will be the same as broadcast as network has to utilize the most conservative scheduling for multicast for these UEs. Thus, we think 33-2a should be merged into 33-2. For compromise, we can only include Type-2 HARQ-ACK codebook into 33-2 and leave type-1 HARQ-ACK codebook as a separate FG. |
| MediaTek | Proposal 3-1a: Since the QC and Spreadtrum clarify the motivation in yesterday’s GTW meeting, it is clear for us. We also have the similar view with them and support the DCI format 1_1 can be split as a separate FG. Proposal 3-1b: Actually, We prefer the FL2’s version than current proposal in FL’3. Besides, we think the PTM retransmission can be separated as separate feature in FL2. We also don’t think HARQ-ACK feedback is as a basic feature for multicast reception since we have agreed that RRC+DCI indication mechanism, which means the HARQ-ACK can be disabled all the time. Regarding the wording for PTM and PTP retransmission, we suggest both of them can be modified as following for clarification from UE’s receivieng perspective: - Support PTM retransmission for multicast====> Support receiving of PTM retransmission for multicast based on G-RNTI - Support PTP retransmission for multicast====> Support receiving of PTP retransmission for multicast based on C-RNTI |
| Huawei, HiSilicon | Support 3-1a. 3-1b. ok in principle. We think ACK/NACK should be the component instead of NACK-only. |
| Qualcomm | Not support 3-1a. For 3-1b, ACK/NACK, NACK-only HARQ-ACK feedback, PTM retransmission as well as PTP retransmission should be separate FGs. But we are fine to merge PTM retransmission with HARQ-ACK feedback as in FL2. The HARQ feedback/PHY-layer retransmission is one of options for multicast reliability, which can be optional. |
| FL4 | proposal 3-1a Summary of companies view - Support: ZTE, Nokia, NSB, Huawei, HiSilicon - Same as unicast - Most of the functionalities related to DCI fields in DCI 1_1 will be optional UE feature, UE has the flexibility to report support or not support for them already - Not Support: Qualcomm, Spreadtrum, MediaTek - DCI format 1_0 can be included in the basic format for multicast. Support DCI format 1_1 with configurable fields to be a separate FG - keep the procedure of DCI size alignment not changed in its implementation, and no additional impact on unicast transmission Given that companies still have different view, companies are encouraged to provide further comments considering the comments provided so far. proposal 3-1b Summary of companies view on whether to support at least one of NACK-only and ACK/NACK based HARQ-ACK feedback as a component of FG 33-2 (FFS which one). - Support: ZTE, Huawei, HiSilicon - If HARQ feedback is not included in the basic FG for multicast, multicast will be the same as broadcast as network has to utilize the most conservative scheduling for multicast for these UEs - Not support: Spreadtrum, MediaTek, Qualcomm - Regarding the reliability issue, there are many other ways, e.g., repetition. No feedback is also beneficial for power saving for UEs. It is very valuable at least for MBS only UEs, and is beneficial for MBS quick commercialization Given that companies still have different view, companies are encouraged to provide further comments considering the comments provided so far. Also, based on the following agreement, it is moderator’s understanding that not necessary to separate FGs for “ACK/NACK based HARQ-ACK feedback” and “enabling/disabling ACK/NACK based HARQ-ACK feedback per G-RNTI by RRC signalling”, since the latter one is the default behaviour for ACK/NACK based HARQ-ACK feedback. Agreement: If the group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is not configured, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling. |
| vivo | Proposal 3-1a: support for separation. Proposal 3-1b: NACK-only and ACK/NACK based HARQ-ACK feedback should be separated from 33-2. As multicast can also support low QoS mode, and for high QoS mode, repetition can improve the reliability as well. |
| OPPO | Proposal 3-1a: Support that DCI format 1_1 can be kept in component 4 of FG 33-2. Proposal 3-1b: Both ACK/NACK and NACK-only based HARQ-ACK feedback can be separated from FG 33-2. |
| NTT DOCOMO | proposal 3-1a: Support proposal 3-1b: We support to merge FGs for “ACK/NACK based HARQ-ACK feedback” and “enabling/disabling ACK/NACK based HARQ-ACK feedback per G-RNTI by RRC signalling”. |
| FL5 | proposal 3-1a Summary of companies view - Support: ZTE, Nokia, NSB, Huawei, HiSilicon, OPPO, DOCOMO - Same as unicast - Most of the functionalities related to DCI fields in DCI 1_1 will be optional UE feature, UE has the flexibility to report support or not support for them already - Not Support: Qualcomm, Spreadtrum, MediaTek, vivo - DCI format 1_0 can be included in the basic format for multicast. Support DCI format 1_1 with configurable fields to be a separate FG - keep the procedure of DCI size alignment not changed in its implementation, and no additional impact on unicast transmission Given that companies still have different view, FFS is added in FG33-2 whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast. proposal 3-1b Summary of companies view on whether to support at least one of NACK-only and ACK/NACK based HARQ-ACK feedback as a component of FG 33-2 (FFS which one). - Support: ZTE, Huawei, HiSilicon - If HARQ feedback is not included in the basic FG for multicast, multicast will be the same as broadcast as network has to utilize the most conservative scheduling for multicast for these UEs - Not support: Spreadtrum, MediaTek, Qualcomm, vivo, OPPO - Regarding the reliability issue, there are many other ways, e.g., repetition. No feedback is also beneficial for power saving for UEs. It is very valuable at least for MBS only UEs, and is beneficial for MBS quick commercialization - multicast can also support low QoS mode Given that companies still have different view, FFS is added in FG33-2 whether to support at least one of NACK-only and ACK/NACK based HARQ-ACK feedback. Also, square brackets are added to component 6 Based on the above, the proposal is updated as follows. Companies are encouraged to check whether the following proposal is acceptable or not. [FL5] High priority proposal 3-1b: - FG 33-2 is separated as following FGs - FG 33-2: current FG 33-2 without the capability for support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling - FFS whether to support at least one of NACK-only and ACK/NACK based HARQ-ACK feedback as a component of FG 33-2, FFS which one - FG 33-2a: Enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - FG 33-2b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - FFS whether to separate the capability for HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast - FFS whether to separate the capability for support of PTP retransmission for multicast - FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast 33. NR_MBS; 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast FFS whether to support at least one of NACK-only and ACK/NACK based HARQ-ACK feedback FFS whether to separate the capability for support of PTP retransmission for multicast; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2a; Enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback per G-RNTI for multicast by RRC signaling; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2b; DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| Huawei, HiSilicon | Ok with FL5 proposal 3-1b. |
| MediaTek | We still think the HARQ-ACK feedback can be optional for multicast since we the repetition is also can ensure the reliability and not all broadcast services is QoS services. Besides, if HARQ-ACK is separated as separate FG, we think the PTM retransmission should be not as a basic FG for multicast if supported. Regarding the enabling/disabling ACK/NACK, we the RRC signaling can indicate the HARQ-ACK is disabled. So, we think it still can as a separate FG. We understand the current proposal is to make more progress with more FFS. However, according to the above comments, we suggest the proposal can be modified as following: High priority proposal 3-1b: - FG 33-2 is separated as following FGs - FG 33-2: current FG 33-2 without the capability for support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling - FFS whether to support NACK-only and ACK/NACK based HARQ-ACK feedback as a component of FG 33-2, FFS which one - FG 33-2a: Enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - FG 33-2b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - FFS whether to separate the capability for HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast - FFS whether to separate the capability for support of PTP retransmission for multicast - FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast 33. NR_MBS; 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast FFS whether to support at least one of NACK-only and ACK/NACK based HARQ-ACK feedback FFS whether to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2a; Enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback per G-RNTI for multicast by RRC signaling; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2b; DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| ZTE | Ok with FL5 proposal 3-1b. |
| vivo | Ok with FL5 proposal 3-1b by additionally adding square brackets to component 7 as component 6 has not been determined yet |
| FL6 | The proposal is updated based on the comments from MediaTek and set for email endorsement. Regarding FG 33-2a, since component 6 in FG 33-2 is in square bracket now, it is not necessary for now. High priority proposal 3-1b: - FG 33-2 is separated as following FGs - FG 33-2: current FG 33-2 - FFS whether to support at least one of NACK-only and/or ACK/NACK based HARQ-ACK feedback as a component of FG 33-2 - FG 33-2a: Enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - FG 33-2b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - FFS whether to separate the capability for HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast - FFS whether to separate the capability for support of PTP retransmission for multicast - FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast 33. NR_MBS; 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast FFS whether to support at least one of NACK-only and/or ACK/NACK based HARQ-ACK feedback FFS whether to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2a; Enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback per G-RNTI for multicast by RRC signaling; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2b; DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| FL | Following was agreed by email endorsement Agreement - FG 33-2 is separated as following FGs - FG 33-2: current FG 33-2 - FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback as a component of FG 33-2 - FG 33-2b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - FFS whether to separate the capability for HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast - FFS whether to separate the capability for support of PTP retransmission for multicast - FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast 33. NR_MBS; 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback FFS whether to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2b; DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |

[FL1] High priority question 3-2:

- Companies are encouraged to provide views on whether FG 33-2-x can be removed

| Company | Comment |
| --- | --- |
| vivo | We think FG 33-2-x is necessary |
| MediaTek | In the ongoing RAN2#116-e meeting, one-to-many mapping between G-RNTI and MBS sessions/services was agreed as following. one-to-many mapping between G-RNTI and MBS sessions is supported and it is assumed that this does not introduce additional specification work.. Since the multiple MBS sessions associated with one G-RNTI is supported, the motivation for supporting multiple G-RNTIs for group-common PDSCH is not clear. In addition, supporting multiple G-RNTIs will increase the probability of false alarm for PDCCH detection, which will degrade the system performance. We think one G-NRTI is sufficient for Rel-17 UE supporting multicast services. |
| ZTE | Ok to have 33-2-x. However, a minimum UE capability should be defined, i.e., if UE doesn’t indicate support of 33-2-x, for example, UE has to support up to 2G-RNTIs for multicast. |
| Nokia, NSB | OK to keep it. |
| Qualcomm | We are fine to have 33-2-x. But may also need RAN2 views on this. |
| FL2 | Given this issue is related to RAN2 discussion, following proposal is made to defer the final decision to RAN2 [FL2] High priority proposal 3-2: - Add a note in FG 33-2-x that whether to remove FG 33-2-x is up to RAN2 |
| Huawei, HiSilicon | At least broadcast and multicast corresponding to G-RNTI should be consider. |
| Spreadtrum | Ok |
| NTT DOCOMO | We are fine to have FG 33-2-x. |
| OPPO | OK to add the note. |
| FL3 | This proposal couldn’t be discussed at the GTW session on Nov. 16. Companies are encouraged to provide further comments on the following proposal [FL3] High priority proposal 3-2: - Add a note in FG 33-2-x that whether to remove FG 33-2-x is up to RAN2 |
| MediaTek | As a compromise, we are ok with the note. |
| Huawei, HiSilicon | I am not quite sure whether it is necessary to leave it up to RAN2 because it was RAN2 sending LS to RAN1 that multiple G-RNTI is considered. |
| FL4 | @Huawei, HiSilicon: Let me clarify the intention of the proposal. It means FG 33-2-x is kept from RAN1 perspective (also considering the LS from RAN2), but final decision whether to keep FG 33-2-x is up to RAN2 since it would be related to RAN2 discussion as mentioned by MediaTek. |
| Vivo | Besides one-to-many mapping between G-RNTI and MBS sessions newly agreed in RAN2, they also agreed to support one-to-one mapping between G-RNTI and MBS session in previous meeting, so we cannot assume that all MBS sessions are always mapped to one single G-RNTI. Multiple G-RNTIs are necessary. We are fine to add the note for RAN2’s further check. |
| OPPO | RAN2’s further check can help to make sure about the relationship between G-RNTI and MBS session(s). |
| FL5 | Following proposal will be set for email endorsement after the confirmation from Huawei [FL5] High priority proposal 3-2: - Add a note in FG 33-2-x that whether to remove FG 33-2-x is up to RAN2 |
| Huawei, HiSilicon | Regarding MTK’s comment, RAN2’s agreement of one-to-many mapping does not mean RAN2 intends to withdraw the support of multiple G-RNTI. RAN1 has been doing the specification work for support of multiple G-RNTI following RAN2’s LS in including defining UE capability of reporting number of G-RNTI supported by UE. Adding a note “whether to remove FG 33-2-x is up to RAN2” may confuse RAN2 that RAN1 may have problem to support multiple G-RNTI, which is actually not the case. FG 33-2-x is the capability of number of G-RNTI for groupcast, candidate values may include value 1. Overall, this proposal does not make sense and may cause additional unnecessary confusion. |
| MediaTek | We support the FL’s proposal. From our perspective, if one-to-many mapping between G-RNTI and MBS sessions is supported, the motivation is not clear for us why introducing the multiple G-RNTI for MBS. Besides, introducing the multiple G-NRTI per serving cell will increase the PDCCH detection false alarm. The reason we want to check RAN2 is that when we require RAN2’s suggestion whether the multiple G-RNTI can be supported, they didn’t achieve the “one-to-many” proposal and think if more services is needed, it is nature to use multiple G-RNTI. However, in the latest finished R2 meeting, they achieved the “one-to-many” proposal. Thus, it is should be up to RAN2’s decision whether the FG is needed. |
| FL6 | Since companies have still different view, no more discussion is necessary in this meeting. |

[FL1] High priority question 3-3:

- Companies are encouraged to provide views on whether to add an FG for multiple G-CS-RNTIs for SPS group-common PDSCHs in addition to FG 33-2-x

| Company | Comment |
| --- | --- |
| vivo | We think it can be covered by 33-2-x |
| MediaTek | The issue related to question 3-2, maybe we can discuss the question 3-2 firstly, and then come back to this issue |
| ZTE | Similar as vivo, it can be covered by 33-2-x. |
| Qualcomm | FG for multiple G-CS-RNTIs should be separate from that of G-RNTIs, which it is on condition of SPS multicast in FG 33-5-1. |
| FL2 | As suggested by MediaTek, this proposal is further discussed after some progress is made for proposal 3-2 |
| Huawei, HiSilicon | It is only for SPS for multicast, not necessarily depending on proposal 3-2. Up to UE capability reporting is fine as for unicast. RAN1 spec impact has been considered for multiple G-CS-RNTI. |
| Spreadtrum | We are fine to add an FG. Multiple G-CS-RNTIs for SPS group-common PDSCHs is one separated issue. |
| NTT DOCOMO | It can be covered by 33-2-x. |
| OPPO | Similar view with MediaTek that it can be further discussed/determined after proposal 3-2. |

Medium priority question 3-4:

- Companies are encouraged to provide views on whether FG 33-2 is supported as a basic FG for MBS

  - Support: ZTE

| Company | Comment |
| --- | --- |
| ZTE | We support to make FG33-2 as a basic FG for multicast. |
| Nokia, NSB | We agree with ZTE. |
| Huawei, HiSilicon | We assume if UE reports support of multicast, FG 33-2 should be the set supported for multicast, so yes it is the basic FG for multicast. |

Medium priority question 3-6:

- Companies are encouraged to provide views on whether the type of FGs 33-2/33-2-x should be per UE, per band, or per FSPC

  - FG 33-2

    - Per UE: Intel, Ericsson

      - Per CC for component 5: Ericsson (need clarification as per CC indication is not supported)

    - Per band: Spreadtrum

    - Per FSPC: Qualcomm

  - FG 33-2-x

    - Per FSPC: Qualcomm

| Company | Comment |
| --- | --- |
| ZTE | We would prefer to make them as per UE with FR1/FR2 differentiation. |
| Nokia, NSB | Per UE with FR1/FR2 differentiation |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. We prefer per FSPC for multicast in Rel17 NR MBS. |

Low priority proposal 3-7:

- Name of FG 33-2 is revised as “Dynamic scheduling for mMulticast for RRC_CONNECTED UEs”

- Components of FG 33-2 are revised as

  - Component 1: Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for multicast.

  - Component 2: [Support of CFR configuration for multicast]

    - Only one CFR is supported for multicast reception

  - Component 3: Support of CORESET and common search space configuration in a CFR for multicast.

  - Component 4: Support of [DCI format 1_0 / 1_1] with CRC scrambled with G-RNTI for multicast.

  - Component 5: Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast in different slots.

  - Component 6: Support of ACK/NACK based Type-1/Type-2 HARQ-ACK feedback codebook based on inter-slot TDM-ed unicast and group-common PDSCH, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling.

- Component of FG 33-2-x is revised as “Capability on max number of G-RNTIs for groupcast multicast”

| Company | Comment |
| --- | --- |

Low priority question 3-8:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-2

  - Add FG 33-1: Nokia, NSB

| Company | Comment |
| --- | --- |

Low priority question 3-9:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 33-2 and 33-2-x which do not have capability ignalling impacts

| Company | Comment |
| --- | --- |

- 33-3-1: Slot-level repetition for group-common PDSCH

In [1], FG 33-3-1 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast. FFS whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107-e meeting.

| [7] | OPPO | Discussion on FG 33-3 For FG 33-3-1 slot-level repetition for group-common PDSCH, there is no necessary to split it into two FGs. The two repetition schemes can be indicated separately, but this indicate can be in one FG. For FG 33-3-1, it is not needed to split it into two FGs for semi-static slot-level repetition and dynamic slot-level repetition. |
| --- | --- | --- |
| [10] | Apple | Slot level repetition Regarding the repetition configuration, the following agreements were reached in last RAN1 meeting. With this agreement two configurations could achieve the purpose of repetition, i.e., Config A and Config B. If UE just supports either Config A or Config B, then - UE supports Config A and gNB configures repetition with Config B, UE would not decode the PDSCH with wrong TDRA table - UE supports Config B and gNB configures repetition with Config A, UE would assume no repetition is configured. Thus, Splitting FG33-3-1 is not preferred. Agreement: For slot-level repetition for group-common PDSCH for RRC_CONNECTED Ues receiving multicast, - (Config A) UE can be optionally configured with pdsch-AggregationFactor. - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table. - If UE is configured with Config B, UE does not expect to be configured with Config A for the same group-common PDSCH.. Proposal 2: Keeping FG 33-3-1 as single FG. |
| [11] | Qualcomm Incorporated | 33. NR_MBS; 33-3-1; Slot-level repetition for group-common PDSCH; - Support slot-level repetition for group-common PDSCH for multicast by using DCI format 1_1. FFS whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition; 33-2, 33-2a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |

Discussion

[FL1] High priority question 4-1:

- Companies are encouraged to provide views on whether to split FG 33-3-1 into 2 separate FGs, i.e., 1st one for semi-static slot-level repetition, 2nd one for dynamic slot-level repetition

  - Keep as a single FG: OPPO, Apple

| Company | Comment |
| --- | --- |
| vivo | Ok for separation if both semi-static slot-level repetition and dynamic slot-level repetition are agreed. |
| MediaTek | Support to split it as two separate FGs |
| ZTE | We propose to keep them in one FG. The essence of slot repetition is soft combining instead of indication method. |
| Qualcomm | Prefer to delay it, which may be related with how to support the slot-level repetition for broadcast. |
|  | Summary of companies view - Keep as a single FG: OPPO, Apple, ZTE, - Split to 2 FGs: vivo (if both are agreed), MediaTek - Wait for the progress in AI 8.12: Qualcomm As suggested by Qualcomm, this question is further discussed after some progress is made in AI 8.12. |
| Huawei, HiSilicon | Ok to wait the discussion for broadcast |
| NTT DOCOMO | We don’t see the need to split. |
| OPPO | Support to keep it in one FG. |

Medium priority question 4-2:

- Companies are encouraged to provide views on whether the type of FG 33-3-1 should be per UE or per FSPC

  - Per FSPC: Qualcomm

| Company | Comment |
| --- | --- |
| ZTE | We would prefer to make it as per UE with FR1/FR2 differentiation. |
| Nokia, NSB | Per UE with FR1/2 differentiation |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. We prefer per FSPC for multicast in Rel17 NR MBS. |

Low priority proposal 4-3:

- Component 1 of FG 33-3-1 is revised as “Support slot-level repetition for group-common PDSCH for multicast by using DCI format 1_1”

| Company | Comment |
| --- | --- |

Low priority question 4-4:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-3-1

  - Add FG for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast: Qualcomm

| Company | Comment |
| --- | --- |

Low priority question 4-5:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 33-3-1 which do not have capability ignalling impacts

| Company | Comment |
| --- | --- |

- 33-3-2 to 33-3-3: Multiplexing of unicast PDSCH and group-common PDSCH

In [1], FGs 33-3-2 to 33-3-4 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107-e meeting.

| [2] | Huawei, HiSilicon, CBN | The IE FeatureSetDownlinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination including supportedBandwidthDL, maxNumberMIMO-LayersPDSCH and supportedModulationOrderDL that can determine an maximum TB size in a slot. For UE supporting intra-slot TDM-ed unicast and group-common PDSCH as FG 33-3-3, the maximum TBS size should be reported and is supposed to be the total maximum TB size for both unicast and group-common PDSCH. Proposal 4: Add a component of “Supported maximum TB size” into FG 33-3-3. |
| --- | --- | --- |
| [3] | ZTE | Another issue is how to capture agreements for FG33-3-3. From our perspective, UE can share the same maximum PDSCH receptions in a slot per CC between unicast and multicast. Thus, the FG33-3-3 can be updated as following. Proposal 2: Update the components description as following. 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between X unicast PDSCH(s) and Y group-common PDSCH(s) in a slot, where X+Y <= Z. Z is the maximum number of TDMed PDSCH receptions capability in a slot per CC according to Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b.  Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast.. |
| [4] | vivo | For multicast, the capability for group-common PDSCH reception doesn’t have to be coupled with HARQ-ACK feedback. In general, there exist three reactions when UE receives group-common PDSCH: does not send any HARQ-ACK feedback; sends NACK-only based HARQ-ACK feedback; sends ACK/NACK based HARQ-ACK feedback. Thus, support of ACK/NACK based HARQ-ACK feedback should be separated from 33-2. Support of FDM-ed HARQ-ACK codebook should be separated from 33-3-2, and support of TDM-ed HARQ-ACK codebook should be separated from 33-3-3. Furthermore, similar as that in NR Rel-15, support of type 1 and type 2 codebook should be separated as different UE capabilities. Regarding the retransmission for multicast, whether to differentiate PTP (re)transmissions for unicast and PTP (re)transmissions multicast is under discussion and thus, it’s better to wait for further progress to determine whether support of PTP retransmission for multicast is a separate UE capability. According the achieved agreements in appendix, support of intra-slot TDM between unicast PDSCH(s) and group-common PDSCH(s), or among multiple group-common PDSCHs in a slot, and support of FDM between unicast PDSCH and group-common PDSCH should also apply to broadcast. Thus, 33-3-2 and 33-3-3 should take 33-1 or 33-2 as prerequisite feature groups. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Mandatory/Optional. 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH; - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot.; 33-1, 33-2; Yes; Optional with capability signalling. 33. NR_MBS; 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s).; 33-1 or 33-2; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-1; ACK/NACK based HARQ-ACK feedback for multicast; - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC ignalling.; 33-2; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-2; TDM-ed Type-1 HARQ-ACK codebook for multicast; - Support TDM-ed Type-1 HARQ-ACK codebook for multicast.; 33-2, 33-3-3, 33-4-1; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-3; FDM-ed Type-1 HARQ-ACK codebook for multicast; - Support FDM-ed Type-1 HARQ-ACK codebook for multicast.; 33-2, 33-3-2, 33-4-1; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-4; Type 2 HARQ-ACK codebook for multicast; - Support Type-2 HARQ-ACK codebook for multicast.; 33-2, 33-4-1; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-5; NACK-only based HARQ-ACK feedback for multicast; - Support NACK-only based HARQ-ACK feedback; 33-2; Yes; Optional with capability signalling. |
| [5] | Spreadtrum Communications | FG33-3-2: Given the above, HARQ-ACK feedback is not the basic operation for multicast. Thus, we prefer to split component 2 and 3 in FG33-3-2 as separated FG. Proposal 4: Component 2 and 3 in FG33-3-2 can be split as separated FG, and as optional UE capability. FG33-3-3: Given the above, HARQ-ACK feedback is not the basic operation for multicast. Thus, we prefer to split component 6 and 7in FG33-3-3as separated FG. Proposal 5: Component 6 and 7 in FG33-3-3 can be split as separated FG, and as optional UE capability. 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH; - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. FFS whether/how to separate the capability for HARQ-ACK codebook; 33-1, 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-3-2-1; HARQ-ACK codebook for FDMed unicast and multicast; - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast.; 33-3-2; Yes; Per UE; No; No; Component 1: {Support, not support} Component 2: {Support, not support}; Optional with capability signalling. 33. NR_MBS; 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). FFS whether/how to separate the capability for HARQ-ACK codebook; 33-1, 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-3-3-1; HARQ-ACK codebook for intra-slot TDM-ed unicast PDSCH and group-common PDSCH; 1. Support TDM-ed Type-1 HARQ-ACK codebook for multicast. 2.Support TDM-ed Type-2 HARQ-ACK codebook for multicast.; 33-3-3; Yes; Per UE; No; No; Component 1: {Support, not support} Component 2: {Support, not support}; Optional with capability signalling. |
| [6] | Nokia, Nokia Shanghai Bell | - 33-3-2: - If CONNECTED_MODE Ues can receive broadcast without supporting multicast, component 1 may need to be separated from 2/3. More clarification needed here. - 33-3-3: - Confirm the FG, details to be futher discussed - Same comment as for 33-3-2. - 33-3-4: - Same comment as above on mixture of broadcast/multicast functionalities - Unclear if intention is to signal candidate values for M, N, K, L or else if UE is expected to always support all values defined in specifications. If explicitly signalled, it needs to be clarified if candidate values are defined as part of RAN1 or RAN2 UE features. - Add pre-requisite ”one of {5-11/5-11a/5-11b}” |
| [9] | Samsung | Pre-requisite For 33-3-2/3, pre-requisite can be either 33-1 or 33-2, but should not be both. Features; Index; Feature group; Components; Prerequisite feature groups. 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH; - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast.; 33-1 or 33-2. 33. NR_MBS; 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. - FFS whether/how to separate the capability for HARQ-ACK codebook; 33-1 or 33-2. |
| [10] | Apple | Multiplexing between multicast and unicast For FG33-3-2, it’s so restrictive to limit only one unicast PDSCH and only one GC-PDSCH FDMed in a slot. Similar as FG3-3-3, it can support K TDMed unicast PDSCH in a slot and L TDMed GC-PDSCH in a slot, but unicast PDSCH and GC-PDSCH are FDMed. With this update, there is no codebooks construction issue, anyway the codebook for unicast PDSCH and for GC-PDSCH are generated separately and concatenated together. The total number of received unicast PDSCHs and GC-PDSCHs in a slot are subject to TDMed PDSCH reception capability defined in Rel-15/Rel-16, i.e.,{2/4/7}. Proposal 3: Support FDM between K TDMed unicast PDSCHs and L TDMed group common PDSCHs in a slot. For the separation of capability for HARQ-ACK codebook in FG33-3-2 and Fg33-3-3, as we comment earlier supporting HARQ-ACK feedback is a UE capability, then the supported HARQ-ACK codedbook types should be independent UE capability with the condition that HARQ-ACK feedback is supported. Proposal 4: Supporting HARQ-ACK codebook types as independent capability. |
| [11] | Qualcomm Incorporated | 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH; - Support of FDM between one unicast PDSCH and one group-common PDSCH in a slot.; 33-1, 33-2; Yes; Per FSPC; N/A; N/A; FFS: maximum data rate of (GC-)PDSCHs in a slot per CC.; Optional with capability signalling. 33. NR_MBS; 33-3-2a; Feedback multiplexing for FDM-ed unicast PDSCH and group-common PDSCH; 1. Support of FDM-ed ACK/NACK-based HARQ-ACK codebook for one unicast and one multicast in a slot.; 33-2b, 33-3-2; Yes; Per FSPC; N/A; N/A; Values = {Type-1 HARQ-ACK codebook only, Type-2 HARQ-ACK codebook only, both}; Optional with capability signalling. 33. NR_MBS; 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH for multicast; - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s).; 33-1, 33-2; Yes; Per FSPC; N/A; N/A; FFS: maximum data rate of (GC-)PDSCHs in a slot per CC.; Optional with capability signalling. 33. NR_MBS; 33-3-3a; Feedback multiplexing for intra-slot TDM-ed unicast PDSCH and group-common PDSCH for multicast; 1. Support of TDM-ed ACK/NACK-based HARQ-ACK codebook for M>= 1 unicast and N>=1 multicast in a slot.; 33-2b, 33-3-3; Yes; Per FSPC; N/A; N/A; Values = {Type-1 HARQ-ACK codebook only, Type-2 HARQ-ACK codebook only, both}; Optional with capability signalling. |
| [12] | MediaTek Inc. | 33-3-2: FDM-ed unicast PDSCH and group-common PDSCH From our understanding, supporting FDM between unicast and GC-PDSCH does not mean FDM-ed HARQ-ACK codebook. Thus, we suggest to separate the FDM-ed HARQ-ACK from the FG 33-3-2. Proposal 8: For FG 33-3-2, the FDM-ed HARQ-ACK codebook capability should be as a separate FG. 33-3-3: Intra-slot TDM-ed unicast PDSCH and group-common PDSCH From our understanding, supporting TDM between unicast and GC-PDSCH does not mean TDM-ed HARQ-ACK codebook. Thus, we suggest to separate the TDM-ed HARQ-ACK from the FG 33-3-2. Proposal 9: For FG 33-3-3, the TDM-ed HARQ-ACK codebook capability should be as a separate FG. |
| [13] | Ericsson | For both of these features, HARQ codebook support also should include unicast and multicast, as it is possible to create a common codebook. The following change is proposed: 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH; - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast and unicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast and unicast.. 33. NR_MBS; 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM-ed Type-1 HARQ-ACK codebook for multicast and unicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast and unicast.. |

Discussion

[FL1] High priority question 5-1:

- Companies are encouraged to provide views on whether/how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3, e.g.,

  - Option 1:

    - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast

    - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast

    - add an FG for Type-2 HARQ-ACK codebook for multicast

  - Option 2:

    - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast

    - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast

  - Option 3:

    - Any other FG structures

  - Note: whether to separate the capability for ACK/NACK based HARQ-ACK feedback is discussed in question 3-1

| Company | Comment |
| --- | --- |
| vivo | Option 1 |
| Support | Support to separate it and Option 1 is preferred. |
| ZTE | We propose to include “Type-2 HARQ-ACK codebook for multicast” in the basic FG for multicast. With this, we are fine to have separate UE FG for “TDM-ed Type-1 HARQ-ACK codebook for multicast” and “FDM-ed Type-1 HARQ-ACK codebook for multicast”. |
| Nokia, NSB | Option 2 |
| Qualcomm | Option 2 |
| FL2 | Given that companies have different view, further discussion is necessary. Proposal is updated based on the comments [FL] Let me correct the typo. Sorry for that. [FL2] High priority proposal 5-1: - Down select one of the followings on how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3 - Option 1: - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for Type-2 HARQ-ACK codebook for multicast - Option 2: - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast - Option 3: - Add the capability for Type-2 HARQ-ACK codebook for multicast as a component in FG 33-2 - add an FG for FDM-ed Type-1 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - Note: whether to separate the capability for ACK/NACK based HARQ-ACK feedback is discussed in question 3-1 |
| Huawei, HiSilicon | We think the current structure of FG 33-3-2 and 33-3-3 are good to have a complete set of a feature. Can accept option 2 as well. |
| Spreadtrum | Option 2 |
| FL3 | This proposal couldn’t be discussed at the GTW session on Nov. 16. Companies are encouraged to provide further comments on the following proposal [FL3] High priority proposal 5-1: - Down select one of the followings on how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3 - Option 1: - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for Type-2 HARQ-ACK codebook for multicast - Option 2: - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast - Option 3: - Add the capability for Type-2 HARQ-ACK codebook for multicast as a component in FG 33-2 - add an FG for FDM-ed Type-1 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - Note: whether to separate the capability for ACK/NACK based HARQ-ACK feedback is discussed in question 3-1 |
| ZTE | Basically, we support Option 3. As we clarified in Section 3, type-2 HARQ-ACK codebook is a mandatory UE feature for unicast, we can adopt the similar handling for multicast. |
| MediaTek | Option 2 is preferred. |
| Huawei, HiSilicon | Option 3 is preferable. |
| Qualcomm | We think the HARQ-ACK feedback is optional feature, so we prefer Option 2 of HARQ-ACK codebooks for multicast. |
| FL4 | Since no companies support Option 1, proposal is updated as follows [FL4] High priority proposal 5-1: - Down select one of the followings on how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3 - Option 1: - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for Type-2 HARQ-ACK codebook for multicast - Option 2: - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast - Option 3: - Add the capability for Type-2 HARQ-ACK codebook for multicast as a component in FG 33-2 - add an FG for FDM-ed Type-1 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - Note: whether to separate the capability for ACK/NACK based HARQ-ACK feedback is discussed in question 3-1 |
| Vivo | We support option 1. In our understanding, unlike construction of type 1 codebook, there is no difference between TDMed and FDMed construction type 2 codebook. Thus, we think separation of TDM-ed and FDM-ed Type-1/2 HARQ-ACK is not necessary |
| NTT DOCOMO | We prefer Option 3. |
| FL5 | Option 1 is revised back based on the comment from vivo. Since companies still have different view, no down selection will be done in this meeting but let’s try to agree on the following proposal to narrow down the scope in the next RAN1 meeting. [FL5] High priority proposal 5-1: - Down select one of the followings on how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3 - Option 1: - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for Type-2 HARQ-ACK codebook for multicast - Option 2: - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast - Option 3: - Add the capability for Type-2 HARQ-ACK codebook for multicast as a component in FG 33-2 - add an FG for FDM-ed Type-1 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - Note: whether to separate the capability for ACK/NACK based HARQ-ACK feedback is discussed in question 3-1 |
| Huawei, HiSilicon | FL5 proposal 5-1, I doubt FDM-ed Type-2 or TDM-ed Type-2 is accurate because Type-2 is essentially generated according to C-/T-DAI. FDM-ed/TDM-ed codebook we are talking about in NR MBS is mainly for Type-1 codebook. |
| MediaTek | Support the FL5 for further discussion and down-selection. |
| ZTE | Similar view as Huawei, we didn’t differentiate FDMed or TDMed for type-2 codebook. |
| vivo | Agree that it is not necessary to differentiate FDMed or TDMed for type-2 codebook, and thus, we support option 1. |
| FL6 | Note that the intention of this proposal is NOT to down select in this meeting but to narrow down the scope in the next RAN1 meeting so that we can discuss this issue in next meeting more efficiently. I would set this proposal for email endorsement. If it is not acceptable, I would just drop it. High priority proposal 5-1: - Down select one of the followings on how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3 - Option 1: - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for Type-2 HARQ-ACK codebook for multicast - Option 2: - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast - Option 3: - Add the capability for Type-2 HARQ-ACK codebook for multicast as a component in FG 33-2 - add an FG for FDM-ed Type-1 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast |
| FL | Following was agreed by email endorsement Agreement - Down select one of the followings on how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3 - Option 1: - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for Type-2 HARQ-ACK codebook for multicast - Option 2: - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast - Option 3: - Add the capability for Type-2 HARQ-ACK codebook for multicast as a component in FG 33-2 - add an FG for FDM-ed Type-1 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast |

Medium priority question 5-2:

- Companies are encouraged to provide views on whether to support FDM between K TDMed unicast PDSCHs and L TDMed group common PDSCHs in a slot for FG 33-3-2.

| Company | Comment |
| --- | --- |
| ZTE | RAN1 has agreed to support FDMed unicast PDSCH and multicast PDSCH. If there are multiple sets of FDMed unicast PDSCH and multicast PDSCH and each set of them is TDMed with another set in the same slot, we think this case can be supported subject to UE capability. |

Medium priority question 5-3:

- Companies are encouraged to provide views on whether UE reports the supported values {M, N, K, L} in FG 33-3-3 or UE is expected to support all candidate values defined in specification if the UE reports the support of FG 33-3-3.

| Company | Comment |
| --- | --- |
| ZTE | From our understanding, we only need to indicate the maximum number of PDSCHs (including both unicast and multicast) that UE is able to receive in one slot. The UE FG description can be updated as following. - Support TDM between X unicast PDSCH(s) and Y group-common PDSCH(s) in a slot, where X+Y <= Z. Z is the maximum number of TDMed PDSCH receptions capability in a slot per CC according to Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b.  Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). |

Medium priority question 5-4:

- Companies are encouraged to provide views on whether to add a component for supported maximum TB size into 33-3-3.

| Company | Comment |
| --- | --- |
| ZTE | We don’t see the motivation to add such a capability considering that the maximum data rate has already been defined since Rel-15. |
| Qualcomm | It is not clear to us whether the maximum TB size per slot in Rel-15 is sufficient to support FDMed/TDMed unicast and multicast or not. |
| Huawei, HiSilicon | To clarify, both FG 33-3-2 and FG33-3-3 needs a max TB size per slot. As in R16 when FDMed/TDMed for multiple is introduced, the max TB size is added in UE feature. It needs to clarify when UE reports maximum TB size, it is for unicast only or the total for both unicast and multicast. |
| Samsung | This should be discussed in WI first whether there will be an TBS restriction for MBS or not. |

Medium priority question 5-5:

- Companies are encouraged to provide views on whether the type of FGs 33-3-2 and 33-3-3 should be per UE or per FSPC

  - Per UE: Spreadtrum

  - Per FSPC: Qualcomm

| Company | Comment |
| --- | --- |
| ZTE | We propose to make them as per UE with FR1/FR2 differentiation. |
| Nokia, NSB | Per UE with FR1/FR2 differentiation |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. We prefer per FSPC for multicast in Rel17 NR MBS. |

Low priority proposal 5-6:

- Components of FG 33-3-2 are revised as

  - Component 2: Support FDM-ed Type-1 HARQ-ACK codebook for multicast and unicast.

  - Component 3: Support FDM-ed Type-2 HARQ-ACK codebook for multicast and unicast.

- Components of FG 33-3-3 are revised as

  - Components 1 to 5: Support TDM between one X unicast PDSCH(s) and one Y group-common PDSCH(s) in a slot, where X+Y <= Z. Z is the maximum number of TDMed PDSCH receptions capability in a slot per CC according to Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b.

    - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s).

  - Component 6: Support TDM-ed Type-1 HARQ-ACK codebook for multicast and unicast.

  - Component 7: Support TDM-ed Type-2 HARQ-ACK codebook for multicast and unicast.

| Company | Comment |
| --- | --- |
| Qualcomm | The unicast should be added for the support of multiplexing of unicast+multicast, or multiple multicast without unicast. |

Low priority question 5-7:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 33-3-2 and 33-3-3

| Company | Comment |
| --- | --- |
| Samsung | We think one of 33-1 or 33-2 is enough for pre-requisite for 33-3-2/3. |

Low priority question 5-8:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 33-3-2 to 33-3-3 which do not have capability ignalling impacts

| Company | Comment |
| --- | --- |

- 33-4: NACK-only based HARQ-ACK feedback for multicast

In [1], FG 33-4 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107-e meeting.

| [6] | Nokia, Nokia Shanghai Bell | - 33-4: - To be mandatorily indicated for Ues supporting 33-2, as it is an essencial functionality for groupcast. |
| --- | --- | --- |
| [8] | Intel Corporation | - FG 33-4: - Add support of enabling/disabling NACK-only feedback by RRC ignalling Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC). 33. NR_MBS; 33-4; NACK-only based HARQ-ACK feedback for multicast; - Support NACK-only based HARQ-ACK feedback. - Support enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signaling; 33-2; Yes; Per UE. |
| [11] | Qualcomm Incorporated | 33. NR_MBS; 33-4; NACK-only based HARQ-ACK feedback for multicast; - Support of NACK-only based HARQ-ACK feedback. - Support PTM retransmission for dynamically scheduled multicast based on G-RNTI; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-4a; More than one NACK-only based HARQ-ACK feedback for multicast in a PUCCH resource; 1. Support of more than one NACK-only based HARQ-ACK feedback for dynamically scheduled multicast in a PUCCH resource; 33-4; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |

Discussion

[FL1] High priority question 6-1:

- Companies are encouraged to provide views on whether to separate the capability for more than one NACK-only based HARQ-ACK feedback for multicast in a PUCCH resource from FG 33-4

| Company | Comment |
| --- | --- |
| vivo | Ok for separation. |
| MediaTek | Support |
| ZTE | This issue is under discussion in AI8.12.2, prefer to wait for the discussion outcome first. Because if it is converged, it basically means this function is not supported and hence no FG is needed. |
| Qualcomm | Yes |
| FL2 | As suggested by ZTE, this question is further discussed after some progress is made in AI 8.12.2 |
| Huawei, HiSilicon | This can wait for more progress under AI 8.12.2 |
| Spreadtrum | Ok to wait the progress in AI8.12.2 |
| NTT DOCOMO | OK to wait the progress in AI 8.12.2. |
| OPPO | Agree to separate the capability for mote than one NACK-only based feedback. |

Medium priority question 6-2:

- Companies are encouraged to provide views on whether the UE supporting FG 33-2 must indicate the support of FG 33-4.

| Company | Comment |
| --- | --- |
| ZTE | This depends whether NACK-only based feedback or ACK/NACK-based feedback should be included in the basic FG for multicast. Our preference is to include ACK/NACK based feedback in the basic FG. Thus, separate FG for NACK-only based feedback is needed and UE supporting FG 33-2 is not required to indicate the support of FG 33-4 |
| Nokia, NSB | Unfortunately our preference seems to be the opposite of ZTE, we would be OK to include NACK-only based feedback in FG 33-2, and have a separate FG on the other cases. |
| Qualcomm | NACK-only based feedback should be a separate FG. |

Medium priority question 6-3:

- Companies are encouraged to provide views on whether the type of FG 33-4 should be per UE or per FSPC

  - Per UE: Intel

  - Per FSPC: Qualcomm

| Company | Comment |
| --- | --- |
| ZTE | Our proposal is to make it as per UE with FR1/FR2 differentiation. |
| Nokia, NSB | Per UE, FR1/2 differentiation. |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. We prefer per FSPC for multicast in Rel17 NR MBS. |

Low priority proposal 6-4:

- Components of FG 33-4 are revised as

  - Component 2: Support FDM-ed Type-1 HARQ-ACK codebook for multicast and unicast.

  - Add a component for support enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signaling

  - Add a component for support PTM retransmission for dynamically scheduled multicast based on G-RNTI

| Company | Comment |
| --- | --- |

Low priority question 6-5:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-4

| Company | Comment |
| --- | --- |

Low priority question 6-6:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 33-4 which do not have capability ignalling impacts

| Company | Comment |
| --- | --- |

- 33-5-1 to 33-5-2: SPS group-common PDSCH for multicast

In [1], FGs 33-5-1 to 33-5-2 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH FFS whether/how to separate the above capabilities from FG 33-5-1 | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration for multicast - Support [M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107-e meeting.

| [2] | Huawei, HiSilicon, CBN | Regarding FG 33-5-1, bullet 4 writes FFS HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. However, we noted that RAN1 has agreed to support HARQ-ACK feedback for SPS group-common PDSCH as follows: Agreement: For RRC_CONNECTED Ues, support HARQ-ACK feedback for SPS group-common PDSCH for MBS - FFS: The retransmission scheme(s) - FFS: The HARQ-ACK details for SPS PDSCH and activation/deactivation, which can be discussed in AI 8.12.2 Therefore, at least for SPS group-common with PDCCH scheduling since the first group-common PDSCH has PDCCH scheduling always, ACK/NACK based feedback should be supported. Proposal 3: Update bullet 4 for FG 33-5-1 to be “support of ACK/NACK based feedback for SPS group-common PDSCH with PDCCH scheduling and FFS on SPS release PDCCH”. |
| --- | --- | --- |
| [4] | vivo | Similar as dynamic scheduled group-common PDSCH reception, capability of HARQ-ACK feedback should be separated from data reception in 33-5-1. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Mandatory/Optional. 33. NR_MBS; 33-4-1; ACK/NACK based HARQ-ACK feedback for multicast; - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC ignalling.; 33-2; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-2; TDM-ed Type-1 HARQ-ACK codebook for multicast; - Support TDM-ed Type-1 HARQ-ACK codebook for multicast.; 33-2, 33-3-3, 33-4-1; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-3; FDM-ed Type-1 HARQ-ACK codebook for multicast; - Support FDM-ed Type-1 HARQ-ACK codebook for multicast.; 33-2, 33-3-2, 33-4-1; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-4; Type 2 HARQ-ACK codebook for multicast; - Support Type-2 HARQ-ACK codebook for multicast.; 33-2, 33-4-1; Yes; Optional with capability signalling. 33. NR_MBS; 33-4-5; NACK-only based HARQ-ACK feedback for multicast; - Support NACK-only based HARQ-ACK feedback; 33-2; Yes; Optional with capability signalling. 33. NR_MBS; 33-5-1; SPS group-common PDSCH for multicast; - Support one SPS group-common PDSCH configuration for multicast - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. FFS whether/how to separate the above capabilities from FG 33-5-1; 33-2; Yes; Optional with capability signalling. |
| [5] | Spreadtrum Communications | FG33-5-1: Given the above, HARQ-ACK feedback is not the basic operation for multicast. Thus, we prefer to split component 2, 3 in FG33-5-1 as separated FG. Proposal 6: Component 2, 3 in FG33-5-1 can be split as separated FG, and as optional UE capability. Regarding repetition for SPS, we have the following agreement made in RAN1#106b-e: Agreement: For slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED Ues. - Config A or Config B can be configured to UE: - (Config A) UE can be optionally configured with pdsch-AggregationFactor per SPS-Config-Multicast. - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Multicast. If UE is configured with Config B, UE does not expect to be configured with Config A for the same SPS group-common PDSCH. - For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value is - Alt1: equal to 1. Dynamic repetition number indication and semi-static repetition number indication for SPS group-common PDSCH are considered. It is similar to repetition indication for dynamic group-common PDSCH. In our mind, there is no necessary to have additional FG for SPS. We are fine that FG 33-3-1 is also applied for SPS group-common PDSCH. FG33-5-2: For the component 1 of FG33-5-2, if taking FG12-2 as reference, it shall be per CFR configuration, and further constraint on per cell or cell group can also be considered. Proposal 7: Component 1 in FG33-5-2 can be revised as ‘Support up to [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group’. 33. NR_MBS; 33-5-1; SPS group-common PDSCH for multicast; - Support one SPS group-common PDSCH configuration for multicast - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. FFS whether/how to separate the above capabilities from FG 33-5-1; 33-2; Yes; Per Band; No; No; Optional with capability signalling. 33. NR_MBS; 33-5-1-1; Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling; - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling.; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-5-1-2; Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling; - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling.; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-5-2; Multiple SPS group-common PDSCH configuration; - Support up to [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - Support up to [M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast; 33-2; Yes; Per UE; No; No; Optional with capability signalling. |
| [7] | OPPO | For FG 33-5-1, at least supporting one SPS GC-PDSCH configuration for multicast is the basic function. The rest components 2~5 can be separated from FG 33-5-1. HARQ-ACK feedback is can be enabled/disabled with different feedback schemes. Furthermore, slot-level repetition can also be separated to an independent FG. - For FG 33-5-1, component 2/3/4 can be separated from this FG. - For FG 33-5-1, component 5 can be separated from this FG into an independent FG. For FG 33-5-2, the association between multiple SPS configuration and CFR is not determined. Furthermore, the two components should be considered separately. - For FG 33-5-2, component 1 and 2 can be separated into two FGs. |
| [8] | Intel Corporation | - FG 33-5-1: - HARQ/ACK feedback support should be separated from this feature group similar to 33-2 |
| [11] | Qualcomm Incorporated | 33. NR_MBS; 33-5-1; SPS group-common PDSCH for multicast; - Support of one SPS group-common PDSCH configuration for multicast - Support of [Nmulticast_SPS >=1 (FFS: value of Nmulticast_SPS)] group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast - Support of DCI format 1_0 with CRC scrambled with G-CS-RNTI for multicast. - Support of slot-level repetition for SPS group-common PDSCH by using DCI format 1_0 - FFS: HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release PDCCH.; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1a; SPS multicast using DCI format 1_1; - Support of DCI format 1_1 with CRC scrambled with G-CS-RNTI for multicast SPS transmission.; 33-5-1; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1b; ACK/NACK-based HARQ ACK feedback for SPS multicast; - Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support of PTM retransmission for SPS group-common PDSCH based on G-CS-RNTI; 33-5-1; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1c; - Support PTP retransmission for SPS group-common PDSCH based on CS-RNTI; 33-5-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1d; NACK-only-based HARQ ACK feedback for SPS multicast; - Support of NACK-only-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support of PTM retransmission for SPS group-common PDSCH based on G-CS-RNTI; 33-5-1; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1e; Slot-level repetition of SPS group-common PDSCH for multicast by using DCI format 1_1; - Support of slot-level repetition for SPS group-common PDSCH by using DCI format 1_1 FFS whether to split FG 33-5-1e into two FGs for semi-static slot-level repetition and dynamic slot-level repetition; 33-5-1, 33-5-1a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1f; HARQ-ACK feedback mode selection for SPS group-common PDSCH without PDCCH scheduling for multicast; - Support of selection of ACK/NACK-based or NACK-only-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling [FFS per G-CS-RNTI].; 33-5-1b, 33-5-1d; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-2; Multiple SPS group-common PDSCH configuration; - Support [N>1] SPS group-common PDSCH configurations per CFR for multicast Support [N>=M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |
| [13] | Ericsson | For SPS it is up to the gNB to configure an SPS configuration for multicast or unicast, and there can be more than one SPS configuration. Moreover, how retransmission is supported for SPS should also be a separate feature. Finally we propose that feature 4 is clarified to be relevant for SPS activation and release. The following change is proposed: 33. NR_MBS; 33-5-1; SPS group-common PDSCH for multicast; - Support SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common activation with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH - Support of PTM retransmission for SPS multicast - Support of PTP retransmission for SPS multicast. |

Discussion

[FL1] High priority question 7-1:

- Companies are encouraged to provide views on whether to separate following capabilities from FG 33-5-1

  - Support of DCI format 1_1 with CRC scrambled with G-CS-RNTI for multicast SPS transmission

  - Support of ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling

  - Support of PTP retransmission for SPS group-common PDSCH based on CS-RNTI

  - Support of NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling

  - Support of slot-level repetition for SPS group-common PDSCH for multicast

    - Whether to separate semi-static slot-level repetition and dynamic slot-level repetition

  - Support of selection of ACK/NACK-based or NACK-only-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling

| Company | Comment |
| --- | --- |
| vivo | Ok for separation, and the separated FGs can be merged with the separated ones from FG 33-2 |
| ZTE | The NACK-only feedback can be merged with FG33-4. The slot-level repetition can be merged with FG 33-3-1. We think we don’t need to have separate FGs for ACK/NACK based feedback for SPS activation/deactivation and SPS PDSCH. |
| Nokia, NSB | We do not see a clear need for separation. |
| Qualcomm | Support |
| FL2 | Summary of companies view - Support of DCI format 1_1 with CRC scrambled with G-CS-RNTI for multicast SPS transmission - Support: Qualcomm, vivo - Not support: Nokia, NSB - Support of ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling - Support: Qualcomm, vivo (merged with FG 32-2x) - Not support: Nokia, NSB, ZTE - Support of PTP retransmission for SPS group-common PDSCH based on CS-RNTI - Support: Qualcomm, vivo - Not support: Nokia, NSB - Support of NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling - Support: Qualcomm, vivo, ZTE (merged with 33-4) - Not support: Nokia, NSB - Support of slot-level repetition for SPS group-common PDSCH for multicast - Support: Qualcomm, ZTE (merged with 33-3-1) - Not support: Nokia, NSB - Whether to separate semi-static slot-level repetition and dynamic slot-level repetition - Support of selection of ACK/NACK-based or NACK-only-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling - Support: Qualcomm, vivo (merged with FG 32-2x) - Not support: Nokia, NSB Given that companies have different view, no additional proposal is made for now but further discussion is necessary. Companies are invited to provide further comment, if any |
| Huawei, HiSilicon | We only think NACK-only for PDSCH without PDCCH can be a separate because it is new in Uu interface for dynamic and SPS. |
| NTT DOCOMO | We have the similar view with ZTE. |
| OPPO | - Support of DCI format 1_1 with CRC scrambled with G-CS-RNTI for multicast SPS transmission No need to separate. - Support of ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling - Support of NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling The above two can be putted into one independent FG. - Support of PTP retransmission for SPS group-common PDSCH based on CS-RNTI No need to separate. - Support of slot-level repetition for SPS group-common PDSCH for multicast - Whether to separate semi-static slot-level repetition and dynamic slot-level repetition Support to keep both in one which is separated from the main FG. |
| FL3 | Companies are encouraged to provide further comments on question 7-1 |
| Spreadtrum | We think it can apply the same principle with FG33-2. Regarding question 7-1, we support all sub-bullets other than the last one. |
| MediaTek | We support the separation. |
| FL4 | Summary of companies view - Support of DCI format 1_1 with CRC scrambled with G-CS-RNTI for multicast SPS transmission - Support: Qualcomm, vivo - Not support: Nokia, NSB, Huawei, HiSilicon, OPPO - Support of ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling - Support: Qualcomm, vivo (merged with FG 32-2x), OPPO (merge with NACK-only based) - Not support: Nokia, NSB, ZTE, Huawei, HiSilicon, DOCOMO - Support of PTP retransmission for SPS group-common PDSCH based on CS-RNTI - Support: Qualcomm, vivo - Not support: Nokia, NSB, Huawei, HiSilicon, OPPO - Support of NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling - Support: Qualcomm, vivo, ZTE (merged with 33-4), Huawei, HiSilicon, DOCOMO (merge with 33-4), OPPO (merge with ACK/NACK based) - Not support: Nokia, NSB - Support of slot-level repetition for SPS group-common PDSCH for multicast - Support: Qualcomm, ZTE (merged with 33-3-1), DOCOMO (merge with 33-3-1), OPPO - Not support: Nokia, NSB, Huawei, HiSilicon - Whether to separate semi-static slot-level repetition and dynamic slot-level repetition - Support of selection of ACK/NACK-based or NACK-only-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling - Support: Qualcomm, vivo (merged with FG 32-2x) - Not support: Nokia, NSB, Huawei, HiSilicon Since this issue would be related to FG 33-2 (dynamic scheduling for multicast), this issue can be discussed after some progress is made for dynamic scheduling |

Medium priority question 7-2:

- Companies are encouraged to provide views on whether the type of FGs 33-5-1 and 33-5-2 should be per UE, per band, or per FSPC

  - FG 33-5-1

    - Per band: Spreadtrum

    - Per FSPC: Qualcomm

  - FG 33-5-2

    - Per UE: Spreadtrum

    - Per FSPC: Qualcomm

| Company | Comment |
| --- | --- |
| ZTE | We propose to make them as per UE with FR1/FR2 differentiation. |
| Nokia, NSB | Per UE, FR1/2 differentiation |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. We prefer per FSPC for multicast in Rel17 NR MBS. |

Low priority proposal 7-3:

- Components of FG 33-5-1 are revised as

  - Component 4: FFS: HARQ-ACK feedback for SPS group-common with PDCCH activation scheduling and SPS release PDCCH

  - Add a component for support of [Nmulticast_SPS >=1 (FFS: value of Nmulticast_SPS)] group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast

  - Add a component for support of PTM retransmission for SPS multicast

- Components of FG 33-5-2 are revised as

  - Component 1: Support up to [N>1] SPS group-common PDSCH configuration [per CFR] for multicast, and up to K SPS group-common PDSCH configuration in a cell group

  - Component 2: Support up to [N>M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast

| Company | Comment |
| --- | --- |

Low priority question 7-4:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 33-5-1 and 33-5-2 which do not have capability cheduled impacts

| Company | Comment |
| --- | --- |

- 33-6-1 to 33-6-3: HARQ-ACK with different priorities for multicast

In [1], FGs 33-6-1 to 33-6-3 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | - Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1 | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107-e meeting.

| [6] | Nokia, Nokia Shanghai Bell | - 33-6-2: - Confirm the FG, details to be futher discussed - 33-6-3: - Confirm the FG, details to be futher discussed |
| --- | --- | --- |
| [8] | Intel Corporation | - FG 33-6: - 33-6-1 should be renamed HARQ-ACK priority indication for multicast in DCI - 33-6-2/3 should be combined into a single FG since these two features are inter-related Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC). 33. NR_MBS; 33-6-1; HARQ/ACK DL priority indication for multicast in DCI; - Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1; 33-2; Yes; Per UE. |
| [9] | Samsung | Adding G-CS-RANTI for FG 33-6-1 For 33-6-1, we assume that activation DCI with G-CS-RNTI would also indicate priority. Then, G-CS-RNTI needs to be added as well. Features; Index; Feature group; Components. 33. NR_MBS; 33-6-1; DL priority indication for multicast in DCI; 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI and G-CS-RNTI for multicast.. |
| [11] | Qualcomm Incorporated | 33. NR_MBS; 33-6-1a; DL priority of multicast ACK/NACK-based feedback for dynamically cheduled multicast; - Support of priority configured for multicast ACK/NACK-based HARQ-ACK feedback of dynamically cheduled multicast Notes: - Two priority indexes are introduced for multicast, with index 0 meaning low priority and index 1 meaning high priority. - The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback.; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-6-1b; DL priority of multicast NACK-only-based feedback for dynamically cheduled multicast; - Support of priority configured for multicast NACK-only-based HARQ-ACK feedback of dynamically cheduled multicast Notes: - Two priority indexes are introduced for multicast, with index 0 meaning low priority and index 1 meaning high priority. - The priority of multicast for NACK-only based feedback is the same as the priority of unicast for the same priority index of HARQ-ACK.; 33-4; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-6-1c; DL priority indication for dynamically scheduled multicast in DCI; - Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast.; 33-2a, 33-6-1a or 33-6-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-6-1d; DL priority of multicast ACK/NACK-based feedback SPS multicast; 1. Support of priority configured for multicast ACK/NACK-based HARQ-ACK feedback of SPS multicast; 33-5-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-6-1e; DL priority of multicast NACK-only-based feedback SPS multicast; 1. Support of priority configured for multicast NACK-only-based HARQ-ACK feedback of SPS multicast; 33-5-1d; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-6-1f; DL priority indication for SPS multicast with PDCCH scheduling; 1. Support of priority indicator field configured in DCI format 1_1 with CRC scrambled with G-CS-RNTI for SPS multicast.; 33-5-1a, 33-6-1d or 33-6-1e; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-6-2; Two ACK/NACK-based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with: different priorities for multicast or for unicast and multicast at a UE.; 1. Supports of two ACK/NACK-based HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE.; 33-2b, 33-6-1c; Yes; Per FSPC; No; No; Optional with capability signalling. 33. NR_MBS; 33-6-3; More than one PUCCH for ACK/NACK-based HARQ-ACK transmission for multicast or for unicast and multicast within a slot; 1. Support of two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot.; 33-6-2; Yes; Per FSPC; No; No; Optional with capability signalling. |
| [12] | MediaTek Inc. | 33-6-3: More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot In the previous RAN1 meeting, up to 2 PUCCH-Config configurations was agreed for multicast PUCCH as copied following. Agreement: For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback, - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively. - FFS other configurations. However, the name for FG 33-6-3 use the wording “more than one PUCCH”, it may cause the confusion. Thus, we prefer to change the name for better understanding. Proposal 10: For FG 33-6-3, the name should be changed as “Up to 2 PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot”. |

Discussion

[FL1] High priority question 8-1:

- Companies are encouraged to provide views on whether to separate following capabilities from FG 33-6-1

  - Support of DL priority configuration (not indication) for multicast

    - Whether to separate ACK/NACK-based HARQ-ACK feedback and NACK-only-based HARQ-ACK feedback

    - Whether to separate dynamically scheduled multicast and SPS multicast

  - Support of DL priority indication for multicast

    - Whether to separate dynamically scheduled multicast and SPS multicast PDCCH activation

| Company | Comment |
| --- | --- |
| Vivo | Not quite clear about the motivation to separate the DL priority configuration (not indication), further elaboration is quite appreciated. |
| MediaTek | Share the similar view with vivo, more clarification is needed. |
| ZTE | The detailed multiplexing handling for ACK/NACK based and NACK-only based for same/different priorities are under discussion in RAN1 now in AI8.12.2. The detailed UE behavior is not clear yet, we would propose to postpone this FG discussion a little bit. |
| Nokia, NSB | The need for separation is not clear. |
| Qualcomm | The multicast feedback is optional. The DL priority for multicast feedback is also optional, which means UE may only support multicast feedback without low and high priority differentiation (low priority by default). |
| FL2 | The motivation is provided from proponent (Qualcomm). Companies are invited to provide further comment, if any. |
| Huawei, HiSilicon | DL priority indication can be optional. However, at least other component should be added together this DL priority indication for which FG33-6-2 can be considered. |
| NTT DOCOMO | We don’t see the need to have separate FG. |
| OPPO | Do not see the clear motivation to separate the FG. |
| FL3 | Companies are invited to provide further comment, if any. |
| ZTE | We don’t see the need to have separate FGs for the above at this stage. |
| MediaTek | Considering the motivation QC’s explanation, we are ok for separation. |
| FL4 | Some companies still don’t see the necessity. No additional proposal is made for now but companies are invited to provide further comment, if any. |

[FL1] High priority question 8-2:

- Companies are encouraged to provide views on whether to merge FG 33-6-3 into FG 33-6-2

| Company | Comment |
| --- | --- |
| vivo | We prefer not merge them. |
| MediaTek | No need to merge them. |
| ZTE | The detailed multiplexing handling for ACK/NACK based and NACK-only based for same/different priorities are under discussion in RAN1 now in AI8.12.2. The detailed UE behavior is not clear yet, we would propose to postpone this FG discussion a little bit. |
| FL2 | There seems no strong request to merge the FGs. Therefore, following proposal is made to confirm FGs 33-6-2/6-3 are necessary. [FL2] High priority proposal 8-2: - FG 33-6-2 is kept as “Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE” as follows - FG 33-6-2 is kept as “More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot” as follows 33. NR_MBS; 33-6-2; Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE.; 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE.; 33-6-1; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-6-3; More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot; 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot.; 33-6-1, 33-6-2; Yes; Per UE; No; No; Optional with capability signalling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |
| Huawei, HiSilicon | We think either one of them should be merged into the capability of DL priority indication. |
| NTT DOCOMO | No need to merge them. |
| OPPO | Do not see the need to merge them. |
| FL3 | Following was agreed at the GTW session on Nov. 16. Agreement - FG 33-6-2 is kept as “Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE” as follows - FG 33-6-3 is kept as “More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot” as follows 33. NR_MBS; 33-6-2; Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE.; 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE.; 33-6-1; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-6-3; More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot; 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot.; 33-6-1, 33-6-2; Yes; Per UE; No; No; Optional with capability signalling. Note that any contents highlighted in yellow mean FFS and to be discussed further. |

Medium priority question 8-3:

- Companies are encouraged to provide views on whether the type of FGs 33-6-1 to 33-6-3 should be per UE or per FSPC

  - FGs 33-6-1

    - Per UE: Intel

    - Per FSPC: Qualcomm

  - FGs 33-6-2

    - Per FSPC: Qualcomm

  - FGs 33-6-3

    - Per FSPC: Qualcomm

| Company | Comment |
| --- | --- |
| Nokia, NSB | Per UE, FR1/2 differentiation |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. We prefer per FSPC for multicast in Rel17 NR MBS. |

Low priority proposal 8-4:

- Name of FG 33-6-1 is revised as “HARQ/ACK DL priority indication for multicast in DCI”

- Components of FG 33-6-1 are revised as

  - Component 1: Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI and G-CS-RNTI for multicast

- Name of FG 33-6-2 is revised as “Two ACK/NACK-based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with: different priorities for multicast or for unicast and multicast at a UE”

- Components of FG 33-6-2 are revised as

  - Component 1: Supports of two ACK/NACK-based HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE.

- Name of FG 33-6-3 is revised as “More than one Up to 2 PUCCH for ACK/NACK-based HARQ-ACK transmission for multicast or for unicast and multicast within a slot”

- Components of FG 33-6-3 are revised as

  - Component 1: Supports of two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot.

| Company | Comment |
| --- | --- |

Low priority question 8-5:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 33-6-1 to 33-6-3 which do not have capability eparateg impacts

| Company | Comment |
| --- | --- |

- 33-7: Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback

In [1], FG 33-7 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#106bis-e meeting.

| [4] | vivo | For NACK-only based HARQ-ACK feedback, there is no consensus to support enabling/disabling HARQ-ACK feedback based on DCI and the motivation to support it is not clear. Thus, ‘ACK/NACK based’ should be kept in 33-7. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Mandatory/Optional. 33. NR_MBS; 33-7; Supporting group-common DCI indicating the enabling/disabling ACK/NACK basedHARQ-ACK feedback; 1. Supports the function of group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback; 33-2; Yes; Optional with capability signalling. |
| --- | --- | --- |
| [8] | Intel Corporation | - FG 33-7: - NACK-only feedback should also be included in this FG |
| [11] | Qualcomm Incorporated | 33. NR_MBS; 33-7; Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback for dynamically scheduled group-common PDSCH; 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback for dynamically scheduled group-common PDSCH; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |

Discussion

Medium priority question 9-1:

- Companies are encouraged to provide views on whether NACK-only based feedback should also be included in FG 33-7.

| Company | Comment |
| --- | --- |
| ZTE | This depends on the ongoing discussion in AI 8.12.2. |
| Nokia, NSB | This depends on outcome of the previous discussions in this document. |

Medium priority question 9-2:

- Companies are encouraged to provide views on whether the type of FG 33-7 should be per UE or per FSPC

  - Per FSPC: Qualcomm

| Company | Comment |
| --- | --- |

Low priority proposal 9-3:

- Name of FG 33-7 is revised as “Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback for dynamically scheduled group-common PDSCH”

- Components of FG 33-7 are revised as

  - Component 1: Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback for dynamically scheduled group-common PDSCH

| Company | Comment |
| --- | --- |

Low priority question 9-4:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-7

| Company | Comment |
| --- | --- |

Low priority question 9-5:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 33-7 which do not have capability eparateg impacts

| Company | Comment |
| --- | --- |

- Other FGs

This section discusses other FGs which are not included in [1].

Following feedbacks are provided in contributions for the RAN1#107-e meeting.

| [2] | Huawei, HiSilicon, CBN | Regarding Type-1 codebook generation for TDM-ed unicast and multicast, the following agreement was achieved with supporting Alt 1 as a UE capability. Agreement: For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions can be configured between the following alternatives from the previous agreement: - Alt 1: - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast. - Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets. - Support of Alt. 1 is a UE capability Such UE capability is needed and can be added as FG 33-8 “Type-1 codebook generation based on k1 values in the intersection of K1 set for unicast and K1 set for multicast” with FG 33-2 as pre-requisite. This capability can be per UE and optional with capability signalling. Proposal 5: Add a new FG 33-8 “Type-1 codebook generation based on k1 values in the intersection of K1 set for unicast and K1 set for multicast” with FG 33-2 as pre-requisite. This capability can be per UE and optional with capability signalling. |
| --- | --- | --- |
| [5] | Spreadtrum Communications | New FGs: Last meeting, we have the following agreement: Agreement: For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions can be configured between the following alternatives from the previous agreement: - Alt 1: - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast. - Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets. - Support of Alt. 1 is a UE capability Thus, the following FG is preferred to be added to reflect the agreement. Proposal 8: Suggest to add one new FG to reflect the following. - For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions as follows - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast 33. NR_MBS; 33-8; Determining PDSCH reception candidate occasions, for TDM-ed unicast and multicast , and for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource; - Supports to determine PDSCH reception candidate occasions as below, for TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast; 33-3-3, 33-3-3-1; Yes; Per UE; No; No; Optional with capability signalling. |
| [11] | Qualcomm Incorporated | 33. NR_MBS; 33-8-1; PUCCH resource configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast; 1. Support of a PUCCH-Config for multicast ACK/NACK-based HARQ-ACK feedback, separate from that of unicast configurations; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-8-2; Up to 2 PUCCH resources configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast; 1. Support of a PUCCH-ConfigurationList for multicast ACK/NACK-based HARQ-ACK feedback, separate from that of unicast configurations; 33-2b, 33-6-1a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-8-3; PUCCH resource configuration for NACK-only-based multicast feedback for dynamically scheduled multicast; 1. Support of a PUCCH-Config for multicast NACK-only-based HARQ-ACK feedback, separate from that of multicast ACK/NACK-based configurations if configured; 33-4; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-8-4; Up to 2 PUCCH resources configuration for NACK-only-based multicast feedback for dynamically scheduled multicast; 1. Support of a eparate PUCCH-ConfigurationList for multicast NACK-only-based HARQ-ACK feedback, separate from that of multicast ACK/NACK-based configurations if configured; 33-4, 33-6-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-8-5; PUCCH resource configuration for ACK/NACK-based multicast feedback for SPS GC-PDSCH; 1. Support of a SPS-PUCCH-AN-List for multicast ACK/NACK-based HARQ-ACK feedback of all SPS configuration(s), separate from that of SPS unicast configurations; 33-5-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |

Discussion

[FL1] High priority question 10-1:

- Companies are encouraged to provide views on whether to introduce a new FG for type-1 codebook generation based on k1 values in the intersection of K_1 set for unicast and K_1 set for multicast

| Company | Comment |
| --- | --- |
| FL2 | Proponent is encouraged to provide the motivation to introduce the FG |
| Spreadtrum | The motivation is below: - We have the agreement, and it should be reflected in the UE capability. - As the agreement shows that, both Alt 1 and Alt 2 can support PDSCH reception candidate occasions determination for TDMed multiplexing for type-1 codebook. Alt 2 is the basic operation, and Alt 1 is optional. Thus, one separated new FG to capture Alt 1 is preferred |
| FL3 | Companies are encouraged to provide comments |
| Huawei, HiSilicon | A capability is needed to reflect the agreement . However, how the capability signaling is reported can be further discussed. In our understanding, it could be a separate UE capability or the same capability of supporting Type-1 codebook. |
| FL4 | Companies are encouraged to provide comments |
| vivo | Support adding the capability and it can be merged with TDM-ed Type-1 HARQ-ACK codebook for multicast |
| FL5 | Since this question may be related to proposal 5-1, this question can be further discuss after some progress is made in proposal 5-1 |

[FL1] High priority question 10-2:

- Companies are encouraged to provide views on whether to introduce new FGs for PUCCH resource configuration as follows:

  - FG for PUCCH resource configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast

  - FG for up to 2 PUCCH resources configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast

  - FG for PUCCH resource configuration for NACK-only-based multicast feedback for dynamically scheduled multicast

  - FG for up to 2 PUCCH resources configuration for NACK-only-based multicast feedback for dynamically scheduled multicast

  - FG for PUCCH resource configuration for ACK/NACK-based multicast feedback for SPS GC-PDSCH

| Company | Comment |
| --- | --- |
| FL2 | Proponent is encouraged to provide the motivation to introduce the FG |
| FL3 | Same as above |
| Huawei, HiSilicon | We don’t see the need to have such separate UE capability. What Qualcomm cited are the agreements of configuration. We assume if UE supports ACK/NACK based feedback for multicast and should support as well the resources configured for this feedback. Same comments for NACK-only and SPS multicast. |
| FL4 | Companies are encouraged to provide comments |
| vivo | In our understanding, PUCCH resource configuration doesn’t involve UE capability. |
| FL5 | Proponent has not provided the motivation to introduce the FG. No further comment is necessary in this meeting. |

- Conclusions

Following agreements were made in this RAN1 meeting.

Agreement

- FG 33-6-2 is kept as “Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE” as follows

- FG 33-6-3 is kept as “More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot” as follows

| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Agreement

- FG 33-2 is separated as following FGs

  - FG 33-2: current FG 33-2

    - FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback as a component of FG 33-2

  - FG 33-2b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

  - FFS whether to separate the capability for HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast

  - FFS whether to separate the capability for support of PTP retransmission for multicast

  - FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast

| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback FFS whether to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast |  | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Agreement

- Down select one of the followings on how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3

  - Option 1:

    - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast

    - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast

    - add an FG for Type-2 HARQ-ACK codebook for multicast

  - Option 2:

    - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast

    - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast

  - Option 3:

    - Add the capability for Type-2 HARQ-ACK codebook for multicast as a component in FG 33-2

    - add an FG for FDM-ed Type-1 HARQ-ACK codebooks for multicast

    - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast

References

[1]	R1-2110587	Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2110780	Rel-17 UE features for NR MBS	Huawei, HiSilicon, CBN

[3]	R1-2110928	Discussion on Rel-17 UE features for NR MBS	ZTE

[4]	R1-2111059	Discussion on UE features for MBS	vivo

[5]	R1-2111120	UE features for R17 NR MBS	Spreadtrum Communications

[6]	R1-2111162	On UE features for NR MBS	Nokia, Nokia Shanghai Bell

[7]	R1-2111306	Discussion on UE features for NR MBS	OPPO

[8]	R1-2111535	Discussion on UE Features for NR MBS	Intel Corporation

[9]	R1-2111779	UE features for NR MBS	Samsung

[10]	R1-2111914	Views on Rel-17 MBS UE features	Apple

[11]	R1-2112257	UE features for MBS	Qualcomm Incorporated

[12]	R1-2112315	Views on UE features for NR MBS	MediaTek Inc.

[13]	R1-2112345	views on NR MBS UE features	Ericsson