3GPP TSG RAN WG1 Meeting #107-e	R1-2112586

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.1.4

Source:	Huawei, HiSilicon

Title:	(Moderator) Summary of CSI enhancements for MTRP and FDD (Round 1)

Document for:	Discussion and Decision

# Introduction

# Summary of CSI enhancement for Rel-17 port selection codebook

Proposal 1: The window size is min(N,N3).

- Note: the UCI payload of i1,6 is ceiling(log2(N-1)) bits regardless of values of N3

Proposal 2: Support to report Port indicator, SCI, and FD indicator in Group 0

- FFS (to be concluded in RAN1 107): Report bitmap in Group 0 or Group 1 without bitmap partition

- FFS (to be concluded in RAN1 107): whether/how to deal with coefficients partition issue when ceil(KNZ/2-v) < 0

- The remaining indicators (except above FFS) are mapped to reporting Groups 0, 1, and 2 same as Rel-16 eType-II PS codebook.

- Note: It is RAN1 common understanding that when UCI omission occurs for Rel-17 PS codebook, the associated CQI does not have to be recalculated conditioned on the PMI after omission.

# Summary of CSI enhancement for Multi-TRP

Proposal 5: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting:

- Two CBSRs can be configured per CodebookConfig, whereas one CBSR is applied to one CMR group in a CMR resource set respectively, i.e. per TRP.

Proposal 7: To confirm the order of UCI payload construction for reported CSIs, down-select one from the following:

- Support Alt 1: modify priority equation, e.g. Section 5.2.5 in 38.214 by introducing priority index corresponding to single-TRP or NCJT measurement hypothesis type

  - It also implies that one CSI reporting setting for NCJT measurement reporting can contain multiple CSI reports each of which corresponds to one single-TRP or NCJT measurement hypothesis

- Support Alt2+Alt 4:

  - Alt 4: modify mapping order of CSI fields of one CSI report, e.g. Table 6.3.2.1.2-3/4/5 in 38.212

    - i.e. introducing mapping order of CSI fields in the order of MTRP CSI, the first TRP CSI, and the second TRP CSI.

  - Alt 2: modifying the table of priority reporting levels for Part 2 CSI for prioritized single-TRP or NCJT measurement hypothesis type, e.g. Table 5.2.3-1 in 38.214

    - introducing the priority reporting level for Part 2 CSI associated with single-TRP or NCJT measurement hypothesis type in the order of:

      - Alt 2-1:

        - Part 2 wideband CSI for CSI reports 1 to N;

        - Part 2 subband CSI for report 1 to N: in the order of MTRP CSI, the first TRP CSI, and the second TRP CSI within each report

      - Alt 2-2:

        - Part 2 wideband CSI for CSI reports 1 to N;

        - Part 2 subband CSI for report 1 to N: even subbands associated with all measurement hypotheses and then odd subbands associated with all measurement hypotheses within each report

      - Down-select either Alt 2-1 or Alt 2-2 in RAN1 107, if Alt 2+Alt 4 is agreed

  - It also implies that one CSI reporting setting for NCJT measurement reporting contains single CSI report which corresponds multiple single-TRP and/or NCJT measurement hypotheses.

| Company | Comments |
| --- | --- |
| Mod | - Alt 1: Motorola, Intel, Qualcomm, MediaTek, DoCoMo - Alt 2+Alt 4: Vivo, Nokia, Apple, Spreadtrum, CMCC - Alt 4 only as RAN1 106bis-e conclusion(almost): ZTE, Oppo, DoCoMo |