3GPP TSG-RAN WG1 Meeting #107-e 	R1-2112963

e-Meeting, November 11–19, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0094 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR sidelink enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2021-12-2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Inclusion of NR sidelink enhancement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of NR sidelink enhancement. Add SCI format 2-C for the decoding of PSSCH and providing inter-UE coordination information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR sidelink enhancement in Rel-17 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.1, 8.4.1.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.213, TS 38. 214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.3.1.1	SCI format 1-A

< Unchanged parts are omitted >

Table 8.3.1.1-1: 2nd-stage SCI formats

| Value of 2nd-stage SCI format field | 2nd-stage SCI format |
| --- | --- |
| 00 | SCI format 2-A |
| 01 | SCI format 2-B |
| 10 | SCI format 2-C |
| 11 | Reserved |

< Unchanged parts are omitted >

8.4.1.3	SCI format 2-C

SCI format 2-C is used for the decoding of PSSCH and providing inter-UE coordination information.

The following information is transmitted by means of the SCI format 2-C:

-	Resource combination(s) – x bits as defined in Clause 8.1.5A of [6, TS 38.214].

-	First resource location(s) – x bits as defined in Clause 8.1.5A of [6, TS 38.214].

< Unchanged parts are omitted >