3GPP TSG RAN WG1 Meeting #107-e 	R1-211xxxx

e-Meeting, November 11th – 19th, 2021

Agenda Item:	7.2.10

Source:	Moderator (Huawei)

Title:	Discussion on dormancy indication Case-1

Document for:	Discussion and Decision

Introduction

This document aims to address the dormancy indication issue raised in [1] per the following.

[107-e-NR-MRDC-CA-01] Email discussion/approval on the issue raised in R1-2111934 using the two alternatives provided as the starting point, until November 17 – Yi Wang (Huawei)

It is suggested to have companies’ initial round of input by the following

- check point: November 15

Discussion

During the preparation phase a majority preference made in Preparation is to take Alt 1 in [1] while there are different preference made via emails considering the following agreements for Case 2 dormancy indication.

Agreement: HARQ-ACK timeline of the Case 2 SCell dormancy indication DCI is defined by adding X symbols to the SPS release PDCCH HARQ-ACK timeline (i.e., X+N symbols) - X = 4, 4, 5, 6 for capability #1 for μ =0,1,2,3 - X = 2, 2, 4 for capability #2 for μ =0,1,2

The above agreement applies to Case 2 SCell dormancy indication DCI without data scheduling, such that the PUCCH timeline is modified to be different from non-dormancy case due to potential interruption time. Now for Case 1 SCell dormancy indication, which was designed to be with data scheduling however was also extended to the cases of e.g. SPS release indication without scheduling data in the specification stage, it is unclear whether the same issue (i.e. interruption) may occur and needs to be accounted for, or the current specification may be maintained without changes.

Moderator assumes the issue has been well understood given previous discussion made for Case 2 and in the CR stage, some alternatives can be directly proposed as below. Draft CRs can be provided after some level of consensus for the proposals. The proposals are generated considering the majority preference is to maintain the function in spec therefore there seems to be a need to modify the timeline, while it is possible to take different approaches. Please see relevant proposal(s) and/or discussion point(s).

## Discussion Point 1

- In line with the function as in current specifications, which one do you prefer or accept?

  - Alt 1-a: no change to the current specifications (in which case a conclusion is possible)

  - Alt 1-b:

    - modify the HARQ-ACK timeline for Case 1 dormancy indication DCI indicating SPS release, using same values per SCS as that for Case 2 SCell dormancy indication, i.e.

      - HARQ-ACK timeline of the Case 1 SCell dormancy indication DCI is defined by adding X symbols to the SPS release PDCCH HARQ-ACK timeline (i.e., X+N symbols)

        - X = 4, 4, 5, 6 for capability #1 for μ =0,1,2,3

        - X = 2, 2, 4 for capability #2 for μ =0,1,2

Moderator Note: For DCI indicating release of UL grant Type 2 PUSCH or deactivating semi-persistent CSI report(s) on PUSCH, there is no HARQ-ACK on PUCCH so there is no need for change. An example TP to TS 38.213 in accordance to Alt 1-b can be as below. Comments for the example TP are also invited if you support/can accept Alt 1-b.

### 38.213 TP1

---------------------------------------- 38.213 TP1 start ----------------------------------------- ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ==== Unchanged parts ==== A UE is expected to provide HARQ-ACK information in response to a detection of a DCI format 1_1 indicating SCell dormancy or in response to a detection of a DCI format 1_1 that indicates SPS PDSCH release and includes a SCell dormancy indication field after N symbols from the last symbol of a PDCCH providing the DCI format 1_1. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the DCI format 1_1, N=7 for μ=0, N=7.5 for μ=1, and N=15 for μ=2; otherwise, N=14 for μ=0, N=16 for μ=1, N=27 for μ=2, and N=31 for μ=3, where μ is the smallest SCS configuration between the SCS configuration of the PDCCH providing the DCI format 1_1 and the SCS configuration of a PUCCH with the HARQ-ACK information in response to the detection of the DCI format 1_1. ==== Unchanged parts ==== ----------------------------------------------- TP1 end ------------------------------------------------

| Company | Alternatives | Comments |
| --- | --- | --- |
| Template | Prefer Alt 1-a Accept Alt 1-b |  |

## Discussion point 2

- If you consider the function can be (partially) removed, which one of the below do you prefer or can be acceptable?

  - Alt 2-a: do not support Case 1 dormancy indication for the DCI for release of semi-persistent PDSCH/PUSCH (including both CS-RNTI and SP-CSI-RNTI), in line with previous RAN1 agreements that Case 1 only refers to cases without scheduling data

    - Note: specification changes seem to be also needed for TS 38.212 in addition to the one submitted for TS 38.213 in [1]. TP examples (TP2, TP3) can be found in Appendix. If you prefer this Alt, comments to the TPs are also invited.

  - Alt 2-b: do not support Case 1 dormancy indication for the DCI scrambled with CS-RNTI

    - Note: this was proposed by one or two companies indicating the same as Case-2 SCell dormancy, by removing the function for DCI indicating SPS/configured grant activation and release. However, a following up question can be whether SP-CSI-RNTI is kept or not, as Case-2 SCell dormancy indication applies to neither. Specification changes seem to be needed for TS 38.212 and TS 38.213. More discussion are needed and no TP available at hand - can be provided later.

  - Alt 2-c: others, please provide details in the ‘Comments’ column of the below table.

| Company | Alternatives | Comments |
| --- | --- | --- |

Conclusion

TBD

References

- R1-2111934, Discussion on Case-1 dormancy operation with data scheduling.

Appendix

## TP2 and TP3 for Alt 2-a

---------------------------------------- 38.213 TP2 start ----------------------------------------- ## 10.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells ==== Unchanged parts ==== If a UE is provided search space sets to monitor PDCCH for detection of DCI format 0_1 and DCI format 1_1 and if one or both of DCI format 0_1 and DCI format 1_1 include a SCell dormancy indication field, - the SCell dormancy indication field is a bitmap with size equal to a number of groups of configured SCells, provided by dormancyGroupWithinActiveTime, - each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured Scells - if the UE detects a DCI format 0_1 or a DCI format 1_1 that does not include a carrier indicator field, or detects a DCI format 0_1 or DCI format 1_1 that includes a carrier indicator field with value equal to 0, and if a DCI format 0_1 is detected and the DCI format 0_1 does not indicate UL grant Type 2 release or deactivate semi-persistent CSI report(s) on PUSCH, or if a DCI format 1_1 is detected and the DCI format 1_1 does not indicate SPS PDSCH release - a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells - a '1' value for a bit of the bitmap indicates - an active DL BWP, provided by firstWithinActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP - a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP - the UE sets the active DL BWP to the indicated active DL BWP ==== Unchanged parts ==== ----------------------------------------------- TP2 end ------------------------------------------------

---------------------------------------- 38.212 TP3 start ----------------------------------------- ##### 7.3.1.1.2 Format 0_1 ==== Unchanged parts ==== The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI: ==== Unchanged parts ==== - SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell. The field is reserved if this format indicates UL grant Type 2 release according to Clause 10.2 of [5, TS 38.213] or deactivates semi-persistent CSI report(s) on PUSCH according to Clause 5.2.1.5.2 of [6, TS 38.214]. ==== Unchanged parts ==== ##### 7.3.1.2.2 Format 1_1 ==== Unchanged parts ==== The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: ==== Unchanged parts ==== - SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell. The field is reserved if this format indicates SPS PDSCH release according to Clause 10.2 of [5, TS 38.213]. ----------------------------------------------- TP3 end ------------------------------------------------