3GPP TSG-RAN WG1 Meeting #107-e	R1-211xxxx

# e-Meeting, November 11th – 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | XXX | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Changes of channel access procedure in TS 37.213 according to MIIT regulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), Lenovo, Nokia, Ericsson, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed e?planations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Minstry of Industry and Information Technology (MIIT) in China has already set the final regulatory rules for operating in the 2.4 GHz band and the 5 GHz band. It is observed that the sensing slot duration for semi-static channel access in 3GPP can not meet the requirement of MIIT regulations for operating in such bands in China. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an optional value of sensing slot duration Tsl = 16us in section 4.3 to meet the minimum idle channel assessment of at least 16us in China for FBE device. |  |  |  |  |  |  |  |  |
| T |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The channel access procedure defined for the semi-static channel access mode in current specifications does not meet the requirements in China. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.3	Channel access procedures for semi-static channel occupancy

Channel access procedures based on semi-static channel occupancy as described in this Clause, are intended for environments where the absence of other technologies is guaranteed e.g., by level of regulations, private premises policies, etc. If a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated by the gNB every Tx within every two consecutive radio frames, starting from the even indexed radio frame at i∙Txx∙Tx with a maximum channel occupancy time Ty=0.95Tx, where Tx= period Tx=Periodin ms, is a higher layer parameter provided in SemiStaticChannelAccessConfig and i∈0,1,…,20Tx-1. For determining a Channel Occupancy Time based on semi-static channel access procedures, duration of any transmission gap within Ty is counted in the channel occupancy time.

In the following procedures in this clause, when a gNB or UE performs sensing for evaluating a channel availability, the sensing is performed at least during a sensing slot duration Tsl=9us, unless longer sensing duration is required (e.g. by level of regulation), in which case sensing is performed within a duration of Tsl=16us. When sensing is performed within a duration of  Tsl=16us,  the channel is considered to be idle if the channel is sensed to be idle for total of at least 5us with at least 4us of sensing occurring in the last 9us time interval in the sensing duration.  The corresponding  XThresh adjustment for performing sensing by a gNB or a UE is described in clauses 4.1.5 and 4.2.3, respectively.

A channel occupancy initiated by a gNB and shared with UE(s) satisfies the following:

-	The gNB shall transmit a DL transmission burst starting at the beginning of the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9us. If the channel is sensed to be busy, the gNB shall not perform any transmission during the current period.

-	The gNB may transmit a DL transmission burst(s) within the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9us if the gap between the DL transmission burst(s) and any previous transmission burst is more than 16us.

-	The gNB may transmit DL transmission burst(s) after UL transmission burst(s) within the channel occupancy time without sensing the channel if the gap between the DL and UL transmission bursts is at most 16us.

-	A UE may transmit UL transmission burst(s) after detection of a DL transmission burst(s) within the channel occupancy time as follows:

-	If the gap between the UL and DL transmission bursts is at most 16us,  the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time without sensing the channel.

-	If the gap between the UL and DL transmission bursts is more than 16us,  the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time after sensing the channel to be idle for at least a sensing slot duration Tsl=9us within a 25us interval ending immediately before transmission.

-	A UE may be indicated by the gNB to transmit UL transmission burst(s) within the channel occupancy time without sensing the channel or after sensing the channel to be idle for at least a sensing slot duration Tsl=9us within a 25us interval ending immediately before transmission.

-	The gNB and UEs shall not transmit any transmissions in a set of consecutive symbols for a duration of at least Tz=max0.05Tx , 100us before the start of the next period.

If a UE fails to access the channel(s) prior to an intended UL transmission to a gNB, Layer 1 notifies higher layers about the channel access failure.