3GPP TSG RAN WG1 #107-e            	                               R1-21nnnnn

e-Meeting, November 11th – 19th, 2021

Source:	Moderator (CATT)

Title:	Summary on email discussion [107-e-NR-Pos-05] for the UE report of parameter NR-TimeStamp

Agenda item:	7.2.8

Document for:	Discussion and Decision

Introduction

This contribution summarizes the discussions and outcomes of email discussion [107-e-NR-Pos-05] triggered by the following Chair’s decision:

[107-e-NR-Pos-05] Email discussion/approval on the UE report of parameter NR-TimeStamp (Aspect #5) until November 17 – Xiaotao (CATT)

Discussion

UE report of parameter NR-TimeStamp

Background

In the draft CR for TS 38.214(R1-2111220) [1], it is proposed that there is misalignment between TS 37.355 [2] and TS 38.214 [3] on whether the parameters ‘dl-PRS-ID-r16’ and ‘NR-TimeStamp’ have to be reported in the UE measurement report.

As shown in the following descriptions in TS 37.355 (section 6.4.3), the TRP ID, i.e. ‘dl-PRS-ID-r16’, has to be reported in the NR-TimeStamp-r16.

-- ASN1START NR-TimeStamp-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, -- Need ON nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON nr-SFN-r16 INTEGER (0..1023), nr-Slot-r16 CHOICE { scs15-r16 INTEGER (0..9), scs30-r16 INTEGER (0..19), scs60-r16 INTEGER (0..39), scs120-r16 INTEGER (0..79) }, ... } -- ASN1STOP

In addition, according to the descriptions in TS 37.355, for DL-TDOA, DL-AoD and Multi-RTT positioning, the parameter ‘NR-TimeStamp-r16’ also has to be reported. For example, in section 6.5.11.4 of TS37.355, the measurements for DL-AoD are shown as follows:

-- ASN1START NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE { nr-DL-AoD-MeasList-r16 NR-DL-AoD-MeasList-r16, ... } NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-AoD-MeasElement-r16 NR-DL-AoD-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126), nr-DL-PRS-RxBeamIndex-r16 INTEGER (1..8) OPTIONAL, nr-DL-AoD-AdditionalMeasurements-r16 NR-DL-AoD-AdditionalMeasurements-r16 OPTIONAL, ... } NR-DL-AoD-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..7)) OF NR-DL-AoD-AdditionalMeasurementElement-r16 NR-DL-AoD-AdditionalMeasurementElement-r16 ::= SEQUENCE { nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-DL-PRS-RSRP-ResultDiff-r16 INTEGER (0..30), nr-DL-PRS-RxBeamIndex-r16 INTEGER (1..8) OPTIONAL, ... } -- ASN1STOP

However, as described in TS 38.214 in the following paragraph in section 5.1.6.5., either ‘dl-PRS-ID-r16’ or ‘NR-TimeStamp-r16’ seems to be an optional parameter in the measurement report.

#### 5.1.6.5 PRS reception procedure [TS 38.214] < Uncorrelated parts are omitted > For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE can report an associated higher layer parameter nr-TimeStamp. The nr-TimeStamp can include the dl-PRS-ID, the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by nr-DL-PRS-ReferenceInfo. < Uncorrelated parts are omitted >

Therefore, there is misalignment between TS 37.355 and TS 38.214 on whether the parameters ‘dl-PRS-ID-r16’ and ‘NR-TimeStamp-r16’ have to be reported in the measurement report.

Issue 1: there is misalignment between TS 38.214 and TS 37.355.

- In TS 37.355, both ‘dl-PRS-ID-r16’ and ‘NR-TimeStamp-r16’ are mandated to be reported.

- In TS 38.214, either ‘dl-PRS-ID-r16’ or ‘NR-TimeStamp-r16’ is optional parameter in the measurement report.

Proposed change

Considering the usage of these parameters, we propose to remove the words ‘can’ in TS 38.214 in order to be aligned with TS 37.355. In this way, ‘NR-TimeStamp-r16’ and its associated parameter ‘dl-PRS-ID-r16’ have to be reported.

The following text proposal is prepared to address raised above issue:

#### 5.1.6.5 PRS reception procedure ### < Unchanged parts are omitted > For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE reports an associated higher layer parameter nr-TimeStamp. The nr-TimeStamp includes the dl-PRS-ID, the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by nr-DL-PRS-ReferenceInfo. ### < Unchanged parts are omitted >

Discussion Round #1

Companies are invited to provide views on the following question for the text proposal above.

### Question 1

- Which option do you prefer to handle the text proposal proposed above?

  - Alt.1 Agree to the draft CR.

  - Alt.2 Endorse the draft CR and include the change in the editor alignment CR.

  - Alt.3 The change is not needed.

| Company Name | Alt | Comments |
| --- | --- | --- |
| Nokia/NSB | Alt 3 | This change is non-essential and the current spec is clear in our view. If you read 214 it says the UE can report this IE. Then if a reader looks at LPP it is clear that the UE has to report this IE as part of the measurement report. We don’t see how the spec could be misinterpreted. |

Conclusion

TBD

Reference

- R1-2111220, “Draft CR on the parameter NR-TimeStamp for Rel-16 Positioning”, CATT.

- TS 37.355, “LTE Positioning Protocol (LPP) (Release 16)”, V16.6.0 (2021-09).

- TS 38.214, “NR; Physical layer procedures for data (Release 16)”, V16.7.0 (2021-09).