3GPP TSG RAN WG1 #107-e                                                        R1-21zzzzz

e-Meeting, November 11th – 19th, 2021

Source: 	Moderator (Intel Corporation)

Title: 	Summary of Preparation Phase for Rel.16 NR Positioning Maintenance [107-e-Prep-AI7.2.8]

Agenda item:	7.2.8

Document for: 	Discussion and Decision

# Introduction

This document provides overview of contributions submitted for R16 NR Positioning maintenance [1]-[8]. In addition, it provides feature lead recommendation and summary of the following RAN1 e-mail discussion assigned by RAN1 chair:

- [107-e-Prep-AI7.2.8] Preparation phase for Rel-16 NR Positioning maintenance: Alexey (Intel)

# Overview of Contributions

## Aspect #1: Correction to PRS QCL Type D

In [1], it is noticed that there is no qcl-Type field/IE in LPP to indicate the PRS-PRS Type D QCL for FR2, however it is referred in TS38.214 clause 5.1.6.5. The following CR is proposed to fix this

#### 5.1.6.5 PRS reception procedure ========================= Unchanged parts ========================= If the UE is configured with DL-PRS-QCL-Info and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are associated with the same dl-PRS-ID. If DL-PRS-QCL-Info is configured to the UE with QCL 'type-D' with a source DL PRS resource then the nr-DL-PRS-ResourceSetId and the nr-DL-PRS-ResourceId of the source DL PRS resource are expected to be indicated to the UE. ========================= Unchanged parts =========================

## Aspect #2: Correction to the Time Stamp

In [2], it is noticed that there is potential ambiguity on the time stamp for UE Rx – Tx time difference measurement. It is proposed to clarify whether the time stamp corresponds to the PRS reception time or positioning SRS transmission time. In addition, corrections to parameter name nr-DL-PRS-ExpectedRSTD-Uncertainty is proposed. Finally, the change of “DL RSTD, UE Rx-Tx time difference” to “DL RSTD or UE Rx-Tx time difference, respectively” is suggested as indicated in provided draft CR below:

#### 5.1.6.5 PRS reception procedure ========================= Unchanged parts ========================= The UE may be indicated by the network that DL PRS resource(s) can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter nr-DL-PRS-ReferenceInfo. The reference indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty. The UE expects the reference to be indicated whenever it is expected to receive the DL PRS. This reference provided by nr-DL-PRS-ReferenceInfo may include a dl-PRS-ID, a DL PRS resource set ID, and optionally a single DL PRS resource ID or a list of DL PRS resource IDs [17, TS 37.355]. The UE may use different DL PRS resources or a different DL PRS resource set to determine the reference for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference than indicated by the network, then it is expected to report the dl-PRS-ID, the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference. The UE may be configured to report quality metrics NR-TimingQuality corresponding to the DL RSTD and UE Rx-Tx time difference measurements which include the following fields: - timingQualityValue which provides the best estimate of the uncertainty of the measurement - timingQualityResolution which specifies the resolution levels used in the timingQualityValue field. The UE expects to be configured with higher layer parameter nr-DL-PRS-ExpectedRSTD, which defines the time difference with respect to the received DL subframe timing the UE is expected to receive DL PRS, and nr-DL-PRS-ExpectedRSTD-Uncertainty, which defines a search window around the nr-DL-PRS-ExpectedRSTD. For DL UE positioning measurement reporting in higher layer parameters NR-DL-TDOA-SignalMeasurementInformation or NR-Multi-RTT-SignalMeasurementInformation the UE can be configured to report the DL PRS resource ID(s) or the DL PRS resource set ID(s) associated with the DL PRS resource(s) or the DL PRS resource set(s) which are used in determining the UE measurements DL RSTD or UE Rx-Tx time difference, respectively. For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE can report an associated higher layer parameter nr-TimeStamp. The nr-TimeStamp can include the dl-PRS-ID, the SFN and the slot number for a subcarrier spacing corresponding to the reception time of the DL-PRS. These values correspond to the reference which is provided by nr-DL-PRS-ReferenceInfo. ========================= Unchanged parts =========================

## Aspect #3: Draft CR on QCL information for a DL PRS resource

In [3], it is noticed that according to current descriptions in TS 38.214, QCL information for a DL PRS resource can be an SSB from a serving cell or non-serving cell. The proposed correction clarifies that an SSB as the QCL information for a DL PRS resource should be from a same serving cell or same non-serving cell as the DL PRS resource.

Specification has ambiguity on which SSB should be used as QCL information for a DL PRS resource.

---- Unchanged texts omitted ---- #### 5.1.6.5 PRS reception procedure <Unchanged parts are omitted> A DL PRS resource is defined by: - nr-DL-PRS-ResourceID determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within a DL PRS resource set. - dl-PRS-SequenceID is used to initialize cinit value used in pseudo random generator as described in Clause 7.4.1.7.2 of [4, TS 38.211] for generation of DL PRS sequence for a given DL PRS resource. - dl-PRS-CombSizeN-AndReOffset defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS 38.211]. - dl-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL PRS resource set slot offset. - dl-PRS-ResourceSymbolOffset determines the starting symbol of a slot configured with the DL PRS resource. - dl-PRS-QCL-Info defines any quasi co-location information of the DL PRS resource with other reference signals. The DL PRS may be configured with QCL 'typeD' with a DL PRS associated with the same dl-PRS-ID, or with rs-Type set to 'typeC', 'typeD', or 'typeC-plus-typeD' with a SS/PBCH Block from a same serving or same non-serving cell as the DL PRS resource. ---- Unchanged texts omitted ----

## Aspect #4: Correction on DL PRS Resource Time Gap

In [4], it is proposed to change the parameter ‘nr-DL-PRS-ResourceSetId’ to ‘nr-DL-PRS-ResourceID’, since it is noticed that there is a misalignment with RAN1 agreement:

Agreement (RAN1#98bis): - Parameter DL-PRS-ResourceRepetitionFactor is configured for a DL PRS Resource Set and controls how many times each DL-PRS Resource is repeated for a single instance of the DL-PRS Resource Set - Values: 1, 2, 4, 6, 8, 16, 32 - Parameter DL-PRS-ResourceTimeGap is configured for a DL-PRS Resource Set - DL-PRS-ResourceTimeGap indicates offset in units of slots between two repeated instances of a DL PRS Resource corresponding to the same DL-PRS Resource ID within a single instance of the DL PRS Resource Set - DL-PRS-ResourceTimeGap is provided only if DL-PRS-ResourceRepetitionFactor is configured and is greater than 1 - Values: 1, 2, 4, 8, 16, 32 - The time duration spanned by one DL PRS Resource set containing repeated DL PRS Resources should not exceed DL-PRS-Periodicity - Note: UE RX beam sweeping is up to UE implementation

The following correction was proposed in [4]:

5.1.6.5 PRS reception procedure < Unchanged parts are omitted > - dl-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceIDSetId within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap if dl-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within one resource set have the same value of dl-PRS-ResourceTimeGap. < Unchanged parts are omitted >

## Aspect #5: Misalignment b/w TS 37.355 and TS 38.214

The following misalignment between TS 37.355 and TS 38.214 is noticed in [5]:

- In TS 37.355 (section 6.4.3), the TRP ID, i.e., ‘dl-PRS-ID-r16’, has to be reported in the NR-TimeStamp. In addition, according to the descriptions in TS 37.355, for DL-TDOA, DL-AoD and Multi-RTT positioning, the parameter ‘NR-TimeStamp’ also has to be reported.

- In TS 38.214, either ‘dl-PRS-ID-r16’ or ‘NR-TimeStamp’ seems to be an optional parameter in the measurement report.

To address misalignment across specifications, it is proposed to remove the word ‘can’ in TS 38.214 as suggested in the CR below:

#### 5.1.6.5 PRS reception procedure ===================== Unchanged parts ===================== For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE reports an associated higher layer parameter nr-TimeStamp. The nr-TimeStamp includes the dl-PRS-ID, the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by nr-DL-PRS-ReferenceInfo. ===================== Unchanged parts =====================

## Aspect #6: Others

References [6]-[8], are not covered in this document as those are not available in Docs folder.

Discussion on Preparation Phase

From FL perspective, all aspects provided in submitted tdocs deserve clarifications and can be recommended for subsequent RAN1 e-mail discussions.

Companies are invited to provide their views on maintenance aspects to be further discussed by RAN1.

Comments from companies:

| Company Name | Comments |
| --- | --- |

Conclusions

In this document, we have provided overview of the contributions submitted to RAN1#107e for R16 NR positioning maintenance.

References

- R1-2110847	Correction to PRS QCL Type D	Huawei, HiSilicon

- R1-2110848	Correction to the time stamp	Huawei, HiSilicon

- R1-2110968	Draft CR on QCL information for a DL PRS resource	ZTE

- R1-2110989	Maintenance on Rel-16 NR positioning	vivo

- R1-2111220	Draft CR on the parameter NR-TimeStamp for Rel-16 Positioning	CATT

- R1-2112265	Remaining issues for Positioning	Qualcomm Incorporated

- R1-2112337	Maintenance for NR positioning support	Ericsson

- R1-2112415	Draft CR on PRS processing	Google Inc.