3GPP TSG RAN WG1 #107-e			R1-2112763

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.1.1

Source: 	Moderator (Samsung)

Title: 	Moderator summary#4 for multi-beam enhancement: ROUND 3

Document for:	Discussion and Decision

## CORESET framework

Summary:

Proposal 1.I: For Rel-17 unified TCI framework, on applying the indicated Rel-17 TCI state to PDCCH reception and the respective PDSCH reception, for intra-cell and inter-cell BM, support per CORESET determination as follows: - For any PDCCH reception on a CORESET [other than CORESET#0] that is associated with [at least or] [USS and/or CSS type 3] set(s) and the respective PDSCH reception, UE always applies the indicated Rel-17 TCI state. - For any PDCCH reception on [CORESET#0 or] a CORESET [(other than CORESET#0)] that is not associated with any [USS and/or CSS type 3] set and the respective PDSCH reception, whether or not UE to apply the indicated Rel-17 TCI state is determined per CORESET by RRC - Note: It was agreed that a UE can receive non-UE dedicated signal/channel only from the serving cell - Above applies only for intra-cell beam indication - [For inter-cell beam management (i.e. when at least one of the RRC-configured TCI states is associated with a PCI different from that of the serving cell) indication, a UE may expect that a CSS and a USS are not associated with a same CORESET] The bracketed texts will be discussed and concluded during maintenance FL Note: 3 open issues to finalize, companies’ views Working Assumption For Rel-17 unified TCI framework, on applying the indicated Rel-17 TCI state to PDCCH reception and the respective PDSCH reception, for intra-cell and inter-cell BM, support per CORESET determination as follows: - For any PDCCH reception on a CORESET [other than CORESET#0] that is associated with [at least or only] [USS and/or CSS type 3] set(s) and the respective PDSCH reception, UE always applies the indicated Rel-17 TCI state. - For any PDCCH reception on [CORESET#0 or] a CORESET [(other than CORESET#0)] that is not associated with any [USS and/or CSS type 3] set and the respective PDSCH reception, whether or not UE to apply the indicated Rel-17 TCI state is determined per CORESET by RRC - Note: It was agreed that a UE can receive non-UE dedicated signal/channel only from the serving cell - Above applies only for intra-cell beam indication - [For inter-cell beam indication, a UE may expect that a CSS and a USS are not associated with a same CORESET] CORESET#0: - Remove brackets (include): ZTE, Ericsson, NTT Docomo, Intel, vivo, Nokia/NSB - Remove text or keep brackets (FFS and address in maintenance): QC, Apple, MTK, Samsung, CMCC, Xiaomi, LG, OPPO, CATT FL assessment: No consensus on removing the brackets for now, continue discussion during maintenance USS and/or CSS Type 3: - Only USS: QC, Ericsson, CMCC, Samsung, Xiaomi, LG, CATT - USS and CSS Type 3: Apple, ZTE, NTT Docomo, Intel, vivo, OPPO, Nokia/NSB FL assessment: ‘USS only’ is default, no consensus on treating CSS Type 3 (non UE dedicated) similarly to USS. Support CORESET association with both CSS and USS: - For both intra- and inter-cell: Ericsson, NTT Docomo, Intel, Samsung (1st pref, NW implementation), ZTE, Nokia/NSB, CATT - Only for intra-cell: Samsung (2nd pref), CMCC, NTT Docomo (2nd pref), LG, OPPO - Not supported: QC, ZTE (except for CORESET#0), MTK, Lenovo/MotM, Apple, Xiaomi, vivo FL assessment: Almost equal support for intra+inter vs none. ‘Only intra’ is a good compromise

Table 1 Restructured CORESET-based framework proposal

Proposal 1.I’: For Rel-17 unified TCI framework, on applying the indicated Rel-17 TCI state to PDCCH reception and the respective PDSCH reception: For discussion purposes, define ‘CORESET A’ and ‘CORESET B’ as follows: - ‘CORESET A’: CORESETs associated with UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with: - USS - [CSS Type 3] - ‘CORESET B’: CORESETs associated with non-UE-dedicated (not associated with UE-dedicated) reception on PDCCH in a CC, comprising CORESETs in association with: - CSS [other than Type 3] - [CORESET#0] For intra-cell BM (when all of the RRC-configured TCI states are associated with the serving cell), support per CORESET determination as follows: - For any PDCCH reception on at least a ‘CORESET A’ and the respective PDSCH reception, UE always applies the indicated Rel-17 TCI state. - For any PDCCH reception on only a ‘CORESET B’ and the respective PDSCH reception, whether or not UE to apply the indicated Rel-17 TCI state is determined per CORESET by RRC - For intra-cell BM, a supported CORESET can be either ‘CORESET A’, ‘CORESET B’, or both For inter-cell BM (when at least one of the RRC-configured TCI states is associated with a PCI different from that of the serving cell), support per CORESET determination as follows: - For any PDCCH reception on only a CORESET A and the respective PDSCH reception, UE always applies the indicated Rel-17 TCI state. - For inter-cell BM, a supported CORESET can be either CORESET A or CORESET B (but not both)

Table 2 Additional inputs: proposal 1.I’

| Company | Input |
| --- | --- |
| Mod V0 | - Share your inputs on proposal 1.I’ |

## Other issues

### Issue 1 (Rel.17 unified TCI framework – note: for intra-cell beam management unless otherwise noted)

Table 3 Summary: issue 1

| # | Issue | Companies’ views |
| --- | --- | --- |
| 1.1 | Proposal 1.A.2: On Rel-17 unified TCI framework, for any SRS resource or resource set that does not share the same indicated Rel-17 TCI state(s) as dynamic-grant/configured-grant based PUSCH and all of dedicated PUCCH resources, but can be configured as a target signal of a Rel-17 UL or, if applicable, joint TCI (hence the Rel-17 UL or, if applicable, joint TCI state pool), Rel-17 mechanism(s) which reuse mechanisms similar to the Rel-15/16 spatial relation info update signaling/configuration design(s) are used to update/configure such SRS (s) with Rel-17 UL or, if applicable, joint TCI state(s). - Applies for both intra-cell and inter-cell beam indication - [In such a case, UE ignores the UL PC parameters associated with the UL or, if applicable, joint TCI state for SRS, and legacy Rel-15/16 UL PC parameter configuration/activation signaling is reused; otherwise, if SRS resource or resource set shares the same indicated Rel-17 TCI state(s) as dynamic-grant/configured-grant based PUSCH and all of dedicated PUCCH resources, UE does not expect legacy Rel-15/16 UL PC parameter configuration for SRS.] - The MAC-CE signaling for the Rel-17 mechanism(s) shall fully reuse, to the fullest possible extent, the MAC-CE for the Rel-15/16 spatial relation info update - Note: Strive, to the fullest possible extent, not to introduce any No new MAC-CE is introduced. The exact details are up to RAN2. - Note: A Rel-17 UE is not required to support both this feature and Rel-16 AP SRS SpatialRelationInfo update within the same band. FL Note: Discussed offline [1] | Support/fine (27): Sony, Nokia/NSB, Ericsson, Samsung, MTK, Fraunhofer IIS/HHI, CMCC, Futurewei, Intel, vivo, NEC, AT&T, NTT Docomo, QC, CATT, Xiaomi, TCL, Lenovo/MotM, Convida, Huawei, HiSi, ZTE, Apple, OPPO Concern: LG, [Qualcomm] |
| 1.2 | Proposal 1.E: On Rel.17 unified TCI framework, for Rel-17 unified TCI, for DL channels/signals that share the same indicated Rel-17 TCI state as UE-dedicated reception on PDSCH/PDCCH (via Rel-17 MAC-CE/DCI TCI state update), the following option on source RSs and QCL-Types is also supported: - Option 3: CSI-RS for CSI is configured for QCL-TypeA and QCL-TypeD source RS FL Note: It was explained that the so-called “circular” issue is avoided in practice via NW implementation, i.e. NW will not configure the same CSI-RS for CSI both as source and target RSs. | Support/fine (22): Huawei/HiSi, Ericsson, CMCC, Samsung, Sony, Qualcomm, Fraunhofer IIS/HHI, Futurewei, MTK, NTT Docomo, AT&T, Lenovo/MotM, Intel, Xiaomi, CATT, TCL, ZTE, Nokia/NSB Concern: Apple (object), OPPO |

Table 4 Additional inputs: issue 1

| Company | Input |
| --- | --- |
| Mod V0 | - Check and update your view in Table 3 - Share more inputs here if needed |

### Issue 4 (MP-UE)

--

# References

| 1 | R1-2111716 | Summary of offline discussion on unified TCI, inter-cell beam management, and MPUE | Moderator (Samsung) |
| --- | --- | --- | --- |