3GPP TSG RAN WG1 Meeting #107-e	R1-2112587

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.1.4

Source:	(Moderator) Huawei, HiSilicon

Title:	Summary of CSI enhancements for MTRP and FDD (Round 2)

Document for:	Discussion and Decision

# Introduction

# Summary of CSI enhancement for Rel-17 port selection codebook

Proposal 3: In Rel-17, the priority value is given by Pril,i,f=v⋅K1⋅f+v⋅ψ(i)+l whereas for ψ(i), down-select one alternative from the following:

- Alt 2: Support non-interleaving between polarization, ψi=i

- Alt 3-2: Support interleave ports across polarizations (starting from port index 0), with the port permutation function  given by  ψi=2i- iL2L-1

| Company | Comments |
| --- | --- |
| Mod | The latest companies’ view so far in Round 2: - Alt 2 : Docomo, Oppo, Intel (1), Samsung (1st), Apple, ZTE, Ericsson (2), LGE, Nokia - Concern: MTK, Fraunhofer - Alt 3-2 : CATT, Fraunhofer, Oppo, Motorola, Sony, Nokia, Intel (2), Vivo, Sony - Concern: ZTE, QC, MTK - Alt 1 : QC (cannot accept Alt3-x), MTK, Ericsson (1) |

Proposal 4-1: Regarding to codebook parameters for Rel-17 PS codebook:

- Alt1: alpha =3/4 is not applicable to 4 and 12 CSI-RS ports

| Company | Comments |
| --- | --- |
| Mod | [Mod] The latest companies’ view so far in Round 2: - Alt 1: Motorola (1), LGE, Qualcomm, Fraunhofer, Nokia(2), Samsung, Docomo, Vivo - Alt 2: Nokia (1), CATT, Intel, Ericsson, ZTE - Majority: Motorola (2), Apple |

Proposal 4-4 (updated #2): For Rel-17 PS codebook:

- Alt 2(updated #2): UCI Group 1 includes the max(0, KNZ2-υ)  highest priority elements of i2,4,l and the max(0, KNZ2-υ)  highest priority elements of i2,5,l (l=1,…,υ). UCI Group 2 includes the min(KNZ-v, KNZ2)   lowest priority elements of i2,4,l and the min(KNZ-v, KNZ2) lowest priority elements of i2,5,l (l=1,…,υ)

| Company | Comments |
| --- | --- |
| Mod | [Mod] What I can see is that most companies are flexible among Alt 1 or Alt 2. However Alt 2 (updated #2) is somewhat slightly preferred by Nokia/Samsung/Qualcomm/CATT - Alt 1: Intel, DOCOMO, ZTE, OPPO, Motorola, Fraunhofer - Alt 2 (updated #2): Qualcomm, Intel, Nokia, Samsung, DOCOMO, ZTE, OPPO, CATT |

# Summary of CSI enhancement for Multi-TRP