n3GPP TSG RAN WG1 #107-e		R1-xxxxxx

e-Meeting, Nov 11th – 19th

Agenda item:	8.14.1

Title:	[Draft] TR section - Coverage evaluation

Source:	Moderator (Qualcomm)

Document for:	Discussion

This document contains XR coverage evaluation results submitted by companies. If agreed, this is going to be coverage evaluation results section in R17 XR SI TR.

Please note that company names are explicitly indicated in the below sections. As discussed in the online session, the company names will be changed to Source XX, Source YY, etc. according to the following table in the final version.

| Source 1 | Apple |
| --- | --- |
| Source 2 | AT&T |
| Source 3 | CATT |
| Source 4 | CEWiT |
| Source 5 | China Unicom |
| Source 6 | CMCC |
| Source 7 | Ericsson |
| Source 8 | FUTUREWEI |
| Source 9 | Huawei |
| Source 10 | Intel |
| Source 11 | InterDigital |
| Source 12 | ITRI |
| Source 13 | LG |
| Source 14 | MediaTek |
| Source 15 | Nokia |
| Source 16 | Qualcomm |
| Source 17 | OPPO |
| Source 18 | vivo |
| Source 19 | Xiaomi |
| Source 20 | ZTE |

================= Start of Coverage Section in TR ==========================

## XR Coverage Evaluation

### Coverage based on Methodology 1

#### FR1

##### DU

Following table captures the company reported XR coverage evaluation results for FR1, DU.

Table 1 XR coverage results for Method 1, FR1, DU, DDDSU

| Deployment environment | Link | Applications | PDB (ms) | # of UEs / cell | XR Coverage |  | source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (dB) | Value (dB) |  |  |
| FR1, DU | DL | VR/AR30 | 10 | Capacity | -121.99 | -121.9 | vivo | Note 1 |
|  |  |  |  |  |  | -118.08 | ZTE |  |
|  |  |  |  |  |  | -126 | Nokia |  |
|  |  |  |  | 1 | -121.63 | -122.9 | Vivo |  |
|  |  |  |  |  |  | -123 | Nokia |  |
|  |  |  |  |  |  | -119 | Ericsson |  |
|  |  | VR/AR45 | 10 | Capacity | -123.19 | -120.39 | ZTE |  |
|  |  |  |  |  |  | -126 | Nokia |  |
|  |  |  |  | 1 | -123 | -123 | Nokia |  |
|  |  | CG30 | 15 | Capacity | -123.2 | -120.41 | ZTE |  |
|  |  |  |  |  |  | -126 | Nokia |  |
|  |  |  |  | 1 | -121 | -123 | Nokia |  |
|  |  |  |  |  |  | -119 | Ericsson |  |
|  | UL | Pose | 10 | 25 | -119.72 | -119.72 | ZTE |  |
|  |  |  |  | 1 | -119 | -119 | Ericsson | Note 2 |
|  |  | AR 1 stream | 30 | Capacity | -118.53 | -117 | vivo |  |
|  |  |  |  |  |  | -120.07 | ZTE |  |
|  |  |  |  | 1 | -119.96 | -122.9 | vivo |  |
|  |  |  |  |  |  | -118 | Ericsson | Note 3 |
| Note 1. Capacity in method 1 equals the minimal value of the DL and UL system capacity results Note 2. - 119dB for DDDUU Note 3. - 119dB for DDDUU |  |  |  |  |  |  |  |  |

Observations

- In Coverage Evaluation Method 1, FR1, DU, # of UE = Capacity, it is observed by Source vivo, ZTE, and Nokia that DL coverage for VR/AR30, VR/AR45, and CG30 are -121.99, -123.19, and -123.2 respectively.

- In Coverage Evaluation Method 1, FR1, DU, it is observed by Source ZTE, vivo that UL coverage for Pose (w/ #UE=25/cell) and AR1 stream (w/ #UE=Capacity/cell) are -119.72 and -118.53 respectively.

- In Coverage Evaluation Method 1, FR1, DU, # of UE =1/cell, it is observed by Source vivo, Nokia, Ericsson, ZTE that DL coverage for VR/AR30, VR/AR45, and CG30 are -121.63, -123, and -121 respectively.

- In Coverage Evaluation Method 1, FR1, DU, # of UE =1/cell, it is observed by Source ZTE, Ericsson, vivo that UL coverage for Pose and AR1 stream are -119 and -119.96 respectively.

Observations

- In Coverage Evaluation Method 1, FR1, DU, VR30, DL coverage is better than UL coverage by 2.63dB.

- In Coverage Evaluation Method 1, FR1, DU, VR45, DL coverage is better than UL coverage by 4dB.

- In Coverage Evaluation Method 1, FR1, DU, AR30, DL coverage is better than UL coverage by 1.67dB.

- In Coverage Evaluation Method 1, FR1, DU, AR45, DL coverage is better than UL coverage by 3.04dB.

- In Coverage Evaluation Method 1, FR1, DU, CG30, DL coverage is better than UL coverage by 1.04dB.

Observations

- In Coverage Evaluation Method 1, FR1, DU, it was observed by the Source Ericsson that coverage of UL Pose is -119dB for both DDDSU and DDDUU.

- In Coverage Evaluation Method 1, FR1, DU, it was observed by the Source Ericsson that coverage of UL AR 1 stream is -118/-119dB for DDDSU/DDDUU respectively.

##### UMa

Following table captures the company reported XR coverage evaluation results for FR1, UMa.

Table 2 XR Coverage results in Method 1, FR1, UMa, DDDSU

| Deployment environment | Link | Applications | PDB (ms) | # of UEs / cell | XR Coverage |  | source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (dB) | Value (dB) |  |  |
| FR1, UMa | DL | VR/AR30 | 10 | Capacity | -136.81 | -132.86 | HW |  |
|  |  |  |  |  |  | -140.76 | ZTE |  |
|  |  |  |  | 1 | -138.69 | -137.19 | HW |  |
|  |  |  |  |  |  | -140.9 | vivo |  |
|  |  |  |  |  |  | -138 | Ericsson |  |
|  |  | VR/AR45 | 10 | Capacity | -136.26 | -132.95 | HW |  |
|  |  |  |  |  |  | -139.56 | ZTE |  |
|  |  |  |  | 1 | -136.58 | -136.58 | HW |  |
|  |  | CG30 | 15 | Capacity | -137.12 | -134.38 | HW |  |
|  |  |  |  |  |  | -139.86 | ZTE |  |
|  |  |  |  | 1 | -137.59 | -137.19 | HW |  |
|  |  |  |  |  |  | -138 | Ericsson |  |
|  | UL | Pose | 10 | 1 | -130.5 | -136.01 | HW |  |
|  |  |  |  |  |  | -125 | Ericsson | Note 2 |
|  |  | AR 1 stream | 30 | 1 | -122.93 | -121.61 | HW | Note 3 |
|  |  |  |  |  |  | -124.2 | vivo |  |
|  |  |  |  |  |  | -123 | Ericsson | Note 4 |
| Note 1. Capacity in method 1 equals the minimal value of the DL and UL system capacity results Note 2. -135dB for DDDUU Note 3. -118.91dB when PDB=15ms, -122.55dB when PDB=60ms Note 4. -127dB for DDDUU |  |  |  |  |  |  |  |  |

Observations

- In Coverage Evaluation Method 1, FR1, UMa,  # of UE =Capacity, it is observed by Sources HW, ZTE, that DL coverage for VR/AR30, VR/AR45, and CG30 are -136.81dB, -136.26dB, and -137.12dB respectively.

- In Coverage Evaluation Method 1, FR1, UMa,  # of UE =1/cell, it is observed by Sources HW, vivo, Ericsson, that DL coverage for VR/AR30, VR/AR45, and CG30 are -138.69dB, -136.58dB, and -137.59dB respectively.

- In Coverage Evaluation Method 1, FR1, UMa,  # of UE =1/cell, it is observed by Sources HW, Ericsson, vivo that UL coverage for Pose and AR1 stream are -130.5dB and -122.93dB respectively.

Observations

- In Coverage Evaluation Method 1, FR1, UMa, VR30, # of UE/cell =1, the DL coverage is better than that of UL by around 8.19dB.

- In Coverage Evaluation Method 1, FR1, UMa, VR45, # of UE/cell =1, the DL coverage is better than that of UL by around 6.08dB.

- In Coverage Evaluation Method 1, FR1, UMa, AR30, # of UE/cell =1, the DL coverage is better than that of UL by around 15.76dB.

- In Coverage Evaluation Method 1, FR1, UMa, AR45, # of UE/cell =1, the DL coverage is better than that of UL by around 13.57dB.

- In Coverage Evaluation Method 1, FR1, UMa, CG30, # of UE/cell =1, the DL coverage is better than that of UL by around 14.66dB.

Observations

- In Coverage Evaluation Method 1, FR1, UMa, # of UE/cell =1, it was observed by Source Ericsson that the coverage of UL Pose with DDDUU is 10dB better than that with DDDSU.

- In Coverage Evaluation Method 1, FR1, UMa, # of UE/cell =1, it was observed by Source Ericsson that the coverage of UL AR 1 stream with DDDUU is 4dB better than that with DDDSU.

- In Coverage Evaluation Method 1, FR1, UMa, # of UE/cell =1, it was observed by Source HW that the coverage of UL AR 1 stream for PDB=15ms, 30ms, and 60ms are given as -118.91dB, -121.61dB, and -122.55dB respectively.

##### InH

Following table captures the company reported XR coverage evaluation results for FR1, InH.

Table 3 XR Coverage results in Method 1, FR1, InH, DDDSU

| Deployment environment | Link | Applications | PDB (ms) | # of UEs / cell | XR Coverage |  | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (dB) | Value (dB) |  |
| FR1, InH | DL | VR/AR30 | 10 | Capacity | -76.62 | -82.24 | ZTE |
|  |  |  |  |  |  | -71 | Nokia |
|  |  |  |  | 1 | -72 | -72 | Nokia |
|  |  | VR/AR45 | 10 | Capacity | -76.06 | -82.12 | ZTE |
|  |  |  |  |  |  | -70 | Nokia |
|  |  |  |  | 1 | -71 | -71 | Nokia |
|  |  | CG30 | 15 | Capacity | -76.59 | -82.18 | ZTE |
|  |  |  |  |  |  | -71 | Nokia |
|  |  |  |  | 1 | -72 | -72 | Nokia |
|  | UL | Pose | 10 | 25 | -79.62 | -79.62 | ZTE |
|  |  | AR 1 stream | 30 | 10 | -79.56 | -79.56 | ZTE |

Observations

- In Coverage Evaluation Method 1, FR1, InH,  # of UE =Capacity, it is observed by Sources ZTE, Nokia that DL coverage for VR/AR30, VR/AR45, and CG30 are -76.62dB, -76.06dB, and -76.59dB respectively.

- In Coverage Evaluation Method 1, FR1, InH, it is observed by Sources HW, Ericsson, vivo that UL coverage for Pose (w/ #UE=25) and AR1 stream (w/ #UE=10) are -79.62dB and -79.56dB respectively.

- In Coverage Evaluation Method 1, FR1, InH,  # of UE =1/cell, it is observed by Sources ZTE, Nokia that DL coverage for VR/AR30, VR/AR45, and CG30 are -72dB, -71dB, and -72dB respectively.

#### FR2

##### DU

Following table captures the company reported XR coverage evaluation results for FR2, DU.

Table 4 XR Coverage results in Method 1, FR2, DU

| Deployment environment | Link | Applications | PDB (ms) | # of UEs / cell, B | XR Coverage |  | TDD format | source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (dB) | Value (dB) |  |  |
| FR2, DU | DL | VR/AR30 | 10 | Capacity | -104.9 | -104.5 | DDDSU | vivo |
|  |  |  |  |  |  | -105.3 | DDDSU | QC |
|  |  |  |  | 1 | -106.9 | -106.9 | DDDSU | vivo |
|  |  | CG8 | 15 | Capacity | -105 | -105 | DDDSU | QC |
|  | UL | Pose | 10 | Capacity | -105.2 | -105.2 | DDDUU | QC |
|  |  | AR 1 stream | 30 | Capacity | -103.35 | -101.9 | DDDSU | vivo |
|  |  |  |  |  |  | -104.8 | DDDUU | QC |
|  |  |  |  | 1 | -106.9 | -106.9 | DDDSU | vivo |

Observations

- In Coverage Evaluation Method 1, FR2, DU,  # of UE =Capacity, it is observed by Sources vivo, QC that DL coverage for VR/AR30 and CG30 are -104.9dB and -105dB respectively.

- In Coverage Evaluation Method 1, FR2, DU,  # of UE =Capacity, it is observed by Sources vivo, QC that UL coverage for Pose and AR 1 stream are -105.2dB and -103.35dB respectively.

- In Coverage Evaluation Method 1, FR2, DU,  # of UE =1/cell, it is observed by Sources vivo that DL coverage for VR/AR30 is -106.9dB respectively.

- In Coverage Evaluation Method 1, FR2, DU,  # of UE =1/cell, it is observed by Sources vivo, QC that UL coverage for AR 1 stream is -106.9dB.

Observations

- In Coverage Evaluation Method 1, FR2, DU, AR30, # of UE/cell=1, the DL coverage is similar with that of UL.

##### InH

Following table captures the company reported XR coverage evaluation results for FR2, InH.

Table 5 XR Coverage results in Method 1, FR2, InH

| Deployment environment | Link | Applications | PDB (ms) | # of UEs / cell | XR Coverage |  | TDD format | source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (dB) | Value (dB) |  |  |
| FR2, nH | DL | VR/AR30 | 10 | Capacity | -86.5 | -86.5 | DDDSU | QC |
|  |  | CG8 | 15 | Capacity | -85 | -85 | DDDSU | QC |
|  | UL | Pose | 10 | Capacity | -84.5 | -84.5 | DDDUU | QC |
|  |  | AR 1 stream | 30 | Capacity | -80.1 | -80.1 | DDDUU | QC |

Observations

- In Coverage Evaluation Method 1, FR2, InH,  # of UE=Capacity, it is observed by Sources QC that DL coverage for VR/AR30 and CG30 are -86.5dB and -85dB respectively.

- In Coverage Evaluation Method 1, FR2, InH,  # of UE=Capacity, it is observed by Sources QC that UL coverage for Pose and AR 1 stream are -84.5dB and -80.1dB respectively.

### Coverage based on Methodology 2

In methodology 2, we evaluate XR coverage with 1 UE per network.

#### FR1

##### DU

Following table captures the company reported XR coverage evaluation results for FR1, DU.

Table 6 XR Coverage results in Method 2, FR1, DU, DDDSU

| Deployment environment | Link | Applications | PDB (ms) | XR Coverage |  | source |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (dB) | Value (dB) |  |
| FR1, DU | DL | VR/AR30 | 10 | -138.93 | -144.58 | vivo |
|  |  |  |  |  | -134.80 | Intel |
|  |  |  |  |  | -137.4 | QC |
|  |  | CG30 | 15 | -138.45 | -135.5 | Intel |
|  |  |  |  |  | -141.4 | QC |
|  | UL | Pose | 10 | -137.45 | -134.6 | Intel |
|  |  |  |  |  | -140.3 | QC |
|  |  | AR 1 stream | 30 | -126.84 | -126.84 | vivo |
|  |  | AR 2 streams | 10,30 | -119.9 | -119.9 | QC |

Observations

- In Coverage Evaluation Method 2, FR1, DU, it is observed by Sources vivo, intel, QC that DL coverage of VR/AR30, CG30 are -138.93dB, -138.45dB respectively.

- In Coverage Evaluation Method 2, FR1, DU, it is observed by Sources intel, QC that UL coverage of Pose, AR 1 stream, AR 2 stream are -137.45 dB, -126.84 dB, -119.9dB respectively.

Observations

- In Coverage Evaluation Method 2, FR1, DU, VR30, the DL coverage is better than that of UL by around 1.48dB.

- In Coverage Evaluation Method 2, FR1, DU, CG30, the DL coverage is better than that of UL by around 1dB.

- In Coverage Evaluation Method 2, FR1, DU, AR30 with 2 UL streams, the DL coverage is better than that of UL by around 19.03dB, which indicates that UL is bottleneck.

##### UMa

Following table captures the company reported XR coverage evaluation results for FR1, UMa.

Table 7 XR Coverage results in Method 2, FR1, UMa, DDDSU

| Deployment environment | Link | Applications | PDB (ms) | XR Coverage |  | source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (dB) | Value (dB) |  |  |
| FR1, UMa | DL | VR/AR30 | 10 | -145.44 | -145.33 | HW |  |
|  |  |  |  |  | -150.07 | Vivo |  |
|  |  |  |  |  | -144.65 | Intel |  |
|  |  |  |  |  | -141.7 | QC |  |
|  |  | VR/AR45 | 10 | -143.85 | -143.85 | HW |  |
|  |  | CG30 | 15 | -147.16 | -146.88 | HW |  |
|  |  |  |  |  | -148.2 | Intel |  |
|  |  |  |  |  | -146.4 | QC |  |
|  | UL | Pose | 10 | -139.45 | -137.81 | HW |  |
|  |  |  |  |  | -139.8 | Intel |  |
|  |  |  |  |  | -140.5 | QC |  |
|  |  | AR 1 stream | 30 | -124.48 | -122.5 | HW | Note 1 |
|  |  |  |  |  | -126.39 | Vivo |  |
|  |  | AR 2 stream | 10,30 | -121.7 | -121.7 | QC |  |
| Note 1. -119.85dB when PDB=15ms, -123.21dB for PDB=60ms |  |  |  |  |  |  |  |

Observations

- In Coverage Evaluation Method 2, FR1, UMa, it was observed by Source HW, vivo, intel, QC that the DL coverage of VR/AR30, VR/AR45, CG30 are -145.44dB, -143.85dB, and -147.16dB respectively.

- In Coverage Evaluation Method 2, FR1, UMa, it was observed by Source HW, intel, QC that the UL coverage of Pose, UL AR 1 stream, and UL AR 2 streams are -139.45, -124.48dB, and -121.7dB respectively.

Observations

- In Coverage Evaluation Method 2, FR1, UMa, VR30, the DL coverage is better than that of UL by around 5.99dB.

- In Coverage Evaluation Method 2, FR1, UMa, VR45, the DL coverage is better than that of UL by around 4.4dB.

- In Coverage Evaluation Method 2, FR1, UMa, CG30, the DL coverage is [better] than that of UL by around 7.71dB.

- In Coverage Evaluation Method 2, FR1, UMa, AR30 with 2 UL streams, the DL coverage is [better] than that of UL by around 23.74dB.

Observations

- For Coverage Evaluation Method 2, FR1, UMa, DL coverage of VR/CG/AR is better than its UL coverage, which indicates that UL is bottleneck.

- Applications with relaxed requirements (e.g., lower data rate, larger PDB) has larger coverage

- UMa has better DL coverage than DU due to higher tx power (5dB).

- UMa and DU have similar UL coverage.

#### FR2

##### DU

Following table captures the company reported XR coverage evaluation results for FR2, DU.

Table 8 XR Coverage Results in Method 2, FR2, DU, DDDSU

| Deployment environment | Link | Applications | PDB (ms) | XR Coverage (dB) |  | source |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean, | Data |  |
| FR2, DU | DL | AR30 | 10 | -127.66 | -127.66 | vivo |
|  | UL | AR 1 stream | 30 | -120.17 | -120.17 | vivo |

Observation

- In Coverage Evaluation Method 2, FR2, DU, it was observed from Source vivo that DL coverage of AR30 is -127.66dB.

- In Coverage Evaluation Method 2, FR2, DU, it was observed from Source vivo that UL coverage of AR 1 stream is -120.17dB.

Observation

- In Coverage Evaluation Method 2, FR2, DU, AR30, the DL coverage is better than that of UL by around 7.49dB.