3GPP TSG RAN WG1 #107-e	                                              R1-xxxxxxx

e-Meeting, November 11th – 19th, 2021

Source:	Moderator (vivo)

Title:	[DRAFT] TR section – Mobility evaluation

Agenda Item:	8.14.1

Document for:	Information

- Foreword

Scope

…

================= (Unchanged part omitted) ==========================

References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

- 3GPP RP-201145: "Revised SI on XR Evaluations for XR"

- 3GPP R1-2104023: “LS on Status Update on XR Traffic”

- 3GPP S4-210614: “FS_XRTRaffic: Permanent document, v0.6.0”

- 3GPP TR 23.501: “System architecture for the 5G System (5GS)”

- 3GPP TR 38.840: “Study on User Equipment (UE) power saving in NR”

- 3GPP R1-2101765, “LS on XR-Traffic Models”

================= (Unchanged part omitted) ==========================

- XR Mobility evaluations

============Start of Text update for TR section – Mobility evaluation =============

Purpose of study

The mobility study is to understand the mobility performance of XR applications. Note that the mobility performance of XR depends on the evaluation assumptions including mobility procedures, traffic models, and user satisfaction criteria, etc. Thus, the mobility metric should be understood as a conditional metric for the give assumption. From this study, we can identify the mobility performance of XR applications in terms of the number of XR packets lost due to a handover (HO) event and the minimum target time interval between HO events.

KPI

The KPI of the mobility evaluation is XR mobility which is defined as {N, T}, where N is the number of consecutive XR packets lost due to a HO event and T is the minimum target time interval between HO events

The details of the mobility evaluation methodologies are found in Annex A.4.

Mobility evaluation results

(Moderator’s note: This section is to capture the evaluation results and the corresponding observations for mobility in the TR.)

Consecutive XR packets lost due to a HO event, N

| F(fps) | PDB (ms) | Y (ms) | N | Source(s) | Note |
| --- | --- | --- | --- | --- | --- |
| 30 | 10 | 0~10 | 0~0.3 | ZTE | Note 1 |
| 60 | 10 | 0~10 | 0~0.6 | ZTE | Note 1 |
|  |  |  | 0 | Huawei, Nokia, Ericsson | Note 2 |
|  |  | 10~30 | 0.6~1 | Huawei, Nokia, Ericsson |  |
|  |  | 30~100 | 1.8~6 | Huawei, vivo, CATT, OPPO, Intel, Nokia, Ericsson, MediaTek |  |
|  |  | 100~1000 | 7.9~51 | OPPO, Intel |  |
|  |  | 1000~4212 | 97~252 | OPPO |  |
|  | 15 | 0~15 | 0 | Nokia, Ericsson | Note 2 |
|  |  | 15~30 | 0.3~1 | Nokia, Ericsson |  |
|  |  | 30~100 | 1.51~6 | vivo, CATT, CMCC, Nokia, Ericsson, MediaTek |  |
|  | 30 | 0~30 | 0 | Nokia, Ericsson | Note 2 |
|  |  | 30~100 | 0.6~5 | vivo, Nokia, Ericsson, MediaTek |  |
| 90 | 10 | 30~100 | 3.78~6.48 | vivo |  |
|  | 15 | 30~100 | 3.33~6.03 | vivo |  |
|  | 30 | 30~100 | 1.98~4.68 | vivo |  |
| 120 | 10 | 0~10 | 0~1.2 | ZTE | Note 1 |
|  |  | 30~100 | 5.04~8.64 | vivo |  |
|  | 15 | 30~100 | 4.44~8.04 | vivo |  |
|  | 30 | 30~100 | 2.64~6.24 | vivo |  |
| 250 | 10 | 0~10 | 0 | Nokia, Ericsson | Note 2 |
|  |  | 10~30 | 2.5~3 | Nokia, Ericsson |  |
|  |  | 30~100 | 7.5~23 | vivo, Nokia, Ericsson, MediaTek |  |
| Note 1: , is considered in the simulation results per agreement. Note 2: N=0, Y < PDB |  |  |  |  |  |

Observation:

- For XR application with F=30 FPS and PDB=10ms, with the range of Y 0~10ms, it is identified from source (ZTE) that the range of N is 0~0.3.

- For XR application with F=60 FPS and PDB=10ms,

  - with the range of Y 0~10ms, it is identified from sources (Huawei, ZTE, Nokia, Ericsson) that the range of N is 0~0.6.

  - with the range of Y 10~30ms, it is identified from sources (Huawei, Nokia, Ericsson) that the range of N is 0.6~1.

  - With the range of Y 30~100ms, it is identified from sources (Huawei, vivo, CATT, OPPO, Intel, Nokia, Ericsson, MediaTek) that the range of N is 1.8~6.

  - With the range of Y 100~1000ms, it is identified from sources (OPPO, Intel) that the range of N is 7.9~51.

  - With the range of Y 1000~4212ms, it is identified from source (OPPO) that the range of N is 97~252.

- For XR application with F=60 FPS and PDB=15ms,

  - with the range of Y 0~15ms, it is identified from sources (Nokia, Ericsson) that the range of N is 0.

  - with the range of Y 15~30ms, it is identified from sources (Nokia, Ericsson) that the range of N is 0.3~1.

  - with the range of Y 30~100ms, it is identified from sources (vivo, CATT, CMCC, Nokia, Ericsson, MediaTek) that the range of N is 1.51~6.

- For XR application with F=60 FPS and PDB=30ms,

  - with the range of Y 0~30ms, it is identified from sources (Nokia, Ericsson) that the range of N is 0.

  - with the range of Y 30~100ms, it is identified from sources (vivo, Nokia, Ericsson, MediaTek) that the range of N is 0.6~5.

- For XR application with F=90 FPS and PDB=10ms, with the range of Y 30~100ms, it is identified from source (vivo) that the range of N is 3.78~6.48.

- For XR application with F=90 FPS and PDB=15ms, with the range of Y 30~100ms, it is identified from source (vivo) that the range of N is 3.33~6.03.

- For XR application with F=90 FPS and PDB=30ms, with the range of Y 30~100ms, it is identified from source (vivo) that the range of N is 1.98~4.68.

- For XR application with F=120 FPS and PDB=10ms, with the range of Y 0~10ms, it is identified from source (ZTE) that the range of N is 0~1.2.

- For XR application with F=120 FPS and PDB=10ms, with the range of Y 0~100ms, it is identified from source (vivo) that the range of N is 0~8.64.

- For XR application with F=120 FPS and PDB=15ms, with the range of Y 30~100ms, it is identified from source (vivo) that the range of N is 5.04~8.64.

- For XR application with F=120 FPS and PDB=30ms, with the range of Y 30~100ms, it is identified from source (vivo) that the range of N is 2.64~6.24.

- For XR application with F=250 FPS and PDB=10ms, with the range of Y 0~10ms, it is identified from sources (Nokia, Ericsson) that the range of N is 0.

- For XR application with F=250 FPS and PDB=10ms, with the range of Y 10~30ms, it is identified from sources (Nokia, Ericsson) that the range of N is 2.5~3.

- For XR application with F=250 FPS and PDB=10ms, with the range of Y 30~100ms, it is identified from sources (vivo, Nokia, Ericsson, MediaTek) that the range of N is 7.5~23.

Minimum target time interval between HO events, T

| X | PE,op | PDB (ms) | Y (ms) | T (s) | Source(s) | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 99% | 0% | 10 | 0~10 | 0 | Huawei, Nokia, Ericsson | Note 1 |
|  |  |  | 20 | 1 | Huawei, Nokia, Ericsson |  |
|  |  |  | 30~100 | 3~9 | Huawei, vivo, Nokia, Ericsson, MediaTek |  |
|  |  | 15 | 0~15 | 0 | Nokia, Ericsson | Note 1 |
|  |  |  | 20 | 0.5 | Nokia, Ericsson |  |
|  |  |  | 30~100 | 2.5~8.5 | vivo, Nokia, Ericsson, MediaTek |  |
|  |  | 30 | 0~30 | 0 | Nokia, Ericsson | Note 1 |
|  |  |  | 30~100 | 1~7 | vivo, Nokia, Ericsson, MediaTek |  |
|  | 0.01%~0.4% | 10 | 0~10 | 0 | Huawei, Nokia, Ericsson | Note 1 |
|  |  |  | 20 | 1.16~1.25 | Huawei, Nokia, Ericsson |  |
|  |  |  | 30~100 | 3.48~14.94 | OPPO, Intel, CATT, vivo, Huawei, Nokia, Ericsson |  |
|  |  |  | 100~1000 | 14.34~93 | OPPO, Intel |  |
|  |  |  | 1000~4212 | 164~466 | OPPO |  |
|  |  | 15 | 0~15 | 0 | Nokia, Ericsson | Note 1 |
|  |  |  | 20 | 0.62~0.83 | Nokia, Ericsson |  |
|  |  |  | 30~100 | 3.1~14.11 | CATT, vivo, CMCC, Nokia, Ericsson |  |
|  |  | 30 | 0~30 | 0 | Nokia, Ericsson | Note 1 |
|  |  |  | 30~100 | 1.2~11.62 | vivo, Nokia, Ericsson |  |
|  | 0.5%~0.7% | 10 | 0~10 | 0 | Nokia, Ericsson | Note 1 |
|  |  |  | 20 | 2.48 | Nokia, Ericsson |  |
|  |  |  | 30~100 | 7~22.4 | vivo, OPPO, MediaTek, Intel, Nokia, Ericsson |  |
|  |  |  | 100~1000 | 28.83~168 | Intel, OPPO |  |
|  |  |  | 1000~4212 | 323~836 | OPPO |  |
|  |  | 15 | 0~15 | 0 | Nokia, Ericsson | Note 1 |
|  |  |  | 20 | 1.24 | Nokia, Ericsson |  |
|  |  |  | 30~100 | 6.2~21.12 | vivo, MediaTek, Nokia, Ericsson |  |
|  |  | 30 | 0~30 | 0 | Nokia, Ericsson | Note 1 |
|  |  |  | 30~100 | 2.48~17.4 | vivo, MediaTek, Nokia, Ericsson |  |
|  | 0.8%~0.9% | 10 | 0~10 | 0 | Nokia, Ericsson |  |
|  |  |  | 20 | 4.96~9.91 | Nokia, Ericsson | Note 1 |
|  |  |  | 30~100 | 14.88~89.2 | Nokia, Ericsson, Intel, vivo, MediaTek |  |
|  |  |  | 142 | 70.43~140.7 | Intel |  |
|  |  | 15 | 0~15 | 0 | Nokia, Ericsson | Note 1 |
|  |  |  | 20 | 2.48~4.95 | Nokia, Ericsson |  |
|  |  |  | 30~100 | 12.4~84.23 | Nokia, Ericsson, vivo, MediaTek |  |
|  |  | 30 | 0~30 | 0 | Nokia, Ericsson | Note 1 |
|  |  |  | 30~100 | 4.96~69.4 | Nokia, Ericsson, vivo, MediaTek |  |
| 95% | 0~0.1% | 10 | 40~100 | 0.84~1.47 | vivo |  |
|  |  | 15 |  | 0.74~1.37 | vivo |  |
|  |  | 30 |  | 0.44~1.06 | vivo |  |
|  | 2.5% | 10 |  | 1.64~2.81 | vivo |  |
|  |  | 15 |  | 1.44~2.61 | vivo |  |
|  |  | 30 |  | 0.86~2.03 | vivo |  |
| Note 1: N=0, Y < PDB |  |  |  |  |  |  |

Observation:

- For XR application with X=99% and PDB=10ms, PE,op=0%,

  - with the range of Y 0~10ms, it is identified from sources (Huawei, Nokia, Ericsson) that the T is 0.

  - with the Y=20ms, it is identified from sources (Huawei, Nokia, Ericsson) that the T is 1.

  - with the range of Y 30~100ms, it is identified from sources (Huawei, vivo, Nokia, Ericsson, MediaTek) that the range of T is 3~9.

- For XR application with X=99% and PDB=15ms, PE,op=0%,

  - with the range of Y 0~15ms, it is identified from sources (Nokia, Ericsson) that the T is 0.

  - with the Y=20ms, it is identified from sources (Nokia, Ericsson) that the T is 0.5.

  - with the range of Y 30~100ms, it is identified from sources (vivo, Nokia, Ericsson, MediaTek) that the range of T is 2.5~8.5.

- For XR application with X=99% and PDB=30ms, PE,op=0%,

  - with the range of Y 0~30ms, it is identified from sources (Nokia, Ericsson) that the T is 0.

  - with the range of Y 30~100ms, it is identified from sources (vivo, Nokia, Ericsson, MediaTek) that the range of T is 1~7.

- For XR application with X=99% and PDB=10ms, the range of PE,op 0.01%~0.4%,

  - with the range of Y 0~10ms, it is identified from sources (Huawei, Nokia, Ericsson) that the T is 0.

  - with the Y=20ms, it is identified from sources (Huawei, Nokia, Ericsson) that the range of T is 1.16~1.25.

  - with the range of Y 30~100ms, it is identified from sources (OPPO, Intel, CATT, vivo, Huawei, Nokia, Ericsson) that the range of T is 3.48~14.94.

- For XR application with X=99% and PDB=15ms, the range of PE,op 0.01%~0.4%,

  - with the range of Y 0~15ms, it is identified from sources (Nokia, Ericsson) that the T is 0.

  - with the Y=20ms, it is identified from sources (Nokia, Ericsson) that the range of T is 0.62~0.83.

  - with the range of Y 30~100ms, it is identified from sources (CATT, vivo, CMCC, Nokia, Ericsson) that the range of T is 3.1~14.11.

- For XR application with X=99% and PDB=30ms, the range of PE,op 0.01%~0.4%,

  - with the range of Y 0~30ms, it is identified from sources (Nokia, Ericsson) that the T is 0.

  - with the range of Y 30~100ms, it is identified from sources (vivo, Nokia, Ericsson) that the range of T is 1.2~11.62.

- For XR application with X=99% and PDB=10ms, the range of PE,op 0.5%~0.7%,

  - with the range of Y 0~10ms, it is identified from sources (Nokia, Ericsson) that the T is 0.

  - with the Y=20ms, it is identified from sources (Nokia, Ericsson) that the T is 2.48.

  - with the range of Y 30~100ms, it is identified from sources (vivo, OPPO, MediaTek, Intel, Nokia, Ericsson) that the range of T is 7~22.4.

- For XR application with X=99% and PDB=15ms, the range of PE,op 0.5%~0.7%,

  - with the range of Y 0~15ms, it is identified from sources (Nokia, Ericsson) that the T is 0.

  - with the Y=20ms, it is identified from sources (Nokia, Ericsson) that the T is 1.24.

  - with the range of Y 30~100ms, it is identified from sources (vivo, MediaTek, Nokia, Ericsson) that the range of T is 6.2~21.12.

- For XR application with X=99% and PDB=30ms, the range of PE,op 0.5%~0.7%,

  - with the range of Y 0~30ms, it is identified from sources (Nokia, Ericsson) that the range of T is 0.

  - with the range of Y 30~100ms, it is identified from sources (vivo, MediaTek, Nokia, Ericsson) that the range of T is 2.48~17.4.

- For XR application with X=99% and PDB=10ms, the range of PE,op 0.8%~0.9%,

  - with the range of Y 0~10ms, it is identified from sources (Nokia, Ericsson) that the T is 0.

  - with the Y=20ms, it is identified from sources (Nokia, Ericsson) that the range of T is 4.96~9.91.

  - with the range of Y 30~100ms, it is identified from sources (Nokia, Ericsson, Intel, vivo, MediaTek) that the range of T is 14.88~89.2.

- For XR application with X=99% and PDB=15ms, the range of PE,op 0.8%~0.9%,

  - with the range of Y 0~15ms, it is identified from sources (Nokia, Ericsson) that the T is 0.

  - with the Y=20ms, it is identified from sources (Nokia, Ericsson) that the range of T is 2.48~4.95.

  - with the range of Y 30~100ms, it is identified from sources (Nokia, Ericsson, vivo, MediaTek) that the range of T is 12.4~84.23.

- For XR application with X=99% and PDB=30ms, the range of PE,op 0.8%~0.9%,

  - with the range of Y 0~30ms, it is identified from sources (Nokia, Ericsson) that the T is 0.

  - with the range of Y 30~100ms, it is identified from sources (Nokia, Ericsson, vivo, MediaTek) that the range of T is 4.96~69.4.

- For XR application with X=95% and PDB=10ms, PE,op less than 0.01%, with the range of Y 40~100ms, it is identified from source (vivo) that the range of T is 0.84~1.47.

- For XR application with X=95% and PDB=15ms, PE,op less than 0.01%, with the range of Y 40~100ms, it is identified from source (vivo) that the range of T is 0.74~1.37.

- For XR application with X=95% and PDB=30ms, PE,op less than 0.01%, with the range of Y 40~100ms, it is identified from source (vivo) that the range of T is 0.44~1.06.

- For XR application with X=95% and PDB=10ms, PE,op=2.5%, with the range of Y 40~100ms, it is identified from source (vivo) that the range of T is 1.64~2.81.

- For XR application with X=95% and PDB=15ms, PE,op=2.5%, with the range of Y 40~100ms, it is identified from source (vivo) that the range of T is 1.44~2.61.

- For XR application with X=95% and PDB=30ms, PE,op=2.5%, with the range of Y 40~100ms, it is identified from source (vivo) that the range of T is 0.86~2.03.

=================(Unchanged part omitted)==========================

- A.4: Evaluation Methodology for Mobility

For XR/CG mobility evaluation, mobility performance is evaluated analytically while taking into account mobility procedures, traffic models, and user satisfaction criteria.

The mobility performance metrics {N, T}, where N is the number of consecutive XR packets lost due to a HO event and T is the minimum target time interval between HO events in ms, are obtained by the following steps:

- Step1. Calculate HO interruption time in ms (Y) for existing HO techniques by directly following the requirements given in TS 38.133.

- Step2. For a Y and the XR traffic pattern characterized by the packet arrival rate in average as determined by the frame generation rate of the flow in fps (F) in 6.1.1.2, and the packet delay budget in ms (PDB):

  - N is estimated as:

N=(Y-PDB)F1000Y≥PDB0 or N=YF1000 Y<PDB

  - T is estimated as:

T=(Y-PDB)100%-PE,op100%-X-PE,opY≥PDB0 or N=Y100%-PE,op100%-X-PE,op Y<PDB

    - where PE,op is packet error rate during time outside of handover procedure and X is packet success rate (baseline: X = 99%, other X value(s) can be also evaluated).

Note 1: mobility evaluation is performed in dense Urban and UMA.

Note 2: T maybe affected by system load, interference, etc.

- Annex B: Source Specific Mobility Performance Evaluation Results

  - Consecutive XR packets lost due to a HO event, N

Table 1  FPS=30, PDB=10ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ZTE | R1-2111351 | 30 | 10 |  |  | 0 | 0 | Note 1 |
| ZTE | R1-2111351 | 30 | 10 |  |  | 2 | 0.06 | Note 1 |
| ZTE | R1-2111351 | 30 | 10 |  |  | 4 | 0.12 | Note 1 |
| ZTE | R1-2111351 | 30 | 10 |  |  | 6 | 0.18 | Note 1 |
| ZTE | R1-2111351 | 30 | 10 |  |  | 8 | 0.24 | Note 1 |
| ZTE | R1-2111351 | 30 | 10 |  |  | 10 | 0.3 | Note 1 |
| Note 1: , is considered in the simulation results per agreement. |  |  |  |  |  |  |  |  |

Table 2  FPS=60, PDB=10ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | R1-2111046 | 60 | 10 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 2.52 |  |
| vivo | R1-2111046 | 60 | 10 | Typical HO | FR1-to-FR1 | 62 | 3.12 |  |
| vivo | R1-2111046 | 60 | 10 | Typical HO | FR1-to-FR1 | 82 | 4.32 |  |
| Huawei | R1-2110811 | 60 | 10 |  |  | 100 | 5.4 |  |
| Huawei | R1-2110811 | 60 | 10 |  |  | 80 | 4.2 |  |
| Huawei | R1-2110811 | 60 | 10 |  |  | 70 | 3.6 |  |
| Huawei | R1-2110811 | 60 | 10 |  |  | 60 | 3 |  |
| Huawei | R1-2110811 | 60 | 10 |  |  | 40 | 1.8 |  |
| Huawei | R1-2110811 | 60 | 10 |  |  | 20 | 0.6 |  |
| Huawei | R1-2110811 | 60 | 10 |  |  | <= 10 | 0 |  |
| ZTE | R1-2111351 | 60 | 10 |  |  | 0 | 0 | Note 4 |
| ZTE | R1-2111351 | 60 | 10 |  |  | 2 | 0.12 | Note 4 |
| ZTE | R1-2111351 | 60 | 10 |  |  | 4 | 0.24 | Note 4 |
| ZTE | R1-2111351 | 60 | 10 |  |  | 6 | 0.36 | Note 4 |
| ZTE | R1-2111351 | 60 | 10 |  |  | 8 | 0.48 | Note 4 |
| ZTE | R1-2111351 | 60 | 10 |  |  | 10 | 0.6 | Note 4 |
| CATT | R1-2111234 | 60 | 10 | Typical HO | FR1-to-FR1 | 66.5 | 3.39 |  |
| CATT | R1-2111234 | 60 | 10 | Typical HO | FR2-to-FR1 | 86.5 | 4.59 |  |
| CATT | R1-2111234 | 60 | 10 | Typical HO | FR1-to-FR2 | 79.4 | 4.164 |  |
| CATT | R1-2111234 | 60 | 10 | Typical HO | FR2-to-FR2 | 59.4 | 2.964 |  |
| CATT | R1-2111234 | 60 | 10 | Conditional HO | FR1-to-FR1 | 56.5 | 2.79 |  |
| CATT | R1-2111234 | 60 | 10 | Conditional HO | FR2-to-FR1 | 76.5 | 3.99 |  |
| CATT | R1-2111234 | 60 | 10 | Conditional HO | FR1-to-FR2 | 69.4 | 3.564 |  |
| CATT | R1-2111234 | 60 | 10 | Conditional HO | FR2-to-FR2 | 49.4 | 2.364 |  |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR1, FR2-to-FR2 | 47 | 2 | Note 1 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR1, FR2-to-FR3 | 192 | 11 | Note 1 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR1, FR2-to-FR4 | 352 | 21 | Note 1 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR2, FR1-to-FR2 | 67 | 3 | Note 1 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR2, FR1-to-FR2 | 212 | 12 | Note 1 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR2, FR1-to-FR2 | 372 | 22 | Note 1 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR1 | 52 | 3 | Note 2 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR1 | 272 | 16 | Note 2 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR1 | 512 | 30 | Note 2 |
| OPPO | R1-2111349 | 60 | 10 |  | FR2-to-FR2 | 87 | 5 | Note 2 |
| OPPO | R1-2111349 | 60 | 10 |  | FR2-to-FR2 | 832 | 49 | Note 2 |
| OPPO | R1-2111349 | 60 | 10 |  | FR2-to-FR2 | 1632 | 97 | Note 2 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR1 | 62 | 3 | Note 3 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR1 | 432 | 25 | Note 3 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR1 | 832 | 49 | Note 3 |
| OPPO | R1-2111349 | 60 | 10 |  | FR2-to-FR1 | 82 | 4 | Note 3 |
| OPPO | R1-2111349 | 60 | 10 |  | FR2-to-FR1 | 452 | 27 | Note 3 |
| OPPO | R1-2111349 | 60 | 10 |  | FR2-to-FR1 | 852 | 51 | Note 3 |
| OPPO | R1-2111349 | 60 | 10 |  | FR2-to-FR2 | 167 | 9 | Note 3 |
| OPPO | R1-2111349 | 60 | 10 |  | FR2-to-FR2 | 2112 | 126 | Note 3 |
| OPPO | R1-2111349 | 60 | 10 |  | FR2-to-FR2 | 4192 | 251 | Note 3 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR2 | 187 | 11 | Note 3 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR2 | 2132 | 127 | Note 3 |
| OPPO | R1-2111349 | 60 | 10 |  | FR1-to-FR2 | 4212 | 252 | Note 3 |
| Intel | R1-2111524 | 60 | 10 |  |  | 47 | 2.2 |  |
| Intel | R1-2111524 | 60 | 10 |  |  | 142 | 7.9 |  |
| Nokia | R1-2111828 | 60 | 10 | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 60 | 10 | Typical HO, Conditional HO |  | 42 | 1.93 |  |
| Nokia | R1-2111828 | 60 | 10 | Typical HO, Conditional HO |  | 62 | 3.13 |  |
| Nokia | R1-2111828 | 60 | 10 |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 60 | 10 |  |  | 20 | 0.60 |  |
| Nokia | R1-2111828 | 60 | 10 |  |  | 40 | 1.81 |  |
| Nokia | R1-2111828 | 60 | 10 |  |  | 60 | 3.01 |  |
| Nokia | R1-2111828 | 60 | 10 |  |  | 80 | 4.22 |  |
| Nokia | R1-2111828 | 60 | 10 |  |  | 100 | 5.42 |  |
| Ericsson | R1-2112160 | 60 | 10 |  |  | 2 | 0 |  |
| Ericsson | R1-2112160 | 60 | 10 |  |  | 20 | 1 |  |
| Ericsson | R1-2112160 | 60 | 10 |  |  | 40 | 2 |  |
| Ericsson | R1-2112160 | 60 | 10 |  |  | 60 | 3 |  |
| Ericsson | R1-2112160 | 60 | 10 |  |  | 80 | 5 |  |
| Ericsson | R1-2112160 | 60 | 10 |  |  | 100 | 6 |  |
| MediaTek | R1-2112296 | 60 | 10 |  |  | 60 | 3.01 |  |
| Note 1: the target cell is known Note 2: target cell is an unknown intra-frequency cell Note 3: target cell is an unknown inter-frequency cell Note 4: , is considered in the simulation results per agreement. |  |  |  |  |  |  |  |  |

Table 3 FPS=60, PDB=15ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | R1-2111046 | 60 | 15 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 2.22 |  |
| vivo | R1-2111046 | 60 | 15 | Typical HO | FR1-to-FR1 | 62 | 2.82 |  |
| vivo | R1-2111046 | 60 | 15 | Typical HO | FR1-to-FR1 | 82 | 4.02 |  |
| CATT | R1-2111234 | 60 | 15 | Typical HO | FR1-to-FR1 | 66.5 | 3.09 |  |
| CATT | R1-2111234 | 60 | 15 | Typical HO | FR2-to-FR1 | 86.5 | 4.29 |  |
| CATT | R1-2111234 | 60 | 15 | Typical HO | FR1-to-FR2 | 79.4 | 3.864 |  |
| CATT | R1-2111234 | 60 | 15 | Typical HO | FR2-to-FR2 | 59.4 | 2.664 |  |
| CATT | R1-2111234 | 60 | 15 | Conditional HO | FR1-to-FR1 | 56.5 | 2.49 |  |
| CATT | R1-2111234 | 60 | 15 | Conditional HO | FR2-to-FR1 | 76.5 | 3.69 |  |
| CATT | R1-2111234 | 60 | 15 | Conditional HO | FR1-to-FR2 | 69.4 | 3.264 |  |
| CATT | R1-2111234 | 60 | 15 | Conditional HO | FR2-to-FR2 | 49.4 | 2.064 |  |
| CMCC | R1-2111632 | 60 | 15 |  |  | 48 | 1.98 |  |
| Nokia | R1-2111828 | 60 | 15 | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 60 | 15 | Typical HO, Conditional HO |  | 42 | 1.63 |  |
| Nokia | R1-2111828 | 60 | 15 | Typical HO, Conditional HO |  | 62 | 2.83 |  |
| Nokia | R1-2111828 | 60 | 15 |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 60 | 15 |  |  | 20 | 0.30 |  |
| Nokia | R1-2111828 | 60 | 15 |  |  | 40 | 1.51 |  |
| Nokia | R1-2111828 | 60 | 15 |  |  | 60 | 2.71 |  |
| Nokia | R1-2111828 | 60 | 15 |  |  | 80 | 3.92 |  |
| Nokia | R1-2111828 | 60 | 15 |  |  | 100 | 5.12 |  |
| Ericsson | R1-2112160 | 60 | 15 |  |  | 2 | 0 |  |
| Ericsson | R1-2112160 | 60 | 15 |  |  | 20 | 1 |  |
| Ericsson | R1-2112160 | 60 | 15 |  |  | 40 | 2 |  |
| Ericsson | R1-2112160 | 60 | 15 |  |  | 60 | 3 |  |
| Ericsson | R1-2112160 | 60 | 15 |  |  | 80 | 4 |  |
| Ericsson | R1-2112160 | 60 | 15 |  |  | 100 | 6 |  |
| MediaTek | R1-2112296 | 60 | 15 |  |  | 60 | 2.71 |  |

Table 4 FPS=60, PDB=30ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | R1-2111046 | 60 | 30 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 1.32 |  |
| vivo | R1-2111046 | 60 | 30 | Typical HO | FR1-to-FR1 | 62 | 1.92 |  |
| vivo | R1-2111046 | 60 | 30 | Typical HO | FR1-to-FR1 | 82 | 3.12 |  |
| Nokia | R1-2111828 | 60 | 30 | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 60 | 30 | Typical HO, Conditional HO |  | 42 | 0.72 |  |
| Nokia | R1-2111828 | 60 | 30 | Typical HO, Conditional HO |  | 62 | 1.93 |  |
| Nokia | R1-2111828 | 60 | 30 |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 60 | 30 |  |  | 20 | 0.00 |  |
| Nokia | R1-2111828 | 60 | 30 |  |  | 40 | 0.60 |  |
| Nokia | R1-2111828 | 60 | 30 |  |  | 60 | 1.81 |  |
| Nokia | R1-2111828 | 60 | 30 |  |  | 80 | 3.01 |  |
| Nokia | R1-2111828 | 60 | 30 |  |  | 100 | 4.22 |  |
| Ericsson | R1-2112160 | 60 | 30 |  |  | 2 | 0 |  |
| Ericsson | R1-2112160 | 60 | 30 |  |  | 20 | 0 |  |
| Ericsson | R1-2112160 | 60 | 30 |  |  | 40 | 1 |  |
| Ericsson | R1-2112160 | 60 | 30 |  |  | 60 | 2 |  |
| Ericsson | R1-2112160 | 60 | 30 |  |  | 80 | 3 |  |
| Ericsson | R1-2112160 | 60 | 30 |  |  | 100 | 5 |  |
| MediaTek | R1-2112296 | 60 | 30 |  |  | 60 | 1.81 |  |

Table 5 FPS=90, PDB=10/15/30ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | R1-2111046 | 90 | 10 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 3.78 |  |
| vivo | R1-2111046 | 90 | 10 | Typical HO | FR1-to-FR1 | 62 | 4.68 |  |
| vivo | R1-2111046 | 90 | 10 | Typical HO | FR1-to-FR1 | 82 | 6.48 |  |
| vivo | R1-2111046 | 90 | 15 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 3.33 |  |
| vivo | R1-2111046 | 90 | 15 | Typical HO | FR1-to-FR1 | 62 | 4.23 |  |
| vivo | R1-2111046 | 90 | 15 | Typical HO | FR1-to-FR1 | 82 | 6.03 |  |
| vivo | R1-2111046 | 90 | 30 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 1.98 |  |
| vivo | R1-2111046 | 90 | 30 | Typical HO | FR1-to-FR1 | 62 | 2.88 |  |
| vivo | R1-2111046 | 90 | 30 | Typical HO | FR1-to-FR1 | 82 | 4.68 |  |

Table 6 FPS=120, PDB=10/15/30ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | R1-2111046 | 120 | 10 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 5.04 |  |
| vivo | R1-2111046 | 120 | 10 | Typical HO | FR1-to-FR1 | 62 | 6.24 |  |
| vivo | R1-2111046 | 120 | 10 | Typical HO | FR1-to-FR1 | 82 | 8.64 |  |
| vivo | R1-2111046 | 120 | 15 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 4.44 |  |
| vivo | R1-2111046 | 120 | 15 | Typical HO | FR1-to-FR1 | 62 | 5.64 |  |
| vivo | R1-2111046 | 120 | 15 | Typical HO | FR1-to-FR1 | 82 | 8.04 |  |
| vivo | R1-2111046 | 120 | 30 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 2.64 |  |
| vivo | R1-2111046 | 120 | 30 | Typical HO | FR1-to-FR1 | 62 | 3.84 |  |
| vivo | R1-2111046 | 120 | 30 | Typical HO | FR1-to-FR1 | 82 | 6.24 |  |
| ZTE | R1-2111351 | 120 | 10 |  |  | 0 | 0 | Note 1 |
| ZTE | R1-2111351 | 120 | 10 |  |  | 2 | 0.24 | Note 1 |
| ZTE | R1-2111351 | 120 | 10 |  |  | 4 | 0.48 | Note 1 |
| ZTE | R1-2111351 | 120 | 10 |  |  | 6 | 0.72 | Note 1 |
| ZTE | R1-2111351 | 120 | 10 |  |  | 8 | 0.96 | Note 1 |
| ZTE | R1-2111351 | 120 | 10 |  |  | 10 | 1.2 | Note 1 |
| Note 1: , is considered in the simulation results per agreement. |  |  |  |  |  |  |  |  |

Table 7 FPS=250, PDB=10ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | R1-2111046 | 250 | 10 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 10.50 |  |
| vivo | R1-2111046 | 250 | 10 | Typical HO | FR1-to-FR1 | 62 | 13.00 |  |
| vivo | R1-2111046 | 250 | 10 | Typical HO | FR1-to-FR1 | 82 | 18.00 |  |
| Nokia | R1-2111828 | 250 | 10 | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 250 | 10 | Typical HO, Conditional HO |  | 42 | 8.00 |  |
| Nokia | R1-2111828 | 250 | 10 | Typical HO, Conditional HO |  | 62 | 13.00 |  |
| Nokia | R1-2111828 | 250 | 10 |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 250 | 10 |  |  | 20 | 2.50 |  |
| Nokia | R1-2111828 | 250 | 10 |  |  | 40 | 7.50 |  |
| Nokia | R1-2111828 | 250 | 10 |  |  | 60 | 12.50 |  |
| Nokia | R1-2111828 | 250 | 10 |  |  | 80 | 17.50 |  |
| Nokia | R1-2111828 | 250 | 10 |  |  | 100 | 22.50 |  |
| Ericsson | R1-2112160 | 250 | 10 |  |  | 2 | 0 |  |
| Ericsson | R1-2112160 | 250 | 10 |  |  | 20 | 3 |  |
| Ericsson | R1-2112160 | 250 | 10 |  |  | 40 | 8 |  |
| Ericsson | R1-2112160 | 250 | 10 |  |  | 60 | 13 |  |
| Ericsson | R1-2112160 | 250 | 10 |  |  | 80 | 18 |  |
| Ericsson | R1-2112160 | 250 | 10 |  |  | 100 | 23 |  |
| MediaTek | R1-2112296 | 250 | 10 |  |  | 60 | 12.5 |  |

  - Minimum target time interval between HO events, T

Table 8  X=99%, PDB=10ms

| Company | Tdoc | PDB (ms) | PE,op | X | Handover type | Handover case | Y (ms) | T (s) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | R1-2111046 | 10 | 0.00% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 4.20 |  |
| vivo | R1-2111046 | 10 | 0.00% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 5.20 |  |
| vivo | R1-2111046 | 10 | 0.00% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 7.20 |  |
| vivo | R1-2111046 | 10 | 0.10% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 4.66 |  |
| vivo | R1-2111046 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 5.77 |  |
| vivo | R1-2111046 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 7.99 |  |
| vivo | R1-2111046 | 10 | 0.50% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 8.36 |  |
| vivo | R1-2111046 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 10.35 |  |
| vivo | R1-2111046 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 14.33 |  |
| vivo | R1-2111046 | 10 | 0.90% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 41.62 |  |
| vivo | R1-2111046 | 10 | 0.90% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 51.53 |  |
| vivo | R1-2111046 | 10 | 0.90% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 71.35 |  |
| Huawei | R1-2110811 | 10 | 0.00% | 99.0% |  |  | 100 | 9.00 |  |
| Huawei | R1-2110811 | 10 | 0.00% | 99.0% |  |  | 80 | 7.00 |  |
| Huawei | R1-2110811 | 10 | 0.00% | 99.0% |  |  | 70 | 6.00 |  |
| Huawei | R1-2110811 | 10 | 0.00% | 99.0% |  |  | 60 | 5.00 |  |
| Huawei | R1-2110811 | 10 | 0.00% | 99.0% |  |  | 40 | 3.00 |  |
| Huawei | R1-2110811 | 10 | 0.00% | 99.0% |  |  | 20 | 1.00 |  |
| Huawei | R1-2110811 | 10 | 0.00% | 99.0% |  |  | <= 10 | 0.00 |  |
| Huawei | R1-2110811 | 10 | 0.14% | 99.0% |  |  | 100 | 10.45 |  |
| Huawei | R1-2110811 | 10 | 0.14% | 99.0% |  |  | 80 | 8.13 |  |
| Huawei | R1-2110811 | 10 | 0.14% | 99.0% |  |  | 70 | 6.97 |  |
| Huawei | R1-2110811 | 10 | 0.14% | 99.0% |  |  | 60 | 5.81 |  |
| Huawei | R1-2110811 | 10 | 0.14% | 99.0% |  |  | 40 | 3.48 |  |
| Huawei | R1-2110811 | 10 | 0.14% | 99.0% |  |  | 20 | 1.16 |  |
| Huawei | R1-2110811 | 10 | 0.14% | 99.0% |  |  | <= 10 | 0.00 |  |
| CATT | R1-2111234 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR1 | 66.5 | 4.697 |  |
| CATT | R1-2111234 | 10 | 0.01% | 99.0% | Typical HO | FR2-to-FR2 | 59.4 | 3.979 |  |
| CATT | R1-2111234 | 10 | 0.03% | 99.0% | Typical HO | FR1-to-FR1 | 66.5 | 4.792 |  |
| CATT | R1-2111234 | 10 | 0.03% | 99.0% | Typical HO | FR2-to-FR2 | 59.4 | 4.061 |  |
| CATT | R1-2111234 | 10 | 0.31% | 99.0% | Typical HO | FR1-to-FR1 | 66.5 | 6.718 |  |
| CATT | R1-2111234 | 10 | 0.31% | 99.0% | Typical HO | FR2-to-FR2 | 59.4 | 5.692 |  |
| CATT | R1-2111234 | 10 | 0.01% | 99.0% | Conditional HO | FR1-to-FR1 | 56.5 | 4.697 |  |
| CATT | R1-2111234 | 10 | 0.01% | 99.0% | Conditional HO | FR2-to-FR2 | 49.4 | 3.979 |  |
| CATT | R1-2111234 | 10 | 0.03% | 99.0% | Conditional HO | FR1-to-FR1 | 56.5 | 4.997 |  |
| CATT | R1-2111234 | 10 | 0.03% | 99.0% | Conditional HO | FR2-to-FR2 | 49.4 | 4.234 |  |
| CATT | R1-2111234 | 10 | 0.31% | 99.0% | Conditional HO | FR1-to-FR1 | 56.5 | 6.185 |  |
| CATT | R1-2111234 | 10 | 0.31% | 99.0% | Conditional HO | FR2-to-FR2 | 49.4 | 5.24 |  |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR1, FR2-to-FR2 | 47 | 4 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR1/FR2-to-FR3 | 192 | 18 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR1, FR2-to-FR4 | 352 | 35 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR2, FR1-to-FR2 | 67 | 6 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR2, FR1-to-FR2 | 212 | 20 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR2, FR1-to-FR2 | 372 | 37 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR1, FR2-to-FR2 | 47 | 4 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR1, FR2-to-FR3 | 192 | 20 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR1, FR2-to-FR4 | 352 | 38 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR2, FR1-to-FR2 | 67 | 6 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR2, FR1-to-FR2 | 212 | 22 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR2, FR1-to-FR2 | 372 | 40 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR1, FR2-to-FR2 | 47 | 7 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR1/FR2-to-FR3 | 192 | 36 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR1, FR2-to-FR4 | 352 | 68 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR2, FR1-to-FR2 | 67 | 11 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR2, FR1-to-FR2 | 212 | 40 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR2, FR1-to-FR2 | 372 | 72 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR1 | 52 | 4 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR1 | 272 | 26 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR1 | 512 | 51 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR2-to-FR2 | 87 | 8 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR2-to-FR2 | 832 | 83 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR2-to-FR2 | 1632 | 164 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR1 | 52 | 5 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR1 | 272 | 29 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR1 | 512 | 56 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR2-to-FR2 | 87 | 9 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR2-to-FR2 | 832 | 91 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR2-to-FR2 | 1632 | 180 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR1 | 52 | 8 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR1 | 272 | 52 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR1 | 512 | 100 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR2-to-FR2 | 87 | 15 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR2-to-FR2 | 832 | 164 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR2-to-FR2 | 1632 | 323 | Note 2 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR1 | 62 | 5 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR1 | 432 | 43 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR1 | 832 | 83 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR1 | 62 | 6 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR1 | 432 | 47 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR1 | 832 | 91 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR1 | 62 | 10 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR1 | 432 | 84 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR1 | 832 | 164 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR2-to-FR1 | 82 | 7 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR2-to-FR1 | 452 | 45 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR2-to-FR1 | 852 | 85 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR2-to-FR1 | 82 | 8 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR2-to-FR1 | 452 | 49 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR2-to-FR1 | 852 | 93 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR2-to-FR1 | 82 | 14 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR2-to-FR1 | 452 | 88 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR2-to-FR1 | 852 | 168 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR2-to-FR2 | 167 | 16 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR2-to-FR2 | 2112 | 212 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR2-to-FR2 | 4192 | 422 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR2-to-FR2 | 167 | 17 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR2-to-FR2 | 2112 | 233 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR2-to-FR2 | 4192 | 464 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR2-to-FR2 | 167 | 31 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR2-to-FR2 | 2112 | 418 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR2-to-FR2 | 4192 | 832 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR2 | 187 | 18 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR2 | 2132 | 214 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.01% | 99.0% |  | FR1-to-FR2 | 4212 | 424 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR2 | 187 | 20 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR2 | 2132 | 236 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.10% | 99.0% |  | FR1-to-FR2 | 4212 | 466 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR2 | 187 | 35 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR2 | 2132 | 422 | Note 3 |
| OPPO | R1-2111349 | 10 | 0.50% | 99.0% |  | FR1-to-FR2 | 4212 | 836 | Note 3 |
| Nokia | R1-2111828 | 10 | 0.00% | 99.0% | DAPS |  | 2 | 0 |  |
| Nokia | R1-2111828 | 10 | 0.00% | 99.0% | Typical HO, Conditional HO |  | 42 | 3.2 |  |
| Nokia | R1-2111828 | 10 | 0.00% | 99.0% | Typical HO, Conditional HO |  | 62 | 5.2 |  |
| Nokia | R1-2111828 | 10 | 0.00% | 99.0% |  |  | 10 | 0 |  |
| Nokia | R1-2111828 | 10 | 0.00% | 99.0% |  |  | 20 | 1 |  |
| Nokia | R1-2111828 | 10 | 0.00% | 99.0% |  |  | 40 | 3 |  |
| Nokia | R1-2111828 | 10 | 0.00% | 99.0% |  |  | 60 | 5 |  |
| Nokia | R1-2111828 | 10 | 0.00% | 99.0% |  |  | 80 | 7 |  |
| Nokia | R1-2111828 | 10 | 0.00% | 99.0% |  |  | 100 | 9 |  |
| Nokia | R1-2111828 | 10 | 0.20% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 10 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 42 | 3.99 |  |
| Nokia | R1-2111828 | 10 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 62 | 6.49 |  |
| Nokia | R1-2111828 | 10 | 0.20% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 10 | 0.20% | 99.0% |  |  | 20 | 1.25 |  |
| Nokia | R1-2111828 | 10 | 0.20% | 99.0% |  |  | 40 | 3.74 |  |
| Nokia | R1-2111828 | 10 | 0.20% | 99.0% |  |  | 60 | 6.24 |  |
| Nokia | R1-2111828 | 10 | 0.20% | 99.0% |  |  | 80 | 8.73 |  |
| Nokia | R1-2111828 | 10 | 0.20% | 99.0% |  |  | 100 | 11.23 |  |
| Nokia | R1-2111828 | 10 | 0.40% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 10 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 42 | 5.31 |  |
| Nokia | R1-2111828 | 10 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 62 | 8.63 |  |
| Nokia | R1-2111828 | 10 | 0.40% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 10 | 0.40% | 99.0% |  |  | 20 | 1.66 |  |
| Nokia | R1-2111828 | 10 | 0.40% | 99.0% |  |  | 40 | 4.98 |  |
| Nokia | R1-2111828 | 10 | 0.40% | 99.0% |  |  | 60 | 8.30 |  |
| Nokia | R1-2111828 | 10 | 0.40% | 99.0% |  |  | 80 | 11.62 |  |
| Nokia | R1-2111828 | 10 | 0.40% | 99.0% |  |  | 100 | 14.94 |  |
| Nokia | R1-2111828 | 10 | 0.60% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 10 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 42 | 7.95 |  |
| Nokia | R1-2111828 | 10 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 62 | 12.92 |  |
| Nokia | R1-2111828 | 10 | 0.60% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 10 | 0.60% | 99.0% |  |  | 20 | 2.48 |  |
| Nokia | R1-2111828 | 10 | 0.60% | 99.0% |  |  | 40 | 7.45 |  |
| Nokia | R1-2111828 | 10 | 0.60% | 99.0% |  |  | 60 | 12.43 |  |
| Nokia | R1-2111828 | 10 | 0.60% | 99.0% |  |  | 80 | 17.40 |  |
| Nokia | R1-2111828 | 10 | 0.60% | 99.0% |  |  | 100 | 22.36 |  |
| Nokia | R1-2111828 | 10 | 0.80% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 10 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 42 | 15.87 |  |
| Nokia | R1-2111828 | 10 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 62 | 25.79 |  |
| Nokia | R1-2111828 | 10 | 0.80% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 10 | 0.80% | 99.0% |  |  | 20 | 4.96 |  |
| Nokia | R1-2111828 | 10 | 0.80% | 99.0% |  |  | 40 | 14.88 |  |
| Nokia | R1-2111828 | 10 | 0.80% | 99.0% |  |  | 60 | 24.80 |  |
| Nokia | R1-2111828 | 10 | 0.80% | 99.0% |  |  | 80 | 34.72 |  |
| Nokia | R1-2111828 | 10 | 0.80% | 99.0% |  |  | 100 | 44.64 |  |
| Nokia | R1-2111828 | 10 | 0.90% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 10 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 42 | 31.71 |  |
| Nokia | R1-2111828 | 10 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 62 | 51.53 |  |
| Nokia | R1-2111828 | 10 | 0.90% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 10 | 0.90% | 99.0% |  |  | 20 | 9.91 |  |
| Nokia | R1-2111828 | 10 | 0.90% | 99.0% |  |  | 40 | 29.73 |  |
| Nokia | R1-2111828 | 10 | 0.90% | 99.0% |  |  | 60 | 49.55 |  |
| Nokia | R1-2111828 | 10 | 0.90% | 99.0% |  |  | 80 | 69.37 |  |
| Nokia | R1-2111828 | 10 | 0.90% | 99.0% |  |  | 100 | 89.19 |  |
| Ericsson | R1-2112160 | 10 | 0.00% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 10 | 0.00% | 99.0% |  |  | 20 | 1.0 |  |
| Ericsson | R1-2112160 | 10 | 0.00% | 99.0% |  |  | 40 | 3.0 |  |
| Ericsson | R1-2112160 | 10 | 0.00% | 99.0% |  |  | 60 | 5.0 |  |
| Ericsson | R1-2112160 | 10 | 0.00% | 99.0% |  |  | 80 | 7.0 |  |
| Ericsson | R1-2112160 | 10 | 0.00% | 99.0% |  |  | 100 | 9.0 |  |
| Ericsson | R1-2112160 | 10 | 0.20% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 10 | 0.20% | 99.0% |  |  | 20 | 1.2 |  |
| Ericsson | R1-2112160 | 10 | 0.20% | 99.0% |  |  | 40 | 3.7 |  |
| Ericsson | R1-2112160 | 10 | 0.20% | 99.0% |  |  | 60 | 6.2 |  |
| Ericsson | R1-2112160 | 10 | 0.20% | 99.0% |  |  | 80 | 8.7 |  |
| Ericsson | R1-2112160 | 10 | 0.20% | 99.0% |  |  | 100 | 11.2 |  |
| Ericsson | R1-2112160 | 10 | 0.40% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 10 | 0.40% | 99.0% |  |  | 20 | 1.7 |  |
| Ericsson | R1-2112160 | 10 | 0.40% | 99.0% |  |  | 40 | 5.0 |  |
| Ericsson | R1-2112160 | 10 | 0.40% | 99.0% |  |  | 60 | 8.3 |  |
| Ericsson | R1-2112160 | 10 | 0.40% | 99.0% |  |  | 80 | 11.6 |  |
| Ericsson | R1-2112160 | 10 | 0.40% | 99.0% |  |  | 100 | 14.9 |  |
| Ericsson | R1-2112160 | 10 | 0.60% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 10 | 0.60% | 99.0% |  |  | 20 | 2.5 |  |
| Ericsson | R1-2112160 | 10 | 0.60% | 99.0% |  |  | 40 | 7.5 |  |
| Ericsson | R1-2112160 | 10 | 0.60% | 99.0% |  |  | 60 | 12.4 |  |
| Ericsson | R1-2112160 | 10 | 0.60% | 99.0% |  |  | 80 | 17.4 |  |
| Ericsson | R1-2112160 | 10 | 0.60% | 99.0% |  |  | 100 | 22.4 |  |
| Ericsson | R1-2112160 | 10 | 0.80% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 10 | 0.80% | 99.0% |  |  | 20 | 5.0 |  |
| Ericsson | R1-2112160 | 10 | 0.80% | 99.0% |  |  | 40 | 14.9 |  |
| Ericsson | R1-2112160 | 10 | 0.80% | 99.0% |  |  | 60 | 24.8 |  |
| Ericsson | R1-2112160 | 10 | 0.80% | 99.0% |  |  | 80 | 34.7 |  |
| Ericsson | R1-2112160 | 10 | 0.80% | 99.0% |  |  | 100 | 44.6 |  |
| Ericsson | R1-2112160 | 10 | 0.90% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 10 | 0.90% | 99.0% |  |  | 20 | 9.9 |  |
| Ericsson | R1-2112160 | 10 | 0.90% | 99.0% |  |  | 40 | 29.7 |  |
| Ericsson | R1-2112160 | 10 | 0.90% | 99.0% |  |  | 60 | 49.5 |  |
| Ericsson | R1-2112160 | 10 | 0.90% | 99.0% |  |  | 80 | 69.4 |  |
| Ericsson | R1-2112160 | 10 | 0.90% | 99.0% |  |  | 100 | 89.2 |  |
| MediaTek | R1-2112296 | 10 | 0.00% | 99.0% |  |  | 60 | 5 |  |
| MediaTek | R1-2112296 | 10 | 0.0% | 99.0% |  |  | 60 | 5 |  |
| MediaTek | R1-2112296 | 10 | 0.5% | 99.0% |  |  | 60 | 9.95 |  |
| MediaTek | R1-2112296 | 10 | 0.5% | 99.0% |  |  | 60 | 9.95 |  |
| MediaTek | R1-2112296 | 10 | 0.9% | 99.0% |  |  | 60 | 49.55 |  |
| MediaTek | R1-2112296 | 10 | 0.9% | 99.0% |  |  | 60 | 49.55 |  |
| Intel | R1-2111524 | 10 | 0.01% | 99.0% | Typical HO | FR1-FR1 | 47 | 3.73 | Note 1 |
| Intel | R1-2111525 | 10 | 0.10% | 99.0% | Typical HO | FR1-FR1 | 47 | 4.107 | Note 1 |
| Intel | R1-2111526 | 10 | 0.20% | 99.0% | Typical HO | FR1-FR1 | 47 | 4.616 | Note 1 |
| Intel | R1-2111527 | 10 | 0.30% | 99.0% | Typical HO | FR1-FR1 | 47 | 5.27 | Note 1 |
| Intel | R1-2111528 | 10 | 0.40% | 99.0% | Typical HO | FR1-FR1 | 47 | 6.142 | Note 1 |
| Intel | R1-2111529 | 10 | 0.50% | 99.0% | Typical HO | FR1-FR1 | 47 | 7.363 | Note 1 |
| Intel | R1-2111530 | 10 | 0.60% | 99.0% | Typical HO | FR1-FR1 | 47 | 9.195 | Note 1 |
| Intel | R1-2111531 | 10 | 0.70% | 99.0% | Typical HO | FR1-FR1 | 47 | 12.25 | Note 1 |
| Intel | R1-2111532 | 10 | 0.80% | 99.0% | Typical HO | FR1-FR1 | 47 | 18.35 | Note 1 |
| Intel | R1-2111533 | 10 | 0.90% | 99.0% | Typical HO | FR1-FR1 | 47 | 36.67 | Note 1 |
| Intel | R1-2111524 | 10 | 0.01% | 99.0% | Typical HO | FR1-FR1 | 142 | 14.34 | Note 1 |
| Intel | R1-2111525 | 10 | 0.10% | 99.0% | Typical HO | FR1-FR1 | 142 | 15.76 | Note 1 |
| Intel | R1-2111526 | 10 | 0.20% | 99.0% | Typical HO | FR1-FR1 | 142 | 17.71 | Note 1 |
| Intel | R1-2111527 | 10 | 0.30% | 99.0% | Typical HO | FR1-FR1 | 142 | 20.22 | Note 1 |
| Intel | R1-2111528 | 10 | 0.40% | 99.0% | Typical HO | FR1-FR1 | 142 | 23.57 | Note 1 |
| Intel | R1-2111529 | 10 | 0.50% | 99.0% | Typical HO | FR1-FR1 | 142 | 28.83 | Note 1 |
| Intel | R1-2111530 | 10 | 0.60% | 99.0% | Typical HO | FR1-FR1 | 142 | 35.29 | Note 1 |
| Intel | R1-2111531 | 10 | 0.70% | 99.0% | Typical HO | FR1-FR1 | 142 | 47 | Note 1 |
| Intel | R1-2111532 | 10 | 0.80% | 99.0% | Typical HO | FR1-FR1 | 142 | 70.43 | Note 1 |
| Intel | R1-2111533 | 10 | 0.90% | 99.0% | Typical HO | FR1-FR1 | 142 | 140.7 | Note 1 |
| Note 1: the target cell is known Note 2: target cell is an unknown intra-frequency cell Note 3: target cell is an unknown inter-frequency cell |  |  |  |  |  |  |  |  |  |

Table 9  X=99%, PDB=15ms

| Company | Tdoc | PDB (ms) | PE,op | X | Handover type | Handover case | Y (ms) | T (s) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | R1-2111046 | 15 | 0.00% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 3.70 |  |
| vivo | R1-2111046 | 15 | 0.00% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 4.70 |  |
| vivo | R1-2111046 | 15 | 0.00% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 6.70 |  |
| vivo | R1-2111046 | 15 | 0.10% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 4.11 |  |
| vivo | R1-2111046 | 15 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 5.22 |  |
| vivo | R1-2111046 | 15 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 7.44 |  |
| vivo | R1-2111046 | 15 | 0.50% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 7.36 |  |
| vivo | R1-2111046 | 15 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 9.35 |  |
| vivo | R1-2111046 | 15 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 13.33 |  |
| vivo | R1-2111046 | 15 | 0.90% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 36.67 |  |
| vivo | R1-2111046 | 15 | 0.90% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 46.58 |  |
| vivo | R1-2111046 | 15 | 0.90% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 66.40 |  |
| CATT | R1-2111234 | 15 | 0.01% | 99.0% | Typical HO | FR1-to-FR1 | 66.5 | 4.192 |  |
| CATT | R1-2111234 | 15 | 0.01% | 99.0% | Typical HO | FR2-to-FR2 | 59.4 | 3.474 |  |
| CATT | R1-2111234 | 15 | 0.07% | 99.0% | Typical HO | FR1-to-FR1 | 66.5 | 4.459 |  |
| CATT | R1-2111234 | 15 | 0.07% | 99.0% | Typical HO | FR2-to-FR2 | 59.4 | 3.696 |  |
| CATT | R1-2111234 | 15 | 0.25% | 99.0% | Typical HO | FR1-to-FR1 | 66.5 | 5.52 |  |
| CATT | R1-2111234 | 15 | 0.25% | 99.0% | Typical HO | FR2-to-FR2 | 59.4 | 4.575 |  |
| CATT | R1-2111234 | 15 | 0.01% | 99.0% | Conditional HO | FR1-to-FR1 | 56.5 | 4.192 |  |
| CATT | R1-2111234 | 15 | 0.01% | 99.0% | Conditional HO | FR2-to-FR2 | 49.4 | 3.474 |  |
| CATT | R1-2111234 | 15 | 0.07% | 99.0% | Conditional HO | FR1-to-FR1 | 56.5 | 4.277 |  |
| CATT | R1-2111234 | 15 | 0.07% | 99.0% | Conditional HO | FR2-to-FR2 | 49.4 | 3.545 |  |
| CATT | R1-2111234 | 15 | 0.25% | 99.0% | Conditional HO | FR1-to-FR1 | 56.5 | 5.996 |  |
| CATT | R1-2111234 | 15 | 0.25% | 99.0% | Conditional HO | FR2-to-FR2 | 49.4 | 4.97 |  |
| CMCC | R1-2111632 | 15 | 0.10% | 99.0% |  |  | 48 | 3.663 |  |
| Nokia | R1-2111828 | 15 | 0% | 99.0% | DAPS |  | 2 | 0 |  |
| Nokia | R1-2111828 | 15 | 0% | 99.0% | Typical HO, Conditional HO |  | 42 | 2.7 |  |
| Nokia | R1-2111828 | 15 | 0% | 99.0% | Typical HO, Conditional HO |  | 62 | 4.7 |  |
| Nokia | R1-2111828 | 15 | 0% | 99.0% |  |  | 10 | 0 |  |
| Nokia | R1-2111828 | 15 | 0% | 99.0% |  |  | 20 | 0.5 |  |
| Nokia | R1-2111828 | 15 | 0% | 99.0% |  |  | 40 | 2.5 |  |
| Nokia | R1-2111828 | 15 | 0% | 99.0% |  |  | 60 | 4.5 |  |
| Nokia | R1-2111828 | 15 | 0% | 99.0% |  |  | 80 | 6.5 |  |
| Nokia | R1-2111828 | 15 | 0% | 99.0% |  |  | 100 | 8.5 |  |
| Nokia | R1-2111828 | 15 | 0.20% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 15 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 42 | 3.37 |  |
| Nokia | R1-2111828 | 15 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 62 | 5.86 |  |
| Nokia | R1-2111828 | 15 | 0.20% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 15 | 0.20% | 99.0% |  |  | 20 | 0.62 |  |
| Nokia | R1-2111828 | 15 | 0.20% | 99.0% |  |  | 40 | 3.12 |  |
| Nokia | R1-2111828 | 15 | 0.20% | 99.0% |  |  | 60 | 5.61 |  |
| Nokia | R1-2111828 | 15 | 0.20% | 99.0% |  |  | 80 | 8.11 |  |
| Nokia | R1-2111828 | 15 | 0.20% | 99.0% |  |  | 100 | 10.60 |  |
| Nokia | R1-2111828 | 15 | 0.40% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 15 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 42 | 4.48 |  |
| Nokia | R1-2111828 | 15 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 62 | 7.80 |  |
| Nokia | R1-2111828 | 15 | 0.40% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 15 | 0.40% | 99.0% |  |  | 20 | 0.83 |  |
| Nokia | R1-2111828 | 15 | 0.40% | 99.0% |  |  | 40 | 4.15 |  |
| Nokia | R1-2111828 | 15 | 0.40% | 99.0% |  |  | 60 | 7.47 |  |
| Nokia | R1-2111828 | 15 | 0.40% | 99.0% |  |  | 80 | 10.79 |  |
| Nokia | R1-2111828 | 15 | 0.40% | 99.0% |  |  | 100 | 14.11 |  |
| Nokia | R1-2111828 | 15 | 0.60% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 15 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 42 | 6.71 |  |
| Nokia | R1-2111828 | 15 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 62 | 11.68 |  |
| Nokia | R1-2111828 | 15 | 0.60% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 15 | 0.60% | 99.0% |  |  | 20 | 1.24 |  |
| Nokia | R1-2111828 | 15 | 0.60% | 99.0% |  |  | 40 | 6.21 |  |
| Nokia | R1-2111828 | 15 | 0.60% | 99.0% |  |  | 60 | 11.18 |  |
| Nokia | R1-2111828 | 15 | 0.60% | 99.0% |  |  | 80 | 16.15 |  |
| Nokia | R1-2111828 | 15 | 0.60% | 99.0% |  |  | 100 | 21.12 |  |
| Nokia | R1-2111828 | 15 | 0.80% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 15 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 42 | 13.39 |  |
| Nokia | R1-2111828 | 15 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 62 | 23.31 |  |
| Nokia | R1-2111828 | 15 | 0.80% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 15 | 0.80% | 99.0% |  |  | 20 | 2.48 |  |
| Nokia | R1-2111828 | 15 | 0.80% | 99.0% |  |  | 40 | 12.40 |  |
| Nokia | R1-2111828 | 15 | 0.80% | 99.0% |  |  | 60 | 22.32 |  |
| Nokia | R1-2111828 | 15 | 0.80% | 99.0% |  |  | 80 | 32.24 |  |
| Nokia | R1-2111828 | 15 | 0.80% | 99.0% |  |  | 100 | 42.16 |  |
| Nokia | R1-2111828 | 15 | 0.90% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 15 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 42 | 26.76 |  |
| Nokia | R1-2111828 | 15 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 62 | 46.58 |  |
| Nokia | R1-2111828 | 15 | 0.90% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 15 | 0.90% | 99.0% |  |  | 20 | 4.95 |  |
| Nokia | R1-2111828 | 15 | 0.90% | 99.0% |  |  | 40 | 24.77 |  |
| Nokia | R1-2111828 | 15 | 0.90% | 99.0% |  |  | 60 | 44.59 |  |
| Nokia | R1-2111828 | 15 | 0.90% | 99.0% |  |  | 80 | 64.41 |  |
| Nokia | R1-2111828 | 15 | 0.90% | 99.0% |  |  | 100 | 84.23 |  |
| Ericsson | R1-2112160 | 15 | 0.00% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 15 | 0.00% | 99.0% |  |  | 20 | 0.5 |  |
| Ericsson | R1-2112160 | 15 | 0.00% | 99.0% |  |  | 40 | 2.5 |  |
| Ericsson | R1-2112160 | 15 | 0.00% | 99.0% |  |  | 60 | 4.5 |  |
| Ericsson | R1-2112160 | 15 | 0.00% | 99.0% |  |  | 80 | 6.5 |  |
| Ericsson | R1-2112160 | 15 | 0.00% | 99.0% |  |  | 100 | 8.5 |  |
| Ericsson | R1-2112160 | 15 | 0.20% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 15 | 0.20% | 99.0% |  |  | 20 | 0.6 |  |
| Ericsson | R1-2112160 | 15 | 0.20% | 99.0% |  |  | 40 | 3.1 |  |
| Ericsson | R1-2112160 | 15 | 0.20% | 99.0% |  |  | 60 | 5.6 |  |
| Ericsson | R1-2112160 | 15 | 0.20% | 99.0% |  |  | 80 | 8.1 |  |
| Ericsson | R1-2112160 | 15 | 0.20% | 99.0% |  |  | 100 | 10.6 |  |
| Ericsson | R1-2112160 | 15 | 0.40% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 15 | 0.40% | 99.0% |  |  | 20 | 0.8 |  |
| Ericsson | R1-2112160 | 15 | 0.40% | 99.0% |  |  | 40 | 4.1 |  |
| Ericsson | R1-2112160 | 15 | 0.40% | 99.0% |  |  | 60 | 7.5 |  |
| Ericsson | R1-2112160 | 15 | 0.40% | 99.0% |  |  | 80 | 10.8 |  |
| Ericsson | R1-2112160 | 15 | 0.40% | 99.0% |  |  | 100 | 14.1 |  |
| Ericsson | R1-2112160 | 15 | 0.60% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 15 | 0.60% | 99.0% |  |  | 20 | 1.2 |  |
| Ericsson | R1-2112160 | 15 | 0.60% | 99.0% |  |  | 40 | 6.2 |  |
| Ericsson | R1-2112160 | 15 | 0.60% | 99.0% |  |  | 60 | 11.2 |  |
| Ericsson | R1-2112160 | 15 | 0.60% | 99.0% |  |  | 80 | 16.2 |  |
| Ericsson | R1-2112160 | 15 | 0.60% | 99.0% |  |  | 100 | 21.1 |  |
| Ericsson | R1-2112160 | 15 | 0.80% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 15 | 0.80% | 99.0% |  |  | 20 | 2.5 |  |
| Ericsson | R1-2112160 | 15 | 0.80% | 99.0% |  |  | 40 | 12.4 |  |
| Ericsson | R1-2112160 | 15 | 0.80% | 99.0% |  |  | 60 | 22.3 |  |
| Ericsson | R1-2112160 | 15 | 0.80% | 99.0% |  |  | 80 | 32.2 |  |
| Ericsson | R1-2112160 | 15 | 0.80% | 99.0% |  |  | 100 | 42.2 |  |
| Ericsson | R1-2112160 | 15 | 0.90% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 15 | 0.90% | 99.0% |  |  | 20 | 5.0 |  |
| Ericsson | R1-2112160 | 15 | 0.90% | 99.0% |  |  | 40 | 24.8 |  |
| Ericsson | R1-2112160 | 15 | 0.90% | 99.0% |  |  | 60 | 44.6 |  |
| Ericsson | R1-2112160 | 15 | 0.90% | 99.0% |  |  | 80 | 64.4 |  |
| Ericsson | R1-2112160 | 15 | 0.90% | 99.0% |  |  | 100 | 84.2 |  |
| MediaTek | R1-2112296 | 15 | 0.0% | 99.0% |  |  | 60 | 4.5 |  |
| MediaTek | R1-2112296 | 15 | 0.5% | 99.0% |  |  | 60 | 8.955 |  |
| MediaTek | R1-2112296 | 15 | 0.9% | 99.0% |  |  | 60 | 44.595 |  |

Table 9  X=99%, PDB=30ms

| Company | Tdoc | PDB (ms) | PE,op | X | Handover type | Handover case | Y (ms) | T (s) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | R1-2111046 | 30 | 0.00% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 2.20 |  |
| vivo | R1-2111046 | 30 | 0.00% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 3.20 |  |
| vivo | R1-2111046 | 30 | 0.00% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 5.20 |  |
| vivo | R1-2111046 | 30 | 0.10% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 2.44 |  |
| vivo | R1-2111046 | 30 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 3.55 |  |
| vivo | R1-2111046 | 30 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 5.77 |  |
| vivo | R1-2111046 | 30 | 0.50% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 4.38 |  |
| vivo | R1-2111046 | 30 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 6.37 |  |
| vivo | R1-2111046 | 30 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 10.35 |  |
| vivo | R1-2111046 | 30 | 0.90% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 21.80 |  |
| vivo | R1-2111046 | 30 | 0.90% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 31.71 |  |
| vivo | R1-2111046 | 30 | 0.90% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 51.53 |  |
| Nokia | R1-2111828 | 30 | 0% | 99.0% | DAPS |  | 2 | 0 |  |
| Nokia | R1-2111828 | 30 | 0% | 99.0% | Typical HO, Conditional HO |  | 42 | 1.2 |  |
| Nokia | R1-2111828 | 30 | 0% | 99.0% | Typical HO, Conditional HO |  | 62 | 3.2 |  |
| Nokia | R1-2111828 | 30 | 0% | 99.0% |  |  | 10 | 0 |  |
| Nokia | R1-2111828 | 30 | 0% | 99.0% |  |  | 20 | 0 |  |
| Nokia | R1-2111828 | 30 | 0% | 99.0% |  |  | 40 | 1 |  |
| Nokia | R1-2111828 | 30 | 0% | 99.0% |  |  | 60 | 3 |  |
| Nokia | R1-2111828 | 30 | 0% | 99.0% |  |  | 80 | 5 |  |
| Nokia | R1-2111828 | 30 | 0% | 99.0% |  |  | 100 | 7 |  |
| Nokia | R1-2111828 | 30 | 0.20% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 42 | 1.50 |  |
| Nokia | R1-2111828 | 30 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 62 | 3.99 |  |
| Nokia | R1-2111828 | 30 | 0.20% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.20% | 99.0% |  |  | 20 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.20% | 99.0% |  |  | 40 | 1.25 |  |
| Nokia | R1-2111828 | 30 | 0.20% | 99.0% |  |  | 60 | 3.74 |  |
| Nokia | R1-2111828 | 30 | 0.20% | 99.0% |  |  | 80 | 6.24 |  |
| Nokia | R1-2111828 | 30 | 0.20% | 99.0% |  |  | 100 | 8.73 |  |
| Nokia | R1-2111828 | 30 | 0.40% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 42 | 1.99 |  |
| Nokia | R1-2111828 | 30 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 62 | 5.31 |  |
| Nokia | R1-2111828 | 30 | 0.40% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.40% | 99.0% |  |  | 20 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.40% | 99.0% |  |  | 40 | 1.66 |  |
| Nokia | R1-2111828 | 30 | 0.40% | 99.0% |  |  | 60 | 4.98 |  |
| Nokia | R1-2111828 | 30 | 0.40% | 99.0% |  |  | 80 | 8.30 |  |
| Nokia | R1-2111828 | 30 | 0.40% | 99.0% |  |  | 100 | 11.62 |  |
| Nokia | R1-2111828 | 30 | 0.60% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 42 | 2.98 |  |
| Nokia | R1-2111828 | 30 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 62 | 7.95 |  |
| Nokia | R1-2111828 | 30 | 0.60% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.60% | 99.0% |  |  | 20 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.60% | 99.0% |  |  | 40 | 2.48 |  |
| Nokia | R1-2111828 | 30 | 0.60% | 99.0% |  |  | 60 | 7.45 |  |
| Nokia | R1-2111828 | 30 | 0.60% | 99.0% |  |  | 80 | 12.43 |  |
| Nokia | R1-2111828 | 30 | 0.60% | 99.0% |  |  | 100 | 17.40 |  |
| Nokia | R1-2111828 | 30 | 0.80% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 42 | 5.95 |  |
| Nokia | R1-2111828 | 30 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 62 | 15.87 |  |
| Nokia | R1-2111828 | 30 | 0.80% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.80% | 99.0% |  |  | 20 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.80% | 99.0% |  |  | 40 | 4.96 |  |
| Nokia | R1-2111828 | 30 | 0.80% | 99.0% |  |  | 60 | 14.88 |  |
| Nokia | R1-2111828 | 30 | 0.80% | 99.0% |  |  | 80 | 24.80 |  |
| Nokia | R1-2111828 | 30 | 0.80% | 99.0% |  |  | 100 | 34.72 |  |
| Nokia | R1-2111828 | 30 | 0.90% | 99.0% | DAPS |  | 2 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 42 | 11.89 |  |
| Nokia | R1-2111828 | 30 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 62 | 31.71 |  |
| Nokia | R1-2111828 | 30 | 0.90% | 99.0% |  |  | 10 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.90% | 99.0% |  |  | 20 | 0.00 |  |
| Nokia | R1-2111828 | 30 | 0.90% | 99.0% |  |  | 40 | 9.91 |  |
| Nokia | R1-2111828 | 30 | 0.90% | 99.0% |  |  | 60 | 29.73 |  |
| Nokia | R1-2111828 | 30 | 0.90% | 99.0% |  |  | 80 | 49.55 |  |
| Nokia | R1-2111828 | 30 | 0.90% | 99.0% |  |  | 100 | 69.37 |  |
| Ericsson | R1-2112160 | 30 | 0.00% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 30 | 0.00% | 99.0% |  |  | 20 | 0.0 |  |
| Ericsson | R1-2112160 | 30 | 0.00% | 99.0% |  |  | 40 | 1.0 |  |
| Ericsson | R1-2112160 | 30 | 0.00% | 99.0% |  |  | 60 | 3.0 |  |
| Ericsson | R1-2112160 | 30 | 0.00% | 99.0% |  |  | 80 | 5.0 |  |
| Ericsson | R1-2112160 | 30 | 0.00% | 99.0% |  |  | 100 | 7.0 |  |
| Ericsson | R1-2112160 | 30 | 0.20% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 30 | 0.20% | 99.0% |  |  | 20 | 0.0 |  |
| Ericsson | R1-2112160 | 30 | 0.20% | 99.0% |  |  | 40 | 1.2 |  |
| Ericsson | R1-2112160 | 30 | 0.20% | 99.0% |  |  | 60 | 3.7 |  |
| Ericsson | R1-2112160 | 30 | 0.20% | 99.0% |  |  | 80 | 6.2 |  |
| Ericsson | R1-2112160 | 30 | 0.20% | 99.0% |  |  | 100 | 8.7 |  |
| Ericsson | R1-2112160 | 30 | 0.40% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 30 | 0.40% | 99.0% |  |  | 20 | 0.0 |  |
| Ericsson | R1-2112160 | 30 | 0.40% | 99.0% |  |  | 40 | 1.7 |  |
| Ericsson | R1-2112160 | 30 | 0.40% | 99.0% |  |  | 60 | 5.0 |  |
| Ericsson | R1-2112160 | 30 | 0.40% | 99.0% |  |  | 80 | 8.3 |  |
| Ericsson | R1-2112160 | 30 | 0.40% | 99.0% |  |  | 100 | 11.6 |  |
| Ericsson | R1-2112160 | 30 | 0.60% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 30 | 0.60% | 99.0% |  |  | 20 | 0.0 |  |
| Ericsson | R1-2112160 | 30 | 0.60% | 99.0% |  |  | 40 | 2.5 |  |
| Ericsson | R1-2112160 | 30 | 0.60% | 99.0% |  |  | 60 | 7.5 |  |
| Ericsson | R1-2112160 | 30 | 0.60% | 99.0% |  |  | 80 | 12.4 |  |
| Ericsson | R1-2112160 | 30 | 0.60% | 99.0% |  |  | 100 | 17.4 |  |
| Ericsson | R1-2112160 | 30 | 0.80% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 30 | 0.80% | 99.0% |  |  | 20 | 0.0 |  |
| Ericsson | R1-2112160 | 30 | 0.80% | 99.0% |  |  | 40 | 5.0 |  |
| Ericsson | R1-2112160 | 30 | 0.80% | 99.0% |  |  | 60 | 14.9 |  |
| Ericsson | R1-2112160 | 30 | 0.80% | 99.0% |  |  | 80 | 24.8 |  |
| Ericsson | R1-2112160 | 30 | 0.80% | 99.0% |  |  | 100 | 34.7 |  |
| Ericsson | R1-2112160 | 30 | 0.90% | 99.0% |  |  | 2 | 0.0 |  |
| Ericsson | R1-2112160 | 30 | 0.90% | 99.0% |  |  | 20 | 0.0 |  |
| Ericsson | R1-2112160 | 30 | 0.90% | 99.0% |  |  | 40 | 9.9 |  |
| Ericsson | R1-2112160 | 30 | 0.90% | 99.0% |  |  | 60 | 29.7 |  |
| Ericsson | R1-2112160 | 30 | 0.90% | 99.0% |  |  | 80 | 49.5 |  |
| Ericsson | R1-2112160 | 30 | 0.90% | 99.0% |  |  | 100 | 69.4 |  |
| MediaTek | R1-2112296 | 30 | 0.0% | 99.0% |  |  | 60 | 3 |  |
| MediaTek | R1-2112296 | 30 | 0.5% | 99.0% |  |  | 60 | 5.97 |  |
| MediaTek | R1-2112296 | 30 | 0.9% | 99.0% |  |  | 60 | 29.73 |  |

Table 9  X=95%, PDB=10/15/30ms

| Company | Tdoc | PDB (ms) | PE,op | X | Handover type | Handover case | Y (ms) | T (s) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | R1-2111046 | 10 | 0.00% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.84 |  |
| vivo | R1-2111046 | 10 | 0.00% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 1.04 |  |
| vivo | R1-2111046 | 10 | 0.00% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 1.44 |  |
| vivo | R1-2111046 | 15 | 0.00% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.74 |  |
| vivo | R1-2111046 | 15 | 0.00% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 0.94 |  |
| vivo | R1-2111046 | 15 | 0.00% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 1.34 |  |
| vivo | R1-2111046 | 30 | 0.00% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.44 |  |
| vivo | R1-2111046 | 30 | 0.00% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 0.64 |  |
| vivo | R1-2111046 | 30 | 0.00% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 1.04 |  |
| vivo | R1-2111046 | 10 | 0.10% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.86 |  |
| vivo | R1-2111046 | 10 | 0.10% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 1.06 |  |
| vivo | R1-2111046 | 10 | 0.10% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 1.47 |  |
| vivo | R1-2111046 | 15 | 0.10% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.75 |  |
| vivo | R1-2111046 | 15 | 0.10% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 0.96 |  |
| vivo | R1-2111046 | 15 | 0.10% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 1.37 |  |
| vivo | R1-2111046 | 30 | 0.10% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.45 |  |
| vivo | R1-2111046 | 30 | 0.10% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 0.65 |  |
| vivo | R1-2111046 | 30 | 0.10% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 1.06 |  |
| vivo | R1-2111046 | 10 | 2.50% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 1.64 |  |
| vivo | R1-2111046 | 10 | 2.50% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 2.03 |  |
| vivo | R1-2111046 | 10 | 2.50% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 2.81 |  |
| vivo | R1-2111046 | 15 | 2.50% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 1.44 |  |
| vivo | R1-2111046 | 15 | 2.50% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 1.83 |  |
| vivo | R1-2111046 | 15 | 2.50% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 2.61 |  |
| vivo | R1-2111046 | 30 | 2.50% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.86 |  |
| vivo | R1-2111046 | 30 | 2.50% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 1.25 |  |
| vivo | R1-2111046 | 30 | 2.50% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 2.03 |  |
| vivo | R1-2111046 | 10 | 4.90% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 39.94 |  |
| vivo | R1-2111046 | 10 | 4.90% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 49.45 |  |
| vivo | R1-2111046 | 10 | 4.90% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 68.47 |  |
| vivo | R1-2111046 | 15 | 4.90% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 35.19 |  |
| vivo | R1-2111046 | 15 | 4.90% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 44.70 |  |
| vivo | R1-2111046 | 15 | 4.90% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 63.72 |  |
| vivo | R1-2111046 | 30 | 4.90% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 20.92 |  |
| vivo | R1-2111046 | 30 | 4.90% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 30.43 |  |
| vivo | R1-2111046 | 30 | 4.90% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 49.45 |  |

Table 10  X=99.9%, PDB=10/15/30ms

| Company | Tdoc | PDB (ms) | PE,op | X | Handover type | Handover case | Y (ms) | T (s) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo | R1-2111046 | 10 | 0.00% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 42.00 |  |
| vivo | R1-2111046 | 10 | 0.00% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 52.00 |  |
| vivo | R1-2111046 | 10 | 0.00% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 72.00 |  |
| vivo | R1-2111046 | 15 | 0.00% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 37.00 |  |
| vivo | R1-2111046 | 15 | 0.00% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 47.00 |  |
| vivo | R1-2111046 | 15 | 0.00% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 67.00 |  |
| vivo | R1-2111046 | 30 | 0.00% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 22.00 |  |
| vivo | R1-2111046 | 30 | 0.00% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 32.00 |  |
| vivo | R1-2111046 | 30 | 0.00% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 52.00 |  |
| vivo | R1-2111046 | 10 | 0.01% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 46.66 |  |
| vivo | R1-2111046 | 10 | 0.01% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 57.77 |  |
| vivo | R1-2111046 | 10 | 0.01% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 79.99 |  |
| vivo | R1-2111046 | 15 | 0.01% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 41.11 |  |
| vivo | R1-2111046 | 15 | 0.01% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 52.22 |  |
| vivo | R1-2111046 | 15 | 0.01% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 74.44 |  |
| vivo | R1-2111046 | 30 | 0.01% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 24.44 |  |
| vivo | R1-2111046 | 30 | 0.01% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 35.55 |  |
| vivo | R1-2111046 | 30 | 0.01% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 57.77 |  |
| vivo | R1-2111046 | 10 | 0.05% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 83.96 |  |
| vivo | R1-2111046 | 10 | 0.05% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 103.95 |  |
| vivo | R1-2111046 | 10 | 0.05% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 143.93 |  |
| vivo | R1-2111046 | 15 | 0.05% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 73.96 |  |
| vivo | R1-2111046 | 15 | 0.05% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 93.95 |  |
| vivo | R1-2111046 | 15 | 0.05% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 133.93 |  |
| vivo | R1-2111046 | 30 | 0.05% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 43.98 |  |
| vivo | R1-2111046 | 30 | 0.05% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 63.97 |  |
| vivo | R1-2111046 | 30 | 0.05% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 103.95 |  |
| vivo | R1-2111046 | 10 | 0.09% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 419.62 |  |
| vivo | R1-2111046 | 10 | 0.09% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 519.53 |  |
| vivo | R1-2111046 | 10 | 0.09% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 719.35 |  |
| vivo | R1-2111046 | 15 | 0.09% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 369.67 |  |
| vivo | R1-2111046 | 15 | 0.09% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 469.58 |  |
| vivo | R1-2111046 | 15 | 0.09% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 669.40 |  |
| vivo | R1-2111046 | 30 | 0.09% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 219.80 |  |
| vivo | R1-2111046 | 30 | 0.09% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 319.71 |  |
| vivo | R1-2111046 | 30 | 0.09% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 519.53 |  |

============End of Text update for TR section – Mobility evaluation =============