3GPP TSG RAN WG1 Meeting #107bis-e			     R1-2200228

Electronic Meeting, January 17th – January 25th, 2022

Source:	NTT DOCOMO, INC.

Title:	Remaiinng issues on PUCCH format 0/1/4 enhancements for NR in FR2-2

Agenda Item:	8.2.3

Document for: 	Discussion and Decision

- Introduction

At the RAN#94-e meeting, status report [1] on supporting NR from 52.6 GHz to 71 GHz was approved and regarded as a completed work.

In this contribution, we discuss remaining issues on PUCCH format 0/1/4 enhancement to support NR from 52.6 GHz to 71 GHz.

- Remining issues on DMRS for enhanced PUCCH format 4

At RAN1#107-e meeting, the following agreement was made [2]:

Agreement - For DMRS of enhanced (multi-RB) PF4, Type-2 low PAPR sequence of length equal to the total number of mapped REs of the PUCCH resource is supported if dmrs-UplinkTransformPrecodingPUCCH is configured and pi2BPSK is configured for the PUCCH resource. - FFS: For DMRS of enhanced (multi-RB) PF4, whether or not Type-1 low PAPR sequence of length equal to the total number of mapped REs of the PUCCH resource is supported if pi2BPSK is configured for the PUCCH resource. - If Type-1 low PAPR sequence of length equal to the total number of mapped REs of the PUCCH resource is not supported for DMRS of enhanced (multi-RB) PF4 in FR2-2 if pi2BPSK is configured for the PUCCH resource, it will be separately discussed whether to support FG 16-6b in FR2-2 in UE feature discussion. - Update the prior agreement from RAN1#104bis-e as follows: Agreement (RAN1#104bis-e): - For DMRS of enhanced PF4, if pi2BPSK is not configured for the PUCCH resource, a Type-1 low PAPR sequence of length equal to the total number of mapped REs of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4 (Alt-1 in agreement from RAN1#104-e).

A remaining issue is whether to support Type-1 low PAPR sequence for DMRS of enhanced PF4 when π/2-BPSK modulation is used as in the FFS in the agreement.

Regarding the conventional sequence of DMRS for PF3 and 4, according to 6.4.1.3.3.1 from TS 38.211[3], Type-2 low PAPR sequence is generated only when π/2-BPSK is used and dmrs-UplinkTransformPrecodingPUCCH is configured. Thus, for other cases, Type-1 low PAPR sequence is generated, i.e., if π/2-BPSK modulation is used for PF3/4, which to generate Type-1 or 2 low PAPR sequence can be switched by the configuration of dmrs-UplinkTransformPrecodingPUCCH.

In Rel-16, Type-2 low PAPR sequence was introduced since it is beneficial in terms of PAPR mitigation compared to Type-1 low PAPR sequence when π/2-BPSK modulation is used, and its benefit is valid even for enhanced PF4 as agreed above. On the other hand, user-multiplexing is not supported for PF4 with Type-2 low PAPR sequence in the current specification. If user-multiplexing is not supported for multi-PRB PF4, i.e., Type-2 low PAPR sequence is used when π/2-BPSK is configured for the PUCCH, there is no advantage compared to PF3. In order to keep the existing functionality of user-multiplexing provided by PF4, we believe the combination of Type-1 low PAPR sequence for DMRS and π/2-BPSK modulation should be kept even for the enhanced PF4. Note that, for DMRS of PF3, if dmrs-UplinkTransformPrecodingPUCCH is not configured but π/2-BPSK is configured, Type-1 low PAPR sequence would be used even for FR2-2 operation. Unless there is any valid technical reason to preclude the combination of Type-1 low PAPR sequence and π/2-BPSK only for PF4, we do not see an issue to continue its support.

Therefore, we suggest supporting Type-1 low PAPR sequence for DMRS of PF4 when π/2-BPSK modulation is used and dmrs-UplinkTransformPrecodingPUCCH is not configured same as the current specification.

Proposal 1: Type-1 low PAPR sequence should be supported for DMRS of PF4 when π/2-BPSK modulation is used and dmrs-UplinkTransformPrecodingPUCCH is not configured.

- Conclusion

In this contribution, we discussed the base sequence design for enhanced PUCCH format 4 DMRS to support NR from 52.6 GHz to 71 GHz. Based on the discussion above, we derived following proposal.

Proposal 1: Type-1 low PAPR sequence should be supported for DMRS of PF4 when π/2-BPSK modulation is used and dmrs-UplinkTransformPrecodingPUCCH is not configured.

References

- RP-212990, “SR for Extending current NR operation to 71GHz”, Qualcomm CDMA Technologies, RAN#94-e, 2021.

- RAN1 #107-e chairman’s notes.

- 3GPP TS 38.211, “NR; Physical channels and modulation”, V16.4.0.