3GPP TSG RAN WG1 #108-e							    R1-21XXXXX

e-Meeting, February 21st - March 3rd

Source:	Moderator (vivo)

Title:	Summary of email discussion [108-e-R16-V2X-02] Alignment CR on TS 38.213

Agenda item:	7.2.4

Document for:	Discussion and Decision

Introduction

This document is to summarize the discussion of the following email thread.

[108-e-R16-V2X-02] Alignment CR on TS 38.213; considering R1-2201073 (first change), R1-2201600, R1-2201603, R1-2201604, R1-2202440 by February 23 – Siqi (vivo)

Companies are highly appreciated providing your inputs before the 1st checkpoint:

- 1st checkpoint: 22th Feb 9:59 AM UTC

Discussion

## TP#1 for incorrect parameter name TDD-UL-ConfigurationCommon in TS 38.213

R1-2201073 [1] proposed the change as captured in TP#1 to align the RRC parameter name between 38.213 and 38.331.

## 16.1 Synchronization procedures - <Unchanged parts omitted> - a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of uslotsSL, respectively - for a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1 - for a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w where - L is the number of symbols in a slot: L=12 if cyclicPrefix = "ECP"; else, L=14 - I1 is 1 if usym*2μ-μref mod L≥ L-Y, else I1 is 0 - I2 is 1 if usym,2*2μ-μref mod L≥L-Y, else I2 is 0 - Y is the sidelink starting symbol index provided by sl-StartSymbol - w is the granularity of slots indication as described in Table 16.1-2 - μref, uslots, usym, uslots,2, usym,2 are the parameters of tdd-UL-DL-ConfigurationCommon as described in clause 11.1, or the parameters of sl-TDD-Configuration as defined in [9.3, TS 38.331] - μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211] - <Unchanged parts omitted>

### Round#1 discussion on TP#1

Do you agree with the proposed change? Please provide your views on TP#1 in the table below.

| Company | Agree or not | Comment |
| --- | --- | --- |

## TP#2 for incorrect parameter name BWP-Sidelink in TS 38.213

R1-2201600 [2], and R1-2202440 [5] identified the same issue but proposed different ways for alignment.

- [2]: Change BWP-Sidelink to sl-BWP-Config

- [5]: Change BWP-Sidelink to sl-BWP

- In the moderator’s understanding, either way could work. Considering that IE BWP-Uplink, which is for configuring an uplink bandwidth part, is used when referring to the active UL BWP in the text to be modified (‘… and BWP-Uplink for the SL BWP and the active UL BWP’), it is slightly preferred to use sl-BWP-Config, which provides the SL bandwidth part configuration, to keep consistency with UL part.

- Thus, moderator suggests the following TP#2 based on [2], the unmodified parts in [2] are removed for conciseness.

BWP-Uplink ::=                      SEQUENCE {

bwp-Id                              BWP-Id,

bwp-Common                          BWP-UplinkCommon                                            OPTIONAL,   -- Cond SetupOtherBWP

bwp-Dedicated                       BWP-UplinkDedicated                                         OPTIONAL,   -- Cond SetupOtherBWP

...

}

SL-BWP-Config-r16 ::=                    SEQUENCE {

sl-BWP-Id                                BWP-Id,

sl-BWP-Generic-r16                       SL-BWP-Generic-r16                                   OPTIONAL,    -- Need M

sl-BWP-PoolConfig-r16                    SL-BWP-PoolConfig-r16                                OPTIONAL,    -- Need M

...

}

#### 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a SL BWP on a carrier, and an active UL BWP on the primary cell, as described in clause 12, a UE determines a set MA of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions for which the UE can multiplex corresponding HARQ-ACK information in a PUCCH transmission in slot nU. The determination is based on: a) a set of slot timing values K1 associated with the SL BWP where K1 is provided by sl-PSFCH-ToPUCCH for DCI format 3_0 or by sl-PSFCH-ToPUCCH-CG-Type1 b) the ratio 2μSL-μUL between the sidelink SCS configuration μSL and the uplink SCS configuration μUL provided by subcarrierSpacing in sl-BWP-Config and BWP-Uplink for the SL BWP and the active UL BWP, respectively c) a configured sidelink resource pool bitmap d) a value of a period of PSFCH transmission occasion resources for a sidelink resource pool provided by a respective sl-PSFCH-Period

### Round#1 discussion on TP#2

Do you agree with the proposed changes? Please provide your views on TP#2 in the table below.

| Company | Agree or not | Comment |
| --- | --- | --- |

## TP#3 for power control in TS 38.213

R1-2201603 [3] proposed changes as captured in TP#3 to correct the subscript of αPFSCH:

### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH - αPFSCHαPSFCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCHαPSFCH=1

### Round#1 discussion on TP#3

Do you agree with the proposed changes? Please provide your views on TP#3 in the table below.

| Company | Agree or not | Comment |
| --- | --- | --- |

## TP#4 for power control in TS 38.213

R1-2201604 [4] proposed changes as captured in TP#4 to correct the subscript of parameters in the procedure of SL Type-1 and Type-2 HARQ-ACK codebook generation.

- Change1: Change oOACK-1ACK to oOACK-1ACK in Clause 16.5.1.1

- Change2: Change oOACK-1ACK to oOACK-1ACK in Clause 16.5.2.1

- Change3: Change OACK to  OACK in Clause 16.5.2.1

- Change4: Change VSAIUL to VT-SAIUL in Clause 16.5.2.2

- <Unchanged parts omitted> #### 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel - <Unchanged parts omitted> The cardinality of the set MA defines a total number M of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions corresponding to the HARQ-ACK information bits. A UE determines o0ACK,o1ACK,…,oOACK-1ACKo0ACK, o1ACK,⋯,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits as ojACK = HARQ-ACK information bit for candidate PSSCH transmission with index j with corresponding PSFCH reception, for 0 ≤j<M, as described in clause 16.5. If the UE does not transmit a PSSCH in an occasion for candidate PSSCH transmission with corresponding PSFCH reception occasion, due to the UE not detecting a corresponding DCI format 3_0, the UE generates a NACK value for the occasion for candidate PSSCH transmission with corresponding PSFCH reception occasion. - <Unchanged parts omitted> #### 16.5.2.1 Type-2 HARQ-ACK codebook in physical uplink control channel - <Unchanged parts omitted> If the UE transmits HARQ-ACK information in a PUCCH in slot n, the UE determines the o0ACK,o1ACK,…,oOACK-1ACKo0ACK, o1ACK,⋯,oOACK-1ACK, for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code: Set m=0 – PDCCH with DCI format 3_0 monitoring occasion index: lower index corresponds to earlier PDCCH with DCI format 3_0 monitoring occasion Set j=0 Set Vtemp=0 Set Vs=∅ Set M to the number of PDCCH monitoring occasions while m<M if PDCCH monitoring occasion m is before an active UL BWP change on the PCell m=M; else if there is a PSFCH reception occasion associated with a PSSCH transmission scheduled by a DCI format in PDCCH monitoring occasion m if VC-SAI,mSL≤Vtemp j=j+1; end if Vtemp=VC-SAI,mSL o4j+VC-SAI,mSL-1ACK = HARQ-ACK information bit Vs=Vs∪4j+VC-SAI,mSL-1 end if end if m=m+1; end while OACK=4⋅j+Vtemp OACK=4⋅j+Vtemp oiACK=NACK for any i ∈0,1,…,OACK-1\Vs i ∈0,1,…,OACK-1\Vs if a SL configured grant Type 1 is configured for a UE, or a SL configured grant Type 2 is configured and activated for a UE, and the SL configured grant provides a grant for PSSCH transmissions with PSFCH reception occasions in a slot n-K1, where K1 is the PSFCH-to-HARQ-feedback timing value for the SL configured grant OACK=OACK+1 OACK=OACK+1; oOACK-1ACKoOACK-1ACK= HARQ-ACK information bit associated with the PSFCH reception occasions associated with the PSSCH transmissions scheduled by the SL configured grant end if - <Unchanged parts omitted> #### 16.5.2.2 Type-2 HARQ-ACK codebook in physical uplink shared channel <Unchanged parts omitted> If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by a DCI format that includes a SAI field, the UE generates the HARQ-ACK codebook as described in clause 16.5.2.1, with the following modifications: - For the pseudo-code for the HARQ-ACK codebook generation in clause 16.5.2.1, after the completion of the m loop, the UE sets Vtemp=VSAIUL Vtemp=VT-SAIUL where VSAIUL VT-SAI UL is the value of the SAI field in the DCI format according to Table 16.5.2.2-1. If a UE - is scheduled for a PUSCH transmission by a DCI format that includes a SAI field with value VT-SAIUL=4, and - has not received any PDCCH within the monitoring occasions for PDCCH with DCI format 3_0 for scheduling PSSCH with corresponding PSFCH reception occasions on a serving cell, and - does not have HARQ-ACK information in response to PSFCH reception occasions associated with a SL configured grant to multiplex in the PUSCH, as described in clause 16.5.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission. Table 16.5.2.2-1: Value of SAI SAI MSB, LSB; VT-SAIUL; Number of PDCCH monitoring occasions in which DCI format 3_0 scheduling PSSCH transmissions with corresponding PSFCH reception occasions is present, denoted as X and X≥1. 0,0; 1; X-1mod4+1=1. 0,1; 2; X-1mod4+1=2. 1,0; 3; X-1mod4+1=3. 1,1; 4; X-1mod4+1=4. <Unchanged parts omitted>

### Round#1 discussion on TP#4

Do you agree with the proposed changes? Please provide your views on TP#4 in the table below.

| Company | Agree or not | Comment |
| --- | --- | --- |

- Summary

- Reference

- R1-2201073, ‘Draft CR on UE procedure for receiving HARQ-ACK on sidelink’, vivo

- R1-2201600, ‘Correction on the parameter name for SL BWP configuration in TS 38.213’, ZTE, Sanechips

- R1-2201603, ‘Editorial modification for PSFCH power control in TS 38.213’, ZTE, Sanechips

- R1-2201604, ‘Correction on SL Type-1 and Type-2 HARQ-ACK codebook in TS 38.213’, ZTE, Sanechips

- R1-2202440, ‘Correction on misalignment for RRC parameters for SL BWP’, Huawei, HiSilicon