3GPP TSG-RAN WG1 Meeting #108-e		          R1-22XXXX

E-Meeting, February 21st – March 3rd, 2022

Title:	[Draft] LS reply on PDCCH skipping

Response to:	R1-2200884 (R2-2201960)

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN1

To:	RAN2

Contact Person:

Name:	Xiaodong SHEN, Weide WU

E-mail Address:	shenxiaodong@vivo.com, Weide.Wu@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	No

1. Overall Description:

RAN1 would like to thank RAN2 for the LS on PDCCH skipping. The following answers have been agreed according to RAN1 discussion.

Question 1: RAN2 would like to know whether PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window for RAR/MsgB reception.

Answer 1: Yes, PDCCH skipping can be applied to C-RNTI(s) and MCS-C-RNTI(s) in at least Type3 PDCCH CSS and USS monitored during RAR/MsgB window for RAR/MsgB reception based on RAN1 agreements till RAN1#107bis-e.

Question 2: RAN2 would like to know if RAN1 prefers to capture the above agreements in RAN1 specification or prefer these to be captured in MAC specification.

Answer 2:

RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification.

- It is RAN1 understanding PDCCH skipping is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending.

Question 3: RAN2 would like to ask RAN1 whether a) Physical layer of UE reports a value of 1 for Wake-up indication bit to higher layer or b) Physical layer of UE does not report Wake-up indication bit to higher layer, in case UE cannot monitor DCP due to PDCCH skipping; or if Approach 3 can be assumed.

Answer 3:

- It is RAN1 understanding that for DRX operation,

  - As DCP is monitored only outside active time, it cannot be missed due to PDCCH skipping as skipping applies only in active time.

Note: RAN1 is discussing whether PDCCH skipping duration can apply to outside active time.

Question 4: RAN2 would like to know whether UE should continue transmitting CSI/SRS during the PDCCH skipping duration or not?

Answer 4: It is RAN1’s understanding that transmission of CSI/SRS is not affected by PDCCH skipping i.e. CSI/SRS is still transmitted during the skipping duration.

# 2. Actions:

RAN1 respectfully asks RAN2 to take above information into account in their work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e    9– 20 May 2022                    E-Meeting

TSG-RAN WG1 Meeting #110      22 – 26 Aug 2022                    Toulouse, FR