3GPP TSG RAN WG1 Meeting #109-e	R1-2203058

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.1.3

Source:	Futurewei

Title:	Considerations on potential solutions for SL positioning

Document for:	Discussion/Decision

# Introduction

In RAN#94e the SID for “Study and improved NR positioning” was approved [RP-213588]. The first objective of the SI is to:

“Study solutions for sidelink positioning considering the following: [RAN1, RAN2]

- Scenario/requirements

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

- Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

- Define evaluation methodology with which to evaluate SL positioning for the use cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2]

  - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1]

  - Study of positioning methods (e.g., TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

  - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

  - Study of positioning architecture and signalling procedures (e.g., configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network-based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.”

In this contribution, we focus on the first AI of the SI SL positioning regarding of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning. For this purpose, we investigate the reference signals for positioning, including signal design, measurements, associated procedures with emphasize on reusing existing reference signals, procedures from sidelink communication and from positioning as much as possible.

# Factors that influence positioning accuracy

Signal Bandwidths and SNR: The channel bandwidth and the received SNR determines positioning accuracy for methods based on time-of-flight/arrival (TOF/TOA) distance measurements. From CRLB analysis, the variance of the TOA measurements for LOS channel is approximately lower bounded as

var(TOA)≥18π2β2SNR

where  denotes the effective signal bandwidth.

β=-∞∞f2S(f)2df-∞∞S(f)2df

and SNR is the signal to noise ratio, f is the frequency and S(f) is the Fourier transform of the transmitted signal. The above inequality implies that higher signal bandwidth improves the TOA measurement accuracy. The investigation of the bandwidth impact on the location accuracy could indicate if the SL positioning solutions need to be extended or not to the unlicensed spectrum.

LOS vs. NLOS channel propagation:

If there is a direct path (LOS) between the anchor and the target nodes, a larger positioning signal bandwidth allows a better resolution of multipath components, which increases the accuracy of finding the first path and thus reducing the error caused by multipath biases for TOA, RTT and TDOA based methods. However, if such direct path (LOS) does not exist and the communication between the anchor and target nodes is done via reflections (NLOS), the range between the anchor and the target nodes is overestimated due to the increased TOF.

Figure 2

As one can notice from Figure 2, the angle of arrival (AOV) and angle of departures (AOD) estimations may be also affected by the NLOS propagation, which leads to low accuracy of the location estimates.

Time synchronization

When a TOF, TOA based positioning method is used, the receiver may use the time stamp from the transmitter to estimate the TOF and thus the range between the transmitter and the receiver. However, even when the channel is LOS, if the clocks at the transmitter and receiver are not synchronized, additional errors are introduced, which affect the ranging and the position estimations. In TR38.855 the network synchronization error is defined as a truncated Gaussian distribution of (T1 ns) rms values between the anchor node and a timing reference source which is assumed to have perfect timing, subject to the largest timing difference of T2 ns, where T2 = 2*T1. That is, the range of timing errors is [-T2, T2]. Two values for T1 were proposed 0ns (perfectly synchronized) and 50ns.

Some positioning methods such as multi-RTT are robust with respect to time synchronization provided that the clock drifts are negligible for the duration when differences of the received time and transmit time of the positioning signals are measured, while others are more sensitive to synchronization errors (such TOA). Therefore, when investigating SL positioning solutions, the synchronization errors between target and anchor nodes should be considered

Proposal 1: The SL positioning study should investigate the BW size, non-ideal synchronization, and NLOS propagation impact on the SL positioning accuracy.

# Methods for location determinations

The RAT-dependent methods for positioning as defined in Rel 16 and Rel 17 [TR38.855, TS 38.305] are based on reference signal (RS) exchanges between the anchor nodes (gNB) and target nodes (UE). In this contribution the target UE is the UE that requires position (location) determination, and the anchor nodes are those nodes (UE, gNB, RSU) that may be considered as reference for relative or absolute positioning of the target UE. The RAT dependent positioning methods were defined in Rel 16 and Rel 17 in addition to the RAT independent methods for positioning such as GNSS, Wi-Fi, Bluetooth, terrestrial beacon systems (TBS), and motion-based sensors. The RAT dependent methods are:

- NR enhanced cell ID methods (NR E-CID) based on NR signals

- Multi-Round Trip Time Positioning (Multi-RTT based on NR signals)

- Downlink Angle-of-Departure (DL-AoD) based on NR signals

- Downlink Time Difference of Arrival (DL-TDOA) based on NR signals

- Uplink Time Difference of Arrival (UL-TDOA) based on NR signals

- Uplink Angle-of-Arrival (UL-AoA), including A-AoA and Z-AoA based on NR signals.

- Hybrid positioning using multiple methods from the list of positioning methods above is also supported

The necessary measurements to support the above methods are defined in TS 38.215.

- Downlink PRS reference signal received power (DL PRS RSRP)

- Downlink PRS reference signal received path power (DL PRS RSRPP)

- Downlink PRS reference signal time difference (DL PRS RSTD)

- UE Rx-Tx time difference

For sidelink positioning scenarios, at least one positioning reference signal is provided via sidelink (PC5), therefore a SL UE may be required to combine and measure sidelink PRS and DL PRS and transmit sidelink PRS and UL PRS.

For sidelink Mode 2 (expected in out of coverage scenarios) the reference nodes (anchor nodes) may be less reliable than for in coverage anchor nodes such as gNB. In these scenarios, it would be preferable to use positioning methods, such multi-RTT, which are more robust with respect to clock synchronization between anchor and target nodes.

Observation 1: For sidelink Mode 2 operation, robust positioning methods, such multi-RTT, are preferable.

Proposal 2: RAN1 should discuss the SL UE support of the positioning methods and measurements defined in Rel-16, and Rel 17.

Proposal 3: For position determination SL UE should support the aggregation of DL PRS resources with SL positioning resources.

One basic requirement for positioning in 5G services is the support of positioning in OOC scenarios when all the devices involved in the SL positioning are out of reach of LMF.

The OOC scenario positioning is part of the SID and required by 5G specifications as in TS 22.104.

“The 5G system shall provide positioning information for a UE that is out of coverage of the network, with accuracy of < [1 m] relative to other UEs that are in proximity and in coverage of the network.”

The TS 22.261 requirement on positioning allows data to be available at the UE, which makes possible UE based positioning and positioning in OOC scenarios.

“The 5G system shall be able to make the position-related data available to an application or to an application server existing within the 5G network, external to the 5G network, or in the User Equipment.”

In the OOC case, it is not clear whether a similar entity to LMF is still necessary and if so where should be located. It seems rather obvious that the target UE should support a function that allows it to compute an estimate of the location.  Such functionality and complexity may depend on the OOC covered scenarios such ranging, relative positioning or absolute positioning.

The SL positioning solutions for OOC scenarios should be able to select a positioning method, to configure and enable the sidelink reference signal transmissions per request or triggered by an event, to select the anchor nodes and enable RRC connectivity if necessary, to obtain the location information or to request S-PRS transmissions, to provide or exchange the location information if requested, to configure and enable the collection of SL positioning measurements to estimate the relative or the absolute position.

Some of the positioning methods involve an exchange between the target node and anchor nodes. For instance, in a UE based positioning multi-RTT the anchor nodes should provide the SL UE target node the measurements of Rx-Tx, which will be combined with the Rx-Tx measurements at the target node to obtain the final position estimate. Another example of data exchange between target node and anchor nodes may be the absolute position coordinates provided by the anchor nodes to the target node.  Such exchange may be carried out only after a RRC connection is established between the target and anchor nodes that could enable data encryption and therefore privacy.

Proposal 4: The SL positioning solutions should support the necessary configurations and controls for OOC SL positioning.

These requirements for the OOC SL positioning solutions may be achieved by two possible approaches. One option is to start from scratch and define new protocols and new signaling that would support the necessary SL positioning methods.

Another option is to build on the existing SL design and to extend the existing protocols with the necessary signaling that implements the SL positioning methods. The Inter UE Coordination (IUC) feature, defined in Rel 17 [4], is a good candidate that may be considered and extended to support the SL positioning solutions for the OOC scenarios. The IUC offers the necessary framework to request and respond for the measurements and location information, to configure and trigger the necessary signaling, and to coordinate the transmissions of the anchor nodes. In addition, reusing the IUC would minimize the specifications impact.

Proposal 5: Consider reusing or extending the IUC framework defined in Rel 17 for OOC SL positioning solutions.

# Rel17 Positioning Reference Signals

The RAT-dependent methods for positioning as defined in Rel 16 and Rel 17 [TR38.855, TR 38.305] are based on reference signal (RS) exchanges between the anchor nodes (gNB) and target UE.  More precisely, the gNB transmits a DL positioning RS (DL PRS) signal. The UE transmits an UL sounding reference signal (UL PRS) based on a configuration provided by SRS-PosResourceSet, which differs from the SRS used for UL channel estimation based on a configuration given by SRS-ResourceSet.

## DL PRS

DL PRS signal as defined in TS38.211 is a length-31 Gold QPSK sequence, where the pseudo-random sequence generator is initialized based the slot number, the DL PRS sequence ID,  nID,seqPRS∈0,1,…,4095 and the OFDM symbol index in the slot to which the sequence is mapped. The PRS sequence ID allows frequency reuse, while the slot and symbol indices allow the TOF, TOA, TDOA and RTT determination.

In time domain, the size of the DL PRS resources is LPRS∈2,4,6,12 symbols and it is given by the higher-layer parameter dl-PRS-NumSymbols.

In frequency domain a PRS resource has a comb distribution (i.e., resource element (RE) spacing in each symbol of DL-PRS Resource) where the comb size KcombPRS∈2, 4, 6,12 is given by the higher-layer parameter dl-PRS-CombSizeN-AndReOffset for a downlink PRS resource configured for RTT-based propagation delay compensation, otherwise by the higher-layer parameter dl-PRS-CombSizeN such that the combination LPRS,KcombPRS is one of {2, 2},{4, 2}, {6, 2}, {12, 2}, {4, 4}, {12, 4}, {6, 6}, {12, 6} and {12, 12}.

The comb distribution allows a wider bandwidth of the RS signal, and therefore as better accuracy for TOA estimation. However, the gaps in frequency generate aliases in time, which may be compensated by the repetition of PRS in time and coherent combining. For instance, if two symbols with comb-4 are coherently combine the result is equivalent with a comb-2 PRS signal.

The frequency offset from symbol to symbol is selected such that there is no staircase pattern. This has the main advantage that increases the robustness (for instance against Doppler shifts) when using just the first symbols for a coherent combining. In addition, the comb design and the offset in frequency between the consecutive symbols offers an increased robustness to wideband fading, and orthogonality with respect to other PRS signals from other TRPs.

A PRS resource is defined by ID, sequence ID {0, … ,4095}, the comb size {2,4,6,12} and the RE offsets for the remaining symbols, resource slot offset (Figure 3), resource symbol offset, and QCL information.

A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources, where each resource has an associated spatial transmission filter (transmission direction).

The PRS resource set [TS 38.214] is characterized by ID, subcarrier spacing, periodicity (of the resource set transmissions), resource list, resource repetition factor (number of repetitions of each resource during an instance of the resource set), resource time gap (a number of slots between resource consecutive repetitions), comb size, resource bandwidth (between 24 PRBs and 272 PRBs in 4 PRBs increments), the start of PRB index, and the number of resource symbols in the PRS slot. The PRS resource set can be located anywhere in the frequency grid via the start of PRB index, which is an offset with respect to the reference frequency Point A.

Figure 3

A PRS resource set is sent by gNB with a periodicity, TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40,  64, 80, 160, 320, 640,  1280, 2560, 5120, 10240  slots.

The PRS resource repetition factor defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and may take values TrepPRS∈1,2,4,6,8,16,32. All the DL PRS resources within one resource set have the same resource repetition factor.

To mitigate interference on weak PRS signals, the strong PRS signals may be muted. The muting pattern is specified by a bit string of {2,4,8,16,32} bits in each cell, where a bit indicates if the PRS transmission in the corresponding slot is muted or not.

A UE in RRC connected state is required to measure the DL PRS only in the active bandwidth part (BWP) and with the same numerology as the active BWP. UE may request the serving gNB measurement gaps that can be used to measure DL PRS outside the active BWP and with different numerology.

For 15 kHz SCS the minimum DL PRS is about 5MHz and the maximum about 50MHz.  With a 120 kHz SCS the DL PRS minimum bandwidth is about 34 MHz, and the maximum bandwidth is about 400 MHz.

## UL PRS

The UL positioning RS (UL PRS) is based on the sounding RS (SRS) and referred as SRS for positioning. The UL PRS sequence is 31-bit Zadoff-Chu, which offers good peak to average ratio. The UL PRS may span in time for {1,2,4,8,12} consecutive OFDM symbols, which can be located anywhere in the slot. Similar with DL PRS, the UL PRS has a comb-N pattern in frequency, with comb size {2,4,8}. Like UL PRS, the DL PRS has a comb offset which defines the relative frequency shifts between consecutive OFDM symbols. The offset offers similar advantage as for DL PRS, i.e., only first few symbols may be considered for TOA measurement through a coherent combining. Like DL PRS, the UL PRS may be transmitted periodically with certain periodicity and slot offset. However, the semi-persistent configuration is activated and deactivated via MAC-CE signaling. An aperiodic UL PRS is transmitted only when UE is instructed by gNB via Downlink Control Information (DCI).  UL PRS supports spatial relationships, where the spatial relation can be either a DL reference signal (SSB, CSI-RS or DL-PRS) or by the previously transmitted SRS or UL-PRS. The UL PRS can also have a spatial relationship with a neighbor TRP.

Another property of the UL PRS is the power transmit control, where UE estimates the UL pathloss for serving and neighbor TRPs based on DL measurements and sets the UL PRS power accordingly.

The UL PRS resource set comprises of one or multiple UL PRS resources, and is defined by resource set ID, resource type (aperiodic, semi-persistent, periodic), alpha the value that characterizes the fractional power control, p0 the desired receive power at TRP, pathloss reference RS and the UL PRS resource list.

The UL resource is described by an ID, transmission comb, resource mapping (symbol location in UL PRS slot), frequency domain shift, bandwidth indication (as part of the Frequency Hopping, which is not used for frequency hopping indication as for SRS case), resource type (periodic, semi-persistent, aperiodic), and the corresponding periodicity, sequence ID used to initialize the pseudo-random group and sequency hopping and a spatial relation information.

Like for DL PRS, the UE may be configured with multiple UL PRS resource sets.

Figure 4, Example of UL PRS resource configuration

# Reference Signals for Sidelink Positioning

## S-SSB

Sidelink Synchronization Signal Block (S-SSB) is a broadcast signal defined in TS 38.211, which is used for the synchronization purposes, and it is composed of the Sidelink Primary Synchronization Signal (S-PSS), Sidelink Secondary Synchronization Signal (S-SSS), and Physical Sidelink Broadcast Channel (PSBCH). There are 672 unique physical layer sidelink synchronization identities, and they are divided into two sets {0,1, ..., 335} and {336, ...,671}. The Sidelink Synchronization Signal ID (SLSSID) indicates the source of time reference (GNSS, gNB or another SL UE (SyncRef UE)), and therefore give an information of the accuracy of the time reference. Prior to start sending S-SSB a SL UE must select its own time reference and advertise it via SLSSID.

In the frequency domain S-SSB occupies 11 Physical Resource Blocks (PRBs), i.e., 132 subcarriers, where S-PSS and S-SSS each occupy 127 subcarriers and are repeated twice during the S-SSB slot. The PSBCH occupies 132 subcarriers for the duration of eight symbols (Figure 5). The first PSBCH symbol serves for automatic gain control (AGC) purposes.  Each S-SSB transmission is repeated several times during each period of 16 subframes. The frequency location of S-SSB is fixed.

Figure 5

The S-SSB may be used primarily by a receiver SL UE to acquire synchronization with the transmitter SL device, or for the target SL UE to measure Time Difference of Arrival (TDOA) between two SyncRef UE that are synchronized with the same reference time. Thus, the target SL UE could estimate relative position to SyncRef UE.

Observation 2: The S-SSB could be adapted to estimate the TDOA between anchor SL devices.

Observation 3: The accuracy of TDOA estimate is increased when the S-SSB originators have the same reference time (SLSSID).

Observation 4: The usage of S-SSB for positioning may enable SL UE positioning in RRC_INACTIVE state.

Proposal 6: Consider supporting the S-SSB based SL position determination.

## Sidelink Positioning Signal (S-PRS)

One of the topics of this SI is

- Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

In out-of-coverage (OOC) scenarios, the target SL UE must rely on the sidelink (PC5) reference signals received from other SL devices (SL UE, RSU) to determine the range, or position. As noted above, the signal bandwidth plays a crucial role in estimation accuracy. The bandwidth of S-SSB may not be sufficient, moreover the S-SSB periodicity (160ms) may be add latency to position determination. Thus, it is necessary that RAN1 defines a SL positioning reference signal (S-PRS) that shares some of the common features of DL and UL PRS such as

- Flexible bandwidth size

- Comb-N distribution in frequency

- Repetition with frequency offset in consecutive symbols

- Different length in time, and periodicity of the resource set

- Aperiodic, semi-static and periodic transmissions

The Zadoff-Chu (ZC) sequence used for uplink PRS offers better PAPR properties (small power variations in time and frequency) with respect to Gold Sequence used for DL PRS.  In our view, such PAPR properties are desirable for SL UE to avoid nonlinear signal distortions.

Proposal 7: RAN1 should consider the UL PRS design as the starting point of the S-PRS design.

A SL UE at the fringe of the network coverage may be required to participate in SL positioning exchange with other SL in partial coverage. Thus, the serving gNB should be able to control and configure the S-PRS UE transmissions in its coverage to minimize interference and maximize capacity.

Proposal 8: Consider whether the S-PRS configuration should be controlled by gNB when SL UEs are in coverage or partial coverage.

# SL Positioning Architecture

TS 38.305 defines functional specifications of UE positioning in NG-RAN. The document defines the positioning methods, positioning architecture and signaling protocols and interfaces. Rel 16 and Rel 17 positioning solutions rely on LTE Positioning Protocol (LPP) and Location Management Function (LMF). Particularly LMF is vital for the orchestration of the positioning methods and protocols.

“For positioning of a target UE, the LMF decides on the position methods to be used, based on factors that may include the LCS Client type, the required QoS, UE positioning capabilities, gNB positioning capabilities and ng-eNB positioning capabilities. The LMF then invokes these positioning methods in the UE, serving gNB and/or serving ng-eNB. The positioning methods may yield a location estimate for UE-based position methods and/or positioning measurements for UE-assisted and network-based position methods. The LMF may combine all the received results and determine a single location estimate for the target UE (hybrid positioning).”

When SL UE that participate in positioning exchange are in coverage of gNB, the existing positioning protocol (LTE Positioning Protocol) should be still supported. The LMF through Uu connections and SL relaying should be able to coordinate and process the measurements for positioning.

At the same time, gNB may forward the measurements from SL UE to LMF to estimate position of SL UE in partial coverage.

Proposal 9: In this study RAN1 should consider as working assumption that LMF is extended to support SL positioning for SL UE in coverage or partial coverage.

Proposal 10: RAN1 should send a LS to RAN2 and RAN3 to ask extending the existing LMF protocol for SL UEs in coverage or partial coverage.

# Conclusions

Proposal 1: The SL positioning study should investigate the BW size, non-ideal synchronization, and NLOS propagation impact on the SL positioning accuracy.

Observation 1: For sidelink Mode 2 operation, robust positioning methods, such multi-RTT, are preferable.

Proposal 2: RAN1 should discuss the SL UE support of the positioning methods and measurements defined in Rel-16, and Rel 17.

Proposal 3: For position determination SL UE should support the aggregation of DL PRS resources with SL positioning resources.

Proposal 4: The SL positioning solutions should support the necessary configurations and controls for OOC SL positioning.

Proposal 5: Consider reusing or extending the IUC framework defined in Rel 17 for OOC SL positioning solutions.

Observation 2: The S-SSB could be adapted to estimate the TDOA between anchor SL devices.

Observation 3: The accuracy of TDOA estimate is increased when the S-SSB originators have the same reference time (SLSSID).

Observation 4: The usage of S-SSB for positioning may enable SL UE positioning in RRC_INACTIVE state.

Proposal 6: Consider supporting the S-SSB based SL position determination.

Proposal 7: RAN1 should consider the UL PRS design as the starting point of the S-PRS design.

Proposal 8: Consider whether the S-PRS configuration should be controlled by gNB when SL UEs are in coverage or partial coverage.

Proposal 9: In this study RAN1 should consider as working assumption that LMF is extended to support SL positioning for SL UE in coverage or partial coverage.

Proposal 10: RAN1 should send a LS to RAN2 and RAN3 to ask extending the existing LMF protocol for SL UEs in coverage or partial coverage.

# References

RP-213588, Study on expanded and improved NR positioning, RAN #94e, December 2021.

TS 38.855, Study on NR positioning support (Release 16), March 2019

TR 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 16), December 2021

TS 38.214, Physical layer procedures for data (Release 17), March 2022

TS 38.211, Physical channels and modulation (Release 17), March 2022