3GPP TSG RAN WG1 #109-e                                              R1-2203630

e-Meeting, May 9th – 20th, 2022

Source:         ZTE

Title:            UE capability on maximum number of layers for MTRP

Agenda item:    7.2.11

Document for:	Discussion and Decision

- Introduction

In RAN1#108-e meeting, an issue on maximum number of layers for multi-TRP was discussed for Rel-16 UE features [1] [2]. However, the consensus was not achieved because of limited discussion time. In this contribution, we provide our views on this issue.

- Discussion

Based on the elaboration in [1][2], in FR2, for single-DCI based MTRP schemes, the parameter maxNumberMIMO-LayersPDSCH is shared among the multi-TRP schemes and with the conventional single-TRP operations. A low-complexity UE implementation may support multi-TRP operations by using two panels, each with only one receiver unit (RxU). For single-TRP operations, UE can report maxNumberMIMO-LayersPDSCH = 2 by using both one-RxU panels for data reception. It also works for the single-DCI based SDM and TDM schemes. However, the single-DCI based FDM schemes cannot support 2 layers since the signals from the two TRPs arrive at the same time and in FR2 it is seldom that one panel can receive signals well from both TRPs simultaneously. Therefore, for single-DCI based FDM schemes, the maximum number of layers should be separately reported from single-TRP and the single-DCI based SDM and TDM schemes.

Similarly, for multi-DCI based MTRP scheme, a low complexity UE equipped with two panels, each with only one RxU may only support 1 layer per PDSCH/TRP by reporting maxNumberMIMO-LayersPDSCH = 2, interpreted as the maximum number of layers across two PDSCHs if having at least one RE overlapped. However, when the two PDSCHs are overlapped in time but non-overlapped in frequency, the UE cannot support 2 layers per PDSCH in FR2 since generally 1 RxUs per panel is used per PDSCH.

Therefore, a FL proposal was suggested as follows in [2]:

Updated FL proposal #1: Introduce a new FG for maximum number of layers for single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17

| Maximum number of layers for single-DCI based multi-TRP FDM scheme A and B and multi-DCI based multi-TRP overlapped in time | 1.For DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH 2.For DL reception of multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH | supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16 | Per FSPC | FR2 only |
| --- | --- | --- | --- | --- |

From our view, the granularity should be per band rather than per FSPC to avoid complicated and large overhead capability signaling.

Furthermore, for multi-DCI based MTRP, it is not good to split the same UE capability report into 2 cases where case 1 is two PDSCH overlapped in at least one RE as described in FG 16-2a-9, and case 2 is two PDSCHs overlapped in time but non-overlapped in frequency. Hence, we suggest using this new FG to override the FG 16-2a-9.

Here is our suggestion:

Proposal #1: Introduce a new FG for maximum number of layers for single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17

| Maximum number of layers for single-DCI based multi-TRP FDM scheme A and B and multi-DCI based multi-TRP overlapped in time | 1. For DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH 2. For DL reception of multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH Note: if this feature is reported, FG 16-2a-9 should not be used. | supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16 | Per band FSPC | FR2 only |
| --- | --- | --- | --- | --- |

- Conclusions

In this contribution, we discuss UE features on maximum number of layers for multi-TRP, the following proposal is suggested.

Proposal #1: Introduce a new FG for maximum number of layers for single-DCI based multi-TRP FDM scheme A/B and multi-DCI based multi-TRP overlapped in time for Rel-17.

| Maximum number of layers for single-DCI based multi-TRP FDM scheme A and B and multi-DCI based multi-TRP overlapped in time | 1. For DL reception of single-DCI based multi-TRP FDM scheme A and scheme B, the maximum number of spatial multiplexing layer(s) supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH 2. For DL reception of multi-DCI based multi-TRP overlapped in time but non-overlapped in frequency, the maximum number of spatial multiplexing layer(s) per PDSCH supported by the UE is half of the value reported in maxNumberMIMO-LayersPDSCH Note: if this feature is reported, FG 16-2a-9 should not be used. | supportFDM-SchemeA-r16 or supportFDM-SchemeB-r16 or multiDCI-MultiTRP-r16 | Per band FSPC | FR2 only |
| --- | --- | --- | --- | --- |

- References

[1] R1-2202084 On UE capability of maximum number of layers for multi-TRP	MediaTek Inc.

[2] R1-2202511 Summary on [108-e-R16-UE-features-MIMO-02] Moderator (NTT DOCOMO, INC.)