3GPP TSG RAN WG1 #109-e	R1-2203761
e-Meeting, May 9th – 20th, 2022

Source:				Panasonic

Title:	Further reduce UE complexity for eRedCap

Agenda item:	9.6.1

Document for:			Discussion and Decision

Introduction

This document discusses further to reduce UE complexity for Rel-18 eRedCap. The UE BW reduction to 5MHz is focused on.

Objective described in the New SID is following:

To further reduce the complexity of RedCap devices, the following should be studied, and the results should be captured in TR 38.8xx: - Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

Discussion

First of all, we identify the following 4 options for “UE BW reduction to 5MHz” in Table 1. Namely, we propose to carefully study whether to reduce the BW of RF/BWP/common channel PRB allocation as well as data allocation.

Table 1: UE BW reduction options to 5MHz

|  | RF bandwidth | BWP bandwidth | Common channel PRB allocation | User data channel PRB allocation |
| --- | --- | --- | --- | --- |
| Option 1 | 5 MHz | 5 MHz | 5 MHz | 5 MHz |
| Option 2 | 20 MHz | 5 MHz | 5 MHz | 5 MHz |
| Option 3 | 20 MHz | 20 MHz | 5 MHz | 5 MHz |
| Option 4 | 20 MHz | 20 MHz | 20 MHz | 5 MHz |

The impact by each option is discussed as follows. As a result, we propose to support Option 4 for DL, and either Option 3 or 4 for UL.

The impact to DL by options 1-3

In options 1-3, the common channel PRB allocation is limited to only 5MHz. Thus, for CD-SSB with 15 kHz SCS and CORESET #0 with 24RB with 15kHz SCS can only be received by a 5 MHz eRedCap UE. If the network configuration is restricted as such for 5MHz UEs, the impact to the co-existing non-5MHz UEs would be large. For example, short latency benefit using 30kHz SCS/CORESET #0 would be unavailable for non-5MHz UEs. Therefore, the deployment scenario is limited. Alternatively, a new design for 5MHz eRedCap -specific CD-SSB, CORESET #0 and MIB/SIB can be studied. But large standardization effort, spec impact (including RAN2/RAN4) are expected, which may be beyond the available TU of eRedCap. In addition, additional network overhead is consumed, which degrades the spectrum efficiency and more network power consumption as these would be always-on transmissions.

The impact to DL by options 1-2

In options 1-2, BW of BWP is limited to only 5MHz. It means that initial DL BWP for non-RedCap or Rel-17 separate initial DL BWP (within 20MHz) for RedCap UE may not be shared to a 5MHz RedCap UE. Therefore, 5MHz-specific initial DL BWP will also be needed, which would result in some spec impact and additional network overhead to decrease the spectrum efficiency and more network power consumption.

The impact to UL by options 1-2

As well as DL, 5MHz eRedCap-specific initial UL BWP will also be needed and then some spec impact is expected. Furthermore, the available value of Msg1-FDM would be limited compared with Rel-17 RedCap. Therefore, there may be impact to the operation of initial access.

Complexity reduction by each option

From the study below, we think Option 4 can achieve moderate complexity reduction. The complexity reduction of Option 1-3 would not be significant compared with than Option 4.

- Option 4: baseband complexity is reduced because higher layer, FEC encoder/decoder and HARQ buffer can be relaxed (compared with Rel-17 RedCap)

- Option 3: complexity is not so reduced in DL from Option 4 because common channel with 20MHz would not be so big TB size

- Option 1/2: complexity is not so reduced from Option 3/4 because the complexity of RF is not so scaled by the bandwidth and FFT/IFFT part is not so contributing part of the UE complexity.

Summary & proposal

As discussed above, Option 4 would have moderate complexity reduction while less spec impact and performance degradation especially on DL is expected than Options 1-3. Therefore, we propose to support Option 4 for DL. In option 4 for DL, whether the user data PRB allocation is contiguous or not should also be discussed. In our view, it can be dis-contiguous as it would not increase complexity so much and it allows better sharing of common channels with non-RedCap UEs and Rel.17 RedCap UEs. For UL, Option 4 is preferable because of DL-UL symmetricity, but Option 3 is also acceptable as spec/performance impact seems a little. It depends the discussion on how fast UL BWP switching can be carried out.

Proposal 1:	Identify the following options for UE bandwidth reduction to 5MHz.

|  | RF bandwidth | BWP bandwidth | Common channel PRB allocation | User data channel PRB allocation |
| --- | --- | --- | --- | --- |
| Option 1 | 5 MHz | 5 MHz | 5 MHz | 5 MHz |
| Option 2 | 20 MHz | 5 MHz | 5 MHz | 5 MHz |
| Option 3 | 20 MHz | 20 MHz | 5 MHz | 5 MHz |
| Option 4 | 20 MHz | 20 MHz | 20 MHz | 5 MHz |

Proposal 2:	Support Option 4 for DL.

Proposal 3:	For DL, user data channel PRB allocation can be dis-contiguous.

Proposal 4:	Support Option either 3 or 4 for UL.

Conclusion

Proposal 1:	Identify the following options for UE bandwidth reduction to 5MHz.

|  | RF bandwidth | BWP bandwidth | Common channel PRB allocation | User data channel PRB allocation |
| --- | --- | --- | --- | --- |
| Option 1 | 5 MHz | 5 MHz | 5 MHz | 5 MHz |
| Option 2 | 20 MHz | 5 MHz | 5 MHz | 5 MHz |
| Option 3 | 20 MHz | 20 MHz | 5 MHz | 5 MHz |
| Option 4 | 20 MHz | 20 MHz | 20 MHz | 5 MHz |

Proposal 2:	Support Option 4 for DL.

Proposal 3:	For DL, user data channel PRB allocation can be dis-contiguous.

Proposal 4:	Support Option either 3 or 4 for UL.