3GPP TSG RAN WG1 #109-e	R1- 2203765

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.1.2

Source: 	Langbo

Title: 	Maintenance of Enhancements for Multi-TRP Deployment

Document for:	Discussion and Decision

# 1	Introduction

In this document, remaining issues on multi-TRP PDCCH enhancement are addressed. Corresponding text proposals are provided also.

# 2         PTRS Resource Mapping for Linked PDCCHs

When two PDCCH candidates are linked, a UE may still be able to detect PDCCH on a single PDCCH candidate. For PDSCH resource mapping, a union of the two linked PDCCH candidates are always rate matched around to avoid ambiguity. This kind of ambiguity should be taken into account also for PTRS resource mapping, i.e., REs used by the union of the two linked PDCCH candidates are excluded for PTRS resource mapping. The following text changes can be considered:

************************************* Start of TP for TS 38.211  **************************************

< Unchanged parts are omitted >

##### 7.4.1.2.2	Mapping to physical resources

The UE shall assume phase-tracking reference signals being present only in the resource blocks used for the PDSCH, and only if the procedure in [6, TS 38.214] indicates phase-tracking reference signals being used.

If present, the UE shall assume the PDSCH PT-RS is scaled by a factor βPT-RS,i to conform with the transmission power specified in clause 4.1 of [6, TS 38.214] and mapped to resource elements k,lp,μaccording to

ak,l(p,μ)=βPT-RS,irk

when all the following conditions are fulfilled

-	 is within the OFDM symbols allocated for the PDSCH transmission

-	resource element  k,lp,μ  is not used for DM-RS, non-zero-power CSI-RS (except for those configured for mobility measurements or with resourceType in corresponding CSI-ResourceConfig configured as 'aperiodic'), zero-power CSI-RS, SS/PBCH block, a detected PDCCH according to clause 5.1.4.1 of [6, TS38.214], a union of linked PDCCH candidates of the detected PDCCH according to clause 5.1.4.1 of [6, TS38.214] or is declared as 'not available' by clause 5.1.4 of [6, TS 38.214]

The set of time indices  defined relative to the start of the PDSCH allocation is defined by

< Unchanged parts are omitted >

************************************** End of TP for TS 38.211 **************************************

# 3         PDSCH resource mapping for Linked PDCCHs

To improve accuracy of channel estimation, NR supports a same precoding across all contiguous PRBs within a CORESET. The resource mapping of PDSCH is performed around all DMRS within the CORESET in case a PDCCH is detected. It is still open whether two linked CORESETs are allowed to be configured with different precoder granularity and how to perform PDSCH resource mapping in case these two CORESETs are both with precoderGranularity set to 'allContiguousRBs'. In our opinion, configuring different precoder granularities for two linked CORESETs does not bring any meaningful benefit. Therefore, we propose that a UE is provided a same precoderGranularity for two linked CORESETs.

Correspondingly, the following text proposal can be considered.

************************************* Start of TP for TS 38.213***************************************

< Unchanged parts are omitted >

## 10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

For search space sets si and sj that include searchSpaceLinking with value sj and si, respectively, a UE monitors, in monitoring occasions with same index according to each of search space sets si and sj in a slot, PDCCH candidates msi,nCI(L) and msj,nCI(L), with msi,nCI(L)=msj,nCI(L), for detection of a DCI format with same information. The UE expects ksi=ksj, osi=osj, Tsi=Tsj, Msi(L)=Msj(L), and a same number of non-overlapping PDCCH monitoring occasions per slot based on corresponding monitoringSymbolsWithinSlot, for search space sets si and sj. For CORESET pi associated with the search space set si and for CORESET pj associated with the search space set sj, the UE is provided tci-PresentInDCI or tci-PresentDCI-1-2 for either none or both of CORESETs pi and pj. For CORESET pi associated with the search space set si and for CORESET pj associated with the search space set sj, the UE is either not provided coresetPoolIndex value of 1 for any of the two CORESETs, or is provided coresetPoolIndex value of 1 for both CORESETs. For CORESET pi associated with the search space set si and for CORESET pj associated with the search space set sj, the UE is provided a same precoderGranularity.

< Unchanged parts are omitted >

*************************************** End of TP for TS 38.213  *************************************

When the same precoderGranularity of two linked CORESETs is set to 'allContiguousRBs',  PDSCH resource mapping should be rate matched around all DMRS REs within the two CORESETS. Hence, we propose the following text proposal.

************************************* Start of TP for TS 38.214***************************************

< Unchanged parts are omitted >

#### 5.1.4.1	PDSCH resource mapping with RB symbol level granularity

< Unchanged parts are omitted >

If a PDSCH scheduled by a PDCCH would overlap with resources in the CORESET containing the PDCCH, the resources corresponding to a union of the detected PDCCH that scheduled the PDSCH and associated PDCCH DM-RS are not available for the PDSCH. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the resources corresponding to a union of the two PDCCH candidates scheduling the PDSCH and the associated PDCCH DM-RS are not available for the PDSCH. When precoderGranularity configured in a CORESET where the PDCCH was detected is set to 'allContiguousRBs', the associated PDCCH DM-RS are DM-RS in all REGs of the CORESET. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213] and precoderGranularity configured in two associated CORESETs is set to 'allContiguousRBs',  the associated PDCCH DM-RS are DM-RS in all REGs of the two CORESETs. Otherwise, the associated DM-RS are the DM-RS in REGs of the PDCCH.

< Unchanged parts are omitted >

*************************************** End of TP for TS 38.214  *************************************

# 4        Conclusions

In this contribution, the remaining issues on multi-TRP PDCCH enhancement for Rel-17 MIMO enhancements are discussed and the corresponding text changes are proposed.