3GPP TSG RAN WG1 #109-e	    			            R1-2203888

e-Meeting, May 9th – May 20th, 2022

Agenda item:	9.1.1.2

Source: 	Samsung

Title: 	Views on two TAs for m-DCI

Document for:	Discussion and Decision

# Introduction

A new work item has been approved to further evolve MIMO in Rel-18 [1]. Rel-18 improvements include identifying and specifying necessary enhancements for uplink MIMO. In particular, one of the focus areas of Rel-18 is multi-TRP operation, where the timing advance can be different for UL transmission to each TRP. To this end, one of the objectives of the Rel-18 MIMO WI is to study and if justified specify support for TAs:

7. Study, and if justified, specify the following

- Two TAs for UL multi-DCI for multi-TRP operation

In this contribution we present Samsung’s views on support of 2 TAs.

# Discussion

In NR, uplink transmissions are synchronized at the antenna interface, i.e., the uplink signals arriving from different users are aligned within one CP. This is required to maintain orthogonality between the uplink users. To achieve time alignment, network estimates the round trip time of each user and advances the UL transmission time from the UE by a time that depends on the round trip time. The round trip time is the sum of the downlink propagation delay and the uplink propagation delay. Initially, the time advance (TA) is estimated using a random access procedure. The TA value is then maintained using a TA command, which is sent from the network to the UE to change the TA value to ensure that the uplink time at the base station is within certain range. When a UE communicates with two TRPs, it can be at a different distance from each TRP, and hence would require different TA values as illustrated in Figure 1. In Figure 1, the DL and UL propagation delays between the UE and TRP A is TpropA each, hence the round trip propagation delay is 2*TpropA. The DL and UL propagation delays between the UE and TRP B is TpropB each, hence the round trip propagation delay is 2*TpropB. When the UE is communicating with TRP A, the time advance is TA,offset + 2*TpropA. When the UE is communicating with TRP B, the time advance is TA,offset + 2*TpropB. Wherein, TA,offset=NTA,offset*Tc.

Figure 1: DL and UL timings at UE and TRPs when UE is communicating with 2 TRPs.

When the UE is communicating with 2 TRPs, there are two aspects to consider:

- How to measure the TA value of the second TRP.

- How to indicate and use the two TA values.

We consider both of these aspects in this section.

## TA Measurement for the second TRP

For a single TRP, the TA value is determined based on the round-trip-time. The round-trip-time is initially determined using a random access procedure, by measuring the time of arrival of PRACH at the base station which includes the effects of the downlink and uplink propagation delays. The TA value is then maintained by having the base station measure the time of arrival of the uplink signal and determining any adjustment needed to maintain it within a certain range. The adjustment is signalled to UE using a Timing Advance MAC CE command. The Timing Advance MAC CE signals a value TA between 0 and 63 (6 bits) that is interpreted as an advance or delay of the current TA value as:

NTA,new=NTA,old+TA-31∙16∙642μ

When the UE communicates with the two TRPs, the UE/network can maintain two TA values one for each TRP. A way to determine the TA value of the second TRP is to trigger a random access procedure towards the second TRP (assuming that the UE already has the TA value to use for the first TRP). This procedure can be triggered by network, for example through a PDCCH order, or it can be triggered by the UE. We can further discuss the conditions to trigger random access procedure or whether to leave it for UE and network implementation.

An alternative method is to use a RACH-less procedure to determine the TA of the second TRP, when the TA value of the first TRP is known. This can be achieved by measuring the differential downlink propagation delay. The same differential propagation delay can then be applied to the uplink transmission. This method avoids the need for triggering a random access procedure, hence has less signalling overhead. However, it requires that the two TRPs are synchronized or at least the difference in reference time is known.

Proposal 1: Study methods for measuring the TA of a second TRP when the TA of a first TRP is known

- Option 1: RACH based methods

- Option 2: Methods based on differential downlink propagation delay.

## Indication and use of two TA values

For the signalling and usage of the TA values, we can consider several options:

- The UE can be signalled a single TA value, for example the TA of TRP A. The UE further measures the differential DL delay. The differential DL is the difference between the DL propagation delay from TRP A and the DL propagation delay from TRP B: Differential DL Propagation Delay=TpropB-TpropA. Assuming symmetric delay on DL and UL channels, which seems to be a reasonable assumption since the same channel is used for DL and UL transmissions, Differential UL Propagation Delay= Differential DL Propagation Delay. If the UE is signalled the TA for TRP A, i.e., TAA, the UE can determine the the UE can determine the UL transmission time to TRP A and consequently, the UL transmission time to TRP B is the UL transmission time to TRP A + Differential uplink propagation delay (differential uplink propagation delay can equal differential DL propagation delay). For this equation to be valid the reference transmit time of TRP A and TRP B should be synchronized (i.e., the same), similarly the reference receive time of TRP A and TRP B should be synchronized (i.e., the same). If the TRPs are not synchronized, and the network is aware of the difference in reference timings, the network can signal the UE the difference in reference time between TRP A and TRP B and the UE can adjust TAB accordingly

- Alternatively, the UE can be signalled two TA values. The TA values can be part of the same TAG ID, i.e., each TAG ID is associated with two TA values, or can be included in two separate TAG IDs. A TCI state can be associated with a TA (e.g., the first TA of a TAG ID or the second TA of the TAG, alternatively, the first TAG ID or the second TAG ID). The usage of a TCI state for determining the UL spatial transmit filter, also determines which TA value to use. The association can also be based on the TRP index or the PCI in case of inter-cell beam management.

Proposal 2: Study methods for the indication and usage of the two TA values. For example;

- Whether to indicate two TA values in the same TAG or indicate two TAG IDs

- Signalling a single TA value and determining the second TA value based on the differential DL propagation delay.

- Association with a TA when two TAs are signalled to the UE.

# Conclusions

The following proposals have been made regarding support of 2 TAs

Proposal 1: Study methods for measuring the TA of a second TRP when the TA of a first TRP is known

- Option 1: RACH based methods

- Option 2: Methods based on differential downlink propagation delay.

Proposal 2: Study methods for the indication and usage of the two TA values. For example;

- Whether to indicate two TA values in the same TAG or indicate two TAG IDs

- Signalling a single TA value and determining the second TA value based on the differential DL propagation delay.

- Association with a TA when two TAs are signalled to the UE.

# References

- RP-213598, “MIMO Evolution for Downlink and Uplink”, 3GPP RAN#94e, e-Meeting, December, 2021.