3GPP TSG- RAN WG1 Meeting #109-e                                                  R1-2204240

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.3.1

Source:	Apple Inc.

Title:	On Evaluation for AI based Beam Management

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our views on evaluation methodology, KPI and performance evaluation results for AI based beam management.

# Evaluation Methodology

Usually, there are 3 key components for AI: data, model and prediction. The machine learning requires a large amount of data for training and validation. With a certain model (neural network) can be trained based on the known data. With the help of data and model, machine learning can provide certain predictions. The evaluation methodology for AI based beam management should follow the same principle.

## Dataset generation

The system level simulator can be used to generate the dataset. Which data to be generated depends on the exact use case. Now that L1-RSRP is used for beam selection and the inter-cell mobility is out of scope, to speed up to dataset generation procedure, to generate the data from a single cell can be sufficient. Traditional evaluation assumption, e.g. gNB and UE antenna architecture, scenario, and so on, for system level simulation can be used for dataset generation.

Besides, another important aspect is spatial consistency. AI is a good tool to predict the spatial correlation between two UEs. Usually for two UEs with high spatial correlation, the best beam could be highly correlated. Thus, spatial consistency modeling becomes an important aspect to reflect such kind of spatial correlation. In 38.901, two levels of spatial consistency are defined: large-scale parameter based spatial consistency and small-scale parameter based spatial consistency. The large-scale parameters include shadow fading, Ricean K factor, delay spread, and angular spread. The small-scale parameters include cluster-specific random delay, shadowing, offset for AoD/AoA/ZoD/ZoA and sign for AoD/AoA/ZoD/ZoA, random coupling of rays, XPR and random phase.

Figure 1 and Figure 2 shows the best beam distribution for the cases with and without small-scale parameter based spatial consistency, where the best beam is selected based on the beam with highest L1-RSRP. It can be observed that the best beam can be more spatially correlated with full spatial consistency modeling.

Figure 1: Best beam distribution with large-scale parameter based spatial consistency only

Figure 2: Best beam distribution with full spatial consistency

Proposal 1: System level simulator can be used to generate the dataset, where the number of cells can be 1 to speed up the dataset generation procedure and full spatial consistency modeling is recommended.

- Other parameters can be selected based on traditional system level evaluation

## AI Model

There can be different kinds of AI models, e.g. neural networks. For different types of datasets and use cases, different neural networks can be used. For example, for spatial domain beam prediction, if the dataset is based on channel impulse response (CIR), CNN might provide better performance. For time domain beam prediction, some RNN networks may provide better performance. The neural network can be optimized for certain use case. Thus, during the use case study phase, it is not necessary to limit the neural network. If a certain neural network might be specified in future, some further study on the neural network architecture could be necessary.

Proposal 2: During the use case study phase, it is not necessary to define a common AI architecture.

## KPI

Usually L1-RSRP is a good metric to judge whether a selected beam is good or not. In Rel-17 intra-cell and inter-cell beam management, L1-RSRP distribution is agreed as a metric for beam management. For AI based beam management, the same metric can be used, where the L1-RSRP measured from the AI predicted beam can be used as the metric. In addition, for time and spatial domain beam prediction, another possible metric is the beam prediction accuracy, with which, it is possible to judge whether the model can be used for beam prediction or not.

Proposal 3: The KPI for AI based beam prediction could be the beam prediction accuracy and the L1-RSRP distribution for the AI predicted beam.

# Simulation results

In R1-2204241, we provide some discussions on the following use cases for AI based beam management:

- Case 1a: Spatial domain beam prediction with measurement for limited number of beams

- Case 1b: Spatial domain beam prediction with FR1 measurements

- Case 2a: Time domain beam prediction based on past measurement results

- Case 2b: Beam dwelling time prediction based on past measurement results

## Simulation Results for Case 1a

For spatial domain beam prediction with measurement for limited number of beams, a classification network is used with fully connected layers. The input is the normalized L1-RSRP for limited number of network beams.

As shown in Figure 3, UE does not need to measure the all the beams, but it only needs to measure a subset of beams at the initial stage and with the help of machine learning, a new beam search space (BSS).can be identified for next step measurement. The final beam selection can be performance based on the measurement result from the BSS.

Figure 3: AI based beam prediction for Case 1a

Table 1 illustrates the results for BSS prediction accuracy. The simulation is based on 100000 L1-RSRP measurement results between each UE and gNBs beam pairs, which are generated from system level simulation. 70% of the data is used for training and 30% is used for validation. The machine learning is based on 1 hidden layer. It can be observed that it is possible to use AI to predict the beam search space (BSS) to reduce the beam management overhead and latency. Detailed simulation results are provided in Table A-1 in appendix.

Table 1: Simulation results for Case 1a

| Number of beams in BSS | Number of measured Beams in the first step |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 |  | 8 |  | 12 |  | 16 |  |
|  | AI | non-AI | AI | non-AI | AI | non-AI | AI | non-AI |
| 1 | 40.45% | 17.49% | 60.58% | 32.52% | 65.91% | 31.27% | 75.83% | 61.73% |
| 2 | 59.32% | 18.18% | 80.40% | 33.12% | 84.99% | 31.81% | 92.58% | 62.23% |
| 4 | 75.47% | 34.20% | 91.72% | 53.48% | 95.37% | 59.19% | 98.50% | 88.33% |
| 6 | 83.70% | 36.61% | 95.58% | 55.50% | 98.08% | 62.04% | 99.53% | 88.68% |
| 8 | 88.81% | 37.77% | 97.43% | 56.78% | 99.16% | 65.03% | 99.85% | 89.42% |

Observation 1: For case 1, AI based beam search space prediction can provide up to 30% gain compared to non-AI based approach.

## Simulation Results for Case 1b

For spatially correlated UEs, the beam network beam could be highly correlated. Based on spatial correlation, it is possible that the network can predict the best beam for some UEs without beam measurement and report. The spatial correlation can be determined based on some fast-fading parameters, e.g. delay/power/direction for each path. It is not easy to make a hard decision on whether two channels are spatially correlated or not. But with the help of machine learning, it is possible to predict whether two channels are spatially correlated, and they should share the same best network beam.

For case 1b, a UE is assumed to work in a FR1+FR2 CA mode. It is possible to determine the spatial correlation based on some FR1 feedback, and the network can predict the FR2 beam based on the FR1 feedback. In this contribution, we select the normalized channel impulse response (CIR) as the FR1 feedback to predict the FR2 beam. Figure 4 illustrates the procedure for FR1 assisted FR2 beam selection.

Figure 4: Procedure for FR1 assisted FR2 beam selection

The input for the neural network is the first 40 samples for the normalized CIR between UE and FR1 cell. 50000 UEs are dropped within a cell with 3 sectors, where 70% UEs are used for training and 30% UEs are used for validation. For FR2, it is assumed 32 network beams are unitized and each UE contains 2 panels with 4 beams per panel. Detailed simulation results are provided in Table A-1 in appendix.

Figure 5-8 illustrates some system level results for AI based beam selection for LOS only status.

Figure 5: RSRP distribution for AI selected beam for LOS only

Figure 6: Distribution of RSRP gap between ideal beam and AI selected beam for LOS only

Figure 7: Distribution of ideal beam for LOS only

Figure 8: Distribution of AI selected beam for LOS only

Figure 9-12 illustrates some system level results for AI based beam selection for mixed NLOS/LOS case.

Figure 9: RSRP distribution for AI selected beam for mixed NLOS/LOS

Figure 10: Distribution of RSRP gap between ideal beam and AI selected beam for mixed NLOS/LOS

Figure 11: Distribution of ideal beam for mixed NLOS/LOS

Figure 12: Distribution of AI selected beam for mixed NLOS/LOS

It can be observed that AI based FR1 assisted FR2 beam selection scheme can provide accurate beam selection with the accuracy of about 70% for mixed NLOS/NLOS case and 90% for LOS only case.

Observation 2: For Case 1b, AI based FR1 assisted FR2 beam selection scheme can provide accurate beam selection with the accuracy of about 70% for mixed NLOS/NLOS case and 90% for LOS only case.

## Simulation Results for Case 2a

Usually, the UE moving direction and speed would not change within small time domain granularity, e.g. within 1 second. The moving direction and speed may be obtained based on some past measurement results. It could be possible to predict the best beam at the next beam report/indication interval. Thus, it can reduce the overhead for beam measurement reference signal and beam report, and it can also reduce the UE power consumption. Figure 13 illustrates the procedure for AI based time domain beam prediction.

Figure 13: Procedure for AI based time domain beam prediction

In the simulation, we assume UE performs beam measure and report for every 40ms, and UE’s moving speed is 30km/h with random orientation. AI is used to predict the beam for the next 40ms. The following 4 different datasets are considered, where the measurement is based on the past 600ms.

- Dataset 1: past best beam index

- Dataset 2: past beam index + RSRP for the best beam

- Dataset 3: past beam index + RSRP for top 2 beams

- Dataset 4: past beam index + RSRP for top 4 beams

Figure 14 and Figure 15 illustrate the L1-RSRP distribution for AI predicted beam for the 4 datasets and the L1-RSRP gap distribution compared to the ideal beam. It can be observed that past beam index + RSRP for the best beam could be sufficient for time domain beam prediction, which can provide about 65% beam prediction accuracy.

Figure 14: L1-RSRP distribution for AI predicted beam

Figure 15: L1-RSRP gap distribution for AI predicted beam compared to ideal beam

Figure 16 and Figure 17 illustrate the L1-RSRP distribution for AI predicted beam for the case with and without UE rotation and the L1-RSRP gap distribution compared to the ideal beam, where the UE rotation speed is assumed to be 360 degree/second. It can be observed that AI can still predict the beam with 65% beam prediction accuracy with UE rotation.

Figure 16: L1-RSRP distribution for AI predicted beam

Figure 17: L1-RSRP gap distribution for AI predicted beam compared to ideal beam

Figure 18 and Figure 19 illustrate the L1-RSRP distribution for AI predicted beam for the LOS only and mixed LOS/NLOS and the L1-RSRP gap distribution compared to the ideal beam, where the UE rotation speed is assumed to be 360 degree/second. It can be observed that AI can still predict the beam with 65% beam prediction accuracy for mixed LOS/NLOS case and 77% beam prediction accuracy for LOS only case.

Figure 18: L1-RSRP distribution for AI predicted beam

Figure 19: L1-RSRP gap distribution for AI predicted beam compared to ideal beam

Observation 3: For case 2a, AI can provide 65% beam prediction accuracy for mixed LOS/NLOS case and 77% beam prediction accuracy for LOS only case.

## Simulation Results for Case 2b

Similar to case 2a, it could be assumed that the UE moving speed and direction would not change within a short time, and it is possible to predict the beam dwelling time, so that the network can trigger beam report at proper time. Thus, it can reduce the overhead for beam measurement reference signal and beam report, and it can also reduce the UE power consumption. Figure 20 illustrates the procedure for AI based time domain beam dwelling time prediction.

Figure 20: Procedure for AI based time domain beam dwelling time prediction

In the simulation, we assume UE performs beam measure and report for every 40ms, and UE’s moving speed is 30km/h with random orientation. AI is used to predict the beam for the next 40ms. The following dataset 2 in section 3.3 is used, where the measurement is based on the past 600ms. The prediction of beam dwelling time is for the next 200ms, where the prediction interval is 40ms.

Figure 21 and Figure 22 provides the beam dwelling time prediction accuracy for LOS only and mixed LOS/NLOS case. It can be observed that AI can provide up to 76% beam dwelling time prediction accuracy.

Figure 21: Beam dwelling time prediction for LOS only

Figure 22: Beam dwelling time prediction for mixed LOS/NLOS

Observation 4: AI can provide up to 76% beam dwelling time prediction accuracy.

Proposal 4: Study following use cases for AI based beam management:

- Case 1a: Spatial domain beam prediction with measurement for limited number of beams

- Case 1b: Spatial domain beam prediction with FR1 measurements

- Case 2a: Time domain beam prediction based on past measurement results

- Case 2b: Beam dwelling time prediction based on past measurement results

# Conclusion

In this contribution, we provided some discussion on evaluation for AI based beam management. Based on the discussion, the following observations have been achieved.

Observation 1: For case 1, AI based beam search space prediction can provide up to 30% gain compared to non-AI based approach.

Observation 2: For Case 1b, AI based FR1 assisted FR2 beam selection scheme can provide accurate beam selection with the accuracy of about 70% for mixed NLOS/NLOS case and 90% for LOS only case.

Observation 3: For case 2a, AI can provide 65% beam prediction accuracy for mixed LOS/NLOS case and 77% beam prediction accuracy for LOS only case.

Observation 4: AI can provide up to 76% beam dwelling time prediction accuracy.

The following proposals have been achieved.

Proposal 1: System level simulator can be used to generate the dataset, where the number of cells can be 1 to speed up the dataset generation procedure and full spatial consistency modeling is recommended.

- Other parameters can be selected based on traditional system level evaluation

Proposal 2: During the use case study phase, it is not necessary to define a common AI architecture.

Proposal 3: The KPI for AI based beam prediction could be the beam prediction accuracy and the L1-RSRP distribution for the AI predicted beam.

Proposal 4: Study following use cases for AI based beam management:

- Case 1a: Spatial domain beam prediction with measurement for limited number of beams

- Case 1b: Spatial domain beam prediction with FR1 measurements

- Case 2a: Time domain beam prediction based on past measurement results

- Case 2b: Beam dwelling time prediction based on past measurement results

# Appendix – Simulation assumption

Table A-1: Simulation Assumption

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban Macro |
| Number of cells | 1, 7 |
| Number of sectors per cell | 3 |
| Number of UEs | 50000, 100000 |
| Carrier frequency | FR1: 3GHz FR2: 28GHz |
| Subcarrier spacing | FR1: 30kHz FR2: 120kHz |
| Bandwidth | FR1: 20MHz FR2: 80MHz |
| gNB antenna configuration | FR1: (M, N, P, Mp, Np, Mg, Ng) = (2, 8, 2, 2, 8, 1, 1) FR2: (M, N, P, Mp, Np, Mg, Ng) = (4, 8, 2, 1, 1, 1, 1) |
| UE antenna configuration | FR1: (M, N, P, Mp, Np, Mg, Ng) = (1, 2, 2, 1, 2, 1, 1) FR2: (M, N, P, Mp, Np, Mg, Ng) = (1, 4, 2, 1, 1, 1, 2) |
| UE distribution | Uniformly distributed |
| UE orientation | Random |
| Slow fading model | 38.901 |
| Fast fading model | 38.901 with spatial consistency |
| FR2 gNB beam pattern | Case 1: 4 beams in vertical * 8 beams in horizontal Case 2: 2 beams in vertical * 6 beams in horizontal |
| FR2 UE beam pattern | 4 beams in horizontal per panel |