3GPP TSG RAN WG1 #109-e			R1-2204311

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.5.2.1

Source: 	CMCC

Title: 	Discussion on solutions for integrity of RAT-dependent positioning techniques

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, a new SID on study on expanded and improved NR positioning was approved [1]. The motivation to extend the integrity to RAT-dependent positioning is justified:

- Improved accuracy, integrity, and power efficiency: - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

In this contribution, we provide our preliminary views on the error source of the integrity for RAT-dependent positioning.

Discussion

Integrity is a measure of the trust in the accuracy of the position-related data and the ability to provide associated alerts, which is an important performance metric to ensure the reliability of a positioning system. In Rel-17, the motivation to include the integrity as a performance metric for both RAT-dependent and RAT-independent positioning was identified; however, due to the heavy workload and limited time, only the integrity of the A-GNSS positioning was studied and specified.

In Rel-16 and Rel-17, the NR positioning mainly focused on the accuracy performance metric, and the positioning accuracy, taking horizontal accuracy as an example, has been improved from < 3m (indoor) / < 10m (outdoor) for 80% UEs to < 1m for 90% UEs for commercial use cases, and to < 0.2m or 0.5m for 90% UEs for IIoT use cases. Mathematically, the accuracy places the cut-off at 90% to ensure that the positioning errors of 90% of UEs should be less than the accuracy requirement; while the integrity focused on the tail of the error distribution, and to keep the probability of hazardous situations extremely low. Hence, integrity is an important metric in use cases such as V2X, real-time tool assignment in assembly line, tracking of forklifts and AGVs in logistics and warehousing, etc.

To study and specify solutions for integrity of RAT-dependent positioning, it should be RAN1’s work to identify the error sources of the RAT-dependent positioning, which is quite different and more complicated when compared to the error sources of A-GNSS integrity. In our views, the potential error sources should include channel states, PRS/SRS configurations, errors in location estimates computation, etc.

Observation 1: The following error sources of RAT-dependent positioning should be considered:

- Channel states;

- PRS/SRS configurations;

- Location estimations computation

Conclusions

In this contribution, we provide our views on the integrity of RAT-dependent positioning, and the following observation is made:

Observation 1: The following error sources of RAT-dependent positioning should be considered:

- Channel states;

- PRS/SRS configurations;

- Location estimations computation

References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, 3GPP TSG RAN Meeting #94e.