3GPP TSG RAN WG1 #109-e			            R1-2204705

e-Meeting, May 9th – 20th, 2022

Agenda Item: 8.7.2

Source: MediaTek Inc.

Title: Maintenance on DCI-based PDCCH Monitoring Adaptation

Document for: Discussion and Decision

# Introduction

In this contribution, the following identified issues for DCI-based PDCCH monitoring adaptation after RAN1#108-e meeting [1]-[5] will be discussed and resolved:

- Application delay and retransmission handling

- Cross-feature interactions

  - PDCCH skipping and RACH/SR

  - PDCCH skipping outside DRX Active Time

  - PDCCH monitoring adaptation and active BWP change

- Working assumptions confirmation

# Application Delay and Retransmission Handling

In RAN1#108-e meeting, there are still FFS’s related to application delay and retransmission handling for SSSG switching and PDCCH skipping, as quoted below

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213 Agreement - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running

Regarding the purpose of minimum applicable scheduling offset (minK0/minK2), it is to achieve UE power saving gain from the following two aspects:

- When there is PDSCH/PUSCH scheduled, UE has a preparation time for “ramping up” data processing unit(s), which allows UE to put the data processing unit(s) in power saving mode before it is notified of data scheduling.

- When there no PDSCH scheduled, UE can save the RF reception for PDSCH by virtue of the PDCCH monitoring outcome known minK0 slot(s) ahead.

By the above, the power saving mechanism with minK0/minK2 is different from that of PDCCH monitoring adaptation. Consequently, it is not logical to have the application delay of PDCCH monitoring adaptation depend on a minimum applicable scheduling offset.

Observation 1: UE power saving gain with non-zero minK0/minK2 is from 1) allowing data processing unit(s) to stay in power saving mode and 2) saving RF reception of PDSCH by knowing no data scheduled earlier, and neither is related to PDCCH monitoring or processing.

Proposal 1: Application delay of R17 PDCCH monitoring adaptation scheme, i.e., SSSG switching or PDCCH skipping triggered by scheduling DCI(s), doesn’t depend on any minimum applicable scheduling offset.

As for the issue of retransmission handling, the main purpose is to avoid blocking retransmission due to a long PDCCH skipping duration. However, the FFS will induce complicated timer behavior, e.g., whether to change monitoring behavior during the drx-RetransmissionTimerDL/UL if UE receives a new indication during the timer, and whether the uplink/downlink scheduling DCI can impact the behavior of rx-RetransmissionTimerDL/UL. Also, since the drx-RetransmissionTimerUL will always be triggered after any uplink data transmission, existing solution cannot provide power saving for such case.

On the other hand, retransmission handling issue can be avoided by network implementation, e.g., applying lower MCS for the last TB or triggering skipping by a (dummy) UL scheduling after confirming all DL TBs are correctly received by UE. Although either of the above network solutions can be further optimized for reduced system overhead, we suggest proceeding with the baseline solution for Rel-17 WI of UE power saving enhancement and leave the potential optimization to Rel-18, e.g, in SI/WI of XR enhancement.

Observation 2: Retransmission handling issue can be avoided by network implementation, e.g., applying lower MCS for the last TB or triggering skipping by a (dummy) UL scheduling after confirming all DL TBs are correctly received by UE. Although either of the above network solutions can be further optimized for reduced system overhead, such optimization can be left to Rel-18.

Proposal 2: No additional UE behavior for retransmission handling is included in Rel-17.

# Cross-Feature Interactions

## PDCCH Skipping and RACH/SR

In RAN1#108-e, it is agreed that the UE will not apply PDCCH skipping during RAR window/MsgB window/contention resolution timer or when SR is pending. However, whether to resume PDCCH skipping after those conditions is not yet decided:

Agreement Answer 2 for R1-2200884 (R2-2201960): RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification. o It is RAN1 understanding PDCCH skipping is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending. • FFS: If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping (i.e., PDCCH skipping is not activated).

When RACH or SR is started, network is expected to response to UE for further handling. In this regard, terminating UE PDCCH skipping as per RACH or SR looks reasonable to avoid any impact to legacy operations. Since UE can be further indicated to perform PDCCH skipping after network finishes handling the UE, UE power consumption impact can be minimized accordingly. By the above, the following proposal is suggested:

Proposal 3: PDCCH skipping is terminated when the RAR window/MsgB window/contention resolution timer is started or when SR is pending.

- Note: UE can be indicated to perform PDCCH skipping after RACH or SR is handled by network.

For the interaction with DRX Active Time, the following issue is highlighted in FL summary [2]:

[High] Proposal 7-3a (v1): The PDCCH skipping applies only in active time Down-select from the following options: Option 1a: the PDCCH skipping duration is considered irrespective the slot(s) are in active time or not, Option 1b: PDCCH skipping is terminated when UE enters outside active time

Since there is no PDCCH monitoring during DRX OFF, the intention of continuing PDCCH skipping is regarded to skip PDCCH monitoring of the upcoming DRX on-duration(s). On the other hand, skipping PDCCH monitoring of a DRX on-duration is already achieved by Rel-16 DCI format 2_6. Consequently, there is no need of duplicated functionality in Rel-17. In this regard, the following proposal is suggested:

Proposal 4: PDCCH skipping is terminated when UE enters outside active time (i.e., Option 1b is supported)

- Note: skipping PDCCH monitoring of DRX on-duration is already achieved by Rel-16 DCI format 2_6

Proposal 5: Adopt the following TP to Section 10.4 of TS 38.213:

<Unchanged parts are omitted> When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. The UE terminates skipping of PDCCH monitoring whenever any of the following condition holds [11, TS 38.321]: - RAR window/MsgB window/contention resolution timer is started - SR is pending - Outside Active Time of a DRX group configured for the serving cell If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on an active DL BWP of a serving cell and the timer is running, the UE <Unchanged parts are omitted>

## PDCCH Monitoring Adaptation and Active BWP Change

During the count-down of SSSG timer or the PDCCH skipping duration, the UE may change the active BWP due to BWP timer expiration or an active BWP change indication by a scheduling DCI. The PDCCH monitoring behavior should be specified for the two cases, respectively. In the case of BWP timer expiration, since there is no DCI indication, UE behavior in the target BWP should be clarified. It is proposed that UE applies the default PDCCH monitoring behavior and resets every SSSG timer according to the related configuration in the target BWP, if available.

In the case that UE is indicated to change active BWP and PDCCH monitoring behavior at the same time, UE should apply the indicated PDCCH monitoring behavior according to the configuration in the target BWP after the active BWP change. If there is no related configuration in the target BWP, UE applies the default PDCCH monitoring behavior in the target BWP. In order to avoid the complexity of time alignment caused by active BWP change with different SCS settings, it is proposed that UE counts the SSSG timer or PDCCH skipping duration after the active BWP change.

Proposal 6: After active BWP change triggered by BWP timer expiration, UE applies the default PDCCH monitoring behavior and resets every SSSG timer according to the related configuration in the target BWP, if available

Proposal 7: If UE is indicated to change active BWP and PDCCH monitoring behavior at the same time,

- UE applies the indicated PDCCH monitoring behavior according to the configuration in the target BWP after the active BWP change

- UE counts the SSSG timer or PDCCH skipping duration after the active BWP change

Proposal 8: Adopt the following text proposal as a new paragraph in the end of Section 10.4 of TS 38.213:

<Unchanged parts are omitted> After active BWP change triggered by BWP timer expiration, UE applies the default PDCCH monitoring behavior and resets every SSSG timer according to the related configuration in the target BWP, if available. If UE is indicated to change active BWP and PDCCH monitoring behavior at the same time, the UE • applies the indicated PDCCH monitoring behavior according to the configuration in the target BWP after the active BWP change • counts the SSSG timer or PDCCH skipping duration after the active BWP change When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. <Unchanged parts are omitted>

# Confirm Working Assumptions

For the following working assumptions in RAN1#108-e, there is no concern from UE perspective, and we proposal to approve all of them:

Proposal 9: Confirm the following working assumption:

Agreement If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI - Otherwise, decrease the timer value by one after each slot. - Note: There is no timer for SSSG switching is running for UE in SSSG#0

Proposal 10: Confirm the following working assumption:

Working assumptions: - When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay. - If SSSG timer expires in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

Proposal 11: Confirm the working assumption without FFS:

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213

# Conclusion

In this contribution, the remaining issues for DCI-based PDCCH monitoring adaption are discussed and resolved. In particular, the following are provided:

Observation 1: UE power saving gain with non-zero minK0/minK2 is from 1) allowing data processing unit(s) to stay in power saving mode and 2) saving RF reception of PDSCH by knowing no data scheduled earlier, and neither is related to PDCCH monitoring or processing.

Proposal 1: Application delay of R17 PDCCH monitoring adaptation scheme, i.e., SSSG switching or PDCCH skipping triggered by scheduling DCI(s), doesn’t depend on any minimum applicable scheduling offset.

Observation 2: Retransmission handling issue can be avoided by network implementation, e.g., applying lower MCS for the last TB or triggering skipping by a (dummy) UL scheduling after confirming all DL TBs are correctly received by UE. Although either of the above network solutions can be further optimized for reduced system overhead, such optimization can be left to Rel-18.

Proposal 2: No additional UE behavior for retransmission handling is included in Rel-17.

Proposal 3: PDCCH skipping is terminated when the RAR window/MsgB window/contention resolution timer is started or when SR is pending.

- Note: UE can be indicated to perform PDCCH skipping after RACH or SR is handled by network.

Proposal 4: PDCCH skipping is terminated when UE enters outside active time (i.e., Option 1b is supported)

- Note: skipping PDCCH monitoring of DRX on-duration is already achieved by Rel-16 DCI format 2_6

Proposal 5: Adopt the following TP to Section 10.4 of TS 38.213:

<Unchanged parts are omitted> When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. The UE terminates skipping of PDCCH monitoring whenever any of the following condition holds [11, TS 38.321]: - RAR window/MsgB window/contention resolution timer is started - SR is pending - Outside Active Time of a DRX group configured for the serving cell If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on an active DL BWP of a serving cell and the timer is running, the UE <Unchanged parts are omitted>

Proposal 6: After active BWP change triggered by BWP timer expiration, UE applies the default PDCCH monitoring behavior and resets every SSSG timer according to the related configuration in the target BWP, if available

Proposal 7: If UE is indicated to change active BWP and PDCCH monitoring behavior at the same time,

- UE applies the indicated PDCCH monitoring behavior according to the configuration in the target BWP after the active BWP change

- UE counts the SSSG timer or PDCCH skipping duration after the active BWP change

Proposal 8: Adopt the following text proposal as a new paragraph in the end of Section 10.4 of TS 38.213:

<Unchanged parts are omitted> After active BWP change triggered by BWP timer expiration, UE applies the default PDCCH monitoring behavior and resets every SSSG timer according to the related configuration in the target BWP, if available. If UE is indicated to change active BWP and PDCCH monitoring behavior at the same time, the UE • applies the indicated PDCCH monitoring behavior according to the configuration in the target BWP after the active BWP change • counts the SSSG timer or PDCCH skipping duration after the active BWP change When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. <Unchanged parts are omitted>

Proposal 9: Confirm the following working assumption:

Agreement If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI - Otherwise, decrease the timer value by one after each slot. - Note: There is no timer for SSSG switching is running for UE in SSSG#0

Proposal 10: Confirm the following working assumption:

Working assumptions: - When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay. - If SSSG timer expires in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

Proposal 11: Confirm the working assumption without FFS:

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213

# Reference

“Draft Report of 3GPP TSG RAN WG1 #108-e”, MCC support, RAN1#108-e

R1-2202895, “Final FL summary of DCI-based power saving adaptation”, Moderator (vivo), RAN 1 #108-e

R1-2202933, “Collection of RAN1-related agreements for UE power saving enhancements for NR (post RAN1 #108-e)”, Rapporteur (MediaTek), RAN1#108-e

R1-2202969, “Corrections on UE power saving enhancements in 38.212”, Huawei, RAN1#108-e

R1-2202953, “Corrections on UE power savings enhancements in NR (38.213)”, Samsung, RAN1#108-e