3GPP TSG RAN WG1 Meeting #109-e    	            R1-2204975

May 9th – 20th, 2022

Source:	Qualcomm Incorporated

Title:	Discussion on Rel-16 span-based PDCCH monitoring

Agenda Item:	7.2.11

Document for: 	Discussion and Decision

# Discussion

In Rel. 16 URLLC, a new span-based PDCCH monitoring capability with limitations on the number of CCEs/BDs per span is specified. For this capability, each span is defined as follows [1]:

# 10 UE procedure for receiving control information … A UE can indicate a capability to monitor PDCCH according to one or more of the combinations = (2, 2), (4, 3), and (7, 3) per SCS configuration of and . A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. Each PDCCH monitoring occasion is within one span. If a UE monitors PDCCH on a cell according to combination , the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of symbols between the first symbol of two consecutive spans, including across slots. A span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends, where the number of symbols of the span is up to . …

Based on the span definition above, for the (2,2) pattern, a UE must be able to perform PDCCH decoding in every symbol. As a consequence, the control decoding overhead under the (2,2) pattern could be significant for a UE. Although PDCCH monitoring on every symbol of a slot is not required under (4,3) and (7,3) patterns, PDCCH processing across up to 3 consecutive symbols, in particular with timing capability 2 for DL and/or UL, is challenging.

Given the mentioned issue, we propose to add additional patterns, in particular (2,1), (4,1) and (7,1), so that the required scheduling flexibility for supporting URLLC services can be maintained, while easing the UE complexity. This can be done by introducing a new FG as follows which includes the new span patterns:

| 11-2x | Rel-16 PDCCH monitoring capability | 1. Supported combination(s) of (X, Y, m). For each reported combination, the UE supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span and the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span 2. Maximum number of DL and UL unicast DCI formats in a span For the set of monitoring occasions which are within the same span: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD |  | pdcch-Monitoring-r16 { pdsch-ProcessingType1-r16{ scs-15kHz-r16 PDCCH-MonitoringOccasions-r16, scs-30kHz-r16 PDCCH-MonitoringOccasions-r16 }, pdsch-ProcessingType2-r16 { scs-15kHz-r16 PDCCH-MonitoringOccasions-r16, scs-30kHz-r16 PDCCH-MonitoringOccasions-r16 } | FeatureSetDownlink-v1610 | n/a | n/a | This capability is signalled for SCS 15 kHz and 30 kHz. For μ =0 and 1, candidate value set for (X, Y, μ): {(7, 1, μ), (4, 1, μ), (2, 1, μ)} For component 1, a list of separate UE capabilities (X, Y, μ) for processing capability #1; For component 1, a list of separate UE capabilities (X, Y, μ) for processing capability #2; | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Since, the (X,Y) patterns and their associated CCE/BD limits are included in 38.213, the following changes are then proposed:

TP to TS38.213 Section 10 (Rel-16)

# 10 UE procedure for receiving control information … A UE can indicate a capability to monitor PDCCH according to one or more of the combinations = (2,1), (2, 2), (4,1), (4, 3), (7,1) and (7, 3) per SCS configuration of and . A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. Each PDCCH monitoring occasion is within one span. If a UE monitors PDCCH on a cell according to combination , the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of symbols between the first symbol of two consecutive spans, including across slots. A span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends, where the number of symbols of the span is up to . … ## 10.1 UE procedure for determining physical downlink control channel assignment … A PDCCH candidate with index for a search space set using a set of CCEs in a CORESET on the active DL BWP for serving cell is not counted for monitoring if there is a PDCCH candidate with index for a search space set , or if there is a PDCCH candidate with index and , in the CORESET on the active DL BWP for serving cell using a same set of CCEs, the PDCCH candidates have identical scrambling, and the corresponding DCI formats for the PDCCH candidates have a same size; otherwise, the PDCCH candidate with index is counted for monitoring. Table 10.1-2 provides the maximum number of monitored PDCCH candidates, , per slot for a UE in a DL BWP with SCS configuration for operation with a single serving cell. Table 10.1-2: Maximum number of monitored PDCCH candidates per slot for a DL BWP with SCS configuration for a single serving cell Maximum number of monitored PDCCH candidates per slot and per serving cell. 0; 44. 1; 36. 2; 22. 3; 20. Table 10.1-2A provides the maximum number of monitored PDCCH candidates, , per span for a UE in a DL BWP with SCS configuration for operation with a single serving cell. Table 10.1-2A: Maximum number of monitored PDCCH candidates in a span for combination (X, Y) for a DL BWP with SCS configuration for a single serving cell Maximum number of monitored PDCCH candidates per span for combination and per serving cell. (2,1), (2, 2); (4,1), (4, 3); (7,1), (7, 3). 0; 14; 28; 44. 1; 12; 24; 36. Table 10.1-3 provides the maximum number of non-overlapped CCEs, , for a DL BWP with SCS configuration that a UE is expected to monitor corresponding PDCCH candidates per slot for operation with a single serving cell. CCEs for PDCCH candidates are non-overlapped if they correspond to - different CORESET indexes, or - different first symbols for the reception of the respective PDCCH candidates. Table 10.1-3: Maximum number of non-overlapped CCEs per slot for a DL BWP with SCS configuration for a single serving cell Maximum number of non-overlapped CCEs per slot and per serving cell. 0; 56. 1; 56. 2; 48. 3; 32. Table 10.1-3A provides the maximum number of non-overlapped CCEs, , for a DL BWP with SCS configuration that a UE is expected to monitor corresponding PDCCH candidates per span for operation with a single serving cell. Table 10.1-3A: Maximum number of non-overlapped CCEs in a span for combination (X, Y) for a DL BWP with SCS configuration for a single serving cell Maximum number of non-overlapped CCEs per span for combination and per serving cell. (2,1), (2, 2); (4,1), (4, 3); (7,1), (7, 3). 0; 18; 36; 56. 1; 18; 36; 56. …

Based on the specified FGs for Rel. 16 URLLC, the following FGs have FG 11-2 as their prerequisite either directly or indirectly: FG 11-2a, 11-2b, 11-2c, 11-2d, 11-2e, 11-2f and 11-2g. With the addition of the proposed FG 11-2x, this FG can be added as a prerequisite to FG 11-2a, 11-2b, 11-2d and 11-2f. However, if the span-based feature is already implemented and there are concerns on backward compatibility, the alternative approach is to create a mirrored version of FG 11-2a, 11-2b, 11-2c, 11-2d, 11-2e, 11-2f and 11-2g which are directly or indirectly related to FG 11-2x.

Proposal: For R16 span-based PDCCH monitoring capability, include (2,1), (4,1) and (7,1) as the new (X,Y) patterns and introduce the following FG:

| 11-2x | Rel-16 PDCCH monitoring capability | 1. Supported combination(s) of (X, Y, m). For each reported combination, the UE supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span and the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span 2. Maximum number of DL and UL unicast DCI formats in a span For the set of monitoring occasions which are within the same span: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD |  | pdcch-Monitoring-r16 { pdsch-ProcessingType1-r16{ scs-15kHz-r16 PDCCH-MonitoringOccasions-r16, scs-30kHz-r16 PDCCH-MonitoringOccasions-r16 }, pdsch-ProcessingType2-r16 { scs-15kHz-r16 PDCCH-MonitoringOccasions-r16, scs-30kHz-r16 PDCCH-MonitoringOccasions-r16 } | FeatureSetDownlink-v1610 | n/a | n/a | This capability is signalled for SCS 15 kHz and 30 kHz. For μ =0 and 1, candidate value set for (X, Y, μ): {(7, 1, μ), (4, 1, μ), (2, 1, μ)} For component 1, a list of separate UE capabilities (X, Y, μ) for processing capability #1; For component 1, a list of separate UE capabilities (X, Y, μ) for processing capability #2; | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Further, apply the following changes to TS38.213 (Rel. 16):

TP to TS38.213 Section 10 (Rel-16):

Reasons for change: Introducing (2,1), (4,1) and (7,1) as new PDCCH monitoring span patterns for R-16 PDCCH monitoring capability to ease UE complexity.

Summary of change: Addition of the new patterns as possible candidates and determining their per-span CCE/BD limits.

Consequence if not adopted: Supporting capability 2 with the new Rel.16 span-based PDCCH has significant UE complexity

# 10 UE procedure for receiving control information … A UE can indicate a capability to monitor PDCCH according to one or more of the combinations = (2,1), (2, 2), (4,1), (4, 3), (7,1) and (7, 3) per SCS configuration of and . A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. Each PDCCH monitoring occasion is within one span. If a UE monitors PDCCH on a cell according to combination , the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of symbols between the first symbol of two consecutive spans, including across slots. A span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends, where the number of symbols of the span is up to . … ## 10.1 UE procedure for determining physical downlink control channel assignment … A PDCCH candidate with index for a search space set using a set of CCEs in a CORESET on the active DL BWP for serving cell is not counted for monitoring if there is a PDCCH candidate with index for a search space set , or if there is a PDCCH candidate with index and , in the CORESET on the active DL BWP for serving cell using a same set of CCEs, the PDCCH candidates have identical scrambling, and the corresponding DCI formats for the PDCCH candidates have a same size; otherwise, the PDCCH candidate with index is counted for monitoring. Table 10.1-2 provides the maximum number of monitored PDCCH candidates, , per slot for a UE in a DL BWP with SCS configuration for operation with a single serving cell. Table 10.1-2: Maximum number of monitored PDCCH candidates per slot for a DL BWP with SCS configuration for a single serving cell Maximum number of monitored PDCCH candidates per slot and per serving cell. 0; 44. 1; 36. 2; 22. 3; 20. Table 10.1-2A provides the maximum number of monitored PDCCH candidates, , per span for a UE in a DL BWP with SCS configuration for operation with a single serving cell. Table 10.1-2A: Maximum number of monitored PDCCH candidates in a span for combination (X, Y) for a DL BWP with SCS configuration for a single serving cell Maximum number of monitored PDCCH candidates per span for combination and per serving cell. (2,1), (2, 2); (4,1), (4, 3); (7,1), (7, 3). 0; 14; 28; 44. 1; 12; 24; 36. Table 10.1-3 provides the maximum number of non-overlapped CCEs, , for a DL BWP with SCS configuration that a UE is expected to monitor corresponding PDCCH candidates per slot for operation with a single serving cell. CCEs for PDCCH candidates are non-overlapped if they correspond to - different CORESET indexes, or - different first symbols for the reception of the respective PDCCH candidates. Table 10.1-3: Maximum number of non-overlapped CCEs per slot for a DL BWP with SCS configuration for a single serving cell Maximum number of non-overlapped CCEs per slot and per serving cell. 0; 56. 1; 56. 2; 48. 3; 32. Table 10.1-3A provides the maximum number of non-overlapped CCEs, , for a DL BWP with SCS configuration that a UE is expected to monitor corresponding PDCCH candidates per span for operation with a single serving cell. Table 10.1-3A: Maximum number of non-overlapped CCEs in a span for combination (X, Y) for a DL BWP with SCS configuration for a single serving cell Maximum number of non-overlapped CCEs per span for combination and per serving cell. (2,1), (2, 2); (4,1), (4, 3); (7,1), (7, 3). 0; 18; 36; 56. 1; 18; 36; 56. …

# Conclusion

In this contribution, we propose the following:

Proposals:

- Introduce the new (X,Y) = (2,1), (4,1) and (7,1) span patterns for Rel. 16 PDCCH monitoring capability.

- Add the new patterns to TS 38.213

- Discuss and decide which alternatives to choose to reflect the dependency between the new FG and FG 11-2a, 11-2b, 11-2c, 11-2d, 11-2e, 11-2f and 11-2g.

- Send a LS to RAN2 to capture the new FG(s).

# Reference

[1] 3GPP TS 38.213