| 1-15 | Proposed conclusion 1-15: On Rel-17 unified TCI framework, if a UE is configured with CrossCarrierSchedulingConfig for a serving cell the value of the DCI field ‘carrier indicator’ corresponds to the value indicated by CrossCarrierSchedulingConfig. - The codepoint indicated by the DCI field ‘Transmission Configuration Indicator’ is applied to the carrier indicated by the DCI field ‘carrier indicator’ and all CCs configured in a same CC list as that carrier, and corresponds to indicated TCI state configured and activated for that carrier and all CCs, respectively. |
| --- | --- |
| 1-20 | Proposal 1-20: To calculate the Type 1 power headroom based on a reference PUSCH, the UE uses the PUSCH power control parameters (i.e., PL-RS, P0, alpha, closed loop index) associated with the indicated joint/UL-TCI state. |
| 2-2A | Proposal 2-2A: On inter-cell beam management, the PDCCH/PDSCH should be rate matched around the SSBs indicated by ssb-PositionsInBurst-r17 for the same PCI as that associated with TCI state of the PDSCH /PDCCH - Send LS to RAN4 on whether there is requirements in RAN4 that assumes UE to measure SSB for L1-RSRP measurement and receiving PDSCH /PDCCH on the same RE in FR1. Revisit this issue after there is RAN4 feedback. |
| 3-3 | Proposal 3-3: On Rel-17 DCI -based beam indication, for both CA and non-CA cases, - TCI update signaling is applied to active all configured BWP(s) in one or a set of CCs - Note: The applied active BWP for the indicated TCI refers to the active BWP in the slot when the TCI is applied - BAT should count the BeamAppTime_r17 in active BWP (s) only - Note: The BAT is counted based on the active BWP(s) in the slot when the DCI is transmitted |
| 4-2 | Proposal 4-2(Original): On Rel.17 enhancements to facilitate UE -initiated panel activation and selection, the bitwidth of the capability index reported in beam report is fixed to 2-bit. |