3GPP TSG RAN meeting #96	RP-221064

Budapest, Hungary, June 06-09, 2022

## Status Report to TSG

Agenda item:	9.3.1.5

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_SL_enh2 |  |  |  |  |
| Unique ID | 940097 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-220300 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 12/2023 | Performance part: 06/2024 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: 7% | Performance Part: 0% | Testing part: xx % |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Kevin Lin |
|  | Company | OPPO |
|  | Email | Kevin.Lin@oppo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

NR sidelink CA operation

- On hold until further checking in RAN#97

NR Sidelink operation in FR1 unlicensed spectrum

Agreement

Type 1 and Type 2 (2A/2B/2C) channel access procedures, transmission gap and LBT sensing idle time requirements specified in TS37.213 for NR-U are taken as baseline for NR sidelink operation in a shared channel.

- FFS conditions for the actual channel access type(s) used for each SL channel and signal transmitted, and based on COT sharing conditions (if supported)

- FFS whether UL CAPC or DL CAPC or both should be used as the baseline,

  - FFS how the channel access priority classes apply to each SL channel and signal

  - FFS sidelink priority levels (PQI or L1 priority), channel and signal mapping to the 4 channel access priority classes. The discussion may involve other WGs.

Agreement

- UE-to-UE COT sharing is supported in NR sidelink operation in a shared channel (SL-U).

  - FFS applicable SL channels and signals (e.g., PSCCH/PSSCH, PSFCH, S-SSB) for shared COT access and any restrictions (e.g. whether the COT can be shared with a single UE or multiple UEs)

  - FFS all other details in compliance with the regulatory requirements

- CP extension (CPE) is supported for NR sidelink operation in a shared channel.

  - FFS all remaining details including applicable scenarios, usage, PHY structure, etc.

Agreement

Channel access procedures for transmission(s) on multiple channels are supported for NR sidelink operation as defined by TS37.213 for NR-U (wherever applicable)

- FFS whether the downlink, uplink and/or semi-static multiple channel access procedure(s) (if supported) from NR-U should be used as a baseline and whether/how they are applied in SL mode 1 and mode 2 operation

Agreement

- The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the allocated resource(s), in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213.

  - FFS whether/how mode 1 resource allocation selection procedure needs to be updated / enhanced due to shared spectrum channel access

- The existing sidelink mode 2 RA schemes are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the selected and/or reserved resources, in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213.

  - FFS whether/how mode 2 resource selection procedure needs to be updated / enhanced due to shared spectrum channel access

- FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects

  - channel access, resource allocation and PHY channel design

- FFS whether/how enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access

- RAN1 to strive for a common solution for channel access for Mode 1 and Mode 2

Agreement

SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline

- Only one SL BWP is (pre-)configured within a carrier

- The SL BWP is (pre-)configured to include one or multiple SL resource pools

- At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets

  - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set

  - FFS: the applicable resource pool

  - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported

- PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets

  - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc.

- FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool

- FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc.

- FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc.

Agreement

For PSCCH and PSSCH in SL-U:

- Both R16/R17 NR SL contiguous RB-based and R16 NR-U interlace RB-based transmissions are considered as starting point

  - RAN1 strives to have unified design for both contiguous RB-based and interlace RB-based transmissions

  - FFS: whether/how to address IBE (In Band Emission) impact

Agreement

For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission (if supported), at least the following candidates can be discussed:

  - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission

    - FFS: Other resource allocation granularity, e.g., RB-level

  - 1 sub-channel equals K interlaces if sub-channel is supported

    - FFS details

  - Other candidates are not precluded

  - FFS: mapping of PSCCH to frequency resources

  - FFS: resource indication in time/frequency domain, e.g., how to handle using one RB set or multiple RB sets, etc.

Agreement

For slot structure in SL-U:

- At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported

  - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission

Agreement

For PSFCH and SL-HARQ in SL-U:

- At least R16 NR SL PSFCH format 0 is supported

  - FFS whether to introduce new PSFCH format

- FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc.

- FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc.

- FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc.

- FFS: whether/how to address PSFCH and related PSSCH in different COTs

Agreement

For S-SSB and synchronization in SL-U:

- FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc.

- Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission

  - Option 1: Using interlaced RB transmission

  - Option 2: S-SSB multiplexing with other SL transmissions in the same slot

  - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

  - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth

- FFS: whether to support 4 symbols S-SSB

  - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above

- FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission

- FFS whether any changes to R16/R17 NR SL synchronization procedure

NR sidelink enhancement in FR2 licensed spectrum

- On hold until further checking in RAN#97

Co-channel coexistence for LTE sidelink and NR sidelink

Agreement

For co-channel coexistence in Rel-18, no changes in the LTE SL specifications are allowed.

Agreement

For co-channel coexistence in Rel-18, Rel-16/17 simulation assumptions are reused for evaluation of solutions, except for the UE dropping model.

- FFS: UE dropping model

Agreement

For the study of co-channel coexistence solutions in Rel-18, the combination of operational modes Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority.

- FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C).

Agreement

For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models:

- UE Dropping Model A: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}.

- UE Dropping Model B: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}.

Companies should mention the UE dropping model and the distribution of each device type (single/dual module) used in their simulation assumptions.

Agreement

Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied.

Agreement

For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence,

- For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module.

  - FFS details on how the NR SL module uses this information.

  - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc.

- FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL.

- FFS: Whether/how device type B should be supported.

#### 2.1.2	Remaining Open issues

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only

- Study and specify enhanced sidelink operation on FR2 licensed spectrum (This part of the work is put on hold until further checking in RAN#97)

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any

## 2.2	RAN2

#### 2.2.1	Agreements

- No scheduled meeting TU since the last RAN

#### 2.2.2	Remaining Open issues

- Mechanism to support NR sidelink CA operation based on LTE sidelink CA operation (This part of the work is put on hold until further checking in RAN#97)

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

- No scheduled meeting TU since the last RAN

#### 2.4.2	Remaining Open issues

- UE Tx and Rx RF requirement for supporting new features introduced in this WI, sidelink frequency bands for single-carrier operation and frequency band combinations for carrier aggregation operation

- UE RRM core requirement for the new features introduced in this WI

- UE demodulation performance requirements

- UE RRM performance requirements

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#109-e

[1]	R1-2203981	Work plan for NR sidelink evolution	OPPO

[2]	R1-2204093	Discussion on V2X use cases, scenarios, and requirements for SL-U	TOYOTA Info Technology Center

[3]	R1-2205113	Contention-Based Sidelink Burst Transmissions	Johns Hopkins University APL

[4]	R1-2203122	On Channel Access Mechanism and Evaluation Methodology for SL-U	Nokia, Nokia Shanghai Bell

[5]	R1-2203146	Channel access mechanism and resource allocation for sidelink operation over unlicensed spectrum	Huawei, HiSilicon

[6]	R1-2203331	Discussion on channel access mechanism for sidelink on unlicensed spectrum	Spreadtrum Communications

[7]	R1-2203365	Discussion on channel access mechanism for SL-U	ZTE, Sanechips

[8]	R1-2203461	Discussion on channel access mechanism for sidelink on unlicensed spectrum	CATT, GOHIGH

[9]	R1-2203561	Channel access mechanism for sidelink on unlicensed spectrum	vivo

[10]	R1-2203645	SL Channel access in unlicensed spectrum	InterDigital, Inc.

[11]	R1-2203657	Discusson on channel access mechanism for sidelink on unlicensed spectrum	China Telecom

[12]	R1-2203693	Channel Access of Sidelink on Unlicensed Spetrum	NEC

[13]	R1-2203703	Channel access mechanism for sidelink on FR1 unlicensed spectrum	Lenovo

[14]	R1-2203713	Discussion on channel access mechanism for sidelink on unlicensed spectrum	LG Electronics

[15]	R1-2203734	Discussion on channel access mechanism for SL-unlicensed	Sony

[16]	R1-2203749	Sidelink channel access on unlicensed spectrum	Panasonic Holdings Corporation

[17]	R1-2203818	Channel access mechanism for sidelink-unlicensed	xiaomi

[18]	R1-2203906	On channel access mehanism for sidelink on FR1 unlicensed spectrum	Samsung

[19]	R1-2203982	Access mechanisms of NR sidelink in unlicensed channel	OPPO

[20]	R1-2204084	SL-U General Access Considerations	CableLabs

[21]	R1-2204097	Discussion of channel access mechanism for sidelink transmission in unlicensed spectrum	FUTUREWEI

[22]	R1-2204194	Discussion on sidelink on unlicensed spectrum	ASUSTeK

[23]	R1-2204247	On channel access mechanism for sidelink on FR1 unlicensed spectrum	Apple

[24]	R1-2204306	Discussion on channel access mechanism for sidelink on unlicensed spectrum	CMCC

[25]	R1-2204382	Discussions on channel access mechanism in SL-U	NTT DOCOMO, INC.

[26]	R1-2204427	Discussions on Channel access mechanism for NR sidelink evolution	Sharp

[27]	R1-2204552	Discussion on channel access mechanism for SL on unlicensed spectrum	WILUS Inc.

[28]	R1-2204580	Disscusion of channel access mechanism for sidelink in unlicensed spectrum	Transsion Holdings

[29]	R1-2204607	NR Sidelink Unlicensed Channel Access Mechanisms	Fraunhofer HHI, Fraunhofer IIS

[30]	R1-2204730	Discussion on Channel access mechanism	MediaTek Inc.

[31]	R1-2204741	Channel access mechanism for SL-U	Ericsson

[32]	R1-2204802	On the Channel Access Mechanisms for SL Operating in Unlicensed Spectrum	Intel Corporation

[33]	R1-2204868	Discussion on channel access mechanism for sidelink on unlicensed spectrum	ROBERT BOSCH GmbH

[34]	R1-2205033	Channel Access Mechanism for Sidelink on Unlicensed Spectrum	Qualcomm Incorporated

[35]	R1-2205082	Considerations on unlicensed channel access for SL-U	Fujitsu Limited

[36]	R1-2203123	On Physical Channel Design Framework for SL-U	Nokia, Nokia Shanghai Bell

[37]	R1-2203147	Physical channel design for sidelink operation over unlicensed spectrum	Huawei, HiSilicon

[38]	R1-2203332	Discussion on Physical channel design for sidelink on unlicensed spectrum	Spreadtrum [35]	Communications

[39]	R1-2203366	Discussion on physical layer structures and procedures for SL-U	ZTE, Sanechips

[40]	R1-2203462	Discussion on physical channel design framework for sidelink on unlicensed spectrum	CATT, GOHIGH

[41]	R1-2203562	Physical channel design framework for sidelink on unlicensed spectrum	vivo

[42]	R1-2203646	SL U physical layer design framework	InterDigital, Inc.

[43]	R1-2203694	Discussion on physical channel design framework	NEC

[44]	R1-2203704	Physical layer design framework for sidelink on FR1 unlicensed spectrum	Lenovo

[45]	R1-2203714	Discussion on physical channel design framework for sidelink on unlicensed spectrum	LG Electronics

[46]	R1-2203735	Discussion on physical channel design framework for SL-unlicensed	Sony

[47]	R1-2203750	Physical channel design for sidelink on unlicensed spectrum	Panasonic Holdings Corporation

[48]	R1-2203819	Physical channel design for sidelink-unlicensed	xiaomi

[49]	R1-2203907	On physical channel design framework for sidelink on FR1 unlicensed spectrum	Samsung

[50]	R1-2203983	Physical channel designs of NR sidelink in unlicensed channel	OPPO

[51]	R1-2204085	Discussion on resources on sidelink operation in unlicensed spectrum	CableLabs

[52]	R1-2204098	Discussion of physical channel designs for sidelink transmission in unlicensed spectrum	FUTUREWEI

[53]	R1-2204121	Discussion on Physical channel design framework for sidelink on unlicensed spectrum	Hyundai Motors

[54]	R1-2204248	On physical channel design framework for sidelink on FR1 unlicensed spectrum	Apple

[55]	R1-2204307	Discussion on physical channel design framework for sidelink on unlicensed spectrum	CMCC

[56]	R1-2204383	Discussions on channel design framework in SL-U	NTT DOCOMO, INC.

[57]	R1-2204428	Discussions on physical channel design framework for NR sidelink evolution on unlicensed spectrum	Sharp

[58]	R1-2204553	Discussion on PHY channel design framework for SL on unlicensed spectrum	WILUS Inc.

[59]	R1-2204581	Disscusion of physical channel design for sidelink in unlicensed spectrum	Transsion Holdings

[60]	R1-2204608	NR Sidelink Unlicensed Physical Channel Design	Fraunhofer HHI, Fraunhofer IIS

[61]	R1-2204731	Discussion on Physical channel design framework	MediaTek Inc.

[62]	R1-2204742	PHY channel design framework for SL-U	Ericsson

[63]	R1-2204803	On the Enhancements to the Physical Channel Design for SL Operating in Unlicensed Spectrum	Intel Corporation

[64]	R1-2205034	Physical Channel Design for Sidelink on Unlicensed Spectrum	Qualcomm Incorporated

[65]	R1-2203124	On Co-channel Coexistence for LTE Sidelink and NR Sidelink	Nokia, Nokia Shanghai Bell

[66]	R1-2203148	Co-channel coexistence for LTE sidelink and NR sidelink	Huawei, HiSilicon

[67]	R1-2203333	Discussion on Co-channel coexistence for LTE sidelink and NR sidelink	Spreadtrum Communications

[68]	R1-2203367	Study on co-channel coexistence for LTE sidelink and NR sidelink	ZTE, Sanechips

[69]	R1-2203463	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	CATT, GOHIGH

[70]	R1-2203563	Co-channel coexistence for LTE sidelink and NR sidelink	vivo

[71]	R1-2203642	On sidelink co-channel coexistence issues	Mitsubishi Electric RCE

[72]	R1-2203658	Discusson on co-channel coexistence for LTE sidelink and NR sidelink	China Telecom

[73]	R1-2203668	Discussion on Sidelink Co-channel Coexistence 	Panasonic Holdings Corporation

[74]	R1-2203695	Co-existence between LTE and NR sidelink	NEC

[75]	R1-2203705	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	Lenovo

[76]	R1-2203715	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	LG Electronics

[77]	R1-2203736	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	Sony

[78]	R1-2203820	Discussion on co-channel coexistence for LTE and NR sidelink	xiaomi

[79]	R1-2203908	On co-channel coexistence for LTE sidelink and NR sidelink	Samsung

[80]	R1-2203984	Discussion on co-channel coexistence for LTE and NR V2X	OPPO

[81]	R1-2204049	Co-channel coexistence for LTE sidelink and NR sidelink	InterDigital, Inc.

[82]	R1-2204249	On co-channel coexistence for LTE sidelink and NR sidelink	Apple

[83]	R1-2204308	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	CMCC

[84]	R1-2204384	Discussions on co-channel coexistence of LTE-SL and NR-SL	NTT DOCOMO, INC.

[85]	R1-2204429	Discussions on co-channel coexistence for LTE sidelink and NR sidelink	Sharp

[86]	R1-2204585	Considerations on co-channel coexistence for LTE SL and NR SL	CAICT

[87]	R1-2204609	Discussion on Co-Channel Coexistence for LTE and NR Sidelink	Fraunhofer HHI, Fraunhofer IIS

[88]	R1-2204652	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	ETRI

[89]	R1-2204720	On co-channel coexistence between LTE sidelink and NR sidelink	MediaTek Inc.

[90]	R1-2204740	Co-channel coexistence between LTE sidelink and NR sidelink	Ericsson

[91]	R1-2204867	On co-channel coexistence between LTE and NR sidelink	ROBERT BOSCH GmbH

[92]	R1-2205035	Co-channel Coexistence Between LTE SL and NR SL	Qualcomm Incorporated

[93]	R1-2203368	Analysis on channel access efficiency of SL-U	ZTE, Sanechips

[94]	R1-2203464	Discussion on the scopes of Rel-18 Sidelink evolution	CATT, GOHIGH

[95]	R1-2203647	SL U QoS management	InterDigital, Inc.

[96]	R1-2203667	Discussion on sidelink operation on unlicensed spectrum	vivo

[97]	R1-2203985	Coexistence evaluation of SL-U and Wi-Fi	OPPO

[98]	R1-2204250	On other topics of NR sidelink evolutions	Apple

[99]	R1-2204743	Other aspects related to SL-U	Ericsson

[100]	R1-2204912	Further consideration on evaluation methodology for sidelink operation over unlicensed spectrum	Huawei, HiSilicon

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template