3GPP TSG RAN WG1 Meeting #110bis-e	R1-2208384

eMeeting, October 10-19, 2022

Agenda Item:	5

Source:	FUTUREWEI

Title:	On the incoming RAN2 LS R1-2205728 on IUC and non-preferred resources

Document for:	Discussion and decision

# Introduction

The LS R1-2205728(R2-2206314) asks the following questions with regards to Rel-17 NR_SL_enh-Core and the inter-UE coordination scenario in which UE B receives a scheme 1 non-preferred resource set from UE A and does not perform sensing:

Q1:	Is the scenario described above a valid scenario or not?

Q2:	If the answer to Q1 is yes, does resource exclusion based on non-preferred resource set needs to be performed by UE B or not?

Q3:	If the answer to Q2 is yes, then, in RAN1’s view, which specification (PHY or MAC) should capture the resource exclusion behavior?

The incoming LS was discussed at RAN1#110 (summary in R1-2208080, draft LS response in R1-2208222), without conclusion.

# Discussion

RAN1 discussed scenarios where sensing results may not be available in the earlier meetings of the Rel-17 WI, and made the agreement in RAN1#106e (see Appendix) to support both preferred and non-preferred resources when sensing results were not available. Sensing results might not be available because of UE capability or (pre-)configuration. For the preferred resource set, the agreement allowed for further discussion on other conditions which might lead to a UE not using available sensing results, which is why the agreement for preferred resources was divided into “Option A” and “Option B” behavior. For non-preferred resources, some details were FFS but it was clear the UE would use sensing results (if available) in addition to excluding the non-preferred resources. Most companies in RAN1#110 also concluded that it is a valid scenario.

As for how to handle, there were roughly three views: do nothing, handle in PHY, handle in MAC. At this stage, we maintain the view that it may be simplest to handle IUC with non-preferred resources and no sensing results in the MAC specification 38.321 with random selection, similar to the case with preferred resources and no sensing results as show below:

3>  if configured by RRC, interUECoordinationScheme1Explicit or interUECoordinationScheme1Condition enabling reception of preferred resource set and non-preferred resource set and when the UE does not have own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE:

4>  randomly select the time and frequency resources for one transmission opportunity from the resources belonging to the received preferred resource set for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

Considering the above discussion, the answers to RAN2’s LS would be “Yes”, “Yes”, and “MAC”. It is also acceptable to us to conclude, as in draft LS R1-2208222, that RAN1 does not have consensus as to how to handle and leave it to RAN2.

# Conclusion

Reply to the LS from RAN2 with the answers to Q1 Q2 and Q3 as “Yes”, “Yes”, and “MAC”. ”. It is also acceptable to us to conclude, as in draft LS R1-2208222, that RAN1 does not have consensus as to how to handle and leave it to RAN2.

# Appendix

Agreement (from RAN1#106e)

In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A:

- For preferred resource set, the following two options are supported:

  - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

    - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result

      - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met

        - FFS: Details of condition(s)

      - This option is supported when UE-B performs sensing/resource exclusion

      - FFS: Other details (if any)

  - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information

    - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set

      - This option is supported at least when UE-B does not support sensing/resource exclusion

        - FFS: Whether the support is conditional or UE capability

      - FFS: Other details (if any)

  - FFS: Other option(s), and other details (if any)

- For non-preferred resource set,

  - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

    - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

      - FFS: Details including

        - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any)

        - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

    - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set

  - FFS: Other option(s), and other details (if any)