3GPP TSG RAN WG1 Meeting #110b	R1-2208397

e-meeting, October 10th – 19th, 2022

Agenda Item:	9.11.3

Source:	MediaTek Inc.

Title:	Disabling of HARQ for IoT NTN

Document for:	Discussion and Decision

- 1 Introduction

In RAN#96e [1], the revised WID on IoT NTN enhancements was endorsed for Release 18. Rel-17 IoT-NTN is considered as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objective for disabling of HARQ feedback.

This contribution aims to discuss aspects related to disabling of HARQ feedback for IoT NTN.

- 2 Discussion on disabling of HARQ feedback

According to the WID, disabling of HARQ feedback for IoT NTN has been agreed in RAN meetings.

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1, RAN2] - Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1] NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

2.1 Analysis of DL peak data rates with repetitions in legacy IoT and IoT NTN

In LTE eMTC and NB-IoT specifications, the HARQ process with adaptive retransmissions is asynchronous on DL and UL. In both directions, the receiver tries to decode the transport block (TB) with soft combining of HARQ re-transmissions. In case the transmission is a new TB, the soft buffer should be flushed. In case a retransmission occurred, soft combining of current HARQ re-transmission and previous HARQ transmissions should be performed. The UE knows a scheduled TB is re-transmitted from the new-data indicator (NDI) as indicated on DCI.

In satellite systems, larger values of RTD can be typically experienced due to the large channel propagation delay between the UE and the satellite, which depends on the satellite orbit and the elevation angle. HARQ stalling occurs due to the longer satellite RTT between the device and the network, where the UE needs to wait for the reception of a packet or the UL grant to transmit a new packet with HARQ re-transmissions.

In SI phase of IoT NTN, link budget results have been captured and summarized in TR 36.763 [2]. The link budget results tables are based on provided guidance as in Appendix. Table 1 summarizes the DL C/N and UL C/N with PC3 (23 dBm), NF=7 dB based on TR 36.763. Considering the low DL C/N and UL C/N in Table1, repetitions are needed in IoT NTN,

Table 1: the DL C/N and UL C/N with PC3 (23 dBm), NF=7 dB

| Case | Satellite orbit | Satellite parameter set | DL C/N | UL C/N 1080 kHz / 360 kHz /180 kHz / 90 kHz / 45 kHz / 30 kHz / 15 kHz / 3.75 kHz |
| --- | --- | --- | --- | --- |
| 1 | GEO | Set 1 | -3.3 dB | -22.0 dB / -17.3 dB / -14.2 dB / -11.2 dB / -8.2 dB / -6.5 dB / -3.5 dB / 2.6 dB |
| 2 | LEO-1200 | Set 1 | 3.6 dB | -14.0 dB / -9.3 dB / -6.3 dB / -3.3 dB / -0.3 dB / 1.5 dB / 4.5 dB / 10.5 dB |
| 3 | LEO-600 | Set 1 | 3.0 dB | -8.6 dB / -3.9 dB / -0.9 dB / 2.2 dB / 5.2 dB / 6.9 dB / 9.9 dB / 16.0 dB |
| 4 | GEO | Set 2 | -8.5 dB | -26.7 dB / -22.0 dB / -19.0 dB / -16.0 dB / -13.0 dB / -11.2 dB / -8.2 dB / -2.2 dB |
| 5 | LEO-1200 | Set 2 | -3.2 dB | -20.8 dB / -16.0 dB / -13.0 dB / -10.0 dB / -7.0 dB / -5.2 dB / -2.2 dB / 3.8 dB |
| 6 | LEO-600 | Set 2 | -3.7 dB | -15.4 dB / -10.6 dB / -7.6 dB / -4.6 dB / -1.5 dB / 0.2 dB / 3.2 dB / 9.3 dB |
| 7 | GEO | Set 3 | -2.2 dB | -24.0 dB / -19.3 dB / -16.3 dB / -13.3 dB / -10.3 dB / -8.5 dB / -5.5 dB / 0.6 dB |
| 8 | LEO-1200 | Set 3 | -2.1 dB | -27.3 dB / -22.5 dB / -19.5 dB / -16.5 dB / -13.5 dB / -11.7 dB / -8.7 dB / -2.7 dB |
| 9 | LEO-600 | Set 3 | -2.1 dB | -21.9 dB / -17.2 dB / -14.1 dB / -11.1 dB / -8.1 dB / -6.4 dB / -3.3 dB / 2.7 dB |
| 10 | LEO-600 | Set 4 | -12.0 dB | -27.0 dB / -23.0 dB / -20.0 dB / -16.9 dB / -13.9 dB / -12.2 dB / -9.2 dB / -3.1 dB |
| 11 | MEO-10000 | Set 5 | -4.5 dB | -25.0 dB / -19.8 dB / -16.8 dB / -13.8 dB / -10.8 dB / -9.0 dB / -6.0 dB / -0.0 dB |

In cellular NB-IoT, the max TBS is 2536 bits with QPSK modulation as shown in Appendix. Figure 1 illustrates the scheduling of DL packets with 2 HARQ processes. The DCI#1 scheduled NPDSCH #1 can be transmitted during the scheduling delay of the DCI#0 and scheduled NPDSCH #0, and network schedules the NPDSCH #1 right next to NPDSCH #0.

Figure 1: 2 HARQ processes for legacy NB-IoT

Table 2 shows DL maximum data rates of NB-IoT for Cellular, LEO, and GEO with HARQ feedback enabling with Max TBS=2536 bits with QPSK. With repetitions scheduled by the eNB, the impact of the satellite RTD on the NB-IoT data rates in GEO due to HARQ stalling is significant with a 61% reduction of peak data rate observed with 16 repetitions compared to cellular; for LEO it is not significant with a 7% reduction of peak data rate observed with 16 repetitions. Without repetitions, the impact of the satellite RTD on the NB-IoT data rates in GEO and LEO is significant. For LEO it is relatively smaller than that in GEO.

Observation 1: Peak data rate reduction with HARQ in LEO satellite scenario compared with cellular is not significant in case repetitions is used.

Table 2: peak data rate of enabling HARQ

|  | 2-HARQ Processes TBS=2536 bits |  |  |  |
| --- | --- | --- | --- | --- |
|  | DL without repetitions for NPDCCH and NPDSCH | DL with 4 repetitions for NPDCCH and NPDSCH | DL with 8 repetitions for NPDCCH and NPDSCH | DL with 16 repetitions for NPDCCH and NPDSCH |
| Cellular | 126.8 kbps | 49.2 kbps | 27.1 kbps | 14.3 kbps |
| LEO | 74.6 kbps (~41% reduction) | 38.7 kbps (~21% reduction) | 23.6 kbps (~13% reduction) | 13.2 kbps (~7% reduction) |
| GEO | 8.7 kbps (~93% reduction) | 7.8 kbps (~84% reduction) | 6.9 kbps (~74% reduction) | 5.6 kbps (~61% reduction) |

2.2 Disabling HARQ feedback in IoT NTN

Disabling of HARQ feedback was discussed in last few meetings and agreements are made in RAN1 and RAN2 for options of disabling HARQ feedback in Rel-18 IoT NTN.

Agreement (RAN1 109) For IoT NTN, to configure/indicate enabling/disabling on HARQ feedback for downlink transmission, one or more of the following options can be considered: - Option 1: per HARQ process via UE specific RRC signaling - Option 2: per HARQ process via SIB signaling - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field) - Option 4: implicitly determined by existing configured/indicated parameter(s) (e.g., repetition number, TBS) - Option 5: per HARQ process via MAC CE - Other options or combinations are not excluded Note: Option(s) for eMTC and NBIoT can be separately discussed. Agreement (RAN1 110) For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options: - Option 1: per HARQ process via UE specific RRC signaling. - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field). - Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS) - Option 6: combinations of some options above. Agreement (RAN1 110) For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options: - Option 1: per HARQ process via UE specific RRC signaling - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field) - Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS) - Option 6: combinations of some options above Agreements (RAN2 119): - Disabling DL HARQ feedback is supported for NB-IoT and eMTC NTN. FFS on UE capability - For UL HARQ operation, introduce two HARQ modes, i.e., HARQ mode A and HARQ mode B in IoT NTN (both NB-IoT and eMTC NTN), similarly to NR NTN - From RAN2 perspective, at least for eMTC, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signalling. FFS for NB-IoT (and especially for CP solution for NB-IOT).

There were agreements in Rel-17 NR NTN on disabling of HARQ feedback.

Agreement (RAN1 102-e): Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling

For disabling of HARQ feedback, RAN2 has made agreement that

- From RAN2 perspective, at least for eMTC, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signalling. FFS for NB-IoT.

Option 1 by reusing NR NTN solutions in IoT NTN, can simplify the discussion and can ensure network flexibility in HARQ feedback enabling/disabling scheduling.

Option 3 and Option 4 make network dynamically configure enabling/disabling but will introduce more spec effect.

From above analysis, we have preference for Option 1 “Enabling/disabling on HARQ feedback for downlink transmission is configurable per HARQ process via UE specific RRC signalling” to be the baseline in IoT NTN.

Option 6 can be further discussed on whether it is needed and how it can be used.

Proposal 1: Enabling/disabling on HARQ feedback for downlink transmission is configurable per HARQ process via UE specific RRC signalling is the baseline in IoT NTN.

2.3 NPDCCH scheduling restriction for disabling HARQ feedback

Agreement (RAN1 110) For a DL HARQ process with disabled HARQ feedback in NB-IoT, at least the following UE behavior(s) can be considered: - Option 1: UE is not expected to receive another NPDCCH carrying a DCI scheduling a NPDSCH for a given HARQ process that starts until X(ms) after the end of the reception of the last NPDSCH for that HARQ process. - X =12 - Option 2: UE is not required to monitor NPDCCH in a period of Y(ms) from the end of reception of the last NPDSCH - Y=12 Note: it may be different UE behaviors for different UE categories (e.g., UE with single/multiple HARQ processes)

In cellular case, for the ending of the received NPDSCH in subframe n, when the UE is not required to transmit a corresponding NPUSCH format 2, UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12 as specified in TS 36.213 Section 16.6.

TS 36.213 Section 16.6 If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

Option 2 is better that UE should not be required to monitor another NPDCCH in any subframe starting from subframe n+1 to subframe n+12, without modifying the legacy behavior. There will be huge equipment complexity and spec impact if the legacy UE behavior is modified.

Proposal 2:  For a DL HARQ process with disabled HARQ feedback in NB-IoT, the legacy procedure with “Option 2: UE is not required to monitor NPDCCH in a period of 12ms from the end of reception of the last NPDSCH” can be reused.

Since Rel-17 IoT NTN solution for no NPDCCH monitoring only applies to the cases that HARQ feedback are enabled, the restriction for disabling HARQ feedback will have no impact on the Rel-17 IoT NTN solution for no NPDCCH monitoring.

Observation 2: Since Rel-17 IoT NTN solution for no NPDCCH monitoring only applies to the cases that HARQ feedback are enabled, the restriction for disabling HARQ feedback will have no impact on the Rel-17 IoT NTN solution for no NPDCCH monitoring

2.4 Others

Agreement (RAN1 109e) For IoT NTN, further study the potential issues due to enabling/disabling on HARQ feedback for downlink transmission - Issue A: SPS PDSCH - Issue B: (N)PDSCH/(N)PDCCH scheduling restriction - Issue C: HARQ feedback for scheduling multiple TB - Issue D: HARQ bundling for eMTC HD-FDD - Issue F: NPRACH capacity - Issue G: Serving cell/satellite change during data transfer (FFS: for eMTC and/or NB-IoT) - Other issues are not excluded Note: The “Issues” in common for eMTC and NB-IoT can be separately discussed.

Multiple TB is one optional feature considering unicast and multicast for Rel-16 NB-IoT. There is no multi-TB HARQ feedback in SC-MTCH. There is multi-TB HARQ feedback in unicast, with consideration of HARQ ACK bundling.

If higher layer parameter sc-mtch-InfoListMultiTB-r16 is enabled and the CRC of the DCI is scrambled by G-RNTI, the Number of scheduled TB for SC-MTCH are 3 bits indicating from 1 to 8 TBs.

If higher layer parameter npdsch-MultiTB-Config is enabled and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI, the Number of scheduled TB for Unicast is 1 bit, where value 0 indicates a single TB is scheduled and value 1 indicates multiple TB are scheduled. For NB-IoT, if Number of scheduled TB for Unicast is present and multiple TB are scheduled, the multi-TB (two TBs) are scheduled together without HARQ process number.

Observation 3: HARQ feedback enabling/disabling can be determined based on the enabling-disabling configuration of the HARQ process for the multi-TB.

When HARQ feedback is disabled, like the case when an NB-IoT UE configured with higher layer parameter sr-WithoutHARQ-ACK-Config, the transmission of a narrowband random-access preamble for SR will be needed.

With high number of UEs in a cell, data resources are more likely than RACH resources to be bottleneck for capacity. Hence, data capacity will likely fail before RACH capacity for many UEs. Unless all HARQ processes are disabled and all UEs transmit RACH at the same time, there will be some HARQ feedback to piggyback SR and to our understanding there should not be any significant RACH capacity issue assuming adequate RACH resource configuration.

Observation 4: With high number of UEs in a cell, data resources are more likely than RACH resources to be bottleneck for capacity.

3 Conclusion

In this contribution, the following proposals were made

Observation 1: Peak data rate reduction with HARQ in LEO satellite scenario compared with cellular is not significant in case repetitions is used.

Proposal 1: Enabling/disabling on HARQ feedback for downlink transmission is configurable per HARQ process via UE specific RRC signalling is the baseline in IoT NTN.

Proposal 2:  For a DL HARQ process with disabled HARQ feedback in NB-IoT, the legacy procedure with “Option 2: UE is not required to monitor NPDCCH in a period of 12ms from the end of reception of the last NPDSCH” can be reused.

Observation 2: Since Rel-17 IoT NTN solution for no NPDCCH monitoring only applies to the cases that HARQ feedback are enabled, the restriction for disabling HARQ feedback will have no impact on the Rel-17 IoT NTN solution for no NPDCCH monitoring

Observation 3: HARQ feedback enabling/disabling can be determined based on the enabling-disabling configuration of the HARQ process for the multi-TB.

Observation 4: With high number of UEs in a cell, data resources are more likely than RACH resources to be bottleneck for capacity.

4 References

- RP-221806, Moderator (MediaTek), Revised WID on IoT NTN enhancements, June 6th – 9th, 2022

- TR 36.763, Release 17

- R1-2205555, Lenovo, Feature lead summary #3 on disabling of HARQ feedback for IoT NTN

- R1-1912125, MediaTek, Delay-tolerant re-transmission mechanisms in NR-NTN, RAN1#99, November 2019

5 Appendix

5.1 Transport block size (TBS) table for NPUSCH in TS 36.213 Section 16.5.1.2

Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |

5.2 link budget analysis in TR 36.763 Section 6.2

Table 6.2.2.1-3: Cases for link budget analysis

| Case | Satellite orbit | Satellite parameter set | Central beam center elevation (deg) | Central beam edge elevation (deg) | Frequency Reuse Factor |
| --- | --- | --- | --- | --- | --- |
| 1 | GEO | Set 1 | 12.5 | 2.3 | 1 |
| 2 | LEO-1200 | Set 1 | 30 | 26.27 | 1 |
| 3 | LEO-600 | Set 1 | 30 | 26.98 | 1 |
| 4 | GEO | Set 2 | 20 | 10.95 | 1 |
| 5 | LEO-1200 | Set 2 | 30 | 22.16 | 1 |
| 6 | LEO-600 | Set 2 | 30 | 23.80 | 1 |
| 7 | GEO | Set 3 | 20.88 | 12.5 | 1 |
| 8 | LEO-1200 | Set 3 | 46.05 | 30 | 1 |
| 9 | LEO-600 | Set 3 | 43.78 | 30 | 1 |
| 10 | LEO-600 | Set 4 | 90 | 30 | 1 |
| 11 | MEO-10000 | Set 5 | 90 | 81.6 | 1 |

| PC3 (23 dBm), NF=7 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 1080 kHz / 360 kHz /180 kHz / 90 kHz / 45 kHz / 30 kHz / 15 kHz / 3.75 kHz |
| 1 | 59 dBW/MHz | 81.6 dBm | -3.3 dB | 19 dB/K | -22.0 dB / -17.3 dB / -14.2 dB / -11.2 dB / -8.2 dB / -6.5 dB / -3.5 dB / 2.6 dB |
| 2 | 40 dBW/MHz | 62.6 dBm | 3.6 dB | 1.1 dB/K | -14.0 dB / -9.3 dB / -6.3 dB / -3.3 dB / -0.3 dB / 1.5 dB / 4.5 dB / 10.5 dB |
| 3 | 34 dBW/MHz | 56.6 dBm | 3.0 dB | 1.1 dB/K | -8.6 dB / -3.9 dB / -0.9 dB / 2.2 dB / 5.2 dB / 6.9 dB / 9.9 dB / 16.0 dB |

| PC5 (20 dBm), NF=9 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 1080 kHz / 360 kHz /180 kHz / 90 kHz / 45 kHz / 30 kHz / 15 kHz / 3.75 kHz |
| 1 | 59 dBW/MHz | 81.6 dBm | -5.3 dB | 19 dB/K | -25.0 dB / -20.3 dB / -17.2 dB / -14.2 dB / -11.2 dB / -9.5 dB / -6.5 dB / -0.4 dB |
| 2 | 40 dBW/MHz | 62.6 dBm | 1.6 dB | 1.1 dB/K | -17.0 dB / -12.3 dB / -9.3 dB / -6.3 dB / -3.3 dB / -1.4 dB / 1.5 dB / 7.5 dB |
| 3 | 34 dBW/MHz | 56.6 dBm | 1.0 dB | 1.1 dB/K | -11.6 dB / -6.9 dB / -3.9 dB / -0.8 dB / 2.2 dB / 3.9 dB / 6.9 dB / 13.0 dB |

| PC3 (23 dBm), NF=7 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 1080 kHz / 360 kHz / 180 kHz / 90 kHz / 45 kHz / 30 kHz / 15 kHz / 3.75 kHz |
| 4 | 53.5 dBW/MHz | 76.1 dBm | -8.5 dB | 14 dB/K | -26.7 dB / -22.0 dB / -19.0 dB / -16.0 dB / -13.0 dB / -11.2 dB / -8.2 dB / -2.2 dB |
| 5 | 34 dBW/MHz | 56.6 dBm | -3.2 dB | -4.9 dB/K | -20.8 dB / -16.0 dB / -13.0 dB / -10.0 dB / -7.0 dB / -5.2 dB / -2.2 dB / 3.8 dB |
| 6 | 28 dBW/MHz | 50.6 dBm | -3.7 dB | -4.9 dB/K | -15.4 dB / -10.6 dB / -7.6 dB / -4.6 dB / -1.5 dB / 0.2 dB / 3.2 dB / 9.3 dB |

| PC5 (20 dBm), NF=9 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 1080 kHz / 360 kHz /180 kHz / 90 kHz / 45 kHz / 30 kHz / 15 kHz / 3.75 kHz |
| 4 | 53.5 dBW/MHz | 76.1 dBm | -10.5 dB | 14 dB/K | -29.7 dB / -25.0 dB / -22.0 dB / -19.0 dB / -16.0 dB / -14.2 dB / -11.2 dB / -5.2 dB |
| 5 | 34 dBW/MHz | 56.6 dBm | -5.2 dB | -4.9 dB/K | -23.8 dB / -19.0 dB / -16.0 dB / -13.0 dB / -10.0 dB / -8.2 dB / -5.2 dB / 0.8 dB |
| 6 | 28 dBW/MHz | 50.6 dBm | -5.7 dB | -4.9 dB/K | -18.4 dB / -13.6 dB / -10.6 dB / -7.6 dB / -4.5 dB / -2.8 dB / 0.2 dB / 6.3 dB |

| PC3 (23 dBm), NF=7 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 1080 kHz / 360 kHz / 180 kHz / 90 kHz / 45 kHz /30 kHz / 15 kHz / 3.75 kHz |
| 7 | 59.8 dBW/MHz | 84.4 dBm | -2.2 dB | 16.7 dB/K | -24.0 dB / -19.3 dB / -16.3 dB / -13.3 dB / -10.3 dB / -8.5 dB / -5.5 dB / 0.6 dB |
| 8 | 33.7 dBW/MHz | 56.3 dBm | -2.1 dB | -12.8 dB/K | -27.3 dB / -22.5 dB / -19.5 dB / -16.5 dB / -13.5 dB / -11.7 dB / -8.7 dB / -2.7 dB |
| 9 | 28.3 dBW/MHz | 50.9 dBm | -2.1 dB | -12.8 dB/K | -21.9 dB / -17.2 dB / -14.1 dB / -11.1 dB / -8.1 dB / -6.4 dB / -3.3 dB / 2.7 dB |

| PC5 (20 dBm), NF=9 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 1080 kHz / 360 kHz /180 kHz / 90 kHz / 45 kHz / 30 kHz / / 15 kHz / 3.75 kHz |
| 7 | 59.8 dBW/MHz | 84.4 dBm | -4.2 dB | 16.7 dB/K | -27.0 dB / -22.3 dB / -19.3 dB / -16.3 dB / -13.3 dB / -11.5 dB / -8.5 dB / -2.4 dB |
| 8 | 33.7 dBW/MHz | 56.3 dBm | -4.1 dB | -12.8 dB/K | -30.3 dB / -25.5 dB / -22.5 dB / -19.5 dB / -16.5 dB / -14.7 dB / -11.7 dB / -5.7 dB |
| 9 | 28.3 dBW/MHz | 50.9 dBm | -4.1 dB | -12.8 dB/K | -24.9 dB / -20.2 dB / -17.1 dB / -14.1 dB / -11.1 dB / -9.4 dB / -6.3 dB / -0.3 dB |

| PC3 (23 dBm), NF=7 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 1080 kHz / 360 kHz / 180 kHz / 90 kHz / 45 kHz / 30 kHz / 15 kHz / 3.75 kHz |
| 10 | 21.4 dBW/MHz | 44 dBm | -12.0 dB | -20.9 dB/K | -27.0 dB / -23.0 dB / -20.0 dB / -16.9 dB / -13.9 dB / -12.2 dB / -9.2 dB / -3.1 dB |

| PC5 (25 dBm), NF=9 dB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 1080 kHz / 360 kHz /180 kHz / 90 kHz / 45 kHz / 30 kHz / 15 kHz / 3.75 kHz |
| 10 | 21.4 dBW/MHz | 44 dBm | -12.0 dB | -20.9 dB/K | -30.0 dB / -26.0 dB / -23.0 dB / -19.9 dB / -16.9 dB / -15.2 dB / -12.2 dB / -6.1 dB |

| Cases | EIRP Density | EIRP per spot | DL C/N | G/T | UL C/N 1080 kHz / 360 kHz / 180 kHz / 90 kHz / 45 kHz / 30 kHz / 15 kHz / 3.75 kHz |
| --- | --- | --- | --- | --- | --- |
| 11 | 45.4 dBW/MHz | 68 dBm | -4.5 dB | 3.8 dB/K | -25.0 dB / -19.8 dB / -16.8 dB / -13.8 dB / -10.8 dB / -9.0 dB / -6.0 dB / -0.0 dB |