3GPP TSG-RAN WG1 Meeting #110bis-e	 R1-2208445

e-Meeting, October 10 - 19, 2022

Agenda Item:	9.1.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission

Document for:	Discussion and Decision

- Introduction

The following agreements were achieved in RAN1#110 meeting to enable 8TX UL operation [1],

Agreement

8TX PUSCH is supported in Rel-18

Agreement

For 8TX PUSCH, at least support

- Ng=1, 2, 4

Note: The above does not restrict the Ng for the non-coherent case

Agreement

For evaluation purpose of codebook alternatives when a precoder based on Rel-15 DL Type I is used, following oversampling ratios are assumed

- (O1, O2) = (1,1), (2,1), (2,2)

- Note: Other values may be used and reported by companies

- Note: When deciding the supported O1, O2 combination, the signalling overhead, performance, UE complexity, etc should be considered

Agreement

RAN1 further studies Alt1b and Alt2a for down-selection of one of the two in RAN1 meeting #110b-e.

Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

Agreement

Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability

- For uplink transmission with rank<=4, single CW is supported

- For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e

The above applies only with regards to the work scope of this agenda item.

Agreement

For SRS configuration for non-codebook UL transmission for an 8TX UE, down-select from

- Alt1: A single SRS resource set configured with up to 8 single-port SRS resources

- Alt2: Up to two SRS resource sets, each configured with up to 4 single-port SRS resources

- Alt3: Support both alternatives.

Agreement

Study low overhead solutions for SRI and/or transmitter precoder matrix indication for codebook-based, and SRI indication for non-codebook-based UL transmission by an 8TX UE,

- FFS using single or separate (exiting or new) fields for the indication, other solutions are not precluded.

- Note: Low overhead schemes for study include those using Rel-15 SRI/TPMI indication mechanisms

In this contribution, we discuss SRI and TPMI enhancement to enable 8TX UL transmission.

- SRI enhancement for enabling 8TX UL transmission

SRI has been used in both codebook (CB) and non-codebook (NCB) based uplink transmission. This section discusses the enhancement of SRI for both CB and NCB PUSCH.

  - CB based UL transmission

For CB based UL transmission, SRI is used to indicate the selected SRS resource for uplink transmission, which is of  bits. Whether enhancement is needed depends on the design of SRS design for 8 TX UL operation. In companion contribution [2], two options are discussed for SRS to enable 8TX. If the number of antenna ports in one SRS resource is increased to 8, then no enhancement is needed for SRI. Meanwhile, if two 4-port SRS resources are used for channel sounding of 8TX, then SRI should be enhanced to support it.

Observation 1: Whether SRI enhancement for enabling 8TX in CB based UL transmission is needed depends on the SRS design for 8TX.

  - NCB based UL transmission

For NCB based UL transmission, SRI is used to indicate one or more SRS resources fromSRS resources to UE, where UE uses the same precoder of selected SRS resource to transmit PUSCH. For 4TX UE, the UE can only be configured with up to 4 single-port SRS resource for a SRS resource set. For 8TX UE, the number of candidate precoders of 8TX UE is up to 8, which requires up to 8 single-port SRS resources. In RAN#110 meeting, it was agreed to down-select from following alternatives for SRS configuration:

- Alt1: A single SRS resource set configured with up to 8 single-port SRS resources

- Alt2: Up to two SRS resource sets, each configured with up to 4 single-port SRS resources

- Alt3: Support both alternatives.

We failed to see the benefits to use two SRS resource sets for 8 TX. In addition, there will be several issues if two SRS resource sets each with up to 4 SRS resources is supported. For example, if , there can be multiple possibilities in configuration across two SRS resource sets, such as 3+3, 2+4, or 4+2, which needs to be further discussed. Hence, we propose to support a single SRS resource set configured with up to 8 single-port SRS resources. Furthermore, a single SRI field is enough to support the indication of selected SRS resources.

Proposal 1: A single SRS resource set configured with up to 8 single-port SRS resources is supported.

In legacy, k (1≤k≤NSRS) SRS resources can be selected from NSRS SRS resource, thus the SRI overhead is  bits, where Lmax is given by the maximum number of layers. For 4TX UE, is up to 4, and the maximum SRI overhead is 4 bits. For 8TX UE, will be increased to 8, thus the maximum SRI overhead needs to be increased to 8 bits, which will result in a large DCI overhead for SRI.

Observation 2: To support 8TX NCB based UL transmission, the legacy SRI field will introduce a large DCI overhead if full flexibility of SRS resource selection is supported.

The SINR of the 8 candidate precoders are evaluated by simulation, and the results are illustrated in Figure 1, where the precoders are derived by SVD of measured channel by UE. The detailed simulation assumptions can be found in Appendix A, where the antenna setups and port layouts at TRP and UE are respectively (12,4,2,1,1,4,4) and (2,2,2,1,1,2,2). In the figure, Precoders 1~8 are eigenvectors corresponding to the eigenvalues in descending order respectively.

Figure 1 The CDF of SINR of candidate precoders for 8TX UE

Observation 3: For 8TX NCB based UL transmission, there is large SINR gap between the precoders used in SRS resources with usage ‘nonCodebook’.

Due to significant SINR gaps, the probability to use some candidate precoders (SRS resources) with low SINR in cases of low-rank PUSCH (rank < 4) will be very small. Therefore, the full flexibility in SRS resource selection by an 8-bit SRI may not be needed. To reduce SRI overhead, a potential solution is to narrow down the number of combinations.

For example, if the number of UL transmission layers is not more than 4, only the combination of 4 best SRS resources are needed for PUSCH transmission. If the number of UL transmission layers is larger than 4, the 4 best SRS resources will always be used, and the remaining SRS resources can be selected by scheduling. In this way, the SRI overhead is expected to be reduced to 4bits.

Proposal 2: To reduce overhead of SRI for NCB PUSCH, reduce the flexibility of SRS resource selection.

- UL 8TX codebook design

In RAN#110 meeting, it was agreed that UL 8TX codebook will be down-selected from Alt1-b and Alt2-a. Both Alt1-b and Alt2-a use NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for partially/non-coherent UEs. For fully-coherent UEs, Alt2-a uses NR Rel-15 UL 2TX/4TX codebooks, while Alt1-b uses NR Rel-15 DL type I codebook. In this section, we discuss the codebook design of both alternatives for fully-coherent UEs and partially/non-coherent UEs.

  - Fully-coherent codebook

In RAN1#109-e meeting, it is agreed to consider Ng>=1 antenna groups for the codebook design of fully coherent UEs, where each group comprises coherent antennas, and antennas across groups can be non-coherent/coherent depending on device types. An example of an antenna group is a panel. In RAN1#110-e meeting, it is agreed to consider at least Ng=1,2,4.

In the design of UL codebook for fully coherent 8TX UEs, it is required to accommodate various UE antenna layouts with different number of antenna groups.

    - Based on Rel-15 DL type I codebook

The Rel-15 DL type I codebook is optimized for highly correlated dual-polarized antennas. To support 8TX UL operation, Rel-15 DL type I codebook with 8 port can be used. Intuitively, Rel-15 DL type I single-panel codebook with 8 port can be used for Ng=1; and Rel-15 DL type I multi-panel codebook with 8 port can be used for Ng>1. However, Rel-15 DL type I multi-panel codebook only supports rank 1~4. Hence, only Rel-15 DL type I single-panel codebook can be considered as the starting point for UL 8TX fully-coherent codebook with up to 8 layers, which is not expected to perform well with multiple panels.

    - Based on Rel-15 UL 4TX codebook

For UL 8TX codebook, a group-wise codebook can be considered, where each group is based on Rel-15 UL 2TX codebook or 4TX codebook. A co-phasing can be indicated between groups, depending on the coherence between them. In this way, the 8 TX antenna ports are grouped into different groups depending on the correlation between each other. Figure 2 and Figure 3 show an example of UL 8TX codebook based on Rel-15 UL 4TX codebook, where 8 TX antenna ports are divided into two groups. Two groups have the same codewords P1 and P2 selected from Rel-15 UL 4TX codebook, and a co-phasing coefficient φ is indicated between two groups. Because the maximal number of transmission layers in Rel-15 UL 4TX codebook is 4, two codewords P1 and P2 need to be indicated for more than 4 transmission. If P1 and P2 are selected from fully coherent codewords, the above 8TX codeword is also fully coherent.

Figure 2 UL 8TX codebook for #layers<=4

Figure 3 UL 8TX codebook for 4<#layers <=8

    - Evaluation results

System-level simulation is performed to compare the performance of two candidate codebooks for UL 8TX. In the simulation, the UE antenna layouts agreed in RAN1#109-e meeting are used, as shown in Table 1. Within each group, antenna elements are spaced by 0.5λ. Both the horizontal and vertical spacings (dG-H, dG-V) between the centers of adjacent antenna groups are 0.8λ. The detailed simulation assumptions can be found in Appendix A.

Table 1 The exemplary cases of UE antenna layout

| Case | Ng | (N1, N2, P) per group | Antenna Layout |
| --- | --- | --- | --- |
| 1 | 1 | (2, 2, 2) (4, 1, 2) |  |
| 2 | 2 | (2, 1, 2) |  |
| 3 | 4 | (1, 1, 2) |  |

For Rel-15 DL type I single-panel codebook, we adopt O1=2，O2=2, codebookMode=1 for antenna layout 1-a, 2-a and 3-a; and O1=4，O2=1, codebookMode=1for antenna layout 1-b, 2-b and 3-b. For Rel-15 UL 4TX, P1 is selected from fully coherent codewords of Rel-15 UL 4TX codebook with rank 1~4, and P2 is selected from fully coherent codewords of Rel-15 UL 4TX codebook with rank 4. When the number of layers is not more than 4, the number of co-phasing coefficient φ is N=8, otherwise N=4. Table 2 lists the number of codewords for DL type I and UL 4TX. It can be observed that DL type I and UL 4TX have similar number of codewords.

Table 2 Number of codewords

| Rank | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Total number of codewords |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| “DL type I” with O1=4，O2=1 | 64 | 128 | 96 | 96 | 32 | 32 | 16 | 16 | 480 |
| “DL type I” with O1=2，O2=2 | 64 | 128 | 96 | 96 | 32 | 32 | 32 | 32 | 512 |
| “UL 4TX” | 128 | 64 | 32 | 16 | 128 | 64 | 32 | 16 | 480 |

Figure 4 shows the throughput comparison between “DL type I” and “UL 4TX” for antenna layouts 1-a, 2-a and 3-a. When Ng=1, corresponding to 1-a, the throughput gains of “UL 4TX” than “DL type I” is -1.29% for P0 = -80 dBm and 3.37% for P0 = -50 dBm. When Ng=2/4, corresponding to 2-a/3-a, the throughput gains of “UL 4TX” than “DL type I” are 5%~8% for P0 = -80 dBm and P0 = -50 dBm. Figure 5 shows the throughput comparison between “DL type I” and “UL 4TX” for antenna layouts 1-b, 2-b and 3-b. It can be observed that the throughput gains of “UL 4TX” than “DL type I” are 3%~5% for all considered cases.

Observation 4: For fully coherent codewords, the average throughput gains of codebook based on UL 4TX than that based on DL type I are -1.29%~8% for antenna layouts 1-a, 2-a and 3-a and 3%~5% for antenna layouts 1-b, 2-b and 3-b.

(a) P0 = -80 dBm                                 (b) P0 = -50 dBm

Figure 4 Throughput comparison between “DL type I” and “UL 4TX” for antenna layouts 1-a, 2-a and 3-a

(a) P0 = -80 dBm                                 (b) P0 = -50 dBm

Figure 5 Throughput comparison between “DL type I” and “UL 4TX” for antenna layouts 1-b, 2-b and 3-b

Since the performance of the codebook based on UL 4TX is better than that of the codebook based on DL type I in most cases. To have a unified codebook for 8TX, we have the following proposal.

Proposal 3：Alt2-a should be supported for UL 8TX codebook.

  - Partially/non-coherent codebook

To have a unified codebook for both partially/non-coherent UEs and fully-coherent UEs, the proposed group-wise codebook structure in Figure 2 and 3 can also be used for partially/non-coherent codebook. If P1 and P2 are selected from partially coherent codewords, the above 8TX codeword support two partially coherent antenna groups. If P1 and P2 are selected from non-coherent codewords, the above 8TX codewords support four partially coherent antenna groups. Furthermore, with the indication of 8x1 antenna selection vector, non-coherent UEs can also be supported. Hence, such a unified codebook structure can support all three coherent capabilities, which greatly reduce the spec work.

Proposal 4：For UL 8TX partially/non-coherent antennas, support

- 12P112φP1         φ=exp(jθ),   θ=0:2πN:(N-1)2πN for rank <= 4, and

- 12P112P212φP1-12φP2       φ=exp(jθ),   θ=0:2πN:(N-1)2πN for rank >4.

- UL precoder indication enhancement

In this section, we discuss the precoder indication.

  - Discussion on field size of DCI precoder indication field

In the existing NR specification, TPMI and TRI are jointly encoded to indicate the selected codeword and rank to UE, which is indicated by the “Precoding information and number of layers” field in DCI. For 4TX UE in Rel-15, the UL transmission layer per UE is up to 4. When the UL transmission layer for 8TX UE is increased to 8, the indication of TPMI and TRI will require a larger field than the legacy. In the existing NR specification, the maximum overhead of “Precoding information and number of layers” field is 6 bits. If UL 8TX codebook is designed based on Rel-15 UL 4TX codebook and reuse the legacy TPMI and TRI indication rule, the maximum size of “Precoding information and number of layers” field may reach up to 12 bits. If UL 8TX codebook is designed based on Rel-15 DL type I codebook with antenna selection vector and reuse the legacy PMI indication rule, the maximum indication overhead may exceed 12 bits.

Observation 5: To support 8TX CB based UL transmission, the field size of TPMI indication field is increased by at least twice than legacy.

Although the indication overhead can be reduced by reducing the number of codewords, such as lower oversampling ratios for DL type 1 and fixed rank for UL 4TX, the performance will be degraded.

  - Discussion on high-resolution UL precoder

To reduce the DCI payload, the existing Rel-15 UL 2TX/4TX codebook and Rel-15 DL Type I codebook, as well as the above-mentioned UL 8TX codebook have limited number of codewords, resulting in a low-resolution precoder. 8TX introduces more spatial degree of freedom compared to 2TX and 4TX, which can improve the UL throughput. To maximize the throughput improvement introduced by 8TX, high-resolution precoder should be considered.

Figure 6 and 7 show UL system-level simulation results for 8TX UE with up to 8 layers. The detailed simulation assumptions can be found in Appendix A, where the antenna setup and port layout at UE is (2,2,2,1,1,2,2). Here we consider DL type 1 single-panel codebook with as a basis with O1=4，O2=4 and codebookMode=1. The label of EigenVector denotes that the precoder of each UE is from the eigenvectors of SVD decomposition of the channel matrix, which represents the upper bound of high-resolution precoder. It can be observed that “EigenVector” precoder can obtain 20~33% throughput gain compared with codebooks of “DL type I”.

(a) 64QAM                                           (b) 256QAM

Figure 6 Throughput for DL type I and EigenVector precoder at 32R

(a) 64QAM                                           (b) 256QAM

Figure 7 Throughput for DL type I and EigenVector precoder at 64R

Observation 6: High-resolution precoder such as eigenvector precoder can obtain 20~33% throughput gain compared with that based on DL type I for UL 8TX.

  - Beamformed CSI-RS based UL precoder indication

As we have discussed above, the field size of DCI indication field for indicating 8TX codebook is increased by at least twice than legacy, that is at least 12 bits, which is a large overhead. In addition, high-resolution precoder will lead to unacceptable indication overhead with the legacy DCI indication method. Hence, it is essential to consider new UL precoder indication method to resolve the above issue.

Beamformed CSI-RS can resolve the issues of large DCI overhead, where CSI-RS is used to indicate the selected precoder by TRP to UE, instead of traditional DCI. Specifically, TRP transmits a beamformed CSI-RS with precoder WDL. Then after the TRP-UE channel H, UE can receive the CSI-RS as a vector PUL with dimension nRxx1 , where nRx is the number of receive antenna ports at UE. Then the received vector can be used as a UL precoder for UE.

PUL=HWDL

By choosing the precoder WDL of CSI-RS, the selected precoder PUL can be indicated to UE with high precision. Because the CSI-RS overhead is not related to the number of codewords, we can design a high-resolution codebook with large codebook size. Even TRP can directly indicate a high-resolution precoder (such as “EigenVector” precoder) to UE without codebook. Note that the considered beamformed CSI-RS based UL precoder indication requires reciprocity between UL and DL channels, which can be used for TDD networks.

To indicate the precoder of one layer, the above beamformed CSI-RS based UL precoder indication requires 1 CSI-RS port. For 8TX UL transmission with up to 8 layers, 8 CSI-RS ports are required, which occupies 8 resource elements. If we use the 8 resource elements to transmit DCI in PDCCH with QPSK modulation, the number of bits to be transmitted is typically 8*2*0.2=3.2, where 0.2 is a typical value of code rate in PDCCH. Compared to the DCI indication overhead in the above-mentioned UL 8TX codebook, the overhead of beamformed CSI-RS based UL precoder indication is greatly reduced.

With the beamformed CSI-RS, the overhead can be reduced and the resolution of indication can be increased. Therefore, we have the following proposal.

Proposal 5: The beamformed CSI-RS should be considered to indicate UL precoders to UE.

- Number of CWs and CW-layer mapping

One remaining issue is whether to use dual CW or single CW for rank>4. As observed from Figure 1, there is a large SINR gap between the candidate precoders. If single CW is used for uplink transmission with rank>4, the performance of the TB will be degraded by the layers with lower SNR. To better match the SNR and the MCS, we prefer to support 2 CWs for rank >4. Furthermore, the CW-layer mapping of Rel-15 PDSCH can also be reused for PUSCH with 8TX. Hence, we have the following proposal.

Proposal 6: Dual CW is used for uplink transmission with rank>4.

Proposal 7: Re-use the CW-layer mapping of Rel-15 PDSCH for uplink transmission with rank>4.

- Conclusions

The contribution discusses the SRI, TPMI (including codebook) enhancement for enabling 8TX UL transmission, based on which the following observations and proposals are made.

Observation 1: Whether SRI enhancement for enabling 8TX in CB based UL transmission is needed depends on the SRS design for 8TX.

Observation 2: To support 8TX NCB based UL transmission, the legacy SRI field will introduce a large DCI overhead if full flexibility of SRS resource selection is supported.

Observation 3: For 8TX NCB based UL transmission, there is large SINR gap between the precoders used in SRS resources with usage ‘nonCodebook’.

Observation 4: For fully coherent codewords, the average throughput gains of codebook based on UL 4TX than that based on DL type I are -1.29%~8% for antenna layouts 1-a, 2-a and 3-a and 3%~5% for antenna layouts 1-b, 2-b and 3-b.

Observation 5: To support 8TX CB based UL transmission, the field size of TPMI indication field is increased by at least twice than legacy.

Observation 6: High-resolution precoder such as eigenvector precoder can obtain 20~33% throughput gain compared with that based on DL type I for UL 8TX.

Proposal 1: A single SRS resource set configured with up to 8 single-port SRS resources is supported.

Proposal 2: To reduce overhead of SRI for NCB PUSCH, reduce the flexibility of SRS resource selection.

Proposal 3：Alt2-a should be supported for UL 8TX codebook.

Proposal 4：For UL 8TX partially/non-coherent antennas, support

- 12P112φP1         φ=exp(jθ),   θ=0:2πN:(N-1)2πN for rank <= 4, and

- 12P112P212φP1-12φP2       φ=exp(jθ),   θ=0:2πN:(N-1)2πN for rank >4.

Proposal 5: The beamformed CSI-RS should be considered to indicate UL precoders to UE.

Proposal 6: Dual CW is used for uplink transmission with rank>4.

Proposal 7: Re-use the CW-layer mapping of Rel-15 PDSCH for uplink transmission with rank>4.

# References

- RAN1 Chairman’s Notes, Toulouse, France, August 22nd – 26th, 2022.

- R1-2208443, “SRS enhancement for TDD CJT and 8 TX operation in Rel-18”, 3GPP RAN#110bis-e, eMeeting, Oct., 10-19, 2022, Huawei, HiSilicon.

# Appendix A

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Carrier Frequency | 3.5G |
| Channel Model | According to the TR 38.901 |
| Scenario | Uma with 500 m ISD |
| Antenna setup and port layouts at TRP | (12,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (12,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 |
| Antenna setup and port layouts at UE | (2,2,2,1,1,2,2) , (1,2,2,2,1,1,2) , (1,1,2,2,2,1,1) (1,4,2,1,1,1,4) , (1,2,2,1,2,1,2) , (1,1,2,1,4,1,1) (𝑑H, 𝑑V) = (0.5, 0.5)𝜆 (𝑑G-H, 𝑑G-V) = (0.8, 0.8)𝜆 |
| TRP antenna height | 30m |
| TRP receiver noise figure | 5dB |
| UE Tx power | 23 dBm |
| UE antenna height | 3m |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| Modulation | up to 256QAM |
| Bandwidth | 20 MHz |
| MIMO scheme | SU-MIMO |
| Rank | Rank adaptation with up to 8 layers |
| Traffic model | Full buffer |
| UE distribution | 100% outdoor (3km/h) |
| BS receiver | MMSE-IRC |
| Network Layout | 1*3 cell, 10 UE pre cell |
| Precoder granularity | Wideband |
| Scheduling granularity | 4RB |
| Power control | Open loop; alpha = 0.8; P0 = -50, -80 dBm |