3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2208449

e-Meeting, October 10 – 19, 2022

Agenda Item:	9.4.1.2

Source:	Huawei, HiSilicon

Title:	Physical channel design for sidelink operation over unlicensed spectrum

Document for:	Discussion and Decision

Introduction

RAN#94-e approved a new work item on NR sidelink evolution [1], which is further updated in RAN#96 [2]. One objective of this WI is to study and specify sidelink on unlicensed spectrum for both mode 1 and mode 2:

Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98. - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

This paper will discuss SL-U physical channel design framework, including consideration on how to reuse existing sidelink designs as well as NR-U design, with necessary modification to accommodate these issues due to operation over unlicensed spectrum. The discussion is organized into a set of sections as follows: bandwidth part and resource pool, slot structure, PSCCH/PSSCH, PSFCH and SL-HARQ, and S-SSB.

Physical channel design

Bandwidth part and resource pool

At RAN1#109-e, following agreements on SL BWP and resource pool were made [3].

Agreement SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc.

Frequency domain configuration

Resource pool configuration

On resource pool configuration, one FFS point is whether/ how to support one SL resource pool can include sub-set of PRBs of one RB set. We do not support such design with the following analysis.

For unlicensed spectrum, before transmission, LBT has to be performed per RB set except short control signaling. Transmissions can also take place on multiple RB sets subject to multi-channel access procedure.

Some companies mentioned one benefit that a resource pool only includes sub-set PRBs of one RB set is to allow FDMed resource pool. However, when a resource pool includes integer number of RB sets, UEs can still be FDMed, e.g., different UEs use different contiguous PRBs or interlaced PRBs. The way that several UEs simultaneously access a LBT channel via their own FDMed resource pool is similar to that several UEs simultaneously access the same resource pool with one LBT channel bandwidth, i.e., all those UEs needs to monitor the LBT channel and sense reservation information.

If a resource pool only includes sub-set of contiguous PRBs or interlaced PRBs of one RB set, a UE can only use part of this RB set if LBT succeeds on this RB set, thus causing resource waste. In addition, for resource pool including only sub-set of contiguous PRBs of one RB set, if bandwidth of the resource pool is less than 80% of 20MHz, OCB requirement cannot be satisfied.

In summary, the design that one SL resource pool can include sub-set of PRBs of one RB set will cause resource waste and may not satisfy OCB requirement, thus not supported.

Observation 1: If a resource pool includes sub-set of contiguous or interlaced PRBs of one RB set, it may result in wasting resource.

Observation 2: The way that several UEs simultaneously access a LBT channel via their own FDMed resource pool is similar to that several UEs simultaneously access the same resource pool with one LBT channel bandwidth.

Observation 3: A resource pool that only includes sub-set of contiguous of PRBs of one RB set may violate OCB requirements.

Proposal 1: Do not support one SL resource pool includes sub-set of PRBs of one RB set.

Usage of intra-cell guard band PRB

On PRBs within intra-cell guard band of two adjacent RB sets, one FFS point is to how such PRBs are used. In NR-U, PRBs within intra-cell guard band can only be used when UE’s LBT is successful on two adjacent RB sets and the UE uses these two RB sets simultaneously for transmission. Otherwise, these PRBs remain unusable. Such design can be reused in SL-U directly.

Proposal 2: SL-U uses PRBs within intra-cell guard band in the same way as NR-U, i.e., PRBs within intra-cell guard band of two adjacent RB sets can only be used when UE’s LBT is successful on these two LBT channels, and the UE uses these two RB sets simultaneously for transmission.

Time domain configuration

In R16/R17 NR sidelink, some slots are excluded from a resource pool, i.e., excluding slots for S-SSB, slots for DL transmission, slots with insufficient number of UL symbols, and reserved slots. Whether the remaining slots are part of resource pool or not are determined based on a bitmap (pre-)configured by higher layers.

Unlicensed band can be accessed by a UE with either 3GPP or non-3GPP channel access technology subject to regulation. Therefore, even under gNB scheduling, there is no guarantee that the channel can be available for DL or UL or SL transmission at a given time. If SL-U considers Uu TDD configuration and excludes slots from SL resource pool as per Rel-16 NR SL design, it would unnecessarily restrict channel access for SL transmissions and reduce COT utilization efficiency for SL-U. As shown in Figure 1, even there is no inter-RAT device, when Uu TDD configuration is considered for the configuration of SL resource pool, the transmission of Uu data before SL slots may cause that SL UE cannot access channel timely and SL slots are wasted until LBT success. In addition, the duration of COT is highly limited and the SL UE have to stop its transmission if subsequent slots are configured for Uu slots even though there is no need for data transmission.  Therefore, we propose that when determining which slots belong to a SL resource pool, Uu TDD configuration is not taken into account.

Figure 1 SL transmission limited by Uu transmission

Proposal 3: In SL-U, do not consider Uu TDD configuration when determining slots belonging to a SL resource pool.

As long as “0” exists in the bitmap configuration, the slots allocated in a resource pool are non-contiguous, resulting in slot-level gap between transmissions. Other UE’s LBT may be successful during such gap, resulting in COT lost. For example, as shown in Figure 2, assuming the (pre-)configured bitmap is {1111000}, there are 3 slot gaps between transmission in logical slot #4 and #5, and there is no transmission during such 3 slots in this resource pool.

In addition, Rel-16 reserved slots are inserted such that the remaining available slots for sidelink is always integer multiple of bitmap length, which further adding slot-level gap in the SL resource pool. If bitmap is set to all “1”, the reserved slots are not needed.

Observation 4: If the bitmap of a resource pool is not set to all “1”s, it will result in slot-level gaps between consecutive transmissions and difficult for the UE to maintain a COT.

Proposal 4: Set the value of bitmap to all “1”s to avoid slot-level transmission gaps in a resource pool.

Figure 2 Logic slots in a resource pool

In R16/R17, S-SSB slots are excluded from resource pool to avoid half-duplex issue between S-SSB and other SL channel/signal. Half-duplex issue still exists in SL-U so that same design can be reused, i.e., R16/17 NR SL S-SSB slots are excluded from resource pool. It is worth noting that SL-U may introduce additional S-SSB occasions to address LBT failure issue, whether or not such additional S-SSB occasions are excluded from the resource pool can be further discussed once the details are clearer.

Proposal 5: R16/17 NR SL S-SSB slots are excluded from resource pool.

Slot structure

Number of starting symbols within a slot

From the following agreement made on slot structure in SL-U at RAN1#109-e [3], a consensus is reached that the slot-based structure in R16/R17 is reused and other transmission structures such as mini-slot are precluded. Due to the uncertainty of channel access over unlicensed band, whether there is only one or more than one starting symbol(s) in a slot still needs further study.

Agreement For slot structure in SL-U: - At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission

A UE’s LBT can be successful at any time, i.e. not necessarily at the slot boundary, since LBT sensing granularity is 9 us which is neither a symbol nor a slot granularity used in NR sidelink. If a UE’s LBT succeeds in the middle of a slot, starting symbol for PSCCH/PSSCH transmission might be missed for transmission if there is only one starting symbol in a slot. Then, the UE has to wait until next slot boundary for sidelink transmission, leading to the risk that the channel might be occupied by other UEs (including non-3GPP RAT UE). This may result in SL-U performance being degraded due to forced and unnecessary delayed channel access.

Observation 5: Since LBT can be successful at any point within a slot, starting symbol for PSCCH/PSSCH transmission might be missed if there is only one starting symbol in a slot, which degrades SL-U performance.

A UE expects to access the channel after LBT success as soon as possible. Instead of one fixed starting symbol for PSCCH/PSSCH transmission as defined in Rel-16, more starting symbols for PSCCH/PSSCH transmission can be introduced to provide more chance for a TX UE to transmit after LBT success. On the other hand, the number of starting symbol for PSCCH/PSSCH transmission should be limited to reduce UE implementation complexity. Thus, two starting symbol for PSCCH/PSSCH transmission is a good balance between performance and UE complexity. These two starting occasions can be regarded as two sets of starting symbol with different symbol length in a slot. R16 NR SL design can be reused, where starting symbol is provided by sl-StartSymbol-r16 with value range as {sym0, sym1, sym2, sym3, sym4, sym5, sym6, sym7}, and symbol length in a slot is provided by sl-LengthSymbols-r16 with value range as {sym7, sym8, sym9, sym10, sym11, sym12, sym13, sym14}.

An example is given in Figure 3, where one starting symbol is #0 symbol with length of 14 symbols, and another starting symbol is #4 symbol with length of 10 symbols. A TX UE can start a transmission at the corresponding candidate AGC occasion after LBT success. In such design, no additional specification impact is needed in terms of, e.g. slot structure, DMRS patterns.

Figure 3 Candidate starting symbols for PSCCH/PSSCH transmission (one starting from #0 and another starting from #4)

Observation 6: Introducing more starting symbols within a slot for the PSCCH/PSSCH transmission is beneficial to provide more channel access chances, but at the cost of increasing UE complexity.

- Two starting symbols within a slot for the PSCCH/PSSCH transmission is a good balance between performance and UE complexity, and it can be regarded as defining two sets of starting symbol with different symbol length in a slot, thus having no additional specification impact in terms of, e.g. slot structure, DRMS patterns, etc.

When a slot includes two starting symbols for the PSCCH/PSSCH transmission, a TX UE transmits AGC and PSCCH symbols at the first available AGC and PSCCH candidate resource location in the slot right after LBT success. The RX UE performs AGC training and PSCCH decoding only at AGC and PSCCH candidate occasions in a slot until it successfully decodes a PSCCH for the initial slot of a COT. After a UE receives COT sharing information and knows the COT structure, the UE does not need to search for multiple AGC and PSCCH candidates in slots within the COT.

Observation 7: When a UE receives COT sharing information and knows the COT structure, the UE does not need to search for multiple starting symbols in slots within the COT.

In the following, we provide evaluation results to show the gain of the proposed scheme, i.e., introducing two starting positions within a slot.

In our simulation, there are 3 clusters and each one includes a center UE and four randomly dropped surrounding UEs, as shown in Appendix Table 1. One of the surrounding UEs generats XR traffic and the other three surrounding UEs generats FTP traffic, and they communicate with the center UE to form a mixed traffic comminication system. We assume 15 ms PDB requirement for XR cloud gaming traffic and 10 ms PDB requirement for XR pose/control traffic, as in TR 38.838. In addition, there are also Wi-Fi devices with FTP traffic and will introduce interfernce to SL-U.

In the baseline scheme, there is only one starting symbol (symbol #0) within a slot for PSCCH/PSSCH transmission. In the proposed sheme, there are two starting symbol (symbol #0 and #4) within a slot for PSCCH/PSSCH transmission. More details about simulation assumptions can be found in Appendix.

As shown in Figure 4, the proposed scheme can achive 52% UPT gain compared with baseline for XR traffic. Such large UPT gain mainly comes from the fact that XR traffic can access the channel for transmission as soon as possible with more opportunities. In our simulation, we assign CAPC=1 to XR traffic since they are important. The maximum sensing durations for Type 1 channel access for  CAPC = 1 is mp*Tsl+Tf+CWp*Tsl=2*9 + 16 + 7*9 = 97 us, which is less than 3-symbol length of a 30 kHz SCS. Thus, the Type-1 channel access of XR traffic can be successful before the start of 4th symbol in a slot and can access the channel quickly thanks to the enhanced slot structure.

52%

Figure 4 Performance comparison between one starting symbol (symbol#0) and two starting symbols (symbol#0, #4) within a slot for PSCCH/PSSCH transmission

Observation 8: Compared with legacy slot structure, i.e., one starting symbol within a slot for PSCCH/PSSCH transmission, introducing two starting symbols can largely increase UPT, especially for traffic with low latency requirement such as XR.

In R16/R17 sidelink design, the starting symbol and length of a SL slot are configurable in accordance with corresponding configurations for UL symbols. However, it would unnecessarily restrict channel access for SL transmissions and disable multi-consecutive slots transmission (MCSt) for SL-U. As shown in Figure 5, assume the starting symbol of a SL slot is configured to symbol #2 and the corresponding length is 10 symbols. It’s impossible to enable MCSt in such configuration since the SL UEs have to stop its transmission if subsequent symbols are configured for Uu transmission, even though there is no need for UL data transmission. Therefore, we propose that the 1st starting symbol within a slot is fixed to symbol #0 and the corresponding length of such slot is 14 symbols for normal CP length and 12 symbol for extended CP length.

The 2nd starting symbol is introduced to increase channel access opportunity and its candidate position should avoid overlapping of AGC/PSCCH symbol and avoid additional specification changes, such as DMRS pattern.

Figure 5 SL transmission limited by NR Uu when sl-StartSymbol-r16 and sl-LengthSymbols-r16 are (pre-) configured to sym2 and sym10

Proposal 6: Support two starting symbols within a slot for the PSCCH/PSSCH transmissions:

- The 1st starting symbol is fixed to symbol 0, and the corresponding length for PSCCH/PSSCH transmissions in a slot is 14 symbols for NCP and 12 symbols for ECP.

- The 2nd starting symbol is M2, and the corresponding length for PSCCH/PSSCH transmissions in a slot is 14-M2 symbols for NCP and 12-M2 symbols for ECP.

  - FFS value of M2.

- Note: the 1st symbol for PSCCH/PSSCH transmission is used for AGC purpose.

Multi-consecutive slots transmission (MCSt)

In Rel-16, gap symbol is introduced at the end of slot, to allow enough time for Tx-Rx switching. Based on multi-slot consecutive transmission, the RX UE may possibly receive continuous transmission from TX UE and gap symbols within consecutive transmissions may not be needed. As shown in Figure 6, when UE 1 transmits data to UE 2 in both slot n and slot (n+1), the GAP symbol on slot n can be used for PSSCH transmission to avoid COT loss and improve resource utilization.

Figure 6 slot structure in case of consecutive MCSt performed by a single TX UE.

However, inter-UE blocking issue may occur if the gap symbol is ignored without considering other UEs transmission in same RB set, i.e. different UE uses different interlace RB to achieve FDM. Therefore, applicable scenarios for gap symbol removal in multi-consecutive slots transmission should be further studied. Note that gap symbol removal issue is also discussed in our companion paper [6].

Proposal 7: When multi-consecutive slots transmission is performed, the gap symbol between two adjacent slots can be used for PSSCH transmission.

- FFS details, e.g., applicable scenarios, etc.

PSCCH/PSSCH

Sub-channel definition

At RAN1#110, following agreement on sub-channel was made [4]. One point that needs further discussion is the definition of sub-channel for interlace RB-based transmission, i.e., whether 1 sub-channel equals K (K>=1) interlaces or 1 sub-channel is fixed as 1 interlace, and whether to confine 1 sub-channel within 1 RB set. We provide our analysis as below.

Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - 1 sub-channel equals K interlace - FFS: whether K is fixed as 1 or (pre-)configured - Discuss whether one or both of the following alternatives are supported - Alt 1: 1 sub-channel is confined within 1 RB set - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool

In NR-U, interlace is defined per carrier, not per RB set, so that 1 interlace may span across multiple RB sets, including intra-cell guard band. One interlace is the frequency domain resource allocation granularity and thus, two-level resource indication, i.e., RB set index and interlace index, is introduced to indicate which RB set(s) and which interlace(s) are used currently. These designs can be reused in SL-U.

1 sub-channel equals 1 or K interlace

Since it’s agreed that one sub-channel is used as frequency domain resource allocation granularity for PSSCH transmission, 1 sub-channel can be defined as 1 interlace. So a UE who needs K interlaces can be satisfied via K sub-channels. It’s obvious that such definition is very flexible to satisfy all kinds of service no matter small data size or large data size.

Observation 9: When 1 sub-channel is defined as 1 interlace, a UE who needs K interlaces can be satisfied via K sub-channels.

Some companies proposed that 1 sub-channel equals K interlaces and K is (pre-)configurable. However, for the case of 30 kHz SCS, there are only 5 interlaces per BWP. When K is configured to 2, 3 or 4, there will be new resource indication issue or resource wastage since 5 is not an integer multiple of 2, 3 or 4. When K is configured to 5, a UE shall use all the RBs of one RB set even with small data size, resulting in resource inefficiency.

Observation 10: For the case of 30 kHz SCS, there are only 5 interlaces per BWP, defining 1 sub-channel equals K interlaces has the following problems:

- When K is configured to 2, 3 or 4, there will be new resource indication issue or resource wastage since 5 is not an integer multiple of 2, 3 or 4.

- When K is configured to 5, a UE shall use all the RBs of one RB set even with small data size, resulting in resource inefficiency.

1 sub-channel confined within 1 RB set or across multiple RB sets

If the sub-channel is confined within one RB set, due to the existence of guardband PRBs, the number of PRBs within a RB set may not be integer multiple of the number of PRBs of 1 sub-channel. Thus, the sub-channel indexing and usage will become very complicated.

In NR-U, interlace is defined per carrier, so that 1 interlace may span across multiple RB sets. Such design can be reused for SL-U, which makes sub-channel indexing simple. Not that although 1 sub-channel spans multiple RB set(s) belonging to a resource pool, it’s still feasible to indicate and use 1 sub-channel in 1 RB set by reusing NR-U two-level resource indication, i.e., RB set index and interlace index.

Observation 11: If the sub-channel is confined within one RB set, due to the existence of guardband PRBs, the number of PRBs within a RB set may not be integer multiple of the number of PRBs of 1 sub-channel. Thus, the sub-channel indexing and usage will become very complicated.

Proposal 8: For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- One sub-channel equals to one interlace and spans across all RB sets belonging to a resource pool.

- Note: the minimum frequency resource size for one PSSCH transmission is one sub-channel in one RB set subject to resource indication design.

Sub-channel indexing

When a resource pool includes multiple RB sets, the RBs belonging to intra-cell guard band between two adjacent RB sets needs to be allocated to sub-channel(s).  For sub-channel index in interlace RB-based transmission, the first sub-channel begins from the lowest RB of the resource pool and then the sub-channels are indexed in ascending order along with interlace.

For example, for 30 kHz SCS shown in Figure 7, the lowest RB locates in interlace #0 and sub-channel #0 is indexed from interlace #0, and then other sub-channels are indexed in increasing order by per interlace (e.g., sub-channel#2 = interlace 2).

Figure 7 Relationship between sub-channel and interlace

Proposal 9: For sub-channel index in SL-U:

- For contiguous RB-based transmission, reuse Rel-16 NR SL sub-channel indexing.

- For interlace RB-based transmission, the first sub-channel begins from the lowest RB of the resource pool and the sub-channels are indexed in ascending order along with interlace.

PSCCH/PSSCH mapping

At RAN1#110, following agreement on resource indication for PSSCH and PSSCH is made [4].

Agreement For PSCCH and PSSCH in SL-U: - Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported

For both contiguous RB-based and interlace RB-based transmissions, PSCCH/PSSCH mapping can have unified design. For the case of contiguous RB-based case, as shown in Figure 8 (a), PSCCH locates in the lowest sub-channel of allocated associated PSSCH sub-channel(s). This reuses Rel-16 PSCCH and PSSCH multiplexing. For the case of interlaced RB-based case, as shown in Figure 8 (b), same principle of Rel-16 can also be reused. For interlaced transmission, the starting frequency location of PSCCH aligns with that of associated PSSCH, and is always within one sub-channel which is equal to one interlace.

In Rel-16 NR-V, resource pool (pre-)configuration ensures that frequency of PSCCH allocation is no larger than sub-channel size, so that a UE is only required to blind decode one PSCCH in a given sub-channel. Such restriction can be reused for interlaced transmission in SL-U to avoid increasing UE blind decoding complexity.

Proposal 10: For PSCCH and PSSCH in SL-U, for both contiguous RB-based and interlaced RB-based transmission:

- PSCCH is always transmitted within one sub-channel

- R16 NR-V PSCCH and PSSCH multiplexing is reused, i.e., PSCCH locates in the sub-channel with the lowest index of the sub-channel(s) of corresponding PSSCH

Figure 8 PSCCH and PSSCH mapping: (a) in terms of contiguous RB allocation; (b) in terms of interlaced RB allocation

Resource indication

At RAN1#110, following agreement on resource indication for PSSCH and PSSCH is made [4]. In this section, we analyze the resource indication for contiguous RB-based and interlace RB-based transmissions, respectively.

Agreement For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication - FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

Contiguous RB-based PSCCH/PSSCH transmission

In NR sidelink, the UE can indicates its candidate resources for each TB by TRIV and FRIV carried in 1st SCI as shown in Figure 9 a).

In frequency domain, because the definition and indexing of sub-channel for contiguous RB-based PSCCH/PSSCH transmission over unlisenced band are same as that in R16/R17 SL design, legcy FRIV can be resued.

In time domain,  supporting MCSt is agreed for SL-U in the last meeting. Some companies may consider enhancement of TRIV, e.g., enhancing TRIV to indicate multiple TBs. We think such TRIV enhancement is unnecessary. Because if a UE indicates multiple TBs in a signle SCI, such as 3 TBs, the UE needs to select candidate reources in consecutive slots for multiple TBs, while each TB may require different number of sub-channels.  It’s hard for a UE to select candidate resource that meet all requirements for each TB. In addition, such enhancement will introduce large specification changes, such as impact on resource selection procedure, since in R16/R17 design a UE selects a candidate resource that only includes one slot and several sub-channels for one TB in one procedure.

In summary, to avoid large spec impact, we propose to reuse legacy TRIV indication mechanism. As shown in Figure 9 b),  the TRIV indication for each TB remains the same with that of R16/R17 sidelink.

Figure 9 Resource indication for contiguous RB transmission for SL-U

Proposal 11: For contiguous RB-based PSCCH/PSSCH transmission, both R16 NR SL TRIV and FRIV are reused for resource indication.

Interlace RB-based PSCCH/PSSCH transmission

It was agreed in RAN1#110 that sub-channel is used as the basic resource allocation granularity for interlaced RB-based transmission.

In NR-U, each interlace uniformly spans on the carrier with the spacing of 10 RBs and its RB(s) may belong to guard band. The number of interlace for 15 kHz and 30 kHz is 10 and 5 respectively. In SL-U, we propose that one sub-channel equals one interlace across all the RB set(s) belonging to resource pool, so that the resource allocation of sub-channel is determined correspondingly.

As shown below, in NR-U, two-level indication, i.e. X+Y, is introduced to indicate allocated interlace and RB set(s) respectively, where RIV and bitmap are used to indicate allocated interlace(s) for 15 KHz SCS and 30 KHz SCS respectively, and RIV is used to indicate allocated RB set(s). Note that the allocated RB set(s) for one UE should be contiguous.

(Below is copied from TS 38.214) For µ=0, the X=6 MSBs of the resource block assignment information indicates to a UE a set of allocated interlace indices m0+l, where the indication consists of a resource indication value (RIV). For 0≤RIV<M(M+1)/2 , l=0,1,⋯L-1 the resource indication value corresponds to the starting interlace index m0 and the number of contiguous interlace indices (). The resource indication value is defined by: … For µ=1, the X=5 MSBs of the resource block assignment information comprise a bitmap indicating the interlaces that are allocated to the scheduled UE. The bitmap is of size M bits with one bitmap bit per interlace such that each interlace is addressable, where M and interlace indexing is defined in Clause 4.4.4.6 in [4, TS 38.211]. The order of interlace bitmap is such that interlace 0 to interlace M-1 are mapped from MSB to LSB of the bitmap. An interlace is allocated to the UE if the corresponding bit value in the bitmap is 1; otherwise the interlace is not allocated to the UE. For DCI 0_0 monitored in a UE-specific search space and DC 0_1 for both µ=0 and µ=1, the Y=log2NRB-set,ULBWPNRB-set,ULBWP+12LSBs of the resource block assignment information indicate to a UE a set of contiguously allocated RB sets for PUSCH scheduled by DCI 0_0 monitored in a UE-specific search space, DCI 0_1 and Type 1 and Type 2 configured grant. The resource allocation field consists of a resource indication value (RIVRB-set). For 0≤RIVRB-set<NRB-set,ULBWP(NRB-set,ULBWP+1)/2 , l=0,1,⋯LRBset-1 the resource indication value corresponds to the starting RB set index NRB-set,ULstart and the number of contiguous RB sets LRB-set . The resource indication value is defined by; if (LRB-set-1)≤NRB-set,ULBWP/2 then RIVRB-set=NRB-set,ULBWP(LRB-set-1)+NRB-set,ULstart else RIVRB-set=NRB-set,ULBWP(NRB-set,ULBWP-LRB-set+1)+(NRB-set,ULBWP-1-NRB-set,ULstart) where NRB-set,ULstart=0,1,⋯NRB-set,ULBWP-1, LRB-set≥1 and shall not exceed NRB-set,ULBWP-NRB-set,ULstart

Observation 12: Two-level resource indication, i.e. X+Y, is introduced in NR-U, where X bits indicate a set of allocated interlace indices and Y bits indicate a set of contiguously allocated RB set(s).

Figure 10 gives two examples of resource indication in NR-U. As shown in left figure, if UE1’s transmission occupies 2 interlaces in one RB set, it can be realized by setting X bits indicating “interlace #0&#1” and setting Y bits indicating “RB set 1”. In the right figure, if UE2’transmission occupies 2 interlaces in two RB set, it can be realized by setting X bits indicating “interlace #0&#1” and setting Y bits indicating “RB set 0&1”.

Similarly, two-level indication can be reused for SL-U for interlace RB-based transmission. In addition to sub-channel indication, indication on which RB set(s) are used for transmission is also needed. This is similar to NR-U where both RB set index and interlace index are indicated in a DCI/CG. When a UE requires multiple sub-channels for PSSCH transmission, these sub-channels are supposed to be adjacent to reduce IBE especially for interlaced based structure. Figure 11 gives an example for SL-U resource indication, where we assume there are two RB sets within the resource pool, and each sub-channel includes RBs that belong to these two RB sets and some sub-channels may also include RBs that belong to the intra-cell guard band. If UE1’s transmission occupies 2 sub-channels in one RB set, it can be realized by setting R16 NR SL FRIV indicating sub-channel #0&#1 and setting Y bits (newly introduced) indicating RB set 1.

Figure 10 Resource indication for interlaced RB transmission in NR-U

Figure 11 Resource indication for interlaced RB transmission in SL-U

In addition, Rel-16 TRIV can still be used for time domain indication for interlaced based transmission since it is not related to frequency transmission structure.

Proposal 12: For interlaced based PSCCH/PSSCH transmission,

- For time domain indication, R16 NR SL TRIV is reused.

- For frequency domain indication,

  - R16 NR SL FRIV is reused to indicate sub-channel index.

  - Additional Y bits, which depends on the number of RB sets within the resource pool, are introduced to indicate a set of contiguously allocated RB set(s) as NR-U.

Unequal sub-channel size issue

It is noted that the number of available RBs of different sub-channel(s) may not be the same due to guard band. As shown in Figure 12, no matter contiguous RB-based or interlaced-RB based configuration, there are cases that the frequency resource allocation for a TB between initial transmission and retransmission are different due to the configuration of guard band.

In Figure 12 (a), the number of PRBs in SubCH#X+1 may be less than the (pre-)configured sub-channel size when SubCH#X+1 includes PRBs belonging to guard band, resulting in the total number of PRBs for initial transmission (if on SubCH#1 and SubCH#2) and retransmission (if on SubCH#X and SubCH#X+1) might be different.

Similarly, in Figure 12 (b), assume one RB set includes 53 PRBs and there are 5 interlaces for 30 KHz SCS. Then there are 3 interlace with 11 PRBs of each interlace and 2 interlaces with 10 PRBs of each interlace in one RB set. When the first 2 interlaces are allocated for initial transmission and the second 2 interlaces are allocated for retransmission, the total number of PRBs for initial transmission is 22 while the total number of PRBs for retransmission is 21, resulting in unequal number of PRBs allocated for (re)transmission of the same TB even though they have been allocated with same number of interlaces/sub-channels.

Figure 12 Unequal number of PRBs between initial transmission and retransmission

Observation 13: Due to the existence of intra-cell guard band PRB, for both IRB and contiguous RB, the available PRBs in different sub-channels may be different, resulting in different number of PRBs for initial and retransmission of same TB.

The issue above will impact TBS calculation. Since initial and retransmission of same TB may have different number of PRBs, it’s not possible to calculate the TBS based on the actual number of PRBs. A straightforward way to solve it is by introducing reference number of PRBs to determine TBS. This is similar to Rel-16 design on introduction of reference number of symbols in a slot, taking into account that PSFCH may or may not be present in a slot subject to resource pool configuration, and resulting in different TBS determination for (re-)transmission of a TB.

Proposal 13: Introduce reference number of PRBs to determine TBS to address the issue of unequal number of PRBs in the allocated frequency resource for (re)transmission of a same TB.

During previous meetings, some companies mentioned a case where the PRBs within a resource pool may not be an integer multiple of the sub-channel size. This case also happens in legacy NR SL and such edge PRBs are not used. We propose to reuse such design.

Proposal 14: For the case where the number of PRBs within a resource pool is not an integer multiple of the sub-channel size, R16 NR SL design is reused, i.e., edge PRBs that does not belong to a sub-channel are not used.

PSFCH and SL-HARQ

PSFCH resources

At RAN1#110, following agreement on locations of PSFCH was made [4]. There are two categories on the locations of PSFCH resources, one is determined by (pre-)configuration and the other is dynamic indication. The details about these two will be discussed as follows.

Agreement If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied - Alt 1: PSFCH resources are (pre-)configured - Alt 2: PSFCH resources are dynamically indicated - Combination of above alternatives are not precluded FFS details of above alternatives

(Pre-)configured PSFCH resource

In NR V2X, resources for PSFCH are (pre-)configured and occur in the resource pool periodically, e.g. in every 1, 2 or 4 slots(s) within a resource pool. However, such periodic PSFCH resources may not be suitable for operations over unlicensed spectrum, because:

- LBT may fail: As shown in Figure 13, it’s possible that the UE’s LBT fails before a (pre-)configured PSFCH resource, so that the UE is unable to transmit HARQ ACK on this resource.

Figure 13 No PSFCH transmission at configured PSFCH resource due to LBT failure

Introducing multiple PSFCH candidate resource is one of potential way to increase PSFCH transmission chances. However, given limited PSFCH resources, the more candidate PSFCH resource introduced, the less multi-user multiplexing supported.

In addition, even if we introduce more PSFCH transmission chances, it’s still possible that those occasions may be occupied by other UEs. For example, as shown in Figure 14, once the (pre-)configured resources within the resource pool are occupied by other RAT UEs, even if there are four or more candidate PSFCH resource for UE 1 and UE 2, they still cannot perform HARQ-ACK reporting as expected. This is because once the COT is occupied by other-RAT UEs, the duration of the occupied COT is usually at millisecond level.

Simply increasing the number of candidates PSFCH occasions will have capacity problem, i.e., the total number of available PSFCH in one slot for a UE will be significantly reduced, and this is particularly worse for groupcast option 2-based HARQ feedback.

Figure 14 PSSCH with multiple PSFCH resources

- Absence of PSFCH transmission: As shown in Figure 15(a), the (pre-)configured PSFCH resource might be close to the beginning of a COT, thus leaves not enough time for the UE to process the data and thus unable to transmit HARQ-ACK on such PSFCH resource. And for the case shown in Figure 15(b), it’s possible that no UE transmits HARQ-ACK on the (pre-)configured PSFCH resource within a COT, e.g., blind retransmission, etc. If either happens, the current COT may be lost because such PSFCH gap is too large that other UE’s LBT might be successful and access the channel. In addition, such PSFCH duration are still taken into account when calculating COT duration, which reduce the available COT duration considering the Maximum COT duration limitation.

Figure 15 No PSFCH transmission may result in COT lost (a) due to processing time constraint; (b) blind retransmission

- No need to have dense or any PSFCH resources in some cases: E.g., for transmitting a large FTP file, of which the remaining PDB is large, HARQ-ACK can be reported once at the end of the COT, rather than multiple times within the COT. Another example is that TX UE may perform blind retransmissions and there is no need to have any PSFCH resource within the COT.

Observation 14: For PSFCH resource, Rel-16 (pre-)configured way is not suitable for unlicensed spectrum because:

- LBT may fail before a (pre-)configured PSFCH resource even with multiple candidate PSFCH resources, so that the UE is unable to transmit HARQ ACK on these resources.

- It’s possible that no UE transmits HARQ-ACK on the (pre-)configured PSFCH resource within a COT, such that the current COT may be lost.

- No need to have dense or any PSFCH resources in a COT, e.g., remaining PDB is large so that no need for frequent HARQ-ACK feedback, blind retransmissions, etc.

Dynamic PSFCH resource

To address the above issues, dynamically indicated PSFCH resources can be introduced, i.e., when a UE’s LBT is successful and initiates a COT, the UE can indicate where the PSFCH resources are within its COT. Note that such design is similar to NR-U, where PUCCH resource can be dynamically scheduled by gNB via introducing signaling a value of the PDSCH-to-HARQ-timing-indicator in the DCI and the UE can obtain the specific timing and resource for HARQ-ACK feedback, thus gNB can effectively utilize resources within a COT by indicating PUCCH resource corresponding to the specific PDSCH.  As shown in Figure 16, the location of PSFCH resources within the COT can be explicitly indicated in COT sharing information, and the UEs who obtains the COT sharing information can know the PSFCH resources within the COT and transmit HARQ-ACK at those indicated occasions. In addition, the dynamically indicated PSFCH resource shall satisfy HARQ RTT gap as in legacy NR SL design.

Figure 16  PSFCH resource location dynamically indicated within the COT

Proposal 15: Support COT initiating UE dynamically indicates PSFCH resources within a COT.

SL HARQ-ACK report on PC5

New PSFCH Format

Given that SL-U is designed to support high data rate services, the packet size might be very large and need to be transmitted in consecutive slots. Thus, PSFCH format 0 becomes less sufficient in term of resource utilization and PSFCH overhead. One possible enhancement is to reuse code-based HARQ feedback supported in NR Uu (including NR-U), which allows RX UE to feedback its HARQ-ACK addressing multiple TBs received from a TX UE.

Observation 15: PSFCH format 0 is insufficient in term of resource utilization and PSFCH overhead, for the case of consecutive TB transmissions of a TX UE to a RX UE.

There are 5 PUCCH formats for HARQ-ACK reporting in NR as shown in Table 6.3.2.1-1 of TS 38.211 (copied below). Based on the length of time-domain symbols occupied by PUCCH formats, the PUCCH formats are classified into the following types:

- Short PUCCH: 1 to 2 symbols, PUCCH format 0 and PUCCH format 2;

- Long PUCCH: 4 to 14 symbols, PUCCH format 1, PUCCH format 3, and PUCCH format 4.

Based on the amount of information bits carried by PUCCH formats, the PUCCH formats are also classified into the following types:

- Small PUCCH: 1 to 2 bits, PUCCH format 0 and PUCCH format 1;

- Large PUCCH: more than 2 bits, PUCCH format 2, PUCCH format 3, and PUCCH format 4.

The short PUCCH format can be used to support faster HARQ response and the large PUCCH format can carry more information. Taking both latency and capacity into consideration, PUCCH format 2, which occupies 1 to 2 symbols in the time domain and 1 to 16 RBs in the frequency domain, can be reused in SL-U to support code-book based PSFCH transmission.

(Below is copied from TS 38.211) … Table 6.3.2.1-1: PUCCH formats. PUCCH format; Length in OFDM symbols; Number of bits. 0; 1 – 2; ≤2. 1; 4 – 14; ≤2. 2; 1 – 2; >2. 3; 4 – 14; >2. 4; 4 – 14; >2.

In NR Uu, the gNB can control the transmission timing by using the HARQ Feedback Timing Indicator field K1 in the DCI. The HARQ Feedback Timing Indicator field K1 indicates a timeslot offset between PDSCH data and HARQ feedback information sent by the UE. If the UE receives PDSCH data in timeslot n, the UE sends corresponding HARQ feedback information in timeslot n+K1. As shown in Figure 17, the UE needs to transmit HARQ-ACK feedback at slot 5 for previous downlink data from slot 0 to slot 3 at the same time by using HARQ codebook.

Figure 17 codebook based HARQ-ACK reporting in NR

Similarly, as shown in Figure 18, a RX UE received consecutive slots transmission from slot n to slot n+3 can perform HARQ-ACK reporting for TB1 ~ TB4 to the corresponding PSSCH slots at slot m. With such design, the PSFCH slots can be present in a sparser and flexible way, and thus saving AGC and GAP overhead compared to PSFCH format 0.

Figure 18 codebook based HARQ-ACK reporting in SL-U

Proposal 16:  Introduce new PSFCH format, to support codebook based SL HARQ-ACK feedback.

SL HARQ-ACK information multiplexed with PSSCH

In NR Uu (including NR-U), UCI can be multiplexed with PUSCH, which can significantly improve the efficiency for transmission of HARQ-ACK information. This kind of multiplexing is also beneficial for sidelink over unlicensed. When the responding UE has its own data to be transmitted, its HARQ-ACK information can be multiplexed with PSSCH within the same COT. This will save two-symbol overhead used for AGC and GAP compared to PSFCH format 0.

As shown in Figure 19, the HARQ-ACK information is conveyed by 2nd-stage SCI and multiplexed with PSSCH to improve spectrum efficiency.

Figure 19 SL HARQ-ACK information multiplexed with PSSCH

Proposal 17:  Support SL HARQ-ACK information to be multiplexed with PSSCH.

PSSCH-PSFCH Mapping relationship

In R16 NR-V, PSFCH format 0 is used to carry HARQ-ACK feedback to respond its associated PSSCH transmission. The mapping between PSSCH and PSFCH is fixed, and no explicit signaling over sidelink is needed to indicate such mapping. This is guaranteed by a periodic PSFCH resources configured at resource level, i.e. for all the UE, every PSSCH can be mapped the earliest PSFCH without confusion.

However, there are always the cases that the PSFCH occasion(s) are interrupted and the RX UE cannot perform HARQ-ACK report at the expected location. For example, as shown in Figure 20, due to processing time, for the last few PSSCH transmissions within the COT, their corresponding HARQ ACK feedback will be located outside the current COT. In such case, those HARQ-ACKs have to be reported in a follow-up COT, which may be occupied by non-sidelink device, resulting in HARQ-ACK reporting failure. In order to address such problem, there can be two potential ways:

- The TX UE can perform LBT to access channel and initiates a COT, and indicate the specific locations for HARQ-ACK reporting for those RX UE that have not yet provided HARQ-ACK information to their received PSSCH in previous COT. Thus, the RX UE can perform codebook-based HARQ feedback at the location indicated by TX UE.

- The RX UE can perform LBT to access channel and initiates a COT, and transmit SL HARQ-ACK information multiplexed with its data.  In this case, the RX UE needs to explicitly indicate specific DST_ID(s), HARQ process ID(s), NDI(s), HARQ-ACK information of previous received PSSCH to TX UE to avoid ambiguity.

Figure 20 PSFCH occasions interrupted by cross-RAT device

Observation 16: For SL-U, it’s possible the PSFCH occasion(s) corresponding to PSSCH are interrupted and the RX UE cannot perform HARQ-ACK report at the expected location.

Proposal 18: Both implicit and explicit PSSCH-PSFCH mapping are supported, FFS the applicable PSFCH format and scenarios.

OCB requirements

At RAN1#110, following agreements on PSFCH transmission to meet OCB requirements were made [4].

Agreement To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details. Agreement Regarding PSFCH transmission, at least the followings alternatives can be further studied - Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s) - Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts) - Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs - FFS details of above alternatives

PSFCH format 0 only occupies one PRB, it cannot meet OCB requirement. To satisfy OCB requirement, RB-based interlace is supported for 15 kHz and 30 kHz SCS. However, since one interlace occupies at least 10 PRBs, if different UEs use different interlaces, the PSFCH resource capacity would be very limited.

Take PSFCH periodicity=4 slots as an example, in 15 kHz, when 1 sub-channel equals 1 interlace, there are only 10 candidate PSFCH resource (10 sub-channels/ interlaces), and 4 slots * 10 sub-channels/interlaces = 40 candidate PSSCH transmission. Even if all these 40 PSSCH transmission are used for unicast, the PSFCH capacity is still very limited, and the situation gets worse when some of the transmission are used for groupcast option 2. To address such issue, one potential way is to introduce a common PSFCH interlace.

As shown in Figure 21, each UE transmits HARQ-ACK information on one PRB as in R16 NR-V, and additionally transmits on the common PSFCH interlace to meet OCB requirement. In such design, all the UEs will transmit on the same common PSFCH interlace (i.e. same PSFCH resource) so that the resource overhead is minimized, and their HARQ-ACK information are transmitted at different PRBs as in R16 NR-V, respectively.

Figure 21 Illustration of common interlace for PSFCH transmission

In order to improve capacity for PSFCH transmission, some companies also mentioned to introduce OCC in time domain or frequency domain. However, since there are only 2 symbols used for PSFCH transmission and one is used for AGC in R16/R17 slot structure, introducing OCC in time domain means more symbols for PSFCH transmission in a slot, which brings potential slot structure change and needs more resources for PSFCH transmission compared to R16/R17 design. Therefore, time-domain OCC should not be pursued.

When frequency domain OCC is considered, the higher OCC density, the higher detection ability required. When both cyclic shift and OCC are imposed to increase PSFCH capacity, the performance of PSFCH detection will decrease largely. There are only OCC-2/4 used in PUCCH format under the consideration of detection performance. Similarly, we think such constraint applies in SL-U.

Observation 17: When both cyclic shift and OCC are imposed to increase PSFCH capacity, the performance of PSFCH detection will decrease largely.

Table 1 below compares the capacity for PSFCH transmission among R16/R17 PSFCH design, Alt 1, Alt 2 without OCC and Alt2 with frequency domain OCC under 30 kHz SCS configuration. Assuming there are 50 PRBs used for PSFCH transmission and 5 sub-channels in one RB set (1 sub-channel = 1 interlace), and PSFCH periodicity is 4 slots.

For Alt2 with OCC, we assume OCC-4 in frequency domain is used for interlaced PSFCH transmission to increase capacity. Considering applying OCC on top of cyclic shift may decrease detection performance, we assume 2 pairs of cyclic shift is acceptable in Alt2-OCC.

From Table 1 below, it can be seen that the capacity of PSFCH transmission of Alt 1 (240) is slightly lower than that of R16/ R17 PSFCH design (300) and much higher than Alt 2 (30 and 40 respectively). In addition, for Alt 2 with or without OCC, it’s infeasible to support groupcast option 2 (see MAC spec below) since there are less than 3 PSFCH resource per PSSCH candidate, which means the group size should be smaller than 3 and thus too unrealistic.

In summary, Alt 1 should be adopted since it can both meet OCB requirements and ensure large capacity for PSFCH transmission.

(Below is copied from TS 38.321) 5> if HARQ feedback is enabled for groupcast: 6> if both a group size and a member ID are provided by upper layers and the group size is not greater than the number of candidate PSFCH resources associated with this sidelink grant: 7> select either positive-negative acknowledgement or negative-only acknowledgement.

Table 1 Capacity comparison between different schemes under 30 kHz SCS configuration

|  | PSFCH R16/R17 (one dedicated PRB) | PSFCH Alt 1 (one common interlace +one dedicated PRB) | PSFCH Alt 2 (w/o OCC) (one dedicated interlace) | PSFCH Alt2 (w/ OCC-4) (one interlace +frequency domain OCC) |
| --- | --- | --- | --- | --- |
| Candidate number of PSFCH transmissions in frequency domain | 50 | 40 | 5 | 5 |
| Total number of PSFCH resources in cyclic shift domain | 6 | 6 | 6 | 2 |
| Total number of PSFCH resources in frequency domain OCC | 0 | 0 | 0 | 4 |
| Total number of PSFCH resources in one PSFCH period | 50*6 =300 | 40*6 =240 | 5*6 =30 | 5*2*4 =40 |
| Number of PSFCH resources per PSSCH candidate Note: there are 4*5=20 PSSCH candidates in one PSFCH period | 300/20=15 | 240/20=12 | 30/20=1.5 | 40/20 = 2 |

Observation 18: If different UEs use different interlaces, the PSFCH resource capacity would be very limited.

Observation 19: The capacity for PSFCH transmission of Alt 1, i.e., one common interlace and one dedicate PRB, is much higher than Alt 2 without/with OCC.

Since there is no interlace structure in NR-U for 60 kHz SCS, additional scheme needs to be introduced to realize similar functionality as common interlace. One simple way is to define a bitmap for common PSFCH resource, e.g., “1”s in bitmap means such PRBs are used for common PSFCH transmission. In summary, the following proposal on PSFCH transmission is given.

Proposal 19: Regarding PSFCH transmission:

- For 15/30 kHz SCS: each PSFCH transmission occupies a common interlace and one dedicated PRB.

- For 60 kHz SCS: each PSFCH transmission occupies a common set of PRBs, which is given by (pre-)configured bitmap, and one dedicated PRB.

SL HARQ-ACK report on UL

On SL HARQ-ACK reporting to the gNB for Mode 1, PSFCH transmission may fail due to LBT failure. One simple way to address this issue is reusing scheme in R16 NR-V, where gNB does not receive feedback from the UE simply assumes the transmission between SL UEs fails.

(Below is copied from TS 38.213) … The UE generates a NACK when, due to prioritization, as described in clause 16.2.4, the UE does not receive PSFCH in any PSFCH reception occasion associated with a PSSCH transmission in a resource provided by a DCI format 3_0 or, for a configured grant, in a resource provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH transmission.

Proposal 20: For the case that LBT fails for PSFCH transmission, reuse R16 SL design on SL HARQ-ACK reporting to the gNB when no SL HARQ-ACK is received.

S-SSB

S-SSB resources

At RAN1#110, following agreement on S-SSB resource is made [4]. Related issues on additional candidate S-SSB occasions need to be discussed.

Agreement If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions - FFS the number and locations of additional candidate S-SSB occasions - FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior

In time domain, the S-SSB is transmitted every 160ms, and can be further configured to have multiple S-SSB slots within a period, depending on SCS. When multiple S-SSB slots are configured, the interval between two adjacent S-SSB slots is determined by NintervalS-SSB. Since R16/R17 S-SSB transmission occasions may be unavailable due to occupation from other-RAT UEs on unlicensed spectrum, additional S-SSB transmission is supported to increase opportunity when LBT is required to perform for S-SSB transmission.

Figure 22 Additional candidate S-SSB occasions

In order to avoid blind detection for additional candidate S-SSB occasions, these slots can be (pre-)configured by an offset to each Rel-16 S-SSB slots by a new RRC signaling. As shown in Figure 22, when the resource pool is configured with 2 S-SSB occasions per 160ms, there will be two additional S-SSB occasions accordingly with a (pre-)configured offset. When both Rel-16 and additional S-SSB occasion are enabled for every 160ms, only if a UE fails to access channel before a Rel-16 S-SSB occasion, it tries to perform LBT before the corresponding additional S-SSB occasion(s), and transmit S-SSB if LBT success. If a UE succeeds its S-SSB transmission at a Rel-16 S-SSB occasion, the additional slots reserved for corresponding S-SSB transmission can be used for PSCCH/PSSCH transmission to improve resource utilization.

Proposal 21: Additional S-SSB occasions belong to resource pool and are determined by a (pre-)configured offset with respect to Rel-16 S-SSB occasions

- When a UE fails to access channel before each Rel-16 S-SSB occasion, it performs LBT before corresponding additional S-SSB occasion for S-SSB transmission.

- When a UE already transmitted S-SSB at Rel-16 S-SSB occasions, the additional S-SSB occasion is not used for S-SSB transmission and can be used for PSCCH/PSSCH transmission.

OCB requirements

At RAN1#110, following agreement on S-SSB and synchronization was made [3].

Agreement For S-SSB and synchronization in SL-U: - No changes on R16 NR SL S-PSS/S-SSS sequence generation - Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS

In NR V2X, the S-SSB always occupies 11 common PRBs, which may not satisfy OCB requirements. Thus, how to satisfy OCB requirements needs further study.

One straightforward way is to use interlaced transmission for S-SSB. However, if using interlaced RB transmission for S-PSS/S-SSS, the total transmit power remains unchanged but the noise increases due to increased bandwidth, thus decreasing SNR and further decreasing synchronization performance. To avoid degradation of synchronization accuracy and SNR, we propose to apply the temporary exemption of OCB requirement [5] to S-PSS/S-SSS and use interlace RB for PBSCH to meet OCB requirements as shown in Figure 23.

For the case of 15 kHz SCS, the frequency domain of S-PSS/S-SSS is less than 2MHz, in order to meet temporary exemption, repeat top 7 REs of S-PSS/S-SSS sequence can be a straightforward way to meet requirements.

For the case of 60 kHz SCS, there is no interlace structure in NR-U. Since 80% of a LBT channel is equivalent to 22.2 RBs (80%*20MHz), one simple way is to span PSBCH to 23 contiguous RBs to meet OCB requirements and apply the temporary exemption of OCB requirement to S-PSS/S-SSS. Repetition of PSBCH is not preferred since the RBs occupied by one more repetition (2 *11RBs =22 RBs) still cannot meet OCB requirement (22.2 RBs), while the RBs occupied by two more repetition (3*11 RBs = 33 RBs) is larger than the RBs of a LBT channel.

(Below is copied from ETSI EN 301 893) … During a Channel Occupancy Time (COT), equipment may operate temporarily with an Occupied Channel Bandwidth of less than 80 % of its Nominal Channel Bandwidth with a minimum of 2 MHz.

Figure 23 Interlace structure for S-SSB

In addition, other options in RAN1#109-e agreement are not preferred for the following reasons.

- Option2 (S-SSB multiplexing with other SL transmissions in the same slot) is not preferred when contiguous RB-based transmission is used for S-SSB and interlace RB-based transmission is used for other signals due to the following reasons:

  - Power splitting will deteriorate the coverage and performance of S-SSB.

  - It seems in Option 2, S-SSB will use contiguous RB, and the FDMed PSCCH/PSSCH still use interlace RB. Such design looks complicated, and cause issues on resource usage, resource indication, interference, etc.

  - Considering different transmissions have different CAPCs and/or CPEs, RAN1 needs to further study issues occurred in channel access to ensure simultaneous transmissions of different signals.

- Option3 (Repetition of S-PSS/S-SSS/PSBCH in frequency domain): Simply repeating S-PSS/S-SSS/PSBCH in frequency domain results in high PAPR/CM of the signal due to the superposition of the time domain signals. In addition, unlike ZC sequences with zero autocorrelation, gold-sequence S-PSS/S-SSS cannot control PAPR/CM by cyclic shifts among repetitions.

- Option4 (S-PSS/S-SSS/PSBCH with wider bandwidth): It was agreed in last meeting that R16 NR SL S-PSS/S-SSS sequence generation cannot be changed. One potential way is repetition of S-PSS/S-SSS/PSBCH as option 3 which is not preferred for reasons mentioned above. As for other potential schemes, compared to the current S-SSB structure, a new structure of S-PSS/S-SSS/PSBCH with wider bandwidth will lead to a lot of standardization work and implementation complexity. In particular, for a given channel bandwidth, different structures may need to be designed for different SCSs.

Proposal 22:  To meet OCB and PSD requirement for S-SSB transmission, for both R16/R17 NR SL S-SSB slots and additional S-SSB slots：

- For S-PSS/S-SSS

    - Apply OCB exemption, where for the case of 15 kHz SCS, repeat top 7 REs of S-PSS/S-SSS to meet minimum of 2 MHz requirement.

- For PSBCH

  - For 15 kHz and 30 kHz SCS, support interlace RB-based transmission for PSBCH

- For 60 kHz SCS, span PSBCH to 23 contiguous RBs to meet OCB requirements.

Conclusions

This contribution includes our considerations on physical channel designs for SL-U, including supporting both contiguous RB and interlaced RB, adopting necessary changes on slot structure to overcome issues due to channel access. In addition, some considerations on physical channels, such as PSCCH, PSSCH, PSFCH (including PSFCH resources and mapping to associate PSSCH), S-SSB (PSBCH) for sidelink operation over unlicensed spectrum are also discussed.

We provide following observations and proposals:

Observation 1: If a resource pool includes sub-set of contiguous or interlaced PRBs of one RB set, it may result in wasting resource.

Observation 2: The way that several UEs simultaneously access a LBT channel via their own FDMed resource pool is similar to that several UEs simultaneously access the same resource pool with one LBT channel bandwidth.

Observation 3: A resource pool that only includes sub-set of contiguous of PRBs of one RB set may violate OCB requirements.

Observation 4: If the bitmap of a resource pool is not set to all “1”s, it will result in slot-level gaps between consecutive transmissions and difficult for the UE to maintain a COT.

Observation 5: Since LBT can be successful at any point within a slot, starting symbol for PSCCH/PSSCH transmission might be missed if there is only one starting symbol in a slot, which degrades SL-U performance.

Observation 6: Introducing more starting symbols within a slot for the PSCCH/PSSCH transmission is beneficial to provide more channel access chances, but at the cost of increasing UE complexity.

- Two starting symbols within a slot for the PSCCH/PSSCH transmission is a good balance between performance and UE complexity, and it can be regarded as defining two sets of starting symbol with different symbol length in a slot, thus having no additional specification impact in terms of, e.g. slot structure, DRMS patterns, etc.

Observation 7: When a UE receives COT sharing information and knows the COT structure, the UE does not need to search for multiple starting symbols in slots within the COT.

Observation 8: Compared with legacy slot structure, i.e., one starting symbol within a slot for PSCCH/PSSCH transmission, introducing two starting symbols can largely increase UPT, especially for traffic with low latency requirement such as XR.

Observation 9: When 1 sub-channel is defined as 1 interlace, a UE who needs K interlaces can be satisfied via K sub-channels.

Observation 10: For the case of 30 kHz SCS, there are only 5 interlaces per BWP, defining 1 sub-channel equals K interlaces has the following problems:

- When K is configured to 2, 3 or 4, there will be new resource indication issue or resource wastage since 5 is not an integer multiple of 2, 3 or 4.

- When K is configured to 5, a UE shall use all the RBs of one RB set even with small data size, resulting in resource inefficiency.

Observation 11: If the sub-channel is confined within one RB set, due to the existence of guardband PRBs, the number of PRBs within a RB set may not be integer multiple of the number of PRBs of 1 sub-channel. Thus, the sub-channel indexing and usage will become very complicated.

Observation 12: Two-level resource indication, i.e. X+Y, is introduced in NR-U, where X bits indicate a set of allocated interlace indices and Y bits indicate a set of contiguously allocated RB set(s).

Observation 13: Due to the existence of intra-cell guard band PRB, for both IRB and contiguous RB, the available PRBs in different sub-channels may be different, resulting in different number of PRBs for initial and retransmission of same TB.

Observation 14: For PSFCH resource, Rel-16 (pre-)configured way is not suitable for unlicensed spectrum because:

- LBT may fail before a (pre-)configured PSFCH resource even with multiple candidate PSFCH resources, so that the UE is unable to transmit HARQ ACK on these resources.

- It’s possible that no UE transmits HARQ-ACK on the (pre-)configured PSFCH resource within a COT, such that the current COT may be lost.

- No need to have dense or any PSFCH resources in a COT, e.g., remaining PDB is large so that no need for frequent HARQ-ACK feedback, blind retransmissions, etc.

Observation 15: PSFCH format 0 is insufficient in term of resource utilization and PSFCH overhead, for the case of consecutive TB transmissions of a TX UE to a RX UE.

Observation 16: For SL-U, it’s possible the PSFCH occasion(s) corresponding to PSSCH are interrupted and the RX UE cannot perform HARQ-ACK report at the expected location.

Observation 17: When both cyclic shift and OCC are imposed to increase PSFCH capacity, the performance of PSFCH detection will decrease largely.

Observation 18: If different UEs use different interlaces, the PSFCH resource capacity would be very limited.

Observation 19: The capacity for PSFCH transmission of Alt 1, i.e., one common interlace and one dedicate PRB, is much higher than Alt 2 without/with OCC.

Proposal 1: Do not support one SL resource pool includes sub-set of PRBs of one RB set.

Proposal 2: SL-U uses PRBs within intra-cell guard band in the same way as NR-U, i.e., PRBs within intra-cell guard band of two adjacent RB sets can only be used when UE’s LBT is successful on these two LBT channels, and the UE uses these two RB sets simultaneously for transmission.

Proposal 3: In SL-U, do not consider Uu TDD configuration when determining slots belonging to a SL resource pool.

Proposal 4: Set the value of bitmap to all “1”s to avoid slot-level transmission gaps in a resource pool.

Proposal 5: R16/17 NR SL S-SSB slots are excluded from resource pool.

Proposal 6: Support two starting symbols within a slot for the PSCCH/PSSCH transmissions:

- The 1st starting symbol is fixed to symbol 0, and the corresponding length for PSCCH/PSSCH transmissions in a slot is 14 symbols for NCP and 12 symbols for ECP.

- The 2nd starting symbol is M2, and the corresponding length for PSCCH/PSSCH transmissions in a slot is 14-M2 symbols for NCP and 12-M2 symbols for ECP.

  - FFS value of M2.

- Note: the 1st symbol for PSCCH/PSSCH transmission is used for AGC purpose.

Proposal 7: When multi-consecutive slots transmission is performed, the gap symbol between two adjacent slots can be used for PSSCH transmission.

- FFS details, e.g., applicable scenarios, etc.

Proposal 8: For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- One sub-channel equals to one interlace and spans across all RB sets belonging to a resource pool.

- Note: the minimum frequency resource size for one PSSCH transmission is one sub-channel in one RB set subject to resource indication design.

Proposal 9: For sub-channel index in SL-U:

- For contiguous RB-based transmission, reuse Rel-16 NR SL subchannel indexing.

- For interlace RB-based transmission, the first sub-channel begins from the lowest RB of the resource pool and the sub-channels are indexed in ascending order along with interlace.

Proposal 10: For PSCCH and PSSCH in SL-U, for both contiguous RB-based and interlaced RB-based transmission:

- PSCCH is always transmitted within one sub-channel

- R16 NR-V PSCCH and PSSCH multiplexing is reused, i.e., PSCCH locates in the sub-channel with the lowest index of the sub-channel(s) of corresponding PSSCH

Proposal 11: For contiguous RB-based PSCCH/PSSCH transmission, both R16 NR SL TRIV and FRIV are reused for resource indication.

Proposal 12: For interlaced based PSCCH/PSSCH transmission,

- For time domain indication, R16 NR SL TRIV is reused.

- For frequency domain indication,

  - R16 NR SL FRIV is reused to indicate sub-channel index.

  - Additional Y bits, which depends on the number of RB sets within the resource pool, are introduced to indicate a set of contiguously allocated RB set(s) as NR-U.

Proposal 13: Introduce reference number of PRBs to determine TBS to address the issue of unequal number of PRBs in the allocated frequency resource for (re)transmission of a same TB.

Proposal 14: For the case where the number of PRBs within a resource pool is not an integer multiple of the sub-channel size, R16 NR SL design is reused, i.e., edge PRBs that does not belong to a sub-channel are not used.

Proposal 15: Support COT initiating UE dynamically indicates PSFCH resources within a COT.

Proposal 16:  Introduce new PSFCH format, to support codebook based SL HARQ-ACK feedback.

Proposal 17:  Support SL HARQ-ACK information to be multiplexed with PSSCH.

Proposal 18: Both implicit and explicit PSSCH-PSFCH mapping are supported, FFS the applicable PSFCH format and scenarios.

Proposal 19: Regarding PSFCH transmission:

- For 15/30 kHz SCS: each PSFCH transmission occupies a common interlace and one dedicated PRB.

- For 60 kHz SCS: each PSFCH transmission occupies a common set of PRBs, which is given by (pre-)configured bitmap, and one dedicated PRB.

Proposal 20: For the case that LBT fails for PSFCH transmission, reuse R16 SL design on SL HARQ-ACK reporting to the gNB when no SL HARQ-ACK is received.

Proposal 21: Additional S-SSB occasions belong to resource pool and are determined by a (pre-)configured offset with respect to Rel-16 S-SSB occasions

- When a UE fails to access channel before each Rel-16 S-SSB occasion, it performs LBT before corresponding additional S-SSB occasion for S-SSB transmission.

- When a UE already transmitted S-SSB at Rel-16 S-SSB occasions, the additional S-SSB occasion is not used for S-SSB transmission and can be used for PSCCH/PSSCH transmission.

Proposal 22:  To meet OCB and PSD requirement for S-SSB transmission, for both R16/R17 NR SL S-SSB slots and additional S-SSB slots：

- For S-PSS/S-SSS

    - Apply OCB exemption, where for the case of 15 kHz SCS, repeat top 7 REs of S-PSS/S-SSS to meet minimum of 2 MHz requirement.

- For PSBCH

  - For 15 kHz and 30 kHz SCS, support interlace RB-based transmission for PSBCH

- For 60 kHz SCS, span PSBCH to 23 contiguous RBs to meet OCB requirements.

References

- RP-213678, “New WID on NR sidelink evolution”, OPPO, LG Electronics, RAN#94e, December 6 - 17, 2021.

- RP-221798, “WID revision: NR sidelink evolution”, OPPO, RAN#96, Budapest, Hungary, June 06 - 09, 2022

- Draft Report of 3GPP TSG RAN WG1 #109-e.

- Draft Report of 3GPP TSG RAN WG1 #110.

- ETSI EN 301 893 V2.1.1, “5 GHz RLAN; Harmonized Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”, May 2017.

- R1-2208448, “Channel access mechanism and resource allocation for sidelink operation over unlicensed spectrum”, Huawei, HiSilicon, RAN1#110bis-e, October, 2022.

- Appendix

Table 1: Summary of simulation assumptions for Indoor SL-U

| Carrier frequency | 5GHz |
| --- | --- |
| Resource pool bandwidth | 80 MHz |
| SCS | 30KHz |
| UE dropping model | Indoor layout and UE dropping model with 3 clusters and each with 5 UEs. Each cluster is a circle, with a central point and radius Rmax = 15 and Rmin = 5, where UEs are randomly dropped within the circle. No overlapping among the 3 clusters. |
| Resource allocation schemes | SL-U: - Enhanced R16 mode 2 (TBs are selected consecutively in a COT) + LBT (performed before transmission). Wi-Fi: IEEE802.11ac with parameters as per TR36.889 |
| Traffic model | Mixed traffic mode for SL-U: - XR: cloud gaming traffic model (8Mbps) and pose/control traffic model defined in TR 38.838. - FTP: FTP 3 as in TR 38.889 λ= 4 Single traffic model for Wi-Fi: - FTP: FTP 3 as in TR 38.889 λ= 2.5 |
| Interference model | 3APs + 1 STAs, , where each AP serves 5 STAs Same flows and load as SL-U. |
| Channel model | InH Mixed Office model |
| UE antenna model | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| AP antenna array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna array configuration | (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| AP Tx power | 23 dBm (total across all TX antennas) |
| SL-U UE | 18 dBm (total across all TX antennas) |
| AP antenna gain | 0 dBi |
| UE/STA antenna gain | 0 dBi |
| AP noise figure | 5 dB |
| UE/STA receiver noise figure | 9 dB |