3GPP TSG RAN WG1 Meeting #110-bis-e                                                                R1-2208524

e-Meeting, Oct 10th – 19th, 2022

Source:              ZTE Corporation

Title:                   Discussion on other aspects for AI beam management

Agenda Item:     9.2.3.2

Document for:   Discussion and Decision

- Introduction

In RAN1#109-e, a number of productive consensuses have been reached on potential specification enhancements for AI/ML-based beam management, such as sub-use case selection and AI/ML deployment [1]. In addition, it has been agreed that the assessment of specification impacts is sub-use-case specific and can be discussed after the sub-use case is finalized. In RAN1#110, relevant conclusions made in RAN1#109-e were further updated with some modifications on wording and some high-level aspects of potential spec impacts were reached [2], which are listed in the following. At the current stage, there are still quite a few options for sub-use cases and associated AI inputs/outputs, making it difficult to concretize and converge the discussion of specification impacts. Therefore, in this contribution, we will make some recommendations on sub-use case details and analyse potential specification impacts.

Agreement: At least for the sub use case BM-Case1 and BM-Case2, support both Alt.1 and Alt.2 for the study of AI/ML model training: - Alt.1: AI/ML model training at NW side; - Alt.2: AI/ML model training at UE side. - Note: Whether it is online or offline training is a separate discussion. Agreement: For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) - Note1: DL Rx beam prediction may or may not have spec impact Agreement: For the sub use case BM-Case1, support the following alternatives for further study: - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A) - Alt.2: Set B is a subset of Set A - Note1: Set A is for DL beam prediction and Set B is for DL beam measurement. - Note2: The beam patterns of Set A and Set B can be clarified by the companies. Agreement: For the sub use case BM-Case2, further study the following alternatives: - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A) - Alt.2: Set B is a subset of Set A (Set A and Set B are not the same) - Alt.3: Set A and Set B are the same - Note1: The beam pattern of Set A and Set B can be clarified by the companies. Agreement: Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output: - Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure) - E.g., N predicted beams can be the top-N predicted beams - Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - FFS: details of Beam angle(s) - FFS: how to select the N DL Tx and/or Rx beams (e.g., L1-RSRP higher than a threshold, a sum probability of being the best beams higher than a threshold, RSRP corresponding to the expected Tx and/or Rx beam direction(s)) - Note1: It is up to companies to provide other alternative(s) - Note2: Beam ID is only used for discussion purpose - Note3: All the outputs are “nominal” and only for discussion purpose - Note4: Values of N is up to each company. - Note5: All of the outputs in the above alternatives may vary based on whether the AI/ML model inference is at UE side or gNB side. - Note 6: The Top-N beam IDs might have been derived via post-processing of the ML-model output Agreement: For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: - Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals - Content/type of the collected data - Other aspect(s) is not precluded Agreement: In order to facilitate the AI/ML model inference, study the following aspects as a starting point: - Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting - Enhanced or new signaling for measurement configuration/triggering - Signaling of assistance information (if applicable) - Other aspect(s) is not precluded Agreement: Regarding the model monitoring for BM-Case1 and BM-Case2, to investigate specification impacts from the following aspects - Performance metric(s) - Benchmark/reference for the performance comparison - Signaling/configuration/measurement/report for model monitoring, e.g., signaling aspects related to assistance information (if supported), Reference signals - Other aspect(s) is not precluded

- AI/ML based beam management

  - Model training and inference at UE/NW side

A consensus was reached in RAN1#109-e that for selected sub use cases BM-Case1 and BM-Case2, both NW-side and UE-side AI/ML model inference are supported for further study. In addition, in RAN1#110, companies also agreed that AI/ML model training can be performed at the NW side or at the UE side. However, it is currently unclear whether the AI/ML model training and inference shall be at the same node or different nodes. The following proposal was provided at the RAN1#110 meeting.

Proposal 2.1.1-2d: For the sub use case BM-Case1 and BM-Case2, at least support Alt.1 and Alt.2 for AI/ML model training and inference for further study: - Alt.1. AI/ML model training and inference at NW side - Alt.2. AI/ML model training and inference at UE side - FFS: Alt.3. AI/ML model training at NW side, AI/ML model inference at UE side

Different alternatives in this proposal correspond to different levels of collaboration as agreed in agenda 9.2.1, where Alt.1 and Alt.2 correspond to collaboration level y, and Alt.3 corresponds to collaboration level z. If all collaboration levels can be supported in agenda 9.2.1, all alternatives in this proposal should be kept open since obvious advantages can be observed for all alternatives. Specifically, Alt.1 and Alt.2 are relatively straightforward model deployment methods that can avoid proprietary model disclosure issues and additional model-related signaling interactions. In Alt.3, model training and model inference are performed on different nodes, which means that the well-trained model needs to be transferred between different nodes.

Alt.3 outperforms Alt.1 and Alt.2 in terms of reporting overhead or beam prediction performance, despite the potential risk of proprietary AI/ML model disclosure. Specifically, compared with the NW-side model inference in Alt.1, the UE doesn’t need to feedback much measured beam quality information to gNB as model inputs, and thus the reporting overhead associated with the UE-side model inference in Alt.3 can be significantly reduced. Compared with the UE-side model training in Alt.2, the NW-side model training in Alt.3 could benefit from the powerful computing power and sufficient storage resources of gNB. Besides, since the channel environment or antenna configuration of different cells may be different, it is difficult to require the model trained on the UE side to work well in different cells, resulting in beam prediction performance loss or frequently model switching/update. However, if the AI/ML model is site-specific trained and transferred to UEs accessed to the cell, the beam prediction performance can be guaranteed with better environmental adaptability. Therefore, if model transfer is supported in agenda 9.2.1, both Alt.1, Alt.2, and Alt.3 can be supported for further study with potential applicability in different scenarios.

Observation 1: For AI/ML model training and inference, Alt.3 outperforms Alt.1 and Alt.2 in terms of reporting overhead and/or beam prediction performance.

Proposal 1: If model transfer is supported in agenda 9.2.1, both Alt.1, Alt.2, and Alt.3 can be supported for further study with potential applicability in different scenarios.

  - Selection of sub use cases

Table I categorizes the sub use cases for AI/ML-based beam management with different inputs, different outputs, different functionalities and so on. All of them have been initially discussed in RAN1#109-e. Besides, BM-Case1 and BM-Case2 are supported by majority companies while other sub use cases are supported by a limited number of companies. In RAN1#110, whether to support any other sub use cases was further discussed but no agreement was achieved.

Table I: Sub use cases and categories

| Category | Sub use case |
| --- | --- |
| Cat1: Spatial-domain DL beam prediction | BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams |
|  | BM-Case3: Beam prediction for higher frequency band (e.g., a band in FR2) based on measurement results of lower frequency band(s) (e.g., a band in FR1) |
|  | BM-Case4: Beam prediction based on UE positioning/trajectory |
|  | BM-Case6: Spatial-domain UL beam prediction for Set A of beams based on measurement results of Set B of beams |
|  | BM-Case9: Joint DL/UL beam pair link prediction |
| Cat2: Time-domain DL beam prediction | BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams |
| Cat3: Others | BM-Case7: beam measurement feedback compression |
|  | BM-Case8: The beam-specific parameter optimization |

Since the time unit of this study item is limited, we suggest to focus on the sub use cases BM-Case1 and BM-Case2 in Rel-18, and other sub use cases can be discussed until solid AI framework has been made in Rel-18. Besides, we also notice some potential drawbacks of other sub use cases, which are detailed in the following.

Regarding the sub-use cases BM-Case6 and BM-Case9, they actually share a similar mechanism with BM-Case1. That is, the optimal beam or beam pair is predicted based on measurement results of partial beams, where deep learning based super-resolution technology is utilized to extract the spatial channel correlations. Therefore, we can focus on BM-Case1 first and try to see if the same framework can be applicable to sub-use cases BM-Case6 and BM-Case9 later.

Regarding the sub-use case BM-Case3, it has strong dependency on the channel information across different frequency ranges. Whether it’s reliable enough should be further studied since different frequency ranges generally have quite different channel characteristics. For the sub-use case BM-Case4, it may be sensitive to acquire movement information of surrounding objects and the current specification doesn’t support UE position to be acquired by NG-RAN (e.g., gNB) node due to privacy issue. Instead, it is more appropriate to use positioning-related measurement or trajectory as an assistance information for temporal beam prediction [3][4]. In this case, we can combine this study with BM-Case2 under the framework of temporal DL beam prediction.

For the sub-use case BM-Case7, whether beam reporting overhead is a big problem should be carefully discussed before this case is selected as a representative use case. At least when the AI/ML model is deployed on the UE side, the UE may only need to report the optimal beam information to the gNB without additional compression related operation, where the associated reporting overhead is acceptable. For the sub use case BM-Case8, compared with predefined (e.g., DFT) codebooks, the codebook construction or beam-specific parameter optimization based on channel characteristics leads to additional complexity and is hard to be aligned among different companies.

Proposal 2: Since the time unit of this study item is limited, we suggest to focus on the sub use cases BM-Case1 and BM-Case2 in Rel-18, and other sub use cases can be discussed until solid AI framework has been made in Rel-18.

  - Spatial-domain DL beam prediction

Spatial-domain beam prediction aims at predicting the optimal beam or beam pair based on measurement results of partial beams or wide beams by utilizing the AI/ML technology. For an AI/ML-based beam prediction model, two basic factors are the input and output. Thus, in order to design a model of a specific sub use case, the input and output need to be clearly defined. Besides, the construction of beam sets for AI/ML input and output also needs to be further refined. In the following, we will elaborate our views on beam set construction, AI input, and AI output, respectively.

Beam set construction:

In RAN1#110, the following agreement regarding the beam set construction was obtained for progress. Wherein, the beam sets for measurement and prediction were considered to be constructed with the same or different beam width.

For the sub use case BM-Case1, support the following alternatives for further study: - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A) - Alt.2: Set B is a subset of Set A - Note1: Set A is for DL beam prediction and Set B is for DL beam measurement. - Note2: The beam patterns of Set A and Set B can be clarified by the companies.

As far as we are concerned, both the hierarchical based method in Alt.1 and the sub-sampling based method in Alt.2 can be considered for spatial domain beam prediction at the early stage of the SI, and down selection may be needed when evaluation results are available. In the sub-sampling based method in Alt.2, the beam sets for measurement and for prediction originate from the same codebook for beamforming. Therefore, the implementation is relatively simple. In contrast, different codebooks may be used in the hierarchical based method in Alt.1. Different methods of forming wide and narrow beams may have a great impact on AI inference performance and the quality of existing communication services. Nevertheless, Alt.1 may match more with the current spec that SSB (wide beams) have to be sent anyway.

In the sub-sampling based method, measurement results on partial beams are used as AI input to predict the beam quality information of the whole beam space. Obviously, the number of beams for measurement (i.e., set B) and for prediction (i.e., set A) is related to the trade-off between inference performance and RS overhead for beam measurement. Specifically, the more beams are required for measurement, the higher would be the RS overhead of beam measurement. At the same time, more channel correlation information can be extracted by the AI model, resulting in a better inference performance. An ideal approach may be to dynamically learn or adjust the number of beams for measurement according to requirements for beam prediction performance. As for the beam pattern for measurement, the training samples required for a fixed beam pattern are relatively small and a stable performance gain can be obtained compared with a random beam pattern [5]. For the case of the random beam pattern being used, each beam may need to be accompanied by additional ID information, resulting in potential standardization efforts.

Observation 2: The number of beams for measurement (i.e., set B) and for prediction (i.e., set A) is related to the trade-off between inference performance and RS overhead for beam measurement.

Proposal 3: The sub-sampling based method in Alt.2 can serve as a starting point for the study of spatial domain beam prediction.

Proposal 4: The association in reference signals between two sets with different beam settings need to be further studied.

AI/ML input:

Many alternatives for AI input were proposed and the following conclusion was obtained in RAN1#109-e.

Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input: - Alt.1: Only L1-RSRP measurement based on Set B - Alt.2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companions in the discussion: Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc. - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: CIR based on Set B - Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose.

The down selection of all alternatives is heavily dependent on evaluation results. For Alt.1 and Alt.4, the necessity of using beam ID as part of the AI input is directly related to the adopted beam pattern. If the beam set B for measurement uses a fixed beam pattern, only the corresponding measured RSRP needs to be input to the AI model, resulting in relatively less standardization work. However, if the beam set B for measurement uses a random beam pattern, it may be critical to include the corresponding beam ID as part of the model input to improve the inference accuracy. Wherein, the beam ID is utilized to identify the position of the corresponding beam in the beam space.

For Alt.2, it is intuitively beneficial to have more available assistance information related to beam shape, UE location, etc., as AI input to improve the beam prediction accuracy. However, more assistance information also results in more complex neural networks and training overhead. According to simulation results provided in our accompanied contribution [5], a fairly good beam prediction performance can be obtained by using measurement results of the beam set B and/or beam IDs without other assistance information. Moreover, much of the assistance information mentioned in Alt.2 is proprietary information of the gNB or UE, which may be difficult to be obtained and shared with another vendor. Therefore, focusing the discussion on Alt.1 and Alt.4 would be a good starting point.

Observation 3: Much of the assistance information mentioned in Alt.2 is proprietary information of the gNB or UE, which may be difficult to be obtained and shared with another vendor.

Proposal 5: Focusing the discussion on Alt.1 and Alt.4 with only L1-RSRP measurement and the corresponding beam ID being taken into account for the AI input would be a good starting point.

AI/ML output:

The following proposal was proposed in RAN1#110, which presented a list of detailed output alternatives for further study.

Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output: - Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure) - E.g., N predicted beams can be the top-N predicted beams - Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - FFS: details of Beam angle(s) - FFS: how to select the N DL Tx and/or Rx beams (e.g., L1-RSRP higher than a threshold, a sum probability of being the best beams higher than a threshold, RSRP corresponding to the expected Tx and/or Rx beam direction(s)) - Note1: It is up to companies to provide other alternative(s) - Note2: Beam ID is only used for discussion purpose - Note3: All the outputs are “nominal” and only for discussion purpose - Note4: Values of N is up to each company. - Note5: All of the outputs in the above alternatives may vary based on whether the AI/ML model inference is at UE side or gNB side. - Note 6: The Top-N beam IDs might have been derived via post-processing of the ML-model output

For beam management, the feedback information includes SSBRI/CRI and L1-RSRP in current specification, which represent the beam indicator and beam quality information, respectively. In Alt.1, only the beam ID and the predicted RSRP are considered as the AI output. Therefore, Alt.1 can provide better standard compatibility and less additional standardization effort. Moreover, if the deployed AI model can predict the beam quality information of all beams, the beam ID can be obtained implicitly from the data structure of the model output vector or matrix, thus avoiding to explicitly define the concept of beam ID. In Alt.2, specific definition of the other information is needed with further evaluation results being provided by the proponents. In Alt.3, the beam angle is included as part of the model output, which is related to the beam implementation issue and hard to be aligned among companies. Besides, in the current specification, beam indication is performed through the configured TCI state. It is also unclear how to establish a corresponding relationship between the output beam angle and the TCI state.

Observation 4: Alt.1 can provide better standard compatibility and less additional standardization effort since only beam ID and the predicted RSRP are considered as the AI output.

Proposal 6: Focus the discussion on Alt.1 as a starting point. Alt.3 can be postponed until the relationship between output beam angle and TCI state is clear.

  - Temporal DL beam prediction

Temporal DL beam prediction aims at predicting future beam quality based on historic measurement results. Just like the spatial-domain beam prediction, we will further elaborate our views on beam set construction, AI input, and AI output respectively in the following.

Beam set constructions:

In RAN1#109-e and RAN1#110, most of the companies mentioned that there was no need to define the connection between the beam set for prediction and the beam set for measurement. However, some companies thought that spatial-temporal beam prediction should not be precluded with the example of using multiple past SSB measurement information to predict best CSI-RS ID, as an illustrative use case. Therefore, the following agreement was obtained in RAN1#110 for further study.

For the sub use case BM-Case2, further study the following alternatives: - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A) - Alt.2: Set B is a subset of Set A (Set A and Set B are not the same) - Alt.3: Set A and Set B are the same - Note1: The beam pattern of Set A and Set B can be clarified by the companies.

For Alt.1 and Alt.2, the beam sets used for measurement and prediction are different in terms of beam widths and/or number of beams. Essentially, the temporal beam prediction in Alt.1 and Alt.2 mixes two different AI models, i.e., an estimation model and a prediction model. The estimation model aims at inferring high-resolution beam quality information based on low-resolution beam measurement information. The prediction model aims at inferring beam quality information of future time instances based on measurement results of past time instances. Wherein, the beam set for measurement of each past measurement instance can be obtained based on a fixed or random beam pattern, or even by learning the beam space information at the last moment. The specific construction of the beam set for measurement will have a great impact on the beam training overhead, model complexity and inference performance.

For Alt.3, the beam set for measurement is the same as the beam set for prediction. Deep learning algorithms such as RNN can be used to extract the time correlation information of the channel to predict the future beam quality. Obviously, Alt.3 is the most straightforward way to implement the temporal beam prediction compared to the other alternatives. Compared with Alt.3, the beam prediction method in Alt.1 and Alt.2 can further reduce the RS overhead for beam measurement. However, in Alt.1 and Alt.2, the selection of the beam set of each past measurement instance is ambiguous and difficult to be aligned across companies. Besides, the difference between the beam set for measurement and the beam set for prediction (like wide beam set B and narrow beam set A) may make it hard for beam reporting and indication.

Consider that Alt.1 and Alt.2 are actually a hybrid method of spatial domain beam prediction and temporal beam prediction. In other word, if the beam set for measurement and the beam set for prediction are different, the spatial domain beam prediction algorithm may be an essential precondition for the temporal beam prediction study in Alt.1 and Alt.2. Thus, Alt.3 can be used as a benchmark, while Alt.1 and Alt.2 are deferred until the evaluation of the spatial domain beam prediction in BM-Case1 has achieved sufficient progress.

Observation 5: The specific construction of the beam set for measurement will have a great impact on the beam training overhead, model complexity and inference performance.

Observation 6: If the beam set for measurement and the beam set for prediction are different, the spatial domain beam prediction algorithm may be an essential precondition for the temporal beam prediction study.

Proposal 7: Regarding the beam set construction, Alt.3 can be used as a benchmark, while Alt.1 and Alt.2 are deferred until the evaluation of the spatial domain beam prediction in BM-Case1 has achieved sufficient progress.

AI/ML input and output:

Regarding the AI/ML input and output for the temporal beam prediction, many kinds of assistance information and different alternatives have been proposed in RAN1#110 and up to companies to explore all possible options. As with the spatial domain beam prediction above, focusing the AI input and output on the measured RSRP and/or beam ID would be a good starting point. The measured RSRP and beam ID represent the beam quality and beam indicator, respectively, where the beam ID can be implicitly indicated by the RS index or TCI state. In this way, the standardization workload and AI model complexity would be relatively low. Incorporating assistance information such as positioning-related measurements as part of AI input may have potential to improve the beam prediction accuracy, depending on further evaluation results. Meanwhile, as mentioned above, some assistance information is proprietary, which is difficult to be shared with another vendor. Therefore, for further performance evaluation and to reduce the standardization workload, the L1-RSRP measurement based on set B can be used as the AI input and the predicted L1-RSRP of DL Tx and/or Rx beams can be used as the AI output.

Proposal 8: For temporal domain beam prediction, focusing the AI input and output on measured RSRP and/or beam ID would be a good starting point, in which case the standardization workload and AI model complexity would be relatively low.

- Potential specification impacts

Regarding the potential spec impacts, it is heavily dependent on the specific sub use cases. Based on the above discussion of the selected sub use cases, we will elaborate our initial view on potential specification enhancements.

Enhanced beam reporting

NW-side beam prediction AI/ML models: If the AI model inference is carried out at the gNB side, sufficient beam measurement results are required as model input. To this end, UE needs to report a large amount of beam measurement information to gNB. Therefore, enhanced beam reporting mechanisms need to be studied, such as further screening, compression, and reporting of the beam measurement results, so as to balance the beam prediction performance and beam reporting overhead. Meanwhile, for L1-RSRP reporting in Rel-17, the maximum RSRP and the differential RSRP are respectively quantized into 7-bit and 4-bit payloads. This quantization method leads to errors in beam reporting. For AI-based beam prediction, the impact of quantization errors on beam prediction accuracy and how to improve the beam reporting accuracy also need further evaluation.

UE-side beam prediction AI/ML models: The deployed AI model on the UE side can extract the spatial correlation of the channel through deep learning-based super-resolution technology, and infers all beam quality based on measurement results on partial narrow beams or wide beams. However, the deployed AI model may be completely transparent to the gNB, which is trained by the UE itself or downloaded directly from a third party. In this case, signaling methods need to be investigated to enable recommendation, configuration, and indication of the beam set for measurement. Consider that the beam set for measurement and the beam set for prediction may be different. Therefore, a mapping relationship between the two sets needs to be established to achieve mutual indication. For example, the UE may report a virtual RS index that implicitly indicates the optimal beam inferred by the deployed AI model at the UE side.

Proposal 9: For NW-side beam prediction AI/ML models, enhanced beam reporting mechanisms such as further screening, compression, and reporting of the beam measurement results need to be studied so as to balance the beam prediction performance and beam reporting overhead.

Proposal 10: For UE-side beam prediction AI/ML models, signaling methods need to be investigated to enable recommendation, configuration, and indication of the beam set for measurement.

Flexible resource configuration and beam indication

In traditional beam management methods, the gNB would sweep all beams at the codebook by sending periodic RS resource sets. In the AI/ML based spatial domain and temporal beam prediction, the gNB only needs to sweep a small range of beams according to the Top-K beams inferred by AI. Therefore, existing resource configuration methods can be enhanced by more flexibly triggering or activating arbitrary resources in a certain resource set. Each resource or resource set can be associated with a time stamp that depicts the effective time of corresponding parameters. Furthermore, considering that the optimal beam at multiple future time instances can be predicted in advance with the AL/ML based temporal beam prediction method, more flexible beam indication and subsequent beam switching methods can be studied. For instance, a list of TCI states and corresponding time stamps can be indicated by the same signaling to be applied in future time instances.

Proposal 11: Study enhanced resource configuration and beam indication mechanism if more flexible triggering or activating approaches are utilized.

Assistance information exchange

In the spatial domain beam prediction and temporal beam prediction, assistance information such as beam direction or positioning-related measurement are used as candidates for AI model input. Considering that the beam implementation information in the existing protocol is completely transparent between gNB and UE. Therefore, enhanced resource configuration and reporting mechanisms need to be investigated to facilitate the exchange of assistance information, which can be either implicit or explicit. More specifically, if the AI inference is performed on the gNB side, the UE may need to feed back receive beam-related information to the gNB, which may include beam ID, beam direction, beam pattern, and so on. Similarly, if the AI  inference is performed on the UE side, the gNB may need to indicate beam-related information in transmission side to the UE. Furthermore, the exchanged assistance information may include indication information related to model activation/deactivation and switching, depending on which side the AI model is deployed on.

Proposal 12: Enhanced resource configuration and reporting mechanisms need to be investigated to facilitate the exchange of assistance information, which can be either implicit or explicit.

Dedicated configuration for Tx-Rx beam pair prediction

P1, P2, and P3 are a set of procedures to acquire and maintain a set of TRP(s) and/or UE beams that can be used for DL and UL transmission/reception, which correspond to initial beam selection, transmit beam refinement, and receive beam refinement, respectively. The P2/P3 procedures can be enabled by configuring the high layer parameter NZP-CSI-RS-ResourceSet -> repetition to be off/on. However, the P1 procedure is still conceptual and has not been specified in the standard. To facilitate the AI/ML based Tx-Rx beam pair prediction, P1 may needs to be specified clearly in the standard with potentially enhanced beam reporting and indication mechanism.

For the NW-side model, in order to collect model input data for beam pair prediction, the gNB needs to accurately know the transmission beam and reception beam corresponding to the reported measured RSRP. To this end, the gNB may need to associate each RS resource set or each RS resource used for beam measurement with a specific reception beam, which may be explicitly or implicitly indicated. At the same time, the UE needs to feed back the relevant receive beam information in the beam reporting. However, the receive beam is up to UE implementation, and its indication and reporting may lead to UE proprietary information disclosure issues.

For the UE-side model, in order to collect model input data for beam pair prediction, the gNB may continuously transmit RS resource sets for beam measurement multiple times without configuring the repetition parameter. The UE autonomously selects the receive beam for measuring each RS resource according to characteristics of the deployed AI model. Obviously, compared with the beam pair prediction on the network side, the beam pair prediction on the UE side brings less standardization work and does not involve sensitive proprietary information disclosure issues.

Observation 7: Compared with the beam pair prediction on the network side, the beam pair prediction on the UE side brings less standardization work and does not involve sensitive proprietary information disclosure issues.

Proposal 13: To facilitate the AI/ML based Tx-Rx beam pair prediction, enhancements on specification can be studied to support P1 with potentially enhanced beam reporting and indication mechanism.

Training data collection

For model fine-tuning/update or detection of AI/ML model performance degradation, AI/ML model providers should be able to initiate model optimization training and request UEs to report relevant training data, including model input data and model output data. For the NW-side model, the gNB can configure two RS resource sets for beam measurement to collect the model input data and model output data, respectively. The specific number of reported beams and reported quantity can be configured by the high layer parameters nrofReportedRS and reportquantity. Alternatively, the gNB can configure only one RS resource set for beam measurement to collect the model input data and model output data at the same time. In this case, the gNB needs to provide additional indication information to instruct the UE to report measurement results of specific beams, so as to use them as the model input. For the UE-side model, the UE can collect training data by triggering the transmission of relevant RS resource sets. Besides, the UE may need to send additional assistance information to the gNB, including the number of RS resources or specific beams required for training data collection. Therefore, further research on enhanced signaling mechanisms and auxiliary information transmission is needed to enable training data collection.

Proposal 14: Further research on enhanced signaling mechanisms and auxiliary information transmission is needed to enable training data collection.

Model monitoring

Limited by generalization capability of the deployed AI model, the beam prediction performance may be significantly degraded if the UE mobile behavior or propagation environment changes. Therefore, model monitoring is essential for performance assurance, i.e. the gNB or UE needs to regularly evaluate the reliability of the deployed AI/ML models. A feasible approach is to periodically transmit or activate a resource set consisting of a set of CSI-RS or alternatively SS blocks that correspond to different downlink beams at the whole beam space. By adopting the traditional exhaustive beam search method, the realistic optimal beam is obtained and compared with the predicted beam obtained by AI inference. If the AI inference fails, or the error probability of the AI inference exceeds a certain threshold during multiple model monitoring, the gNB or UE should update the model parameters or fallback to the traditional beam management methods.

The model monitoring method is directly related to the deployment node of the AI model. More specifically, for the NW-side model, the gNB can compare the optimal beam information obtained by the AI model inference with the optimal beam information obtained by the UE measurement, so as to evaluate whether the deployed model is effective. For the UE-side model, the UE may report the predicted optimal beam information and the measured optimal beam information (i.e., labels) in a single reporting instances. Then, the gNB judges whether the performance of the deployed UE-side model is reliable according to UE reporting contents. Or, more directly, the UE notifies the gNB of the model monitoring result implicitly or explicitly in a reporting instance. Based on the model monitoring result, the gNB performs further operations such as AI model activation/deactivation, model switching, and fallback.

Observation 8: Model monitoring can be performed by comparing the predicted optimal beam with the realistic optimal beam, which is obtained by measuring one or more resource sets consisting of CSI-RS or alternatively SS blocks that correspond to different downlink beams at the whole beam space.

- Conclusions

In this contribution, we discuss the potential specification impacts and enhancements for AI/ML based beam management. We have the following observations and proposals.

Observations:

Observation 1: For AI/ML model training and inference, Alt.3 outperforms Alt.1 and Alt.2 in terms of reporting overhead and/or beam prediction performance.

Observation 2: The number of beams for measurement (i.e., set B) and for prediction (i.e., set A) is related to the trade-off between inference performance and RS overhead for beam measurement.

Observation 3: Much of the assistance information mentioned in Alt.2 is proprietary information of the gNB or UE, which may be difficult to be obtained and shared with another vendor.

Observation 4: Alt.1 can provide better standard compatibility and less additional standardization effort since only beam ID and the predicted RSRP are considered as the AI output.

Observation 5: The specific construction of the beam set for measurement will have a great impact on the beam training overhead, model complexity and inference performance.

Observation 6: If the beam set for measurement and the beam set for prediction are different, the spatial domain beam prediction algorithm may be an essential precondition for the temporal beam prediction study.

Observation 7: Compared with the beam pair prediction on the network side, the beam pair prediction on the UE side brings less standardization work and does not involve sensitive proprietary information disclosure issues.

Observation 8: Model monitoring can be performed by comparing the predicted optimal beam with the realistic optimal beam, which is obtained by measuring one or more resource sets consisting of CSI-RS or alternatively SS blocks that correspond to different downlink beams at the whole beam space.

Proposals:

Proposal 1: If model transfer is supported in agenda 9.2.1, both Alt.1, Alt.2, and Alt.3 can be supported for further study with potential applicability in different scenarios.

Proposal 2: Since the time unit of this study item is limited, we suggest to focus on the sub use cases BM-Case1 and BM-Case2 in Rel-18, and other sub use cases can be discussed until solid AI framework has been made in Rel-18.

Proposal 3: The sub-sampling based method in Alt.2 can serve as a starting point for the study of spatial domain beam prediction.

Proposal 4: The association in reference signals between two sets with different beam settings need to be further studied.

Proposal 5: Focusing the discussion on Alt.1 and Alt.4 with only L1-RSRP measurement and the corresponding beam ID being taken into account for the AI input would be a good starting point.

Proposal 6: Focus the discussion on Alt.1 as a starting point. Alt.3 can be postponed until the relationship between output beam angle and TCI state is clear.

Proposal 7: Regarding the beam set construction, Alt.3 can be used as a benchmark, while Alt.1 and Alt.2 are deferred until the evaluation of the spatial domain beam prediction in BM-Case1 has achieved sufficient progress.

Proposal 8: For temporal domain beam prediction, focusing the AI input and output on measured RSRP and/or beam ID would be a good starting point, in which case the standardization workload and AI model complexity would be relatively low.

Proposal 9: For NW-side beam prediction AI/ML models, enhanced beam reporting mechanisms such as further screening, compression, and reporting of the beam measurement results need to be studied so as to balance the beam prediction performance and beam reporting overhead.

Proposal 10: For UE-side beam prediction AI/ML models, signaling methods need to be investigated to enable recommendation, configuration, and indication of the beam set for measurement.

Proposal 11: Study enhanced resource configuration and beam indication mechanism if more flexible triggering or activating approaches are utilized.

Proposal 12: Enhanced resource configuration and reporting mechanisms need to be investigated to facilitate the exchange of assistance information, which can be either implicit or explicit.

Proposal 13: To facilitate the AI/ML based Tx-Rx beam pair prediction, enhancements on specification can be studied to support P1 with potentially enhanced beam reporting and indication mechanism.

Proposal 14: Further research on enhanced signaling mechanisms and auxiliary information transmission is needed to enable training data collection.

- References

- R1-2205695, Session notes for 9.2 (Study on AI/ ML for NR air interface), Ad-hoc Chair (CMCC), 3GPP TSG RAN1#109-e, May 2022.

- R1-2207874, Summary#4 for other aspects on AI/ML for beam management, Moderator (OPPO), 3GPP TSG RAN1#110, August 2022.

- RP-210620, Enhancements on predictable mobility for beam management, ZTE Corporation, 3GPP TSG RAN#91-e, March 2021.

- RWS-210481, Enhancements on predictable mobility for beam management, ZTE Corporation, 3GPP TSG RAN REL-18 workshop, June 2021.

- R1-2208523, Evaluation on AI for beam management, ZTE Corporation, 3GPP TSG RAN1#110-bis-e, October 2022.