3GPP TSG RAN WG1 Meeting#110bis-e		R1-2208531

Online, October 10th – 19th, 2022

Agenda item:	9.3.1

Source:	KT Corp.

Title:	Discussion on evaluation on NR duplex operation

Document for:	Discussion

# Introduction

In the last RAN1 #110 meeting, following working agreements had been made as below [1]:

Working assumption:

- Adopt the following table for gNB-gNB channel model and gNB-UE channel model.

|  | Dense urban, Urban macro | Indoor office |
| --- | --- | --- |
| Large-scale channel parameters | FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa in TR 38.901 (hUE =25m), - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m) FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa in TR 38.901 (hUE =25m) - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m) | FR1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) FR2-1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) |
| Fast fading parameters | FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa O2O in TR 38.901 (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa O2O in TR 38.901 (hUE=25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 | FR1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE=3m), ASA and ZSA statistics updated to be the same as ASD and ZSD FR2-1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m), ASA and ZSA statistics updated to be the same as ASD and ZSD |

Working Assumption

| Parameters | Indoor office | Urban macro / Dense Urban Macro layer | Dense Urban with 2-layer |
| --- | --- | --- | --- |
| Layout | Single layer Indoor floor: (12BSs per 120m x 50m) | Single layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. | Two layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. Micro layer: According to previous agreement - Baseline: 3 Micro BSs per Macro BS - Optional: 6, or 9 Micro BSs per Macro BS |
| Inter-BS (2D) distance | 20m [TR 38.802 Table A.2.1-11] | 500m for Urban Macro [TR 38.802 Table A.2.1-11] 200m for Dense Urban Macro layer [TR 38.802 Table A.2.1-1] | Macro-to-macro: 200m Minimum Macro-to-micro-center distance: 105m Minimum Micro-center-to-micro-center distance: 57.9m |
| Minimum BS-UE (2D) distance | 0m [TR 38.802 Table A.2.1-11] | 35m [TR 38.802 Table A.2.1-11] | Macro-to-UE: 35m Micro-to-UE: 10m [TR 38.802 Table A.2.1-11] |
| Minimum UE-UE (2D) distance | FFS | FFS :3m [TR 38.802 Table A.2.1-11] | FFS: 3m [TR 38.802 Table A.2.1-11] |
| BS antenna height | 3 m [TR 38.802 Table A.2.1-1] | 25 m [TR 38.802 Table A.2.1-1] | 25m for macro cells and 10m for micro cells [TR 38.802 Table A.2.1-1] |

Working Assumption

For UE-UE channel model, reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2 as baseline, and other models are not precluded.

UE-UE channel model

|  | Dense urban, Urban macro | Indoor hotspot |
| --- | --- | --- |
| Large-scale channel parameters | FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843(*), penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Option 2: UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-13 in TR38.802 FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802 | FR1: - Option1 : UE-to-UE: A.2.1.2 in TR36.843 (*) - Option 2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) |
| Fast fading parameters | FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH) for indoor to indoor, and 3D UMi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Optioin 2: UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. | FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA |
| (*): For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-13 in TR38.802. |  |  |

In this contribution, open issues from the last RAN1 meetings are discussed.

# Discussion

For gNB-gNB channel model and gNB-UE channel model, dense urban, urban macro and indoor office scenario is being considered for both FR1 and FR2-1. Large-scale channel parameters and fast fading parameters are being considered to adopt parameters used in TR 38.901 [2]. These parameters had been used for various studies in RAN involving channel modelling unless separate channel modelling study had been done. We propose to re-use the same parameters to keep consistent with other channel models in RAN.

Proposal 1: Confirm current Working assumption on parameters for gNB-gNB and gNB-UE channel model

For UE-UE (2D) distance, it is still FFS whether to adapt 3m which was used in TR 38.802 [3]. In TR 38.802 System level simulation assumptions has been made specific to flexible duplex where for all scenarios including dense urban, urban macro, indoor hotspot scenario, minimum UE-UE (2D) distance has been set to 3m. We don’t see any other strong motivation on evaluating other value for UE to UE distance and we propose to reuse this value for evaluation on NR duplex operation.

Proposal 2: Minimum UE-UE (2D) distance to be set to 3m

For UE-UE channel model, KT believes that this should also be consistent with gNB-gNB and gNB-UE channel model where parameters used in TR 38.901 had been proposed to be adopted for FR1 and FR2-1 as a baseline. As using other parameter may result inconsistent result, KT proposes to confirm current working assumption as a baseline.

Proposal 3: Confirm current Working assumption on parameters for UE-UE channel model

# Conclusion

Based on the discussion in the above section, the following proposal is suggested.

Proposal 1: Confirm current Working assumption on parameters for gNB-gNB and gNB-UE channel model

Proposal 2: Minimum UE-UE (2D) distance to be set to 3m

Proposal 3: Confirm current Working assumption on parameters for UE-UE channel model

# Reference

- R1-2208321, Report of 3GPP TSG RAN1#110 meeting, RAN1 #110bis-e

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100GHz.

- 3GPP TR 38.802, Study on New Radio Access Technology. Physical Layer Aspects