3GPP TSG-RAN WG1 Meeting #110bis-e		R1-2208586

e-Meeting, October 10th – 19th, 2022

Title:	[DRAFT] Reply LS on IUC with non-preferred resource set

Response to:	R1-2205728 (R2-2206314)

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	vivo [RAN1]

To:	RAN2

Cc:

Contact person:	Zichao Ji

jizichao@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN1 thanks RAN2 for the LS asking the inter-UE coordination scenario where UE B receives IUC Scheme 1 non-preferred resource set from UE A, but UE B does not perform sensing in the resource pool associated with the non-preferred resource set. RAN1 would provide the following answers to RAN2’s questions.

Q1:	Is the scenario described above a valid scenario or not?

Answer: This scenario may occur, e.g., when the UE A sends the condition-triggered non-preferred resource set while the UE B requesting a preferred resource set. However, it may not be a valid scenario, e.g., if the UE B does not support receiving IUC Scheme 1 non-preferred resource set.

Q2:	If the answer to Q1 is yes, does resource exclusion based on non-preferred resource set needs to be performed by UE B or not?

Answer: In the valid case, it is up to UE implementation to handle this, i.e., whether to ignore the non-preferred resource set (e.g., to wait until a preferred resource set is available), or to perform resource exclusion based on the non-preferred resource set (e.g., in a random selection procedure or to trigger another sensing procedure). When sensing results are available for UE B, the UE behavior is defined according to the following RAN1 agreement. Otherwise, the UE B is not required to exclude the resources based on the non-preferred resource set.

Agreement - When UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A or different UE-As, when UE-B has own sensing results - No RAN1 specification change to TS 38.214 is deemed necessary on how it uses the non-preferred resource set in its resource (re)selection - It is up to UE-B implementation to use the preferred resource set in its resource (re)selection for transmissions to the UE A providing the preferred resource set

Q3:	If the answer to Q2 is yes, then, in RAN1’s view, which specification (PHY or MAC) should capture the resource exclusion behavior?

Answer: No specification change is required regarding resource exclusion behaviour when UE B does not have sensing results.

# 2	Actions

To RAN2

RAN1 respectfully asks RAN2 to take the above information into account in related work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG RAN WG1 Meeting #111			November 14th – 18th, 2022   			Toulouse

TSG RAN WG1 Meeting #112			February 27th – March 03rd, 2023		Athens