3GPP TSG RAN WG1 #110bis-e 	R1-2208591

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft] CR on the UE behavior when PDCCH candidate overlaps with SSBs for inter-cell beam measurement in the same REs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-core |  |  |  |  |  | Date: |  |  | 2022/9/30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1#109-e meeting, the following working assumption was agreed and the common understanding is to confirm whether to apply it or not when RAN4 LS feeds back. On inter-cell beam management, the PDCCH /PDSCH should be rate matched around the SSBs indicated by ssb-PositionsInBurst-r17 for the same PCI as that associated with TCI state of the PDSCH /PDCCH.. According to the working assumption, RAN1 sent an LS to RAN4 asking whether there are requirements in RAN4 that assumes UE to measure SSB for L1-RSRP measurement and receive PDCCH on the same RE in FR1. And, the response from RAN4 was provided in R4-2215029, copied below: - There is no RAN4 requirement or scheduling restriction that is related to UE L1-RSRP measurement and reception of PDSCH/PDCCH on the same RE in FR1 for inter-cell beam management. - In RAN4 common understanding, if SSB has the same SCS as PDSCH/PDCCH or UE supports the capability simultaneousRxDataSSB-DiffNumerology, UE may be able to measure on SSBs while receiving PDSCH/PDCCH on overlapped REs simultaneously in FR1, though decoding performance degradations and/or additional UE receiver complexities may be expected if SSB and PDCCH/PDSCH overlap on the same RE.. In this meeting, we should confirm whether the working assumption is correct or not based on the reply LS from RAN4. According to the reply from RAN4, the case of L1-RSRP measurement and reception of PCSCH on the same RE will lead to decoding performance degradations and/or additional UE receiver complexities. To avoid performance degradation or additional UE complexity, extending the rate match pattern to the SSB resource for inter-cell beam management in FR1 is a straightforward method. |
| Summary of change: |  | When PDCCH candidate overlaps with SSBs for inter-cell beam measurement in at least one REs, the UE would not monitor the PDCCH candidate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not clarified, UE may need to measure on SSBs for inter-cell beam management while receiving PDCCH on overlapped REs simultaneously in FR1, which would lead to decoding performance degradations and/or require additional UE receiver complexities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

10 	UE procedure for receiving control information

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG except for PDCCH monitoring in Type0/0A/2/2A -PDCCH CSS sets where the UE is not required to apply the procedures in this clause for the SCG

-	When the procedures are applied for MCG, the terms 'secondary cell', 'secondary cells' , 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms 'secondary cell', 'secondary cells', 'serving cell', 'serving cells' in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term 'primary cell' in this clause refers to the PSCell of the SCG.

A UE monitors a set of PDCCH candidates in one or more CORESETs on the active DL BWP on each activated serving cell configured with PDCCH monitoring according to corresponding search space sets where monitoring implies receiving each PDCCH candidate and decoding according to the monitored DCI formats.

In the remaining of this clause, when a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1

-	a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates

-	the start of the PDCCH reception is the start of the earlier PDCCH candidate

-	the end of the PDCCH reception is the end of the PDCCH candidate that ends later

The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10 (except clause 10.4), 11.1, and 11.1.1.

If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the active DL BWP of the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs

-	per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or

-	per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability, or

-	per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability

The remaining of this clause, including clause 10.1, considers that a UE is provided monitoringCapabilityConfig for a serving cell. If the UE is not provided monitoringCapabilityConfig for the serving cell, corresponding statements that the UE is provided monitoringCapabilityConfig for the serving cell are substituted as follows

-	for SCS configuration μ∈0, 1, 2, 3, the UE monitors PDCCH on the active DL BWP of the serving cell for maximum numbers of PDCCH candidates and non-overlapping CCEs per slot as in Tables 10.1-2 and 10.1-3.

-	for SCS configuration μ∈5, 6, the UE monitors PDCCH on the active DL BWP of the serving cell for maximum numbers of PDCCH candidates and non-overlapping CCEs per group of Xs slots according to combination Xs,Ys=(4, 1) for μ=5 and Xs,Ys=(8, 1) for μ=6 as in Tables 10.1-2B and 10.1-3B.

The UE does not expect to monitor PDCCH with SCS configuration μ=6 before the UE is provided dedicated higher layer parameters.

A UE can indicate a capability to monitor PDCCH according to one or more of the combinations X,Y = (2, 2), (4, 3), and (7, 3) per SCS configuration of μ=0 and μ=1. A span is a number of consecutive symbols in a slot where the UE is configured to monitor PDCCH. Each PDCCH monitoring occasion is within one span. If a UE monitors PDCCH on a cell according to combination X,Y, the UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including across slots. A span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends, where the number of symbols of the span is up to Y.

If a UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for more than one of the multiple combinations X,Y, the UE monitors PDCCH on the cell according to the combination X,Y, from the more than one combinations X,Y, that is associated with the largest maximum number of MPDCCHmax,X,Y,μ and CPDCCHmax,X,Y,μ defined in Table 10.1-2A and Table 10.1-3A. The UE expects to monitor PDCCH according to the same combination X,Y in every slot on the active DL BWP of a cell.

For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots. Groups of Xs slots are consecutive and non-overlapping and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots.

If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively.

For μ=6, if the UE indicates a capability to monitor PDCCH according to multiple combinations Xs,Ys and a configuration of search space sets to the UE for PDCCH monitoring on a serving cell results to a separation of every two consecutive groups of Ys slots that is not smaller than Xs for more than one combinations Xs,Ys, of the multiple combinations Xs,Ys, the UE monitors PDCCH on the cell according to the combination Xs,Ys, from the more than one combinations Xs,Ys, that is associated with the largest maximum number of MPDCCHmax,Xs,μ and CPDCCHmax,Xs,μ defined in Table 10.1-2B and Table 10.1-3B.

A UE capability for PDCCH monitoring per slot, or per group of Xs slots according to combination Xs,Ys, or per span on an active DL BWP of a serving cell is defined by a maximum number of PDCCH candidates and non-overlapped CCEs the UE can monitor per slot, or per group of Xs slots according to combination Xs,Ys, or per span, respectively, on the active DL BWP of the serving cell.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

-	does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1,

the UE is not required to monitor the PDCCH candidate.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

-	does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon,

the UE is not required to monitor the PDCCH candidate.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in SSB-MTCAdditionalPCI for a serving cell, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SSB-MTCAdditionalPCI,

the UE is not required to monitor the PDCCH candidate.

< Unchanged parts are omitted >