3GPP TSG-RAN WG1 Meeting #110bis-e 	R1-2208610

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | x | rev | x | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for partial sensing resource selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022/9/24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to the agreements in RAN1 #106bis-e and #107-e, the sensing window for CPS is defined by the range of slots [n+TA, n+TB], which can by applied in the following cases: periodic transmission and aperiodic transmission in a pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, and aperiodic transmission in a pool with periodic reservation for another TB (sl-MultiReserveResource) disabled. Only for periodic transmission, n+TA is M logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS; but for aperiodic transmission, no matter the periodic reservation for another TB (sl-MultiReserveResource) is enabled or disabled, TA and TB are both selected such that UE has sensing results starting at M consecutive logical slots before t'y0SL and ending at Tproc,0 + Tproc,1 slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y’ candidate slots. However, in the current 38.214, t'y0SL used to deriving CPS window in the above mentioned cases are obtained from the first slot of the selected Y candidate slots of PBPS and the CPS window for periodic transmission and aperiodic transmission in a pool with periodic reservation for another TB (sl-MultiReserveResource) enabled are the same, which are not compliant with the RAN1 previous agreements. - When a UE performs periodic-based partial sensing, the UE monitors one PSO if sl-Additional-PBPS-Occasion is not (pre-)configured or two PSO if sl-Additional-PBPS-Occasion is (pre-)configured, where a PSO is a set of slots according to ty-k×Preserve''SL. The value of k indicates the index of PSO to be monitored. However , it is confusing to use the natation of “k” to indicate the index of the PSO in “the UE shall monitor slots at ty-k×Preserve''SL” and the number of the PSO in “The UE monitors k sensing occasions…”, which should be clarified. - The CPS monitoring window in re-evaluation or pre-emption checking is [n+TA, n+TB], whick is incorrected captured as [n+TA, n+TB]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Clause 8.1.4, capture the following corrections: - Clarifications of the CPS window when triggered by aperiodic transmission. - Clarifications of the sensing occasions when sl-Additional-PBPS-Occasion is (pre-)configured or not (pre-)configured. - Some editorial changes on re-evaluation or pre-emption |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity on partial sensing resource selection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SL, where t'ySL is a slot of the selected candidate slots and Preserve' is Preserve converted to units of logical slot according to clause 8.1.7. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

The value of Preserve corresponds to sl-PBPS-OccasionReservePeriodList if (pre-)configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList.

The UE monitors k’ sensing occasions determined by sl-Additional-PBPS-Occasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) and k’=1 if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one and k’=2 if sl-Additional-PBPS-Occasion is (pre-)configured. t'y0SL is the first slot of the selected Y candidate slots of PBPS.

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, the sensing window is defined by the range of slots [n+TA, n+TB]. If Prsvp_TX≠0, n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, Tproc,0SL, Tproc,1SL are in units of physical time/slots, andPrsvp_TX≠0  the value of M is (pre-)configured with the sl-CPS-WindowPeriodic. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31. If Prsvp_TX=0, TA and TB are both selected such that the UE has sensing results starting at M consecutive logical slots before t'y0SL, where t'y0SL is the  first slot of the selected Y’ candidate slots and the value of M is (pre-)configured with the sl-CPS-WindowAperiodic. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31.When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

When the UE performs contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) disabled and if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the  first slot of the selected Y’ candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter partialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given periodicity corresponding to Preserve, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots.

<Unchanged parts omitted>

When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0,

-	During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot t'yiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to t'y+q×Prsvp_TX'SL, where t'ySL is a slot index of Y candidate slots used in the initial resource (re)selection.

-	t'yiSL is the first candidate slot after slot n+T3.

-	The UE performs PBPS for the remaining Y candidate slots according to ty'-k×Preserve''SL, wheret'y'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than t'yiSL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured.

-	The UE performs CPS starting from M logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL.

-	By default, M is 31 unless (pre-)configured with another value, by sl-CPS-WindowPeriodic.

When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0,

-	Candidate resource set (SA) is initialized to the remaining Y' candidate slots starting from slot t'yiSL and ending at the last slot of the Y' candidate slots, where t'yiSL is the first candidate slot after slot n+T3.

-	It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled.

-	UE performs CPS starting from at least M consecutive logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL.

-	For minimum size M of the CPS monitoring window [n+TA, n+TB] [n+TA, , by default, M is 31 unless (pre-)configured with another value, by sl-CPS-WindowAperiodic.

When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0.