3GPP TSG RAN WG1 #110bis-e		                                                                 R1-2208714

e-Meeting, October 10th – 19th, 2022

Title:	Draft reply LS on Pemax,c of S-SSB transmission

Response to:	R1-2208348/ R4-2214421

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	ZTE, Sanechips [to be RAN1]

To:	RAN4

Cc:	RAN2

Attachment:	-

Contact Person:

Name:	Yuzhou Hu

E-mail Address:	hu.yuzhou@zte.com.cn

Overall description

RAN1 thanks RAN4 for the LS (R1-2208348/ R4-2214421) on the Pemax,c parameters for S-SSB transmission. In the LS, RAN4 raised a question as follows:

Question 1: RAN4 agreed not to define PEMAX,c for S-SSB configured Tx Power in TS38.101-1 as highlighted yellow below and would like to know if this can have any impact to RAN1/2, for example, Pcompensation defined in idle mode procedure in TS 38.304.

6.2E.4          Configured transmitted power for V2X

6.2E.4.1    General

The NR V2X UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f, c = MIN {PEMAX,c,  PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + DTIB,c , P-MPRc), PRegulatory,c }

PCMAX_H,f, c = MIN {PEMAX,c, PPowerClass, V2X,  PRegulatory,c }

where

-  PCMAX,f,c is configured for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-  For the total transmitted power PCMAX,PSSCH/PSCCH, PEMAX,c is the value given by IE sl-maxTransPower, defined by TS 38.331.

-  For the total transmitted power PCMAX,S-SSB, the PCMAX_L,f,c and PCMAX_H,f,c are defined as follows:

PCMAX_L,f,c = MIN {PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + DTIB,c , P-MPRc), PRegulatory,c}

PCMAX_H,f,c = MIN {PPowerClass, V2X,  PRegulatory,c}

Answer 1:

From RAN1’s perspective, the total transmitted power PCMAX,S-SSB  should be defined per carrier, instead of per resource pool, thus the description of S-SSB configured Tx Power in TS38.101-1 as highlighted yellow above is reasonable, and there is no impact on RAN1 about the agreement RAN4 made on Pemax,c of S-SSB transmission.

Actions:

To RAN4:

RAN1 respectfully requests RAN4 to take above responses into account for the future work.

Date of Next TSG-RAN1 Meetings:

RAN1#111			14th -18th Nov 2022				Toulouse, France

RAN1#112			27 February to 3 March 2023			Athens, Greece