3GPP TSG RAN WG1 Meeting #110bis-e	R1-2208717

# E-meeting, October. 10th – 19th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | draft | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Condition 2-A-1 for scheme 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Condition 2-A-1 on IUC scheme 2 needs further clarification on conditions for UE to be UE-B when at least one of UEs scheduling conflicting TBs does not set indicationUEB flag to 1, and we continue to support following proposal in RAN1#109-e meeting: Proposal in RAN1#109-e: - For Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A, - If there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and, if sl-IndicationUE-B is (pre)configured to ‘Enabled’, ‘Conflict information receiver flag’ field is set to 1, that UE is UE-B.. - For SCI format 1-A, a field is named as ‘Conflict information receiver flag’ in TS38.212, while the same filed is named as ‘indicationUEB flag’ in TS38.213. |
| Summary of change: |  | - Adding the related description on conditions for UE to be UE-B for Condition 2-A-1 in TS38.213. - Changing ‘indicationUEB flag’ to ‘conflict information receiver flag’ to in TS38.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to determine UE-B for Condition 2-A-1 of Scheme 2 is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

    - 16.3.0 UE procedure for transmitting PSFCH with control information

< Unchanged parts are omitted >

A first UE determines a UE for providing the conflict information to in a PSFCH as follows

-	if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception,

-	detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE,

-	detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and

-	determines that the first and second resources overlap in time and frequency

-	the PSFCH occasions for resource conflict information of the second UE and the third UE are valid

-	the conflict information receiver flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 = 'enabled'

-	determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information

-	determines to transmit to the second UE the PSFCH with the conflict information

-	determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1

-	determines to transmit to the second UE the PSFCH with the conflict information, if only for the second UE, the PSFCH occasion for resource conflict information is valid and the conflict information receiver flag in SCI Format 1-A is set to 1, if indicationUEBScheme2 = 'enabled'

< Unchanged parts are omitted >