3GPP TSG RAN WG1 #110bis		R1-2208794

e-meeting, October 10th– 19th, 2022

Source:	OPPO

Title:	CSI enhancement for high/medium UE velocities and coherent JT

Agenda Item:	9.1.2

Document for:	Discussion and Decision

Introduction

In RAN1#110 e-meeting, Rel-18 CSI enhancement was discussed with the following agreements:

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP with NTRP>1 TRP/TRP-groups, support NTRP={1, 2, 3, 4} with equal priority.

Agreement

For the Rel-18 Type-II codebook for CJT mTRP, support RI={1,2,3,4}.

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP with NTRP>1 TRP/TRP-groups, the following is supported:

- The CMR comprises K>1 NZP CSI-RS resources, where one resource corresponds to one TRP/TRP-group (i.e. K=NTRP)

  - Each of the CSI-RS resources has a same number of CSI-RS ports

- Note: The terms TRP and TRP-group are used for discussion purposes only (no spec impact is implied).

Agreement

For the Rel-18 Type-II codebook for CJT mTRP, support the following two modes:

- Mode 1: Per-TRP/TRP-group SD/FD basis selection which allows independent FD basis selection across N TRPs / TRP groups. Example formulation (N = number of TRPs or TRP groups):

W1,1W2,1Wf,1H⋮W1,NW2,NWf,NH

- Mode 2: Per-TRP/TRP group (port-group or resource) SD basis selection and joint/common (across N TRPs) FD basis selection. Example formulation (N = number of TRPs or TRP groups):

W1,1W2,1WfH⋮W1,NW2,NWfH

- Striving for the two modes to share commonality in detailed designs such as parameter combinations, basis selection, TRP (group) selection, reference amplitude, W2 quantization schemes.

- FFS: Depending on the decision on SCI design, whether additional per-TRP/TRP-group amplitude scaling and/or co-phase is needed or not, and whether they are a part of W2s

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, down-select one from the following codebooks structures:

- Alt2A: Doppler-domain basis commonly selected for all SD/FD bases, e.g. W1W2(Wf⨂Wd)H

  - Note that Wd may be the identity as a special case

- Alt2B: Doppler-domain basis independently selected for different SD/FD bases

  - Note that Wd may be the identity as a special case

- Alt3. Reuse Rel-16/17 (F)eType-II codebook with multiple W2 and a single W1 and Wf report.

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, on the DD/TD basis waveforms:

- Down-select or combine from the following Doppler-/time-domain basis waveforms:

  - Alt1. Orthogonal DFT

    - TBD (by RAN1#110bis): whether rotation is used or not

    - FFS: identical or different rotation factors for different SD components

  - Alt2. Identity (i.e. no Doppler-/time-domain compression)

- FFS: Whether Doppler-/time-domain (DD/TD) basis vector length (N4) is RRC-configured or reported by the UE

- FFS: Whether the number of selected DD/TD basis vectors (for Alt1) is RRC-configured or reported by the UE

Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the assumption of the UE-side prediction

- On the definition of UE-side prediction, down-select one from the following alternatives:

  - Alt1. UE “predicting” channel/CSI after the slot with a reference resource

  - Alt2. UE “predicting” channel/CSI after slot n (where the CSI is reported)

  - Alt3. UE “predicting” channel/CSI after the slot where CSI-RS resides

Agreement

The Rel-18 TRS-based TDCP reporting comprises stand-alone auxiliary feedback information to enable refinement of CSI reporting configuration, and/or codebook configuration parameters, and/or (to be confirmed in RAN1#110) gNB-side CSI prediction

- Not conditioned on other UCI parameters

- Not reported together with CQI/PMI/RI/(CRI) associated with a codebook

  - Note: This does not prevent TDCP reporting from being multiplexed with other UCI parameters on PUCCH and/or PUSCH

- Note: Aperiodic reporting is supported (per agreed Alt1 in RAN1#109-e)

Agreement

On the Type-II codebook refinement for CJT mTRP, down-select from the following TRP selection/determination schemes (where N is the number of cooperating TRPs assumed in PMI reporting) by RAN1#110bis-e:

- Alt1. N is gNB-configured via higher-layer (RRC) signalling

  - The N configured TRPs are gNB-configured via higher-layer (RRC) signalling

  - Note: only one transmission hypothesis is reported

- Alt2. N is UE-selected and reported as a part of CSI report where N∈{1,..., NTRP}

  - N is the number of cooperating TRPs, while NTRP is the maximum number of cooperating TRPs configured by gNB

  - In this case, the selection of N out of NTRP TRPs is also reported (FFS: exact reporting scheme)

  - FFS: Configuration of NTRP TRPs and the value of NTRP, whether explicit or implicit

  - Note: only one transmission hypothesis is reported. UE is not mandated to calculate CSI for multiple transmission hypotheses.

- Alt3. The UE reports CSI corresponding to K transmission hypotheses

  - The N configured TRPs per hypothesis are gNB-configured via higher-layer (RRC) signalling

  - FFS: supported value(s) of K, and whether the K transmission hypotheses are gNB-configured or UE-reported

  - FFS: Whether the same N value or possibly different N values

- Alt4. The UE reports CSI corresponding to K transmission hypotheses where N is UE-selected and reported as a part of CSI report where N∈{1,..., NTRP}

  - N is the number of cooperating TRPs per hypothesis, while NTRP is the maximum number of cooperating TRPs configured by gNB

  - In this case, the selection of N out of NTRP TRPs is also reported (FFS: exact reporting scheme)

  - FFS: Configuration of NTRP TRPs and the value of NTRP, whether explicit or implicit

  - FFS: Whether the same N value or possibly different N values

FFS: Whether S-TRP transmission hypothesis is also reported

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group and Strongest Coefficient Indicator (SCI) design, for each layer, down-select one from the following alternatives by RAN1#110bis-e:

- Alt1. One group comprises one polarization across all TRPs/TRP-groups (Cgroup,phase=1, Cgroup,amp=2), one (common) SCI across all TRPs/TRP groups

- Alt2. One group comprises one polarization for one TRP/TRP-group (Cgroup,phase=N, Cgroup,amp=2N), per-TRP/TRP-group SCI

  - FFS: Quantization of N strongest coefficients

- Alt3. One group comprises one polarization for one TRP/TRP-group with a common phase reference across TRPs/TRP-groups (Cgroup,phase=1, Cgroup,amp=2N)

  - FFS: SCI, per-TRP/TRP-group vs. one (common) SCI across all TRPs/TRP groups

  - FFS: Quantization of N strongest coefficients

- Alt4. For a selected TRP/TRP-group, one group comprises one polarization, and for remaining N-1 TRPs/TRP-groups, one group comprises one polarization across remaining N-1 TRPs/TRP-groups (Cgroup,amp=2+2=4), with a common phase reference across all of N TRPs/TRP-groups (Cgroup,phase=1)

  - FFS: The selected TRP/TRP-group

FFS: The need for “strongest” TRP/TRP-group indicator in addition to SCI(s)

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, support DD/TD (compression) unit (analogous to PMI sub-band for Rel-16 codebook) as a codebook parameter.

- FFS: whether this parameter is defined as a function of another parameter

- FFS: whether this is used for PMI only or PMI/CQI

Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities assuming the UE-side prediction, on the definition of UE-side prediction, down-select one from the following alternatives by RAN1#110bis-e:

- Alt1. UE “predicting” channel/CSI after the slot with a reference resource

- Alt2. UE “predicting” channel/CSI after slot n (where the CSI is reported)

- Alt3. UE “predicting” channel/CSI after the slot where CSI-RS resides

Agreement

For the Rel-18 Type-II codebook for CJT mTRP based on the Rel-16 Type-II codebook, SD basis and FD basis are separate, each fully reusing the legacy Rel-16 DFT-based design

Agreement

The Rel-18 Type-II codebook for CJT mTRP comprises refinement of the following codebooks:

- Refinement of the Rel-16 eType-II regular codebook

- Refinement of the Rel-17 FeType-II port selection (PS) codebook, based on the same design details as the refinement of the Rel-16 eType-II regular codebook, except for the supported set of parameter combinations

Strive to maintain as much commonality between the Rel-16 and Rel-17 codebook enhancements to minimize workload.

Vivo and Lenovo raised concerns on the workload due to this agreement

Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, down-select one from the following alternatives by RAN1#110bis-e:

- Alt1.B:  l ≥ nref

  - nref (a CSI reference resource slot) as boundary

- Alt2.B: l ≥ n

  - n (report slot) as boundary

Agreement

For the Rel-18 TRS-based TDCP reporting, down select one of the following alternatives by RAN1#110bis-e:

- AltA. Based on Doppler profile

  - E.g., Doppler spread derived from the 2nd moment of Doppler power spectrum, average Doppler shifts, Doppler shift per resource, maximum Doppler shift, relative Doppler shift, etc

- AltB. Based on time-domain correlation profile

  - E.g. Correlation within one TRS resource, correlation across multiple TRS resources

  - Note: The correlation over one or more lags of TRS resource may be considered.  The lags may be within one TRS burst or different TRS bursts

- AltC: CSI-RS resource and/or CSI reporting setting configuration parameter(s) to assist network

  - E.g. gNB configures UE with multiple choices on what to assist (e.g. two or more CSI-RS/report periodicities, or precoding schemes depending mainly on UE velocity), then UE report according to configuration; parameters correspond to CSI reporting periodicity, codebook type, etc.

Note: Different alternatives may or may not apply to different use cases

Agreement

For the Rel-18 TRS-based TDCP reporting, the use case of “aiding gNB-side CSI prediction” is refined to “aiding gNB implementation in CSI prediction for TDD”

In this contribution, we further discuss the details of CSI enhancement for high/medium UE velocities and coherent JT.

Discussion

  - CSI enhancement for high/medium mobility UEs

- Codebook structure/ basis selection

Time domain DFT compression and no compression are proposed for codebook structure. In our view, TD DFT basis is superior to identity basis. Even for N2=2, DFT basis show good performance gain over identity basis particularly at low overhead as show in figure 1, not mention the overhead of identity basis increase linearly as N4 increase. Supporting two type of W2 encoding/reporting scheme complex UE implementation and cost more specification effort, but the benefit is unclear to us.  A unified solution is appreciated.

Since precoder is invariant to circular shifting for the same reason as frequency domain, rotation factor maybe not needed for orthogonal DFT basis.

Figure 1 TD basis

Observation 1: DFT basis outperform identity basis at low overhead, the gain is about 10% for N2=2

Proposal 1: Support Doppler-domain basis commonly selected for all SD/FD basis e.g. W1W2(Wf⨂Wd)H .

- UE side prediction

For a given reporting window (e.g. 10ms), we compared the performance of Alt1.B and Alt.2B for measurement window configuration from 10ms to 60ms, The N4 value is 5 and 3 for Alt 1.B and Alt 2.B respectively. gNB’s refinement based on the reporting before CSI reporting slot n is not assumed for Alt 1B. We don’t see clearly benefit of Alt 1.B if gNB precode as UE suggested. As Alt 2B cost lower UE complexity, we slightly prefer to Alt 2.B.

Proposal 2:  Support Alt 2.B, i.e., reporting slot n as prediction boundary.

We show the performance of N4 >= 1 in figure 3. The measurement window is set to {16, 24, 32} ms respectively. There are {4, 6, 8} CSI-RS occasions for time unit 4 slots and {8, 12, 16} CSI-RS occasions for time unit 2 slots. We assumed time unit equals CSI-RS spacing. Frequency-time domain LMMSE is used for channel prediction where covariance is measured from Wmeas. Reporting window size is prediction horizon (from the latest CSI-RS occasion). The overhead for each setting of W_CSI is about 300 bits, R16 PC6 is the reference.  Although the prediction is less reliable as W_CSI increase, the performance gain is still obvious. Moreover, supporting N4 > 1 could reduce the normalized overhead. At medium velocity, precoder may only hold on in duration of 1~2 ms, supporting N4=1 only may be quite wasteful in terms of CSI-RS and CSI overhead.

Figure 3, performance vs Wcsi

Based on the evaluation result, we support N4>=1.

Proposal 3:  Support N4>=1.

CSI enhancement for coherent JT transmission

Two modes were agreed for Rel-18 Type-II codebook for CJT, where the FD basis is independently selected for mode 1 across N TRPs/TRP groups and jointly selected for mode 2. For SD basis, based on the definition of mode 1 and mode 2, it should be separately selected for each TRP/TRP group for both modes. Considering one CSI-RS resource would be configured for each TRP/TRP group as agreed, the SD basis should be selected per CSI-RS resource.

For the value of L for SD basis, it can be configured per CSI-RS resource or common across N CSI-RS resources. Separate configuration can provide additional flexibility, though it would also introduce ambiguity on the UCI payload especially for CSI part 1, if the value of L is different for TRPs and the selected TRP/CSI-RS resource is also reported by UE via UCI. On the other hand, separate configuration can also avoid unnecessary overhead. For example, for two TRPs with L=2 and L=4, if common L=4 is configured, then some of the bitmap for the first TRP would be wasted with 0 always. Hence, separate configuration of L for different TRPs is preferred.

Proposal 4: For SD basis selection for mode 1 and mode 2, support per-CSI-RS-resource SD basis selection with separate configuration of L for N CSI-RS resources.

Since different CSI-RS resources are associated with different TRPs/TRP groups, and the FD basis can be independently or joint selected for N TRPs/TRP groups, the position of non-zero coefficient (NZC) should be allowed to be different for different CSI-RS resources. That is, separate bitmaps for NZC are reported for different CSI-RS resources. Considering the UCI payload, the total number of NZC should be restricted. Compared to per CSI-RS resource NNZC constraint, joint NNZC constraint across N CSI-RS-resources can provide additional flexibility with the same UCI payload.

Proposal 5: For per-layer and per-polarization bitmap, support separate bitmaps for N CSI-RS resources and joint NNZC constraint across N CSI-RS-resources.

For type II codebook refinement for CJT, UE can report the CSI of N TRP/TRP groups out of NTRP TRP/TRP groups. The value of N can be configured by gNB via RRC signaling or reported by UE as part of CSI. The benefit of gNB configuration is simplification of UCI design based on known N, and lower UE complexity without the processing to select the value of N. However, since gNB cannot derive the DL CSI, it would be difficult for gNB to determine the value of N. If a suitable value cannot be configured, the transmission performance would be degraded. On the contrary, UE can select and report a proper N based on the measured DL CSI, as well as the N selected TRPs/TRP groups. The value N can be determined based on some simple calculation, e.g. RSRP instead of CSI, and would not introduce much complexity. The value of N and corresponding TRP/TRP group information can be reported in CSI part 1, and used to determine the payload of CSI part 2 with CSI of the N TRPs/TRP groups. UCI payload ambiguity can be avoided. From the perspective of performance, N reported by UE (Alt.2) is preferred.

Proposal 6: On TRP selection schemes for Type-II codebook refinement for CJT mTRP, support Alt2 where N is UE-selected and reported as a part of CSI report.

Conclusion

In this contribution, we discuss the possible enhancements to CSI feedback for mobility and coherent JT. To summarize, we have the following proposals:

Observation 1: DFT basis outperform identity basis at low overhead, the gain is about 10% for N2=2

Proposal 1: Support Doppler-domain basis commonly selected for all SD/FD basis e.g. W1W2(Wf⨂Wd)H .

Proposal 2:  Support Alt 2.B, i.e., reporting slot n as prediction boundary.

Proposal 3:  Support N4>=1.

Proposal 4: For SD basis selection for mode 1 and mode 2, support per-CSI-RS-resource SD basis selection with separate configuration of L for N CSI-RS resources.

Proposal 5: For per-layer and per-polarization bitmap, support separate bitmaps for N CSI-RS resources and joint NNZC constraint across N CSI-RS-resources.

Proposal 6: On TRP selection schemes for Type-II codebook refinement for CJT mTRP, support Alt2 where N is UE-selected and reported as a part of CSI report.

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator), 3GPP TSG RAN Meeting #94e, Dec. 6th – 17th, 2021.

Appendix

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, 2GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Type II overhead reduction Other configuration is not precluded. |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz for 15kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| UE distribution |  | outdoor only |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |