3GPP TSG RAN WG1 #110bis-e			R1-2208819

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on starting slot and pre-condition in re-evaluation and pre-emption checking for partial sensing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - tyiSL is defined as the first candidate slot after slot n+T3 in both periodic and aperiodic case of re-evaluation and pre-emption： Agreement When UE is triggered to perform re-evaluation and pre-emption checking for periodic transmission (Prsvp_TX≠0) in slot n, - During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starts from slot tyiSL and ends at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to [q x Prsvp_Tx + tySL], where tySL is a slot index of Y candidate slots used in the initial resource (re)selection. - tyiSL is the first candidate slot after slot n+T3. …… Agreement When UE is triggered to perform re-evaluation and pre-emption checking for aperiodic transmission (Prsvp_TX=0) in slot n, - The candidate resource set (SA) is initialized to the remaining Y’ candidate slots that starts from slot tyiSL and ends at the last slot of the Y’ candidate slots. - tyiSL is the first candidate slot after slot n+T3. ……. The current definition of tyiSL will cause that the pre-selected or reserved resource is not included within the candidate resource set (SA) before resource exclusion. For example, a pre-selected resource is located at slot m and re-evaluation is triggered at slot m - T3. When tyiSL is the first candidate slot after slot m, the pre-selected resource is not included in the candidate resource set and resource re-selection is always triggered in this case. Therefore, tyiSL should be the first candidate slot from (i.e., not earlier than) slot n+T3. - Re-evaluation and pre-emption checking are not always triggered simultaneously. Therefore, it is better to change “and” to “and/or” in the main bullet of re-evaluation and pre-emption part for partial sensing. - Furthermore, the UE behaviour for performing CPS should be under the condition that UE performs CPS. The pre-condition of UE performing CPS is omitted for aperiodic transmissions in re-evaluation and pre-emption part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change “after” to “from” in the definition of tyiSL. - Change “and” to “and/or” in the main bullet of re-evaluation and pre-emption part for partial sensing. - Add “When UE performs at least contiguous partial sensing” for aperiodic transmissions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Re-evaluation and pre-emption cannot work and resource re-selection is always triggered. - Cause confusion when only re-evaluation or pre-emption is triggered by higher layer. - The pre-condition of performing re-evaluation and/or pre-emption checking for aperiodic transmissions is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<omitted text>

When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and/or pre-emption checking, and if Prsvp_TX≠0,

-	During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot t'yiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to t'y+q×Prsvp_TX'SL, where t'ySL is a slot index of Y candidate slots used in the initial resource (re)selection.

-	t'yiSL is the first candidate slot from slot n+T3.

-	The UE performs PBPS for the remaining Y candidate slots according to ty'-k×Preserve''SL, wheret'y'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than t'yiSL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured.

-	The UE performs CPS starting from M logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL.

-	By default, M is 31 unless (pre-)configured with another value. by sl-CPS-WindowPeriodic.

When the UE performs at least contiguous partial sensing, and when the UE is triggered to perform re-evaluation and/or pre-emption checking, and if Prsvp_TX=0,

-	Candidate resource set (SA) is initialized to the remaining Y' candidate slots starting from slot t'yiSL and ending at the last slot of the Y' candidate slots, where t'yiSL is the first candidate slot from slot n+T3.

-	It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled

-	UE performs CPS starting from at least M consecutive logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL

-	For minimum size M of the CPS monitoring window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by sl-CPS-WindowAperiodic.

When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0.

<omitted text>