3GPP TSG RAN WG1 #110bis-e		R1-2208824

e-Meeting, October 10th – 19th, 2022

Source:	OPPO

Title:	Discussion on dynamic resource sharing in co-channel coexistence of LTE and NR SL

Agenda Item:	9.4.2

Document for:	Discussion and Decision

Introduction

The WID on NR SL evolution was revised in RAN#97-e with the following objective[1]:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4]

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

- Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A

According to the above RAN guidance in yellow highlight on co-channel coexistence, we continue to discuss the details of dynamic resource pool sharing and provide the corresponding system level simulation result in this paper.

Discussion

  - Dynamic resource pool sharing

During the last few meetings, the following agreements for dynamic resource pool sharing were achieved:

Agreement Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied. Agreement For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module. - FFS details on how the NR SL module uses this information. - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc. - FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL. - FFS: Whether/how device type B should be supported. Working assumption Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module. Agreement For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints: - NR SL resource pool is configured with 15 kHz SCS. - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools - For NR PSFCH (if configured), at least the following alternatives are studied: - Alt 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions. - FFS: Avoiding PSFCH transmissions can be performed by the UE transmitting PSFCH and/or the UE transmitting PSSCH. - Alt 2: NR SL UEs use a periodically repeating set of PSFCH slots. - FFS: periodicities of the set.

The main motivation to study the dynamic resource pool sharing is full utilization of all available radio resources allocated to either LTE or NR-based SL communications while transmission collisions can be largely avoided. And this advantage has been justified by the simulation result provided in section 2.2 especially in the urban scenario. In Figure 3, it can be observed that the semi-static partitioning will bring the performance loss for one of RATs compared with dynamic sharing method in the congested urban scenario. The performance loss is mainly due to the high congestion in the resource pool configured for one RAT but underutilization for the other RAT in semi-static case. In the last few meetings, it was already agreed that the feasibility of dynamic resource sharing as possible solutions for co-channel coexistence is to be studied. Therefore, we propose to support the dynamic resource sharing in current release considering the benefit and performance gain of it.

Proposal 1: Dynamic resource pool sharing between LTE SL and NR SL should be supported.

Based on the updated objective of co-channel coexistence, type A devices with dual modules ought to be considered with higher priority in dynamic resource sharing mechanism. For type A devices, LTE module will share sensing and resource reservation information to NR module. However, it is still unclear what the sensing and reservation information refers to. If the information is the SCI received by LTE module, some extra exchanges between LTE and NR module are needed such that NR module is able to understand the SCI format of LTE especially for the time gap and RIV fields. For example, the subframes for S-SSB transmissions and reserved subframes are shared from LTE module to NR module, so NR module can determine the subframes that may belong to LTE SL resource pool and further achieve reserved/indicated resources within the set of logical subframes. On the other hand, if the information refers to reserved resources determined based on the SCI received by LTE module, NR module can use the reserved resources directly during resource exclusion. In addition, the SL RSRP measurement result should be additionally included in the shared information as well.

Proposal 2: At least one of the following should be included in the information shared from LTE module to NR module:

- SCI monitored by LTE module or reserved resources determined based on the SCI monitored by LTE module

- SL RSRP measurement result

Proposal 3: If LTE module forwards its received SCI to NR module directly, the configuration used to determine the logical subframes of LTE SL need to be shared to NR module as well.

Another open issue is how NR module utilizes the sensing and resource reservation information from LTE module. In our view, it should be used during resource exclusion in physical layer of NR module because physical layer can guarantee that the candidate resource set SA contains a sufficient number of resources after excluding the resources occupied by LTE SL. In addition, it is much easier to handle the case in physical layer when some configurations of resource pool between LTE and NR SL are different. For example, if the subchannel size of LTE and NR SL resource pool is different, physical layer can deal with this case without extra enhancement because the resource overlapping in physical layer also includes partial overlapping.

Proposal 4: The sensing and resource reservation information shared by LTE module is used in physical layer of NR module as part of resource exclusion process.

From the perspective of NR module, it may use the shared information from LTE module during initial (re-)selection, re-evaluation and pre-emption checking. In current specification, the resource exclusion procedure in physical layer is common to initial selection, re-evaluation and pre-emption checking. Hence, if the shared information from LTE module is applied for resource exclusion in the physical layer of NR module, it is natural that it should be reused for re-evaluation and pre-emption as well. For example, the shared information is utilized to perform re-evaluation and/or pre-emption checking by NR module and trigger resource re-selection for corresponding resources.

Proposal 5: NR module ought to use the shared sensing and resource reservation information from LTE module for initial selection, re-evaluation and pre-emption checking.

The in-device coexistence between LTE SL and NR SL in different frequency channels was studied and discussed in Rel-16. And it was mainly handled by dropping the transmission of LTE or NR SL based on priority comparison, which may cause the performance loss of the RAT with lower priority. In co-channel coexistence work, the preferred approach is to avoid in-device collision instead of dropping transmissions. For instance, LTE module forwards the selected SL grant to NR module such that NR module can exclude the candidate resource overlapping with the SL grant in time domain.

Proposal 6: NR module performs resource exclusion based on the SL grant determined by LTE module to address the in-device coexistence issue in the same frequency channel.

During the last meeting, two alternatives in the yellow highlighted part of above agreements were proposed to address the AGC issue caused by PSFCH of NR SL. In alternative 1, NR SL UE tries to avoid performing PSFCH transmission in the slots which may be occupied by LTE SL UE. More specifically, NR SL UE may drop PSFCH transmission or exclude PSSCH resource during resource exclusion when the corresponding PSFCH overlaps with the resources reserved by LTE SL UE. In our understanding, alternative 1 has a large impact on the HARQ procedure in NR SL, which will lead to the degradation of reliability. For HARQ feedback option 1 (i.e., NACK-Only), if RX UE drops PSFCH transmission due to the collision with LTE SL, TX UE will think the data transmission has been decoded successfully and then doesn’t perform retransmissions anymore. For HARQ feedback option 2 (i.e., ACK-or-NACK), the PSFCH dropping will cause more retransmissions and congestion level increases in this way. Furthermore, even if no LTE transmission / reservation is detected by the LTE module in a subframe that overlaps with NR PSFCH slot, it does not guarantee that the transmission of PSFCH by the NR module will not cause interference / collision to LTE transmissions due to both hidden node issue and initial transmissions of LTE SL which may not be reserved/indicated in advanced.

The original intention of alternative 2 is that NR SL UE uses a set of periodic PSFCH resources and LTE SL UE can avoid using these resources by RSSI detection. In our opinion, alternative 2 cannot work because of different definitions for logical slots in LTE and NR SL. In LTE SL, RSSI is detected within the subframes that may belong to the LTE SL resource pool. However, PSFCH period is configured in the NR SL resource pool. That is, PSFCH resources are located within the slots that belong to the resource pool. Therefore, although RSSI period could be the multiples of PSFCH period, they are used in different set of logical slots. In addition, it is hard to align the subframes that may belong to LTE SL resource pool and the slots of NR SL resource pool due to different configurations of LTE and NR SL, such as the S-SSB configuration, bitmap of resource pool and so on.

Therefore, in order to address the AGC issue, the most straightforward way is to study dynamic resource sharing with the constraint that the slots containing PSFCH resource and the subframes of LTE SL resource pool are configured in a TDM manner.

Proposal 7: PSFCH resources of NR SL should be configured in a TDM manner with the resource pool of LTE SL.

  - System level simulation result

In this section, we provide the system level simulation results of both LTE and NR V2X UE deployed in the same frequency channel to evaluate the performance of reliability when different collision avoidance mechanisms (i.e., dynamic sharing of resources and semi-static partitioning) are used. As for the dynamic sharing mechanism, LTE and NR UE are configured with overlapped resource pool and NR UE can active avoid the collision with LTE UE based on the sensing result of LTE module. It should be noted that the LTE module of NR UE is only used for SCI collection and RSRP measurement in our simulation, so there is no overcrowding issue. For semi-static partitioning, the TDM resource pools are configured for LTE and NR UE. In our simulation, three typical scenarios are simulated and the corresponding simulation results are shown in Figure 1 to Figure 3, respectively. Furthermore, other simulation assumptions can be found in the appendix.

Scenario 1：NR UE with periodic traffic + LTE UE with periodic traffic + Freeway layout

Scenario 2：NR UE with aperiodic traffic + LTE UE with periodic traffic + Freeway layout

Scenario 3：NR UE with periodic traffic + LTE UE with periodic traffic + Urban layout

Figure 1 and Figure 2 illustrate the PRR of LTE and NR UE in scenario 1 and scenario 2, it is observed that there is no much difference for the performance of LTE UE or NR UE when dynamic sharing mechanism and semi-static approach are simulated. This is mainly because a relatively small number of UEs in freeway and the ratio of the slots included in NR resource pool to the subframes included in LTE resource pool is set according to the traffic of NR and LTE UE. It should be noted that the around 2% performance loss for the semi-static case of NR UE in scenario 2 is mainly due to the mismatch between the aperiodic traffic of NR UE and the periodic configured TDM resource pool.

Observation 1: There is no much difference for the PRR of LTE UE or NR UE when dynamic sharing and semi-static configuration are applied in freeway scenario.

However, although we still try to adjust the ratio of slots/subframes between NR and LTE resource pool in the semi-static case of scenario 3, the simulation results always show the performance loss of one RAT and the performance gain for another RAT compared with the dynamic sharing approach. In other words, it is hard to find a proper TDM ratio between NR and LTE resource pool in urban to avoid the performance loss of NR-based or LTE-based V2X RAT.

Observation 2: It is hard to find a proper TDM ratio between NR and LTE resource pool in urban scenario.

Figure 1 PRR of LTE and NR UE in scenario 1

Figure 2 PRR of LTE and NR UE in scenario 2

Figure 3 PRR of LTE and NR UE in scenario 3

Conclusion

In this contribution, we discussed the mechanism of dynamic resource pool sharing in co-channel coexistence, we have following observations and proposals:

Observation 1: There is no much difference for the PRR of LTE UE or NR UE when dynamic sharing and semi-static configuration are applied in freeway scenario.

Observation 2: It is hard to find a proper TDM ratio between NR and LTE resource pool in urban scenario.

Proposal 1: Dynamic resource pool sharing between LTE SL and NR SL should be supported.

Proposal 2: At least one of the following should be included in the information shared from LTE module to NR module:

- SCI monitored by LTE module or reserved resources determined based on the SCI monitored by LTE module

- SL RSRP measurement result

Proposal 3: If LTE module forwards its received SCI to NR module directly, the configuration used to determine the logical subframes of LTE SL need to be shared to NR module as well.

Proposal 4: The sensing and resource reservation information shared by LTE module is used in physical layer of NR module as part of resource exclusion process.

Proposal 5: NR module ought to use the shared sensing and resource reservation information from LTE module for initial selection, re-evaluation and pre-emption checking.

Proposal 6: NR module performs resource exclusion based on the SL grant determined by LTE module to address the in-device coexistence issue in the same frequency channel.

Proposal 7: PSFCH resources of NR SL should be configured in a TDM manner with the resource pool of LTE SL.

References

- RP-221938, “WID revision: NR sidelink evolution”, RAN #97e

- 3GPP RAN1 #109-e Chairman’s Notes, May 9th – 20th, 2022

- 3GPP RAN1 #110-e Chairman’s Notes, August 22nd – 26th, 2022

Appendix

Table 1 Simulation assumption for scenario 1

|  | NR UE | LTE UE |
| --- | --- | --- |
| Layout and UE dropping | Freeway Option A LTE UE:50% NR UE:50% |  |
| Solution for Inter-RAT interference | Case A: NR and LTE UE co-exist in a resource pool dynamically Case B: NR and LTE UE are configured with TDM resource pool separately (slots for NR: slots for LTE = 4:1). |  |
| Traffic model | Periodic traffic Model 2 specified in 37.885 with following changes: -Inter-packet arrival time: 50 ms -Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 -Latency requirement: 50 ms | Periodic traffic Model 1 specified in 37.885: -Inter-packet arrival time: 100 ms -Packet size: Pattern of {300 bytes, 190 bytes, 190 bytes, 190 bytes, 190 bytes} with random starting point for each UE -Latency requirement: 100 ms |
| Cast type | Broadcast (to NR UE only) | Broadcast (to LTE and NR UE) |
| Resource selection | R16 mode 2 full sensing (includes re-evaluation) | R14 mode 4 full sensing (includes backward indication and RSSI ranking) |
| Subchannel size | 20PRB |  |
| SCS | 15KHz |  |
| Bandwidth | 20MHz |  |
| Carrier frequency | 6GHz |  |
| Channel Model | TR 37.885 V2V Channel Model |  |
| Retransmission | 1 |  |

Table 2 Simulation assumption for scenario 2

|  | NR UE | LTE UE |
| --- | --- | --- |
| Layout and UE dropping | Freeway Option A LTE UE:50% NR UE:50% |  |
| Solution for Inter-RAT interference | Case A: NR and LTE UE co-exist in a resource pool dynamically Case B: NR and LTE UE are configured with TDM resource pool separately (slots for NR: slots for LTE = 4:1). |  |
| Traffic model | Aperiodic traffic Model 1 specified in 37.885 with following changes: -Inter-packet arrival time: 15 ms + an exponential random variable with the mean of 15 ms -Packet size: Uniformly random in the range between 200 bytes and 2000 bytes with the quantization step of 200 bytes -Latency requirement: 15 ms | Periodic traffic Model 1 specified in 37.885: -Inter-packet arrival time: 100 ms -Packet size: Pattern of {300 bytes, 190 bytes, 190 bytes, 190 bytes, 190 bytes} with random starting point for each UE -Latency requirement: 100 ms |
| Cast type | Broadcast (to NR UE only) | Broadcast (to LTE and NR UE) |
| Resource selection | R16 mode 2 full sensing (includes re-evaluation) | R14 mode 4 full sensing (includes backward indication and RSSI ranking) |
| Subchannel size | 20PRB |  |
| SCS | 15KHz |  |
| Bandwidth | 20MHz |  |
| Carrier frequency | 6GHz |  |
| Channel Model | TR 37.885 V2V Channel Model |  |
| Retransmission | 1 |  |

Table 3 Simulation assumption for scenario 3

|  | NR UE | LTE UE |
| --- | --- | --- |
| Layout and UE dropping | Urban Option A LTE UE:50% NR UE:50% |  |
| Solution for Inter-RAT interference | Case A: NR and LTE UE co-exist in a resource pool dynamically Case B: NR and LTE UE are configured with TDM resource pool separately (slots for NR: slots for LTE = 9:1 or 8:2 or 7:3 or 6:4 or 5:5). |  |
| Traffic model | Periodic traffic Model 2 specified in 37.885 with following changes: -Inter-packet arrival time: 50 ms -Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 -Latency requirement: 50 ms | Periodic traffic Model 1 specified in 37.885: -Inter-packet arrival time: 100 ms -Packet size: Pattern of {300 bytes, 190 bytes, 190 bytes, 190 bytes, 190 bytes} with random starting point for each UE -Latency requirement: 100 ms |
| Cast type | Broadcast (to NR UE only) | Broadcast (to LTE and NR UE) |
| Resource selection | R16 mode 2 full sensing (includes re-evaluation) | R14 mode 4 full sensing (includes backward indication and RSSI ranking) |
| Subchannel size | 20PRB |  |
| SCS | 15KHz |  |
| Bandwidth | 20MHz |  |
| Carrier frequency | 6GHz |  |
| Channel Model | TR 37.885 V2V Channel Model |  |
| Retransmission | 1 |  |