3GPP TSG RAN WG1 #110bis-e R1-2208865 e-Meeting, October 10th – 19th, 2022 CR-Form-v12.2. DRAFT CHANGE REQUEST. 38.213; CR; -; rev; -; Current version:; 17.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on HARQ-ACK retransmission. Source to WG:; OPPO. Source to TSG:; R1. Work item code:; NR_IIOT_URLLC_enh-Core; Date:; 2022-9-30. Category:; F; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19). Reason for change:; In RAN1 #107b, the following agreement was made for HARQ-ACK retransmission: Agreement For one-shot triggering of HARQ-ACK re-transmission, the HARQ_retx_offset is indicated by the bits of the MCS field for transport block 1.. When maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks, there are two MCS fields in DCI format 1_1. In current specification, the HARQ_retx_offset in indicated by a MCS field without any restriction.. Summary of change:; The MCS field for TB 1 in DCI format 1_1 is used to indicate the timing offset which is used to determine the retransmitted HARQ-ACK CB.. Consequences if not approved:; Unclear UE implementation when the UE receives a DCI format 1_1 which triggers HARQ-ACK retransmission, and maxNrofCodeWordsScheduledByDCI indicates reception of two transport blocks.. Clauses affected:; 9.1.5. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:; This is the first version for this CR..

### 9.1.5	HARQ-ACK codebook retransmission

*** Unchanged text is omitted ***

If the HARQ-ACK retransmission indicator field value in the DCI format 1_1 or 1_2 is '1', the UE determines slot m as m=n-l where l is determined by a one-to-one mapping in ascending order among the values of the MCS field for transport block 1 in the DCI format 1_1 or the MCS field in the DCI format 1_2 and the values from -7 to 24.

*** Unchanged text is omitted ***