3GPP TSG RAN WG1 #110bis-e		R1-2208875

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.1.4.1

Source:	Google

Title:	On Simultaneous Multi-Panel Transmission

Document for:	Discussion/Decision

# Introduction

In RAN1 #110, the following agreements on simultaneous multi-panel transmission have been achieved.

Working assumption Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme - In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e. - FFS: FDM-A scheme - FFS: FDM-B scheme - FFS: SFN-based transmission scheme For schemes other than SDM, final decision to support or not will be made in RAN1#110bis-e. Agreement For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2} for single CW case. - FFS on layer combinations of {1+3} and {3+1} considering the performance gain, system/UE complexity, specification efforts, etc. - FFS: the option of using layer combination of 0+n and n+0 for dynamic switch between single-panel and STxMP (n=1 or 2, 3 or 4). - This applies to SDM with 1 CW at least. Agreement To enhance the DMRS port indication for SDM scheme of STxMP PUSCH transmission in single-DCI based mTRP system, study the following aspects: - Whether the indicated DMRS ports can be in different or same CDM group? - How to determine the port partition among PUSCH layers. - How to map DMRS ports with PUSCH layers from different panels. - Whether to use one DCI field or two DCI fields for DMRS port indication - How to indicate layer combination that is used to partition DMRS port partition among PUSCH layers. - Other aspects are not precluded. Agreement Study and evaluate STxMP PUCCH based on the following: - For single-DCI based STxMP PUCCH transmissions, companies to provide the detailed description of the scheme being evaluated along with evaluation results in contribution. - For multi-DCI based STxMP PUCCH transmissions, transmitting two PUCCH resources with independent UCI payload to different TRPs with different UE panels that are fully or partially overlapping in time domain and partially/fully/non-overlapping in frequency domain can be considered. - Note: Companies can reuse the EVM assumptions of Rel-18 STxMP as agreed in RAN1#109-e (other than the parameters that are specific to PUSCH) as well as Rel-17 EVM for PUCCH as agreed in RAN1#102-e (PUCCH format, # of RBs/symbols, UCI payload, and Frequency hopping as shown below). - Baseline scheme can be Rel-15 PUCCH or Rel-17 mTRP PUCCH repetition. Parameters; Potential values. Baseline scheme; Rel-15 PUCCH or Rel-17 mTRP PUCCH repetition. PUCCH format; Format 1 and 3. Other PUCCH Formats can be optionally considered.. # of RBs/symbols; PUCCH Format 1: 4 symbols, 1 RB PUCCH Format 3: 4 and 8 symbols, 1 RB Other combinations are not precluded.. UCI payload; 2 bits for PUCCH Format 1 (and Format 0, if considered). Companies to report assumptions on other PUCCH Formats. Frequency hopping; Reported by companies.

In this contribution, we provide some discussions on simultaneous multi-panel transmission.

# General aspects

Several general assumption should be clarified on the enhancement of STxMP:

- Maximum number of active UE panels

- Maximum number of physical UE panels

- Number of ports per UE panel

- Enhancement for connected mode or connected mode + idle mode

Regarding the maximum number of active UE panels, since only up to 2 TRPs are supported. It is reasonable to assume up to 2 active UE panels for a UE, where each panel can be used for uplink signal toward one TRP. On the maximum number of physical UE panels, usually this can be considered an implementation related issue. But it should be clarified that whether the Rel-18 STxMP would introduce any restriction on the number of physical UE panels. In addition, the number of ports per panel should be clarified. Usually the number of ports per panel should be 2. Last, it needs to be clarified whether the enhancement should focus on connected mode only or both connected mode and idle mode. Since the mTRP can be enabled for connected mode UE, it should be more reasonable to consider STxMP for connected mode only. Thus, the PUSCH enhancement for MsgA/Msg3 should be precluded.

In addition, usually UE cannot always support STxMP as a result of overheating issue. When a UE keeps communicating with a gNB from a Tx panel, the overheating issue could happen for this panel. The UE has to turn off the panel. If there is no additional Tx panel, the UE cannot support STxMP when such overheating issue happens. Therefore, it is necessary to study the solution to the overheating issue for STxMP.

Proposal 1: Rel-18 STxMP only considers UE with up to 2 active panels.

Proposal 2: Rel-18 STxMP supports UEs with more physical panels than active panels.

Proposal 3: The number of antenna ports per UE panel should be assumed to be 2.

Proposal 4; Rel-18 STxMP only considers the enhancement for connected mode UE, i.e. enhancement for MsgA/Msg3 PUSCH should be precluded.

Proposal 5: Study the solution to the overheating issue for STxMP, where the UE has to turn off one panel when overheating is detected for that panel.

# STxMP on PUSCH

## Single DCI based STxMP

In RAN1 #110, the following working assumption on transmission scheme is achieved.

Working assumption Support the following schemes for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme - In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e.

Among the candidate schemes, SDM scheme is the only one that can increase the system throughput. Therefore, SDM scheme should be supported. Thus, the working assumption should be confirmed. One open issue is whether to support SFN based transmission scheme in addition to SDM scheme. With regard to reliability, SFN scheme should be considered.

Proposal 6: Confirm the working assumption to support SDM scheme for sDCI based STxMP PUSCH transmission.

Proposal 7: Support SFN scheme for sDCI based STxMP PUSCH transmission.

### SDM scheme

For SDM scheme, the first issue is to determine the indicated DMRS ports combinations. In RAN1 #110, the following agreement was achieved.

Agreement For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2} for single CW case. - FFS on layer combinations of {1+3} and {3+1} considering the performance gain, system/UE complexity, specification efforts, etc. - FFS: the option of using layer combination of 0+n and n+0 for dynamic switch between single-panel and STxMP (n=1 or 2, 3 or 4). - This applies to SDM with 1 CW at least.

As the number of ports per panel should be 2 and number of active panels should be 2, the total number of indicated DMRS ports should be 4 and the port combination 1+3 and 3+1 should be precluded. In addition, the combination 0+n and n+0 for dynamic switching between single-panel and STxMP could depend on supported schemes and how to configure the transmission scheme from the candidate schemes for PUSCH.

Proposal 8: For SDM based STxMP PUSCH, do not support layer combination {1+3} and {3+1}.

Proposal 9: For SDM based STxMP PUSCH, defer the decision for layer combination {0+n} and {n+0} after more details on transmission schemes for STxMP are agreed.

The second issue should be precoder indication for each panel. Since the beamformed channels observed in two panels can be different, the digital precoder should be different. Therefore, it is necessary to indicate two precoders for PUSCH transmission. Further, two SRS resource sets for CB/NCB should be required, where each SRS resource set correspond to one panel. Thus each SRS for CB can be configured with up to 2 ports, and the number of SRS resources for a SRS resource set for NCB should be up to 2.

For CB based transmission scheme, the gNB can indicate two sets of RI/PMI. The indication of DMRS port should be based on the indicated RIs. For NCB based transmission, the gNB can indicate two SRIs, where each SRIs corresponding to a panel. The number of indicated SRIs for the each SRIs can provide the indication of DMRS ports combination.

Proposal 10: For SDM based STxMP PUSCH, support to configure two SRS resource sets for CB/NCB, where each SRS resource set corresponds to one panel

- The number of ports for SRS resource set for CB should be up to 2.

- The number of SRS resources for a SRS resource set for NCB should be up to 2.

Proposal 11: For SDM based STxMP PUSCH, support to indicate two sets of RI/TPMI for DG/CG PUSCH for codebook-based transmission.

Proposal 12: For SDM based STxMP PUSCH, support to indicate two SRIs for DG/CG PUSCH for non-codebook based transmission.

### SFN scheme

For SFN scheme, the precoder indication should depend on the number of configured SRS resource sets for CB/NCB. There can be the following two options:

- Option 1: One SRS resource set for CB/NCB

- Option 2: Two SRS resource sets for CB/NCB

In option 1, the gNB can configure two TCI states for an SRS resource in an SRS resource set for CB/NCB. The UE can transmit the SRS in SFN manner, which is the same as the PUSCH. In option 2, the gNB can indicate one TCI state for each SRS resource in an SRS resource set for CB/NCB. To measure the uplink CSI for SFN manner, the gNB may need to configure fully overlapped resources for both SRS resource sets. Further, the gNB may need to send two DCIs to trigger the two SRS resource sets. Therefore, option 1 and option 2 can achieve the same functionality, but option 1 can reduce the SRS overhead compared to option 2. Therefore, the SFN scheme should be supported based on one SRS resource set for CB/NCB.

Proposal 13: For SFN based STxMP, support to configure one SRS resource set for CB/NCB.

## Multi-DCI based STxMP

The multi-DCI based operation can provide more flexible PUSCH multiplexing schemes. In Rel-16, the PUSCHs scheduled by mDCI can only be multiplexed in TDMed manner. With the helpful of STxMP, the PUSCHs can be multiplexed in a more complicated manner: FDMed manner, SDMed manner, hybrid FDM/TDM/SDM manner. Thus the PUSCHs may be configured with partially or fully overlapped resources.

Proposal 14: For mDCI based STxMP on PUSCH, support to multiplex the PUSCHs triggered by two DCIs with partially and fully overlapped resources.

For the PUSCH scheduled by each DCI, the gNB can grant the resources independently. But the PUSCH should only be transmitted from a single panel. Thus the maximum number of DMRS ports for the PUSCH should be up to 2. For each TRP, a separate SRS resource set for CB/NCB should be configured. Therefore, the gNB can configure 2 SRS resource sets where each SRS resource set can be associated with one CORESETPoolIndex. The number of ports for SRS resource set for CB should be up to 2.

Proposal 15: For mDCI based STxMP on PUSCH, support to indicate up to 2 DMRS ports in the DCI.

Proposal 16: Support to configure 2 SRS resource sets for CB/NCB, where each SRS resource set is associated with each CORESETPoolIndex.

- The number of ports for SRS resource set for CB should be up to 2.

- The number of SRS resources for a SRS resource set for NCB should be up to 2.

# STxMP on PUCCH

To support STxMP on PUCCH, in general, there can be two options:

- Option 1: To transmit UCI on two PUCCH resources with one beam indicated per resource

- Option 2: To transmit UCI on one PUCCH resource with two beams indicated

In option 1, more flexible PUCCHs multiplexing schemes can be supported. The two PUCCH resources can be multiplexed in FDM/TDM/SDM manner, which is like mDCI based PUSCH. In option 2, the gNB can indicate two TCI states for a PUCCH resource, and the UCI can be transmitted in different repetitions with different beams for the PUCCH resource. The PUCCH multiplexing schemes could not be as flexible as option 1. Since some multiplexing manner is hard to be configured, e.g. partial overlapped based multiplexing scheme. Therefore, compared to option 2, option 1 is preferred.

Proposal 17: Support to transmit UCI on two PUCCH resources, where each PUCCH resource is indicated with one TCI.

# Conclusion

In this contribution, we provided discussion on STxMP enhancement. Based on the discussion, the following proposals have been achieved.

Proposal 1: Rel-18 STxMP only considers UE with up to 2 active panels.

Proposal 2: Rel-18 STxMP supports UEs with more physical panels than active panels.

Proposal 3: The number of antenna ports per UE panel should be assumed to be 2.

Proposal 4; Rel-18 STxMP only considers the enhancement for connected mode UE, i.e. enhancement for MsgA/Msg3 PUSCH should be precluded.

Proposal 5: Study the solution to the overheating issue for STxMP, where the UE has to turn off one panel when overheating is detected for that panel.

Proposal 6: Confirm the working assumption to support SDM scheme for sDCI based STxMP PUSCH transmission.

Proposal 7: Support SFN scheme for sDCI based STxMP PUSCH transmission.

Proposal 8: For SDM based STxMP PUSCH, do not support layer combination {1+3} and {3+1}.

Proposal 9: For SDM based STxMP PUSCH, defer the decision for layer combination {0+n} and {n+0} after more details on transmission schemes for STxMP are agreed.

Proposal 10: For SDM based STxMP PUSCH, support to configure two SRS resource sets for CB/NCB, where each SRS resource set corresponds to one panel

- The number of ports for SRS resource set for CB should be up to 2.

- The number of SRS resources for a SRS resource set for NCB should be up to 2.

Proposal 11: For SDM based STxMP PUSCH, support to indicate two sets of RI/TPMI for DG/CG PUSCH for codebook-based transmission.

Proposal 12: For SDM based STxMP PUSCH, support to indicate two SRIs for DG/CG PUSCH for non-codebook based transmission.

Proposal 13: For SFN based STxMP, support to configure one SRS resource set for CB/NCB.

Proposal 14: For mDCI based STxMP on PUSCH, support to multiplex the PUSCHs triggered by two DCIs with partially and fully overlapped resources.

Proposal 15: For mDCI based STxMP on PUSCH, support to indicate up to 2 DMRS ports in the DCI.

Proposal 16: Support to configure 2 SRS resource sets for CB/NCB, where each SRS resource set is associated with each CORESETPoolIndex.

- The number of ports for SRS resource set for CB should be up to 2.

- The number of SRS resources for a SRS resource set for NCB should be up to 2.

Proposal 17: Support to transmit UCI on two PUCCH resources, where each PUCCH resource is indicated with one TCI.