3GPP TSG RAN WG1 #110bis-e		R1-2208877

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.2.1

Source:	Google

Title:	On General Aspects of AI/ML Framework

Document for:	Discussion/Decision

# Introduction

In RAN1 #110, the following agreements on general aspects of AI/ML Framework have been achieved.

Agreement Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Note: some of them to be refined - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative and pending terminology definition. Note: More aspects may be added as study progresses. Agreement The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Performance - Intermediate KPIs - Link and system level performance - Generalization performance - Over-the-air Overhead - Overhead of assistance information - Overhead of data collection - Overhead of model delivery/transfer - Overhead of other AI/ML-related signaling - Inference complexity - Computational complexity of model inference: FLOPs - Computational complexity for pre- and post-processing - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte) - Training complexity - LCM related complexity and storage overhead - FFS: specific aspects - FFS: Latency, e.g., Inference latency Note: Other aspects may be added in the future, e.g. training related KPIs Note: Use-case specific KPIs may be additionally considered for the given use-case. Working Assumption Terminology; Description. Online training; An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.). Offline training; An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions.. Note: It is encouraged for the 3gpp discussion to proceed without waiting for online/offline training terminologies. Working Assumption Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion. Terminology; Description. AI/ML model delivery; A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc.. Note: Companies are encouraged to bring discussions on various options and their views on how to define Level y/z boundary in the next RAN1 meeting.

In this contribution, we provide some discussion on general aspects of AI/ML framework.

# Collaboration Levels

In RAN1 #109, the following agreement on collaboration levels was achieved.

Agreement Take the following network-UE collaboration levels as one aspect for defining collaboration levels - Level x: No collaboration - Level y: Signaling-based collaboration without model transfer - Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary

One important aspect is whether the AI/ML is implemented based on a one-side operation or two-side operation. Such different options could have some impact on the signaling on AI/ML life cycle management. For example, if the AI/ML is in gNB side only, it may not be necessary for gNB to send signaling to UE with regard to AI/ML model activation/deactivation, AI/ML model selection and so on. While, if the AI/ML is in both gNB and UE side, the signaling could be different from the one side model, since gNB and UE should maintain the same understanding on AI/ML life cycle management. Therefore, it is not sufficient to differentiate the collaboration levels based on whether there is model transfer or not.

In addition, the boundary between Level x and Level y is unclear. From the 3 use cases, CSI/BM/positioning, signal for measurement and report should always be needed.  Therefore, there is no actual use case without any collaboration. But the key point is whether both sides know the AI/ML is actually enabled, so that signaling on AI/ML life cycle management is needed.

Based the discussion above, whether any life cycle management related signaling is needed should be the key factor to differentiate the collaboration. The collaboration level should be defined as follows:

- Level 0: Collaboration without AI/ML life cycle management related signaling and without AI/ML model transfer

- Level 1: Collaboration with AI/ML life cycle management related signaling and without AI/ML model transfer

- Level 2: Collaboration with AI/ML life cycle management related signaling and AI/ML model transfer

Proposal 1: Support the following collaboration levels for AI/ML:

- Level 0: Collaboration without AI/ML life cycle management related signaling and without AI/ML model transfer

- Level 1: Collaboration with AI/ML life cycle management related signaling and without AI/ML model transfer

- Level 2: Collaboration with AI/ML life cycle management related signaling and AI/ML model transfer

# AI/ML Life Cycle Management

In RAN1 #110, it was agreed to study several aspects on LCM for AI/ML as follows:

Agreement Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative Note: More aspects may be added as study progresses.

## Data Collection

There could be two use cases for data collection: one is the data collection for online training, and the other is the data collection for model monitoring. Compared to online training, offline training should be a more typical implementation choice. In Rel-18, data collection for online training should be deprioritized. The AI/ML cannot always produce perfect prediction. Thus, model monitoring could be necessary. Therefore, for data collection for LCM, the study should focus on data collection for model monitoring.

Proposal 2: The data collection for LCM should focus on data collection for model monitoring, and data collection for other purposes should be deprioritized in Rel-18.

## Model training

For AI/ML, offline training should be a more typical implementation choice compared to online training. Thus, the model training for LCM should focus on offline training. The online training can be considered in future releases if a deployment need is justified. For offline training, more than one models can be trained with regard to different scenarios and use cases.

For one-side model, the study should consider both cases when the model is trained in NW side or UE side. The model training in the NW side could be much easier than model training in the UE side, as the scenario for a gNB is already fixed but the UE needs to consider several different kinds of scenarios. However, the NW may not be able to get some data for training. For example, for AI/ML based positioning, the NW may not be able to get the actual UE position. Thus, the model training for one-side model should consider cases that the model is trained in NW side or UE side.

For two-side mode, there could be several options: 1) The models are trained in NW side and UE downloads the model from NW; 2) The models are trained in UE side and the UE uploads the model to NW; 3) The models are trained in both NW and UE side separately. Option 3) could potentially cause some mismatch between the NW and UE, which could be more complicated and potentially require more specification work. Rel-18 should focus on the first and the second option.

Proposal 3: Model training should focus on offline training in Rel-18, where more than one models can be trained with regard to different scenarios and use cases.

Proposal 4: For 1-side mode, Rel-18 SI should consider the following cases:

- Case 1a: The model is trained in NW side

- Case 1b: The model is trained in UE side.

Proposal 5: For 2-side mode, Rel-18 SI should consider the following cases:

- Case 2a: The models are trained in NW side and UE downloads the model from NW

- Case 2b: The models are trained in UE side and the UE uploads the model to NW

## Model inference operation

For 1-side mode, there can be two cases: one is that the model inference is in the same side as the model training; the other is that the model inference is in a different side from the model training. For the second case, AI/ML model transfer could be necessary, which could require more overhead compared to the first case. Thus, Rel-18 should focus on the first case. For 2-side mode, the model inference should be performed in both NW and UE sides separately. There should be no other cases.

Another aspect for model inference operation is to consider the UE complexity for parallel model inference. A UE may be configured with multiple reports for one or more than one use cases simultaneously. For example, a UE may be configured with multiple ML based CSI reports based on different CMRs, which requires the UE to perform the model inference operation simultaneously. Further, a UE may also be configured to perform ML based CSI report and ML based beam report. Thus, parallel model inference for the same or different AI/ML models should be studied.

Proposal 6: For 1-side mode, Rel-18 should focus on the scenario that the model inference and training are in the same side.

Proposal 7: Study parallel model inference based on the same or different AI/ML models.

## Model selection/activation/deactivation/switching/fallback operation

The model selection/switching should be applicable to the case when more than one AI/ML models are configured, defined or activated. For 1-side mode, if the model training and inference are in the same side, transparent model selection/switching should be sufficient. For 2-side mode, such model selection/switching could be necessary. The typical use case for the 2-side mode is CSI compression. Several models may be preconfigured for different scenarios or different performance (compression ratio). For models for different scenarios, the model can be selected by the NW. But for models with different performance, the model can be selected by the UE.  The UE may report the CSI with a model with proper compression ratio that can fit for the payload size restriction to avoid CSI omission.

The model activation/deactivation/fallback operation can be performed more dynamically. If the NW identifies an AI/ML model cannot work well based on model monitoring, it can deactivate the AI/ML model and fallback to non-ML based solution.  If the NW identifies an AI/ML model works better than non-ML based solution, it can activate the AI/ML model. As the performance for the AI/ML model could change quickly, which may depend on the channel condition, it is better to consider some lower layer signaling, e.g., MAC CE, for model activation/deactivation/fallback operation.

Proposal 8: For 1-side mode, the model selection/switching should be transparent.

Proposal 9: For 2-side mode, the model selection/switching can be configured by the NW or reported by the UE

Proposal 10: Consider to use lower layer signaling, e.g., MAC CE, for model activation/deactivation/fallback operation.

## Model monitoring

The model monitoring is a necessary step to decide whether to switch a new model or fallback to a non-ML based operation. For 1-side mode, the model monitoring can be done at the same side with the model inference and training. The other side may or may not provide some information to assist the model monitoring. For example, for spatial domain beam prediction, if the inference is in the NW side, the UE may report the best beam measured from all the DL RSs to the NW from non-ML based operation so that NW can decide whether the model is good enough for further spatial domain beam prediction. However, if the inference is in the UE side, the UE may measure all the DL RSs to identify the best NW beam and compared the performance for the non-ML selected beam and the ML selected beam. Then the UE can determine whether the AI/ML model can still work.

For 2-side mode, the model monitoring can be in both sides. Taking CSI compression as an example, the model monitoring can be in either NW side or UE side. The model monitoring can be based on the input of the CSI compression, i.e., uncompressed CSI, and the output of the CSI decompression, i.e., decompressed CSI. Then this may require one side to inform the uncompressed CSI or the decompressed CSI to the other side. Another possible way is to consider some performance related metric, e.g., BLER. If the BLER based on the decompressed CSI is higher than a threshold, it can be considered that the AI/ML based CSI compression cannot work well.

Proposal 11: For 1-side mode, the model monitoring should be performed at the same side with the model inference and training, and study necessary information from the other side to assist the model monitoring

Proposal 12: For 2-side mode, further study the following options for model monitoring

- Option 1: The model monitoring is based on the input for the AI/ML model in transmitter side and the output for the AI/ML model in receiver side

- Option 2: The model monitoring is based on some performance related metric, e.g., BLER, based on the output for the AI/ML model

## Model transfer and update

In RAN1 #109, the following conclusion has been achieved, where it is not expected to specify any AI/ML models.  Thus, for model transfer, the signaling would not be designed particularly based on one or s set of AI/ML models.

Conclusion As indicated in SID, although specific AI/ML algorithms and models may be studied for evaluation purposes, AI/ML algorithms and models are implementation specific and are not expected to be specified.

Since AI/ML models are not expected to be specified, the model transfer and update procedure could be deprioritized. One possible way for model transfer is to introduce some reserved bits in RRC signaling for model downloading or UE capability signaling for model uploading. The interpretation of such reserved bits could be handled through IODT. For each use case, it is possible that more than one models may be configured, and one or more than one models may be activated. For example, for CSI compression, it is possible that multiple models could be configured and activated, where each model correspond to one rank.

Proposal 13: Since AI/ML models are not expected to be specified, the model transfer and update procedure could be deprioritized.

## UE capability

The AI/ML based operation may require UE to perform inference. A high-performance UE may use dedicated hardware, neural processing unit (NPU), to process the inference. However, a low-performance UE may use general processing unit (GPU) to handle such operation. The high-performance UE may be able to proceed the inference with a short processing delay, but it may not be able to process multiple AI/ML operations simultaneously since the number of NPUs could be limited. The low-performance UE may be able to process multiple AI/ML operations as it does for non-AI/ML based operations, e.g. CSI report with multiple CPUs. However, the low-performance UE may not be able to proceed the AI/ML based operation with a small delay. Thus, for AI/ML based operation, the following UE types should be considered:

- Type 1 UE (low performance UE): AI/ML based operation is based on general processing unit (GPU)

- Type 2 UE (high performance UE): AI/ML based operation can be based on neural processing unit (NPU)

Proposal 14: For AI/ML based operation, the following UE types should be considered:

- Type 1 UE (low performance UE): AI/ML based operation is based on general processing unit (GPU)

- Type 2 UE (high performance UE): AI/ML based operation can be based on neural processing unit (NPU)

# Conclusion

In this contribution, we provided discussion on general aspects of AI/ML. Based on the discussion, the following proposals have been achieved.

Proposal 1: Support the following collaboration levels for AI/ML:

- Level 0: Collaboration without AI/ML life cycle management related signaling and without AI/ML model transfer

- Level 1: Collaboration with AI/ML life cycle management related signaling and without AI/ML model transfer

- Level 2: Collaboration with AI/ML life cycle management related signaling and AI/ML model transfer

Proposal 2: The data collection for LCM should focus on data collection for model monitoring, and data collection for other purposes should be deprioritized in Rel-18.

Proposal 3: Model training should focus on offline training in Rel-18, where more than one models can be trained with regard to different scenarios and use cases.

Proposal 4: For 1-side mode, Rel-18 SI should consider the following cases:

- Case 1a: The model is trained in NW side

- Case 1b: The model is trained in UE side.

Proposal 5: For 2-side mode, Rel-18 SI should consider the following cases:

- Case 2a: The models are trained in NW side and UE downloads the model from NW

- Case 2b: The models are trained in UE side and the UE uploads the model to NW

Proposal 6: For 1-side mode, Rel-18 should focus on the scenario that the model inference and training are in the same side.

Proposal 7: Study parallel model inference based on the same or different AI/ML models.

Proposal 8: For 1-side mode, the model selection/switching should be transparent.

Proposal 9: For 2-side mode, the model selection/switching can be configured by the NW or reported by the UE

Proposal 10: Consider to use lower layer signaling, e.g., MAC CE, for model activation/deactivation/fallback operation.

Proposal 11: For 1-side mode, the model monitoring should be performed at the same side with the model inference and training, and study necessary information from the other side to assist the model monitoring

Proposal 12: For 2-side mode, further study the following options for model monitoring

- Option 1: The model monitoring is based on the input for the AI/ML model in transmitter side and the output for the AI/ML model in receiver side

- Option 2: The model monitoring is based on some performance related metric, e.g., BLER, based on the output for the AI/ML model

Proposal 13: Since AI/ML models are not expected to be specified, the model transfer and update procedure could be deprioritized.

Proposal 14: For AI/ML based operation, the following UE types should be considered:

- Type 1 UE (low performance UE): AI/ML based operation is based on general processing unit (GPU)

- Type 2 UE (high performance UE): AI/ML based operation can be based on neural processing unit (NPU)