3GPP TSG RAN WG1 #110bis-e		                                                                  R1-2208921

e-meeting, October 10th – 19th, 2022

Agenda Item:	8.11

Source:	CATT, GOHIGH

Title:	Discussion on resource exclusion behavior with non-preferred resource set

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss the remaining issue on UE-B’s resource exclusion behavior with non-preferred resource set, the corresponding CR can be found in our contribution [1].

Discussions

In RAN1#107-e meeting, the following agreement on non-preferred resource set exclusion was achieved [2].

Agreement For Scheme 1 with non-preferred resource set, - Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

And the related exclusion behavior is endorsed in TS 38. 214 Section 8.1.4C as follows [3]:

- 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement.

Based on the agreement and specification text, UE-B only excludes the candidate resources overlapping with the non-preferred resource set. The potential issue is the resource collisions caused by periodical transmission cannot be avoided especially for condition-based scheme 1, where the resource coordination window identified by UE-A implementation is not necessarily consistent with resource selection window identified by UE-B. As shown in Figure 1, resource coordination window is partially overlapped with UE-B’s resource selection window. According to current specification, UE-B will only exclude resource R2 from the candidate resource set, but resource R3 will be ignored in resource exclusion procedure, which is outside UE-B’s resource selection window and is not overlapped with any candidate resources. Although the selected resource R1 is not collided with non-preferred resource set directly, UE-B’s next periodical transmission will occupy the same resource and will overlap with non-preferred resource R3.

Observation 1: The non-preferred resource set exclusion behavior based on current agreement and specification will cause potential collisions for periodical transmissions, especially for condition-based scheme 1.

Figure 1: Potential resource collision with non-preferred resource set for periodical transmission

Based on above discussion and proposed example, in additional to these resources overlapping with non-preferred resource set, UE-B should also exclude the ones from the candidate resources, which will cause potential collisions between UE-B’s periodical transmissions and non-preferred resource set.

Proposal 1: In additional to these resources overlapping with non-preferred resource set, UE-B should also exclude the ones from the candidate resources, which will cause potential collisions between UE-B’s periodical transmissions and non-preferred resource set.

Conclusions

In this contribution, the potential issues about non-preferred resource set exclusion are discussed and the solution is proposed. Our proposals and observations are given as follows:

Observation 1: The non-preferred resource set exclusion behavior based on current agreement and specification will cause potential collisions for periodical transmissions, especially for condition-based scheme 1.

Proposal 1: In additional to these resources overlapping with non-preferred resource set, UE-B should also exclude the ones from the candidate resources, which will cause potential collisions between UE-B’s periodical transmissions and non-preferred resource set.

References

  - R1-2208920, “Correction on resource exclusion behavior with non-preferred resource set”, CATT, GOHIGH, Sep. 2022.

  - 3GPP TSG RAN WG1 Meeting RAN1 #107-e Chairman's Notes, Nov. 2021.

  - Physical layer procedures for data (Release 17), V17.3.0, Sep. 2022.