3GPP TSG RAN WG1 # 110bis-e		                                                          R1-2208932

e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on BD/CCE budge of scheduling cell(s) for the features extending NR operation to 71 GHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-0X-XX |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When r17monitoringcapability of monitoringCapabilityConfig is configured, the following capability configurations on BD/CCE budget of scheduling serving cell(s) are not complete - Ncells,r15/r17cap-r17is defined for determining BD/CCE budget of scheduling serving cell(s) for μ=0,1,2. - Ncells,r16/r17cap-r17is defined for determining BD/CCE budget of scheduling serving cell(s) for μ=0,1. - Ncells,r16/{r17,r15}cap-r17is defined for determining BD/CCE budget of scheduling serving cell(s) for μ=0,1. - Ncells,r15/{r17,r16}cap-r17is defined for determining BD/CCE budget of scheduling serving cell(s) for μ=0,1,2,3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding missing monitoringCapabilityConfig configurations for determining BD/CCE budge of scheduling serving cell(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature of extending NR operation to 71GHz is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------- Start of text proposal to TS 38.213 v17.3.0 -----------

10.1 UE procedure for determining physical downlink control channel assignment

<Unchanged Text Omitted>

If a UE

-	does not report pdcch-BlindDetectionCA, pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2, or pdcch-BlindDetectionCA3, or is not provided BDFactorR, γ=R

-	reports pdcch-BlindDetectionCA, pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2, or pdcch-BlindDetectionCA3, the UE can be indicated by BDFactorR either γ=1 or γ=R

If a UE is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is not provided monitoringCapabilityConfig, or is provided monitoringCapabilityConfig = r15monitoringcapability and is not provided CORESETPoolIndex, with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor, on the active DL BWPs of the scheduling cells,

-	more than MPDCCHtotal,slot,μ=MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHtotal,slot,μ=CPDCCHmax,slot,μ non-overlapped CCEs per slot for each scheduled cell when the scheduling cell is from the Ncells,0DL,μ downlink cells, or

-	more than MPDCCHtotal,slot,μ=γ∙MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHtotal,slot,μ=γ∙CPDCCHmax,slot,μ non-overlapped CCEs per slot for each scheduled cell when the scheduling cell is from the Ncells,1DL,μ downlink cells

-	more than MPDCCHmax,slot,μ PDCCH candidates or more than CPDCCHmax,slot,μ non-overlapped CCEs per slot for CORESETs with same coresetPoolIndex value for each scheduled cell when the scheduling cell is from the Ncells,1DL,μ downlink cells

Ncellscap is replaced by Ncells,r15cap-r16, if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability. Ncellscap is replaced by Ncells, r15/r17cap-r17, if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability. Ncellscap is replaced by Ncells, r15/ {r16, r17} cap-r17, if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability.

If a UE

-	is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is not provided monitoringCapabilityConfig, or is provided monitoringCapabilityConfig = r15monitoringcapability and is not provided coresetPoolIndex,

-	with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cell(s) using SCS configuration μ, where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, and

-	a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell,

the UE is not required to monitor more than MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j  PDCCH candidates or more than CPDCCHtotal,slot,μ=Ncellscap⋅CPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the Ncells,0DL,μ+Ncells,1DL,μ downlink cells. Ncellscap is replaced by Ncells, r15cap-r16 if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability. Ncellscap is replaced by Ncells, r15/r17cap-r17, if a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability. Ncellscap is replaced by Ncells, r15/ {r16, r17} cap-r17.If a UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability.

For each scheduled cell from the Ncells,0DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot.

For each scheduled cell from the Ncells,1DL,μ downlink cells, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell

-	more than minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot

-	more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates or more than minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ non-overlapped CCEs per slot for CORESETs with same coresetPoolIndex value

If a UE is configured with Ncells,r16DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ≤Ncellscap-r16, the UE is not required to monitor, on the active DL BWP of the scheduling cell, more than MPDCCHtotal,(X,Y),μ=MPDCCHmax,(X,Y),μ PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=CPDCCHmax,(X,Y),μ non-overlapped CCEs per span for each scheduled cell when the scheduling cell is from the Ncells,r16DL,(X,Y),μ downlink cells. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability, Ncellscap-r16 is replaced by Ncells,r16cap-r16. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r16monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability,  Ncellscap is replaced by Ncells, r16/r17cap-r17. If a UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability,  Ncellscap is replaced by Ncells, r16/ {r15, r17} cap-r17.

If a UE is configured only with Ncells,r16DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs

-	per set of spans on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells within every X symbols, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y and any pair of spans in the set is within Y symbols, where first X symbols start at a first symbol with a PDCCH monitoring occasion and next X symbols start at a first symbol with a PDCCH monitoring occasion that is not included in the first X symbols

-	per set of spans across the active DL BWP(s) of all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells, with at most one span per scheduling cell for each set of spans, otherwise

where Ncells,r16DL,j is a number of configured cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration j. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability, Ncellscap-r16 is replaced by Ncells,r16cap-r16. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig = r16monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability,  Ncellscap is replaced by Ncells, r16/r17cap-r17. If a UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability and monitoringCapabilityConfig = r17monitoringcapability,  Ncellscap is replaced by Ncells, r16/ {r15, r17} cap-r17.

For each scheduled cell from the Ncells,r16DL,(X,Y),μ downlink cells using combination X,Y, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell, more than minMPDCCHmax,(X,Y),μ,MPDCCHtotal,(X,Y),μ PDCCH candidates or more than minCPDCCHmax,(X,Y),μ,CPDCCHtotal,(X,Y),μ non-overlapped CCEs per span.

A UE does not expect to be configured CSS sets, except for CSS sets provided by searchSpaceMCCH, searchSpaceMTCH or by SearchSpace in PDCCH-ConfigMulticast for DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI, that result to corresponding total, or per scheduled cell, numbers of monitored PDCCH candidates and non-overlapped CCEs per slot, per group of Xs slots for a corresponding combination Xs,Ys, or per span that exceed the corresponding maximum numbers per slot, or per group of Xs slots for a corresponding combination Xs,Ys, or per span, respectively.

For same cell scheduling or for cross-carrier scheduling, a UE does not expect a number of PDCCH candidates, and a number of corresponding non-overlapped CCEs per slot, or per group of Xs slots for a corresponding combination Xs,Ys, or per span, on a secondary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the secondary cell per slot, or per group of Xs slots for a corresponding combination Xs,Ys, or per span, respectively. If a UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, except the first span of each slot, the UE does not expect a number of PDCCH candidates and a number of corresponding non-overlapped CCEs per span on the primary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the primary cell per span.

If a UE is configured with Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs of the scheduling cells, and with Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ of the Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells using any combination Xs,Ys for a group of Xs slots for PDCCH monitoring, where μ=56Ncells,r17,0DL,μ+γ∙Ncells,r17,1DL,μ≤Ncellscap-r17, the UE is not required to monitor, on the active DL BWP of the scheduling cell,

-	more than MPDCCHtotal,Xs,μ=MPDCCHmax,Xs,μ PDCCH candidates or more than CPDCCHtotal,Xs,μ=CPDCCHmax,Xs,μ non-overlapped CCEs per group of Xs slots for each scheduled cell when the scheduling cell is from the Ncells,r17,0DL,Xs,μ downlink cells, or

-	more than MPDCCHtotal,Xs,μ=γ∙MPDCCHmax,Xs,μ PDCCH candidates or more than CPDCCHtotal,Xs,μ=γ∙CPDCCHmax,Xs,μ non-overlapped CCEs per group of Xs slots for each scheduled cell when the scheduling cell is from the Ncells,r17,1DL,Xs,μ downlink cells, or

-	more than MPDCCHmax,Xs,μ PDCCH candidates or more than CPDCCHmax,Xs,μ non-overlapped CCEs per group of Xs slots for CORESETs with same coresetPoolIndex for each scheduled cell when the scheduling cell is from the Ncells,r17,1DL,Xs,μ downlink cells

If the UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs, Ncellscap-r17 is replaced by Ncells,r17/r15cap-r17. If the UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r16monitoringcapability and downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs, Ncellscap-r17 is replaced by Ncells,r17/r16cap-r17. If the UE is configured with downlink cells for which the UE is provided monitoringCapabilityConfig = r15monitoringcapability and monitoringCapabilityConfig = r16monitoringcapability and downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs, Ncellscap-r17 is replaced by Ncells,r17/{r15,r16}cap-r17. If, for one or more of the cells, the UE is provided with monitoringCapabilityConfig = r16monitoringcapability, γ=1.

If a UE is configured Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the active DL BWPs of the scheduling cells, and with Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ of the Ncells,r17,0DL,μ+Ncells,r17,1DL,μ downlink cells using any combination Xs,Ys for a group of Xs slots for PDCCH monitoring, where μ=56Ncells,r17,0DL,μ+γ∙Ncells,r17,1DL,μ>Ncellscap-r17, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,Xs,μ=Ncellscap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=56Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j PDCCH candidates, or more than CPDCCHtotal,Xs,μ=Ncellscap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=56Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j non-overlapped CCEs, per group of Xs slots on the active DL BWP(s) of scheduling cell(s) from the Ncells,r17,0DL,Xs,μ+Ncells,r17,1DL,Xs,μ downlink cells where Ncells,r17,0DL,j+Ncells,r17,1DL,j is a number of configured cells with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration j. If the UE is configured Ncells,r17,0DL,j+Ncells,r17,1DL,j downlink cells for which the UE is provided both monitoringCapabilityConfig = r15monitoringcapability or monitoringCapabilityConfig = r16monitoringcapability, and monitoringCapabilityConfig = r17monitoringcapability for the active DL BWP, Ncellscap-r17 is replaced by Ncells,r17/r15cap-r17, or by Ncells,r17/r16cap-r17, or by Ncells,r17/{r15,r16}cap-r17, respectively, and MPDCCHtotal,Xs,μ=Ncells, refcap-r17⋅MPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=06Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j, CPDCCHtotal,Xs,μ=Ncells,refcap-r17⋅CPDCCHmax,Xs,μ⋅Ncells,r17,0DL,Xs,μ+γ∙Ncells,r17,1DL,Xs,μj=06Ncells,r17,0DL,j+γ∙Ncells,r17,1DL,j, and Ncells,refcap-r17 is one of Ncells,r17/r15cap-r17, Ncells,r17/r16cap-r17, or Ncells,r17/{r15,r16}cap-r17, respectively. If, for one or more of the cells, the UE is provided with monitoringCapabilityConfig = r16monitoringcapability, γ=1.

For each scheduled cell from the Ncells,r17,0DL,Xs,μ downlink cells using any combination Xs,Ys for a group of Xs slots, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell, more than minMPDCCHmax,Xs,μ,MPDCCHtotal,Xs,μ PDCCH candidates or more than minCPDCCHmax,Xs,μ,CPDCCHtotal,Xs,μ non-overlapped CCEs per group of Xs slots.

For each scheduled cell from the Ncells,r17,1DL,Xs,μ downlink cells using any combination Xs,Ys for a group of Xs slots, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell,

-	more than minγ∙MPDCCHmax,Xs,μ,MPDCCHtotal,Xs,μ PDCCH candidates or more than minγ∙CPDCCHmax,Xs,μ,CPDCCHtotal,Xs,μ non-overlapped CCEs per group of Xs slots

-	more than minMPDCCHmax,Xs,μ,MPDCCHtotal,Xs,μ PDCCH candidates or more than minCPDCCHmax,Xs,μ,CPDCCHtotal,Xs,μ non-overlapped CCEs per group of Xs slots for CORESETs with the same coresetPoolIndex value.

For cross-carrier scheduling, the number of PDCCH candidates for monitoring and the number of non-overlapped CCEs per span or per slot or per group of Xs slots are separately counted for each scheduled cell.

The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration μ in a slot if the UE is not provided monitoringCapabilityConfig for the primary cell or if the UE is provided monitoringCapabilityConfig = r15monitoringcapability for the primary cell, or in the first span of each slot if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, or in a group of Xs slots for a corresponding combination Xs,Ys if the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the primary cell, according to the following pseudocode.

If for the USS sets for scheduling on the primary cell the UE is not provided coresetPoolIndex for first CORESETs, or is provided coresetPoolIndex with value 0 for first CORESETs, and is provided coresetPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring.

In the following pseudocode, if the UE is provided monitoringCapabilityConfig = r16monitoringcapability for the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,(X,Y),μ and CPDCCHmax,(X,Y),μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,(X,Y),μ and CPDCCHtotal,(X,Y),μ respectively.

In the following pseudocode, if the UE is provided monitoringCapabilityConfig = r17monitoringcapability for the primary cell, MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are replaced by MPDCCHmax,Xs,μ and CPDCCHmax,Xs,μ respectively, and MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ are replaced by MPDCCHtotal,Xs,μ and CPDCCHtotal,Xs,μ respectively.

For all search space sets that a UE monitors PDCCH on the primary cell within a slot n, or within a group of Xs slots for a corresponding combination Xs,Ys, or within a span in slot n, denote by Scss a set of CSS sets, except for CSS sets provided by searchSpaceMCCH, searchSpaceMTCH or by SearchSpace in PDCCH-ConfigMulticast for DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI, with cardinality of Icss and by Suss a set of USS sets and CSS sets provided by searchSpaceMCCH, searchSpaceMTCH or by SearchSpace in PDCCH-ConfigMulticast for DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI with cardinality of Juss for scheduling on the primary cell. The location of search space sets sj, 0≤j<Juss, in Suss is according to an ascending order of the search space set index.

.

------------------------ End of Text proposal to TS 38.213 v17.3.0 ----------