3GPP TSG RAN WG1 #110bis-e                                                                           R1-2208986

e-Meeting, October 10th – 19th, 2022

Source:	CATT

Title:	Discussion on further complexity reduction for eRedCap UE

Agenda Item:	9.6.1

Document for:	Discussion and Decision

Introduction

In Rel-17, a new type of UE with reduced capability, i.e. RedCap UE, was introduced to support scenarios with middle transmission requirements, such as industrial sensors, video surveillance, and wearables. To further expand the market for RedCap use cases with relatively low cost, low energy consumption, and low data rate requirements, further complexity reduction is considered in Rel-18. After a short study phase, a new WID was approved in RAN#97-e to further reduce UE complexity for RedCap UE in Rel-18 [1], i.e. eRedCap.

In this contribution, we provide our views on these potential solutions for Rel-18 eRedCap evolution. In addition, we analyze coexistence issue between Rel-18 eRedCap UE, non-RedCap UE and Rel-17 RedCap UE.

Discussion

UE BB bandwidth reduction

Reducing BB bandwidth is one of the solutions for complexity reduction, as stated by the WID [1].

- Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth.

The above description mainly inherits from Option BW3 as recommended in TR 38.865 [2]. Based on the definition above, we can see that a BWP up to 20 MHz can still be configured, just like Rel-17 RedCap UE in FR1. The only difference is that the frequency domain resource for PDSCH/PUSCH is restricted within 5 MHz within the BWP.

Observation 1: For Rel-18 eRedCap UE with 5 MHz BB bandwidth, a BWP up to 20 MHz can be configured, while only the frequency domain resource for PDSCH/PUSCH is restricted within 5 MHz within the BWP.

Clarification of allocable PRB number within 5 MHz

So far, RAN4 has defined the maximum transmission bandwidth of different UE channel bandwidths. For a bandwidth of 5 MHz, as specified in TS 38.101-1, the maximum PRB number is 25 for SCS = 15 kHz and 11 for SCS = 30 kHz. Such definition is under the assumption that RF bandwidth is 5 MHz. Therefore, some PRBs at the edge are reserved to alleviate neighbor band interference, leading to the consequence that the BB bandwidth is significantly smaller than 5 MHz.

To be specific, in current TS 38.101-1 [3]:

- Maximum transmission bandwidth of SCS = 15 kHz is 4.5 MHz (15 kHz * 12 * 25),

- Maximum transmission bandwidth of SCS = 30 kHz is 3.96 MHz (30 kHz * 12 * 11).

However, for Rel-18 eRedCap UE, only the BB bandwidth for PDSCH/PUSCH is restricted in 5 MHz, while the RF bandwidth is kept as 20 MHz. For signals/channels other than PDSCH/PUSCH, e.g. CORESET or CSI-RS, they can use a bandwidth up to 20 MHz. Therefore, RAN4 definition on PRB number of 5 MHz transmission bandwidth (in RF) is not suitable for this case.

From view of BB bandwidth reduction only, the allocable PRB number no larger than 5 MHz should be:

- BB bandwidth of SCS = 15 kHz is 4.86 MHz (15 kHz * 12 * 27),

- BB bandwidth of SCS = 30 kHz is 4.68 MHz (30 kHz * 12 * 13).

Table 1 summarizes the different PRB numbers of ‘5 MHz for RF+BB’ and ‘5 MHz for BB only’.

Table 1 PRB number within 5 MHz.

| SCS | RAN4-defined maximum transmission bandwidth (5 MHz for both RF and BB) | Maximum PRB number within 5 MHz for BB bandwidth reduction only |
| --- | --- | --- |
| 15 kHz | 25 | 27 |
| 30 kHz | 11 | 13 |

We need to clarify the allocable PRB number first for Rel-18 eRedCap UE. This not only makes the specification more clear, but also impacts the relaxed constraint for Option PR1. From our view, larger PRB number is preferred for higher flexibility and better resource utilization.

Proposal 1: For Rel-18 eRedCap UE with 5 MHz BB bandwidth, confirmed the allocable PRB number:

- For SCS with 15 kHz, the maximum PRB number is 27.

- For SCS with 30 kHz, the maximum PRB number is 13.

Restricting the bandwidth of PDSCH/PUSCH

The following observation is captured in TR 38.865 [2]:

For BW3, the main contributors of the complexity reduction are the following functional blocks: - Baseband: Post-FFT data buffering - Baseband: Receiver processing block - Baseband: LDPC decoding - Baseband: HARQ buffer - Baseband: UL processing block Note 1: BW3 may have different degrees of impacts on the post-FFT data buffering depending on the scheduling aspects (cross-slot scheduling, RF retuning, etc.).

During the study, it is observed that companies have different understandings on the cost of post-FFT data buffering for Option BW3. In general, without any optimization, eRedCap UE still needs to buffer the post-FFT data of the whole BWP, until the UE finishes PDCCH detection and DCI decoding. Then, only after FDRA information is acquired from DCI, eRedCap UE can pick out the PDSCH data within 5 MHz from the whole BWP bandwidth.

Figure 1 illustrates the post-FFT data buffering in case no optimization is applied.

Figure 1 UE buffers post FFT data of the whole BWP, without any optimization.

If it is desired to reduce the complexity of post-FFT data buffering, some scheduling optimization is required.  Two directions can be considered. One direction is to ensure that the delay between scheduling PDCCH and scheduled PDSCH is larger than the required time for PDCCH detection and DCI decoding, i.e. cross-slot scheduling. Another direction is to utilize some prior knowledge on the used 5 MHz, and thus the UE knows where to buffer the used 5 MHz before decoding DCI. The frequency domain resource of the PDSCH is within the 5 MHz that known beforehand. Figure 2 illustrates these two optimizations directions.

Figure 2 UE buffers post FFT data within 5 MHz, with scheduling optimization.

Therefore, we should determine whether to introduce scheduling optimization, for the purpose of reducing post-FFT data buffering. In general, if scheduling optimization is not introduced, the complexity reduction of Option BW3 is almost the same as Option PR3, since both of them need to buffer post-FFT data of the whole BWP. In this case, PR3 may be even more attractive, since the allocated PRBs can span the whole BWP flexibly. Since Option BW3 is targeted, we prefer to further reduce the UE complexity as much as possible. Scheduling optimization should be studied for the purpose of reducing post-FFT data buffering.

Proposal 2: Further study scheduling optimization for the purpose of reducing post-FFT data buffering, at least including the following directions.

- Cross-slot scheduling.

- Prior knowledge of used 5 MHz.

Peak data rate reduction

Another possible solution for complexity reduction is to limit the peak data rate. Following the recommendation of TR 38.865 [2], Option PR1 is also included in the WID [1].

- UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap.

The key factor of Option PR1 is to reduce the complexity of HARQ buffer, as well as the related decoding and processing block. Hence, the constraint of (vLayers, Qm, f) can be reduced from 4 to X. Smaller X helps larger complexity reduction. However, the TBS should still be able to fulfill the targeted data rate of Rel-18 eRedCap UE, i.e. 10 Mbps. Therefore, X should be carefully determined, considering the potential combination with Option BW3:

- If PR1 is standalone, X is determined based on the assumption of BW=20 MHz. In this case, X can be as low as 0.8. To provide some margin, X=1 is also considerable.

- If PR1 is combined with BW3, X is determined based on the assumption of BW=5 MHz. In this case, X=3 can be considered.  Otherwise, the target data rate may not be fulfilled if smaller X is adopted.

We have the following proposal:

Proposal 3: For peak data rate reduction,

- If it is standalone, the constraint of vLayers·Qm·f can be reduced to 0.8 or 1.

- If it is combined with bandwidth reduction, the constraint of vLayers·Qm·f can be reduced to 3.

Coexistence with non-RedCap UE and Rel-17 RedCap UE

Separate initial DL/UL BWP

In Rel-17, separate initial DL/UL BWP is introduced for RedCap UE. One important purpose is to avoid restricting the bandwidth of legacy initial DL/UL BWP. Another purpose is to alleviate the potential congestion in the legacy initial DL/UL BWP. The design of separate initial DL/UL BWP sufficiently considers the characteristics of RedCap UE. For example, the corresponding bandwidth is no larger than 20 MHz in FR1.

For Rel-18 eRedCap UE, we do not see strong motivation to introduce new separate initial DL/UL BWP. Reusing Rel-17 separate initial DL/UL BWP is preferred. Note that eRedCap UE can support a BWP up to 20 MHz, which will not further restrict the bandwidth of Rel-17 separate initial DL/UL BWP. There is also no evidence showing that congestion becomes a serious problem for Rel-17 separate initial DL/UL BWP. On the other hand, if new separate initial DL/UL BWP is introduced for Rel-18 eRedCap UE, the fragmentation of the carrier bandwidth is difficult for gNB to handle.

Proposal 4: Rel-17 separate initial DL/UL BWP can be reused for Rel-18 eRedCap UE. No new separate initial DL/UL BWP is introduced in Rel-18.

Recall that, the configuration of separate initial DL/UL BWP is carried by legacy SIB1. However, both legacy SIB1 and corresponding scheduling common PDCCH may occupy a bandwidth larger than 5 MHz, i.e. up to the largest size of CORESET#0 (17.28 MHz). According to the simulation result averaged from companies [2], serious coverage loss is observed for both SIB1 and common PDCCH. Fortunately, coverage recovery is not so urgent, and PUSCH is still the bottleneck channel in most cases. Rel-18 eRedCap UE should be assumed to be able to read legacy SIB1. Whether further optimization is needed, e.g. to improve the coverage is FFS.

Proposal 5: Rel-18 eRedCap UE is assumed to be able to read legacy SIB1.

- FFS further optimization for SIB1 transmission/reception is needed or not.

Early indication for Rel-18 eRedCap UE

Similar to Rel-17, there may be some cases where early indication is not needed for Rel-18 eRedCap UE. For instance, if legacy initial DL/UL BWP is within 20 MHz which is shared by all UEs, and PDSCH/PUSCH is scheduled within 5 MHz before finishing initial access, early indication is unnecessary. This means early indication for Rel-18 eRedCap UE should not be mandated for network.

Observation 2: Early indication for Rel-18 eRedCap UE is useless in some cases.

On the other hand, early indication is still helpful especially when different scheduling strategies of some channels are desired. For example, if eRedCap UE is identified by Msg1, the corresponding Msg2 can be scheduled in a cross-slot scheduling manner or scheduled within pre-known 5 MHz to reduce the post-FFT data buffering, while such restriction will not impact non-RedCap UE or Rel-17 RedCap UE.

Observation 3: Early indication in Msg1 for Rel-18 eRedCap UE can reduce post-FFT data buffering for Msg2 by cross-slot scheduling or pre-known used 5 MHz, while non-RedCap UE and Rel-17 RedCap UE will not be impacted.

Note that early indication in Msg3 is mandatory for Rel-17 RedCap UE. It is realized by LCID report, which does not impact physical layer. This mechanism, in our view, can be easily reused without much effort. But the final determination can be left to RAN2 since it is a RAN2 solution. For early indication in Msg1, the specification impact is larger if early indication for Rel-18 eRedCap UE is independent from Rel-17 RedCap UE. As discussed in Section 2.1.2, whether early indication in Msg1 (different from Rel-17 RedCap UE) is supported or not depends on whether we would like to reduce the cost of post-FFT data buffering.

In summary, we have the following two proposals:

Proposal 6: Early indication in Msg1 for Rel-18 eRedCap UE is supported and configurable,

- Sharing RACH resource with non-RedCap UE is supported.

- Sharing RACH resource with Rel-17 RedCap UE is supported.

- FFS whether separate RACH resource (different from that of non-RedCap UE and Rel-17 RedCap UE) is supported or not, which depends on whether scheduling optimization for reducing post-FFT data buffering (at least for Msg2) is supported or not.

Proposal 7: Whether/how to support early indication in Msg3 for Rel-18 eRedCap UE is left to RAN2.

Cell access/barring of Rel-18 eRedCap UE

For Rel-17 RedCap UE, whether it can access to a cell or not depends on the following cell-specific indications:

- Cell barring indication in MIB

- Presents of RedCap-specific IFRI in SIB1

- Rx-specific indication for RedCap UE in SIB1

- Half-duplex-specific indication in SIB1 (in paired-spectrum)

We think the above cell access/barring indication can be reused for Rel-18 eRedCap UE. It is unlikely that a cell supporting HD-FDD for Rel-17 RedCap UE cannot support HD-FDD for Rel-18 eRedCap UE. Similarly, a cell forbidding the access of 1Rx Rel-17 RedCap UE may not be interested in serving 1Rx Rel-18 eRedCap UE. Whether additional cell access/barring indication for Rel-18 eRedCap UE is needed or not can be FFS. But the final determination can be left to RAN2.

Proposal 8: Rel-18 eRedCap UE shares the same cell access/barring indication and mechanism with Rel-17 RedCap UE.

- FFS additional cell access/barring indication for Rel-18 eRedCap UE.

- Final decision is up to RAN2.

Conclusion

In this contribution, we share our views on further complexity reduction for Rel-18 eRedCap UEs. The observations and proposals are summarized as follows:

Observation 1: For Rel-18 eRedCap UE with 5 MHz BB bandwidth, a BWP up to 20 MHz can be configured, while only the frequency domain resource for PDSCH/PUSCH is restricted within 5 MHz within the BWP.

Observation 2: Early indication for Rel-18 eRedCap UE is useless in some cases.

Observation 3: Early indication in Msg1 for Rel-18 eRedCap UE can reduce post-FFT data buffering for Msg2 by cross-slot scheduling or pre-known used 5 MHz, while non-RedCap UE and Rel-17 RedCap UE will not be impacted.

Proposal 1: For Rel-18 eRedCap UE with 5 MHz BB bandwidth, confirmed the allocable PRB number:

- For SCS with 15 kHz, the maximum PRB number is 27.

- For SCS with 30 kHz, the maximum PRB number is 13.

Proposal 2: Further study scheduling optimization for the purpose of reducing post-FFT data buffering, at least including the following directions.

- Cross-slot scheduling.

- Prior knowledge of used 5 MHz.

Proposal 3: For peak data rate reduction,

- If it is standalone, the constraint of vLayers·Qm·f can be reduced to 0.8 or 1.

- If it is combined with bandwidth reduction, the constraint of vLayers·Qm·f can be reduced to 3.

Proposal 4: Rel-17 separate initial DL/UL BWP can be reused for Rel-18 eRedCap UE. No new separate initial DL/UL BWP is introduced in Rel-18.

Proposal 5: Rel-18 eRedCap UE is assumed to be able to read legacy SIB1.

- FFS further optimization for SIB1 transmission/reception is needed or not.

Proposal 6: Early indication in Msg1 for Rel-18 eRedCap UE is supported and configurable,

- Sharing RACH resource with non-RedCap UE is supported.

- Sharing RACH resource with Rel-17 RedCap UE is supported.

- FFS whether separate RACH resource (different from that of non-RedCap UE and Rel-17 RedCap UE) is supported or not, which depends on whether scheduling optimization for reducing post-FFT data buffering (at least for Msg2) is supported or not.

Proposal 7: Whether/how to support early indication in Msg3 for Rel-18 eRedCap UE is left to RAN2.

Proposal 8: Rel-18 eRedCap UE shares the same cell access/barring indication and mechanism with Rel-17 RedCap UE.

- FFS additional cell access/barring indication for Rel-18 eRedCap UE.

- Final decision is up to RAN2.

References

- RP-222675, New WID on enhanced support of reduced capability NR devices, Ericsson, RAN#97-e, e-Meeting, September 12th – 16th, 2022.

- TR 38.865, Study on further NR RedCap UE complexity reduction, v18.0.0.

- TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, v17.6.0.