3GPP TSG RAN WG1 #110-bis-e		    													             R1-2209057

e-Meeting, October 10th – 19th, 2022

Source: 	Intel Corporation

Title:	Evaluation Methodologies for SL Operating in FR-2 Licensed Band

Agenda item:	9.4.3

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN meeting #94, a new work item related to NR Sidelink (SL) evolution was approved, whose description has been recently updated during the latest RAN meeting [1] to include an additional agenda item starting from RAN1 #110-e devoted to update the evaluation methodology to enable evaluations of SL FR2 in licensed band. In fact, as part of the objectives of this Working Item (WI), that following aspects were included:

- Study and specify enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2, RAN4] (Determine in RAN#98-e whether to continue the study or study + specification work for FR2 until the end of R18) - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Work is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

In the context of updating the evaluation methodology to enable evaluations of SL FR2 in licensed band, the remaining of this document provides our view on the necessary changes required to study enhancements for SL FR2, while view on other aspects are provided in our companion contributions [3-5].

# General discussion

During the initial standardization of NR SL for Rel-16 some features to support SL FR2 were already developed. Some examples of such features are operation with 120 kHz SCS and phase tracking reference signals. However, beam management has not been standardized yet. As so far in the framework of 3GPP, the beam management operation for the SL has never been standardized for any past release and this could be considered as a major additional step. For FR2, beam management is essential as in this frequency range all current systems rely on antenna arrays with analog beamforming to enable sufficient coverage.

Figure 1 – Illustration of SL FR2 beamforming for V2X.

Figure 1 illustrates the SL FR2 beamforming operation. Note that beamforming is used at both the transmitter and receiver. It is also important to note that to achieve a spherical (or semi-spherical) coverage multiple antenna panels are necessary.

Figure 2 – Operation modes of SL FR2.

In our understanding it is necessary to also support mode-1 and mode-2 operation for SL FR2 beam management. As illustrated in Figure 2 these represent the gNB controlled and autonomous resource allocation in combination with beam management.

## Frequency range

In our current understanding, there is no FR2-2 licensed spectrum available (note we do not consider the available ITS spectrum in FR2-2 in Europe as a licensed spectrum).

  - There is currently no licensed FR2-2 band

The current WID does only specify licensed FR2 spectrum. In our interpretation this does mean that only FR2-1 does need to be studied. Thus, only evaluation methodology for this frequency range will be specified. In addition, currently only up to 120 kHz SCS is specified for the SL, if SL operation in FR2-2 should be supported it is also necessary to add support for a larger SCS.

  - Define evaluation methodology only for FR2-1

## Link level evaluation

No simulation parameters for link level simulations for systems operating with a carrier frequency above 6 GHz were defined in 37.885 [2]. For beam management in the context of the SL single links are not the major target for the evaluation. Thus, as there is limited benefit from beam management-based evaluations using link level simulations and as this would be a major effort to define a SL channel model for SL link level evaluations for systems with a carrier frequency above 6 GHz there is no need to define a SL FR2 LLS evaluation methodology.

  - No SL FR2 LLS evaluation methodology is defined

# System level simulations methodology

In the latest version of the NR SL evaluation methodology in 37.885 [2] most of the simulation models for SLS regarding operation in the frequency range for FR2-1 is already defined. The definition that can be reused are regarding:

- Evaluation Scenario

- UE drop and mobility modeling

- BS and UE-type RSU deployment

- Antenna model

- Traffic model

- Pathloss model

- Shadowing model

- Fast fading model

Thus, it is reasonable to use the status of 37.885 [2] as the starting point for the SL FR2 beam management evaluation assumption methodology.

  - Use FR2-1 SLS evaluation methodology defined in 37.885 as a starting point

## Performance metrics

The main missing part in the current version of 37.885 [2] regarding beam management are the performance metrics. One metric to define the communication quality with after beam acquisition needs to be defined. To also consider the changing quality over time we think the CDF of the wideband SINR with beamforming is defined to represent this. In addition, one metric should also capture the time that is required for the initial beam acquisition as this can be used to compare different signaling and reference signal designs.

  - Define the following performance metrics for the SL beam management evaluations:

    - CDF of wideband SINR with beamforming

    - Time for beam acquisition

# Conclusions

In this contribution, we discussed updated to the SL evaluation methodology. Further, we summarize the proposals as follows:

  - Define evaluation methodology only for FR2-1

  - No SL FR2 LLS evaluation methodology is defined

  - Use FR2-1 SLS evaluation methodology defined in 37.885 as a starting point

  - Define the following performance metrics for the SL beam management evaluations:

    - CDF of wideband SINR with beamforming

    - Time for beam acquisition

  - There is currently no licensed FR2-2 band

# References

- RP-221938, “WID revision: NR sidelink evolution”, OPPO, Electronic Meeting, September 2022.

- TR 37.885, “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR”, v15.3.0, June 2019.

- R1-2209054, “Channel Access Mechanisms for SL Operating in Unlicensed Spectrum”, Intel Cooperation, Electronic Meeting, October 2022.

- R1-2209055, “Physical Channel Design for SL Operating in Unlicensed Spectrum”, Intel Cooperation, Electronic Meeting, October 2022.

- R1-2209056, “Considerations for LTE Sidelink and NR Sidelink Co-channel Coexistance”, Intel Cooperation, Electronic Meeting, October 2022.