3GPP TSG RAN WG1 #110bis-e	                                                   	R1-2209074

e-Meeting, October 10th – 19th, 2022

Source:	Intel Corporation

Title:	On Timing Advance Management

Agenda item:	9.12.2

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss timing advance (TA) management for Rel-18 mobility enhancement WI [1]

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Timing Advance management [RAN1, RAN2] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

# L1/L2 based mobility framework

The goal of L1/L2 based mobility is to allow a serving cell change based on L1/L2 signalling without a trigger from a RRC-Reconfiguration (reconfiguration with sync) message used to perform handover. The motivation is to reduce the latency, overhead and interruption time. A high-level signal flow over the air-interface for L1/L2 based mobility is shown below:

Figure 1: High level signal flow for L1/L2 based mobility

As shown above, a framework for L1/L2 based mobility may comprise of the following signal-flows:

- RRC configuration for multiple candidate target cells: In order to enable a low-overhead L1/L2 based mobility, a significant portion of the target cell information (SpCellConfig) can be pre-configured to a UE via RRC. This can be done for one or more target cells. This includes RACH configuration and a type of RACH procedure (including full-RACH, 2-step RACH or RACH-less) associated with a target cell

- L1 measurement reporting: L1 measurement and reporting for multiple candidate target cells can be faster than L3 measurement and reporting. This is addressed in our companion contribution in [XXX]

- RRC configuration for the target cell: Further RRC configuration specifically associated for the target cell can be used to provide more information needed for connecting to the target cell

- L1/L2 mobility trigger: A L1/L2 mobility trigger indicates to a UE a particular target cell for handover

- RACH procedure/UL Transmission: Upon receiving a L1/L2 mobility trigger and if a new TA for the target cell is desired, a UE initiates a RACH procedure. If TA is known, then UE initiates an UL transmission to the target cell.

# TA management specifications in RAN1

In terms of TA management, based on the above framework for L1/L2 based mobility, the following specification aspects could be identified:

Pre-configuration of RACH and RACH procedure types: A UE could be configured with a RACH procedure type for each candidate target cell and an associated RACH configuration. This could allow both CFRA and CBRA, 2-step RACH as well as RACH-less access options. A RACH-less access option can be envisioned in a manner similar to LTE where a rach-Skip configuration allows a UE to perform PUSCH transmission to a target PCell with a TA value of 0 or the latest TA value used for a source cell.

TA acquisition based on L1/L2 mobility trigger: A L1/L2 mobility trigger could be used to provide the following information to a UE

  - Index to a configured candidate target cell

  - RACH preamble for CBRA or CFRA indication

  - SSB index of the target cell

  - PRACH Mask

A L1/L2 mobility trigger could be based on DCI or MAC-CE. Upon receiving a L1/L2 mobility trigger, a UE initiates a RACH procedure to acquire a TA value or initiates an UL transmission with an known TA value.

Proposal-1: Consider RRC pre-configuration of RACH and RACH procedure types (including RACH-less) for one or more candidate target cells

Proposal-2: Consider defining a L1/L2 mobility trigger (DCI or MAC-CE) that includes an index to a configured candidate target cell, RACH preamble, SSB index of the target cell and a PRACH Mask

# Conclusions

Proposal-1: Consider RRC pre-configuration of RACH and RACH procedure types (including RACH-less) for one or more candidate target cells

Proposal-2: Consider defining a L1/L2 mobility trigger (DCI or MAC-CE) that includes an index to a configured candidate target cell, RACH preamble, SSB index of the target cell and a PRACH Mask

# References

- RP-213565, New WID on Further NR mobility enhancements, MediaTek, 3GPP TSG RAN #94e, Electronic Meeting, Dec. 6 - 17, 2021