3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209084

# e-meeting, 10 – 19 October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RB set size for Rel-17 IAB FDM multiplexing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1#110 RAN3 sent an LS to RAN1 requesting confirmation on whetherRB set configuration as specified in TS 38.473 was clear. RAN1 was unable to reach consensus on whether the provided specification was clear, since the interpretation of the specification is unambiguous; however, as noted by RAN3 the following agreement specifying minmum RB set size, made in RAN1#105-e has not been captured in in existing Rel-17 specification. Agreement The minimum resource size for configuring the frequency domain granularity is a set of N RBs: - Candidate values for N: {4, 8, 16, other values TBD} - N is at least the # PRBs that are corresponding to the MT’s # PRBs of an RBG). - FFS: Scaling or configuration of N based on system BW or size of IAB-MT BWP Since this agreement was made in RAN1 it should be captured in the relevant RAN1 specs; however, there may be some further discussion necessary on the final interpretation which is the relevant RBG |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update TS 38.213 to reflect that the minimum RB set size configured is at least the number of PRBs corresponding to the to RBG size configuration 1 of the smallest BWP configured for the IAB node. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Previously made RAN1 agreement will be reverted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | x.x.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the CR |  |  |  |  |  |  |  |  |

14 Integrated access-backhaul operation

< Unchanged parts are omitted >

With reference to slots of an IAB-DU cell, the IAB-DU can be provided an indication of hard, soft or unavailable type per RB set for symbols configured as downlink, uplink or flexible in a slot by Frequency-Domain HSNA Configuration List [16, TS 38.473]. The RB set size and the number of RB sets are configured by RB-Set-Configuration [16, TS 38.473], with the minimum configurable RB set size not less than the number of PRBs corresponding to the to RBG size ‘Configuration 1’ of the smallest BWP configured for the IAB node. If an indication of hard, soft or unavailable type is not provided for an RB set of a symbol in a slot, the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB set of the symbol in the slot. If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by Frequency-Domain HSNA Configuration List [16, TS 38.473] when the IAB-node uses simultaneous transmission and reception in the slot.

< Unchanged parts are omitted >